3GPP TSG-RAN WG2 Meeting #122	R2-2306363

Incheon, South Korea, 22– 26 May 2023

Agenda item:	7.11.2.1

Source:	Nokia, Nokia Shanghai Bell

Title:	Control plane aspects of multicast reception in RRC_INACTIVE state

WID/SID:	NR_MBS_enh-Core - Release 18

Document for:	Discussion and Decision

# 1	Introduction

The Rel-18 work item on Enhancements of NR Multicast and Broadcast services has been agreed in [1] with the objectives including:

- Specify support of multicast reception by UEs in RRC_INACTIVE state [RAN2, RAN3]

  - PTM configuration for UEs receiving multicast in RRC_INACTIVE state [RAN2]

  - Study the impact of mobility and state transition for UEs receiving multicast in RRC_INACTIVE.  (Seamless/lossless mobility is not required) [RAN2, RAN3]

Rel-17 work specified multicast reception for UEs in RRC_CONNECTED state, whereas Rel-18 WID targets especially cells with a large number of UEs. In addition to the varied limitations on the network side for keeping all UEs receiving a multicast service in RRC_CONNECTED state, receiving multicast transmission in RRC_INACTIVE state by the UEs is assumed to be more power efficient [1].

In this paper, we provide an analysis of the configuration principles to operate the feature of multicast reception in RRC_INACTIVE state, in addition to the principles for mobility aspects. Furthermore, we provide an analysis of the RRC state transitions, and the details on notifications to be provided to the UEs. Previous agreements in RAN2#119-e, RAN2#119bis-e, RAN2#120, RAN2#121 and RAN2#121bis-e meetings [2][3][4][5][6] are considered and open issues are targeted.

# 2	Principles for MBS reception in RRC_INACTIVE state

## 2.1	Delivery Mode for Multicast Reception in RRC_INACTIVE state

Configuration of the Delivery Mode for Multicast Reception in RRC_INACTIVE State

The following related agreements were made in RAN2#121 regarding the information delivered to the UE to receive multicast in RRC_INACTIVE state, and the means to provide such information to the UE [5]:

- When network configures UE to receive multicast in INACTIVE state, RRCRelease message with suspendconfig can be used to deliver the PTM configuration. Other dedicated RRC messages will not be used to provide PTM configuration for MBS multicast for INACTIVE.

- If network finds it useful, the PTM configuration for the (single) serving cell can be configured to UE before the session activation, and UE stores the configuration. When session is activated, UE can receive multicast in INACTIVE state by applying the configuration without going back to RRC_CONNECTED, if not updated by MCCH after being configured.

- Optionally, Multicast MCCH configuration for the serving cell can also be provided in dedicated signalling. Understanding is we are not optimizing mobility case because of this.

- Serving cell will not provide the PTM configuration of neighbour cells from other gNBs.

- FFS whether the network can provide PTM configuration for intra-gNB cells.

- UE shall join in the multicast session before receiving multicast in RRC INACTIVE.

- We introduce a new MCCH logical channel for multicast in INACTIVE (different from broadcast MCCH)

Furthermore, the following agreement was made in RAN2#121bis-e meeting:

Similar to Rel-17 broadcast reception procedure, UE acquires new SIB and multicast MCCH to get PTM configuration after cell reselection.

In our understanding, PTM configuration in the above agreements refer to the content of MCCH, i.e., information needed by the UE to receive multicast in RRC_INACTIVE state excluding the information about how to receive MCCH. The latter is referred as Multicast MCCH configuration. With this understanding, above agreements lead to:

- RRC release with suspendConfig can be used to deliver both Multicast MCCH configuration and PTM configuration to the UE, while sending the UE to RRC_INACTIVE state, for the serving cell.

- When such operation is performed as in first bullet above, PTM configuration of the inter-gNB cells will not be provided to the UE.

- The UE in RRC_INACTIVE state starts monitoring the MCCH transmitted in the cell for possible notifications or updates.

- When the UE reselects to a new cell, it checks the SIB/MCCH of the cell to receive the multicast MCCH configuration and PTM configuration to receive multicast, if available.

The following are still open:

- Whether in RRC release gNB can provide PTM configuration and/or Multicast MCCH configuration of neighbor intra-gNB cells.

- Whether SIB/MCCH of one cell can provide Multicast MCCH configuration and/or PTM configuration of the neighbor cells.

- What is the condition at the UE to start monitoring MCCH?

Regarding (1), a UE in RRC_INACTIVE state receiving a multicast service would need to read SIB/MCCH to receive the configuration of the new camped cell to continue receiving the multicast service. In such cases, interruptions may occur due to the delay for receiving the configuration of the new camped cell. One possibility to reduce such interruptions is as the following:

A UE can be provided with the full MCCH information (i.e., Multicast MCCH configuration + PTM configuration) of the cells of the same gNB (intra-gNB cells) via the RRC release message with suspendConfig. For that, the size of the RRC release message should be taken into account, i.e., such intra-gNB cells’ configuration should be provided if the size of it stays in reasonable limits. To reduce the RRC release message size, a part of Multicast MCCH configuration rather than full Multicast MCCH configuration could be provided to the UEs, e.g., only the CFR information can be provided and not the MCCH scheduling details. With CFR information of MCCH and MTCH (Which are same) and PTM configuration, the UE can immediately start decoding data when going from RRC_CONNECTED to RRC_INACTIVE; or when reselecting a new intra-gNB cell. If the UE does not have a valid SIB/MCCH (e.g., MCCH content changed), it can obtain the newer version of the SIB/MCCH that is periodically broadcasted in the cell, i.e., the UE can immediately start looking for multicast data, and in parallel, decode SIB/MCCH to test validity of its own configuration. For future changes, the UE would monitor MCCH, as well.

Proposal 1a: In Rel-18, for multicast reception by UEs in RRC_INACTIVE state:

- A UE may be provided regarding MCCH information:

  - with the full MCCH information (i.e., Multicast MCCH configuration + PTM configuration) of the cells of the same gNB (intra-gNB cells) via the RRC release message with suspendConfig; or

  - only CFR of Multicast MCCH configuration and PTM configuration (MCCH scheduling details can be avoided)

Proposal 1b: The UE in RRC_INACTIVE state can immediately start receiving the multicast data, e.g., when going from RRC_CONNECTED to RRC_INACTIVE state, after initial cell selection to a cell or after reselecting a cell for which UE has been provided configuration

Proposal 1c: The UE still monitors SIB/MCCH for possible changes in multicast configuration and when UE does not have configuration via suspendConfig.

Regarding (2), as for (1), neighbor cell’s Multicast MCCH configuration and/or PTM configuration would be beneficial to reduce interruptions during cell reselection in RRC_INACTIVE state. Also, excessive Xn signalling should be avoided. For that reason, we believe that SIB/MCCH can be used to indicate Multicast MCCH configuration and PTM configuration of the intra-gNB neighbor cells, as long as SIB/MCCH size stays in reasonable limits.

Proposal 2: SIB/MCCH can be used to indicate Multicast MCCH configuration and PTM configuration of the intra-gNB neighbor cells, as long as SIB/MCCH size stays in reasonable limits. FFS exact information provided in SIB/MCCH messages.

Regarding (3), some companies have proposed to include a flag in RRC release to indicate the UE that the service will be provided in RRC_INACTIVE state, for the UE to start monitoring MCCH. In our view, this approach is not feasible, as transmitting data to the UEs in RRC_INACTIVE state is per cell decision. For example, if such flag is not given to the UE for the serving cell while sending the UE to RRC_INACTIVE, would this UE not monitor MCCH for a particular multicast service also in a neighbor cell? Note that information regarding inter-gNB cells will not be provided in RRC release message. For that reason, we propose that a UE in RRC_INACTIVE state that joined successfully via NAS signalling to an MBS session(s) shall monitor SIB/MCCH for that respective TMGI(s).

Proposal 3: A UE in RRC_INACTIVE state that joined successfully via NAS signalling to an MBS session shall monitor SIB/MCCH for that particular TMGI, irrespective of what UE receives in RRC release with suspendConfig.

Details of MCCH Used for Multicast Reception in RRC_INACTIVE State

New MCCH logical channel was agreed to be used for multicast reception in RRC_INACTIVE state, along with a new SIB. This avoids the usage of the same MCCH-RNTI for broadcast to be used also for multicast. Thus, a new MCCH-RNTI for multicast is needed.

Proposal 4: A new MCCH-RNTI is specified to receive the new MCCH for multicast.

# 3	Principles of Mobility for Multicast Reception in RRC_INACTIVE State

Possible Mobility Scenarios for Rel-18 UEs Receiving Multicast Services

The following table summarizes all mobility and state transition cases and necessary actions for a UE receiving a multicast transmission in one delivery mode from the source cell and in same or different delivery mode from the target cell:

| Target Cell Source Cell | RRC_CONNECTED (Rel-17 multicast delivery mechanism) | RRC_INACTIVE |
| --- | --- | --- |
| RRC_CONNECTED (Rel-17 multicast delivery mechanism) | Scenario 1 Example use case: Source and target cells are providing multicast to the UE in RRC_CONNECTED state, i.e., Rel-17 multicast reception. Action: Rel-17 handover of MRBs. | Scenario 2 Example use case: Source cell is providing multicast to the UE in RRC_CONNECTED state, target cell is providing multicast to the UE in RRC_INACTIVE state and no other unicast service exists for this UE to be handed over. Action: A handover command sending the UE to RRC_INACTIVE, releasing source MRB(s) and indicating the target SIB/MCCH/MTCH configuration. |
| RRC_INACTIVE | Scenario 3 Example use case: Source cell provides multicast to the UE in RRC_INACTIVE state and target cell provides in RRC_CONNECTED state. Action: UE reselects the target cell, checks SIB/MCCH and cannot find the service provided to UEs in RRC_INACTIVE. UE shall reconnect unless session is inactive. Warning: means shall be provided to enable RRC_INACTIVE UE to differentiate the cases where: - Target cell does not currently provide service to UEs in RRC_INACTIVE state, but rather only UEs in RRC_CONNECTED state - Where the session is deactivated. For that, we provide more details in Section 4. | Scenario 4 Example use case: Source and target cells are providing multicast to the UE in RRC_INACTIVE state. Action: UE reselects the target cell, UE reads new broadcasted SIB/MCCH/MTCH configuration and continues reception. |

Scenario 1 is already supported.

For scenario 2, we propose that RAN2/3 further studies the mechanisms and the details to provide the necessary enhancements for the needed mobility and state transitions. A target node of the handover should be able to directly send the UE to RRC_INACTIVE state, rather than first accepting UE in RRC_CONNECTED state, i.e., conventional handover of the UE, and then sending another RRC release with suspendConfig to send the UE to RRC_INACTIVE state, as such process would require redundant signalling.

Rather than that, a target node should be able to directly send the UE to RRC_INACTIVE state via handover command.

For scenario 3 to properly function, a notification in SIB/MCCH is needed, and this is provided in Section 4. Scenario 4 is rather simple, however, means shall be provided to guide the UE in RRC_INACTIVE state to the best possible neighbor frequency, which is discussed in the next section.

Proposal 5: RAN2/3 further studies the mechanisms and the details to provide necessary enhancements for the needed mobility and state transitions, where the UE is in RRC_CONNECTED state receiving a multicast service in a source cell and to be handed over to a target cell that can serve the UE in RRC_INACTIVE state.

Cell Reselection Optimization for UEs in RRC_INACTIVE State Receiving Multicast Services

Based on previous RAN2 agreement, a Rel-18 UE in RRC_INACTIVE state should be able to move without connecting among the cells in the RNA.

Frequency prioritization may be provided to the UE for cell reselection for multicast reception in RRC_INACTIVE, detailed mechanism on how to identify the frequency info (e.g., SAI, USD, or frequency info directly provided by network) is FFS.

No need to define a mechanism other than the frequency prioritization, i.e., per cell based prioritization in cell re-selection, to help UE choose the right cell to camp on.

With the above agreements, RAN2 agreed define a mechanism to guide the UE to a frequency/cell that provides the service for service continuity purposes, similar to Rel-17 broadcast frequency prioritization feature. Furthermore, RAN2 agreed not to use a cell-based prioritization, e.g., cell reselection should not be guided by neighbor cell list.

In our view, FSAI-based mechanism from broadcast cell reselection is to be reused, as the mechanism is well-known by the companies and significant amount of effort was spent in Rel-17 to define such complete behavior:

  - RAN nodes are configured with FSAI to frequency mapping, and this information is exchanged among neighbor gNBs.

  - UEs are informed about FSAI to MBS service mapping via service announcement (UEs can also be directly given frequencies rather than FSAIs mapping to a multicast service).

  - RAN node signals SIBx (possibly similar to broadcast SIB21) including FSAI to neighbor frequencies (intra-frequency neighbors as well).

  - UE prioritizes neighbor frequencies during cell reselection in RRC_INACTIVE state.

For that reason, coordination with SA2 for the provision of FSAIs (as AF/MB-SMF configuration/involvement would be needed) is required.

Proposal 6: For multicast service continuity, the UE in RRC_INACTIVE state uses FSAI-based frequency prioritization mechanism of Rel-17 broadcast.

Proposal 7: RAN2 sends an LS to SA2 regarding the feasibility of provisioning of FSAIs also for multicast services.

Neighbor Cell List for UEs in RRC_INACTIVE State Receiving Multicast Services

RAN2 made the following agreement in the last meeting:

The neighbor cell list mechanism for multicast reception in RRC_INACTIVE may be configured e.g. it can be used by UE to resume RRC connection if service is not available in the re-selected cell by NCL, without reading MCCH in the re-selected cell, in some aspects similar to Rel-17 NCL mechanism in MBS broadcast.

However, the details of the content and the usage of such list is missing. For example, should the gNB provide a neighbor cell list to provide the neighbour cells where a specific multicast service is available for the UEs in RRC_INACTIVE state? Or should the gNB provide the neighbour cells where a specific multicast service is available for the UEs in any RRC state?

From the agreement, it is understood that the intention of the NCL is to enable UE behavior, so that the UE does not read MCCH in the reselected neighbor cell in case the service is not provided to the UEs in RRC_INACTIVE state. For that reason, we propose that the neighbor cell list indicates the services that are provided to the UEs in RRC_INACTIVE state.

Proposal 8: Neighbor cell list indicates the services that are provided to the UEs in RRC_INACTIVE state in the neighbor cells.

Furthermore, how would gNB get to know of neighbor gNBs’ delivery, i.e., whether multicast setup is established with the core network and whether the service is provided to the UEs in RRC_INACTIVE? Currently, there are no means for such information exchange. Therefore, RAN3 should specify the procedure regarding neighbor gNBs’ information exchange on multicast delivery to UEs in RRC_INACTIVE state.

Proposal 9: An LS is sent to RAN3 to define the signalling for information exchange on multicast delivery to UEs in RRC_INACTIVE state between neighbor gNBs.

RRC_INACTIVE Cell Reselection Multicast Service Continuity

Rel. 17 multicast operation supports synchronization of PDCP COUNT/Sequence number (SN) among gNBs, so that when the UE moves from one cell to another in the RRC_CONNECTED state, i.e., handover, then the UE can continue receiving the same multicast service.  However, this synchronization of the PDCP COUNT/SN is not mandatory, resulting that UEs are unaware of whether the PDCP COUNT/SN is synchronized among the gNBs.  This is not a problem in Rel. 17 multicast reception as the multicast in Rel. 17 is supported only for RRC_CONNECTED UEs, and network takes care of mobility. This is however a problem for  Rel. 18 RRC_INACTIVE UEs, because these RRC_INACTIVE UEs should manage themselves the minimization of packet loss during the mobility, by continuing the ongoing PDCP COUNT/SN, when applicable, i.e., when the network achieves PDCP COUNT/SN synchronization among the gNBs.

Observation 1: Rel. 18 RRC_INACTIVE UEs are unaware whether PDCP COUNT/SN synchronization is applied in the network among the gNBs. Therefore, UE in RRC_INACTIVE state would not know whether it can assume PDCP COUNT/SN continuation when selecting a new cell for camping.

Proposal 10: gNB to include information indicating that PDCP COUNT/SN is synchronized among some gNBs. It can include the list of neighbor gNBs/cells where the PDCP is synchronized.  This informing can take place via an indication to the UEs that is broadcast over MCCH channel or sent to individual UEs within an RRC release message with suspendConfig.

If the PDCP COUNT/SN is synchronized among gNBs, the UE could ideally continue receiving the same multicast session in a newly selected cell in the RRC_INACTIVE state without re-establishing the PDCP entity in the new cell. However, the UE is unaware of which MBS radio bearer (MRB) in the source cell maps to which MRB in the target cell.

Observation 2: The UE in RRC_INACTIVE state is unaware which MRB in the source cell maps to which MRB in the target cell. Although PDCP COUNT/SN synchronization is applied across gNBs, the UE cannot achieve service continuity without such knowledge.

Proposal 11: To provide the service continuity when PDCP COUNT/SN synchronization is applied across cells, mapping of Logical Channel IDs (LCIDs) in the source and neighbor cells can be indicated to the UE. As LCID has 1-to-1 mapping with MRBs, the UE can map the source cell’s MRBs to the target cell’s MRBs to achieve service continuity.

Proposal 12: RAN2 to discuss whether such information should be exchanged over the Xn interface among gNBs. Information exchange can include the list of QFIs (QoS flow identifiers) which maps to LCID as an identifier for this LCID.

Proposal 13: If the PDCP COUNT/SN synchronization is not applied among the gNBs, the RRC_INACTIVE UEs can follow Rel. 17 MBS broadcast principle for achieving the service continuity.

# 4	Management of Multicast Delivery to UEs in RRC_INACTIVE State

With the understanding explained in previous sections on how the multicast delivery to the UEs in RRC_INACTIVE state is configured, several elements are needed to be considered to manage the delivery of the multicast sessions to the UEs in RRC_INACTIVE state, which will be discussed in the upcoming sections.

## 4.1 Indication of Multicast Session Activation/Deactivation to UEs in RRC_INACTIVE State

Enhanced Group Paging and Special UEs

The following agreements were made in RAN2#119bis-e meeting on multicast session activation/deactivation:

- Rel-18 UE in INACTIVE can be informed when the session is activated (Details FFS).

- As a baseline, group paging can be used to inform Rel-18 UE(s) about the session activation (Details FFS, e.g., UE behavior when receiving such group notification).

Furthermore, the following was agreed last time:

Rel-18 UE can stay in RRC_INACTIVE and start monitoring corresponding G-RNTI upon an enhanced group paging (e.g., upon session activation or data transmission resumed). Details FFS.

Therefore, RAN2 agreed to enhance group paging, such that the Rel-18 UEs in RRC_INACTIVE state do not go to RRC_CONNECTED state after group paging, in case the gNB decides to use the delivery mode that enables the reception of a multicast MBS session in a cell by the UEs in RRC_INACTIVE state.

Note that for Rel-17 UEs, group paging would anyway be there when the session is activated.

Observation 3: Group paging would be there when the session is activated for Rel-17 UEs that can be in RRC_INACTIVE/IDLE state, and such group paging messages can also be obtained by Rel-18 UEs in RRC_INACTIVE state.

In our view, the group paging message can easily be enhanced with an indication that the Rel-18 UEs can stay in RRC_INACTIVE state and start receiving the multicast transmission. For a Rel-18 UE, this would be an indication to start reading MCCH or even the data if the UE has already the PTM configuration (via RRC release or in MCCH). If this indication is not present in the group paging message, group paging would guide Rel-17 and Rel-18 UEs to RRC_CONNECTED state, which then would be served by the Rel-17 delivery method of multicast.

Without enhancing the group paging message, a Rel-18 UE will get confused when seeing a group paging message for Rel-17 UEs: should the Rel-18 UE move to RRC_CONNECTED or should it stay in RRC_INACTIVE?

Furthermore, such indication is compatible with RRC_INACTIVE reception being a per cell decision, as such indication can be provided within the paging message by the gNB depending on many dynamic parameters, e.g., the audience size, congestion at the gNB.

Observation 4: If enhancements to the group paging message are not made, a Rel-18 UE that receives the group paging, which is aimed for getting Rel-17 UEs to RRC_CONNECTED for multicast delivery, would always need to transition to RRC_CONNECTED state - even if the gNB desires the UE to stay in RRC_INACTIVE for multicast delivery.

Proposal 14: Group paging message is enhanced with a flag to enable RRC_INACTIVE UEs to stay in RRC_INACTIVE state, in case the gNB is to provide the multicast session with the delivery mode that enables the reception of a multicast session in a cell by the UEs in RRC_INACTIVE state.

In addition, the following agreement was made in the last meeting:

A "special UE" identified by MBS assistance information from 5GC can be released to RRC_INACTIVE (e.g., when the session is deactivated). FFS how can network enable such UE to resume to RRC_CONNECTED upon session activation

As agreed by SA2 WG, there may be some UEs, e.g., frequent talkers of a public safety session or platoon leaders, that are preferably kept in RRC_CONNECTED state when receiving a multicast session. A gNB can be informed about such UEs by the core network via different means. In that case, even though some other Rel-18 UEs are to be staying in RRC_INACTIVE state when the session is activated, such UEs would still be desired to go to RRC_CONNECTED state. When such UE receives a group paging message, i.e., either Rel-17 group paging or enhanced group paging as proposed above, it should go into RRC_CONNECTED state. For the Rel-17 group paging, anyway the UE would reconnect via current means. However, for the enhanced group paging using the extra paging indication to keep other UEs in RRC_INACTIVE, such UEs are to be informed to still go to RRC_CONNECTED state. Otherwise, extra unicast paging would be required towards such UEs. For such informing, while releasing UE to RRC_INACTIVE, the UE can be configured with the information that it should always reconnect in case of a specific multicast session activation.

Observation 5:  gNB based on information from core and from some other internal information may desire to get only some of the Rel-18 UEs, e.g., special UEs, to RRC_CONNECTED from RRC_INACTIVE, when the session gets activated.

Proposal 15: Special UEs, e.g., the ones preferred by 5GC to be served in RRC_CONNECTED for a multicast session, can be configured when UEs are released to RRC_INACTIVE state, e.g., to always come to RRC_CONNECTED state in case of session activation.

MCCH Enhancements for Session Activation Status

Moreover, additional mechanisms are needed in addition to the group paging for indication of activation/deactivation of a multicast session to UEs in RRC_INACTIVE state. For example, a UE in RRC_INACTIVE state may be out of the service area of the multicast session initially, but then may go into a cell within the service area. This UE may have missed the group paging performed by the gNB (as above).

Observation 6: A UE in RRC_INACTIVE state may be out of the service area of the multicast session initially, but then may go into a cell within the service area. This UE may have missed the group paging performed by the gNB for session activation.

Furthermore, the following was agreed in the last RAN2 meeting:

When a UE reselects to a cell for which PTM configuration is not available in multicast MCCH, the UE initiates RRC resumption procedure for an active multicast session it is interested to receive or continue receiving.

When camping in a new cell, a UE can check broadcasted SIBx/MCCH to see whether the configuration of RRC_INACTIVE multicast session reception is provided. There are two possibilities:

- SIBx/MCCH contains the configuration of RRC_INACTIVE multicast session reception

In this case, the UE would understand the session is active and it can receive the service in RRC_INACTIVE state.

- SIB/MCCH does not contain the configuration of RRC_INACTIVE multicast session reception

Either the service is inactive, or the service is provided only to the UEs in RRC_CONNECTED state, e.g., audience size is small, therefore, gNB decides on Rel-17 multicast delivery method. In the former case, the UE does not need to reconnect, whereas in the latter, the UE should reconnect to receive the multicast session.

To overcome above mentioned ambiguity in (2), an indication can be provided in SIBx/MCCH, indicating whether a service is active/inactive. The indication should rather be in SIBx rather than MCCH in order to reduce the time UE needs to receive the information (faster reading by the UE, as MCCH would require reading SIBx first).

Last for deactivation, RAN2 made the following agreements:

For one UE already in RRC_INACTIVE, it can stay in RRC_INACTIVE and stop monitoring corresponding G-RNTI upon events like session deactivation/temporary no data.

FFS which option to take: enhanced group paging or enhanced MCCH, to enable Rel-18 UE to stay in RRC_INACTIVE and stop monitoring corresponding G-RNTI upon events like session deactivation/temporary no data.

A session status indication in SIBx/MCCH would also solve the problem of missed group paging due to UE being out of the service area, the ambiguity mentioned above after cell reselection and the problem of indication of session deactivation to the UEs.

Furthermore, if enhanced group paging is used for deactivation, a Rel-17 UE will also be able to receive such paging, as the gNB may have released the Rel-17 UE to RRC_INACTIVE once the session gets deactivated or when there is lack of data coming from the core for a while.

Observation 7: If enhanced group paging is used for deactivation, a Rel-17 UE will also be able to receive such paging, as the gNB may have released the Rel-17 UE to RRC_INACTIVE once the session gets deactivated or when there is lack of data coming from the core for a while.

Observation 8: A session activation status in SIBx or MCCH  solves the problem of missed group paging due to UE being out of the service area, the ambiguity mentioned above after cell reselection and the problem of indication of session deactivation to the UEs.

Proposal 16: An indication is provided in SIBx (or MCCH), indicating whether a service is active or deactive.

## 4.2 Enabling/disabling of Multicast Data Delivery to UEs in RRC_INACTIVE State

- The following agreement was made previously by RAN2 on enabling/disabling multicast data delivery to UEs in RRC_INACTIVE state [3]:

- It is up to gNB to decide whether a multicast session may be received by UE(s) in INACTIVE. FFS what information gNB may be provided to form such decision (related to SA2 discussion).

- Rel-17 multicast delivery mode provides significant benefits to the UE compared to reception of multicast in RRC_INACTIVE state, as it offers HARQ feedback that can feed MCS selection and enable retransmissions. Moreover, if the reception of a multicast session by the UEs in RRC_INACTIVE is enabled, the gNB should transmit the data in all beams similarly to any broadcast channels, as the gNB would not be aware under which beam the UE is located, which decreases spectral efficiency compared to Rel-17 mechanism where the gNB is aware of the location of the UE within beam accuracy (and does not transmit data multiple times in different beams).

- Observation 9: Rel-17 multicast delivery mode provides significant benefits to the UE compared to reception of multicast in RRC_INACTIVE state, as it offers HARQ feedback that can feed MCS selection and enable retransmissions.

- Observation 10: If the reception of a multicast session by the UEs in RRC_INACTIVE is enabled, the gNB should transmit the data in all beams, as the gNB would not be aware under which beam the UE is located, which may decrease spectral efficiency compared to Rel-17 mechanism where the gNB is aware of the location of the UE (and not transmit data multiple times in different beams).

- At least due to above observations, based on the RAN2 agreement, a gNB is able to decide when to disable the reception of the multicast data by the RRC_INACTIVE UEs, and switch them to RRC_CONNECTED state, if the number of UEs in the cell receiving multicast transmission is relatively low and their return to RRC_CONNECTED state would not affect scalability.

- In our view, for deciding on whether to provide a multicast session to UEs in RRC_INACTIVE state, a gNB that uses delivery of multicast session to the UEs in RRC_INACTIVE state should estimate periodically the number of UEs in RRC_INACTIVE state in the cell that are receiving a multicast session, if the multicast session is active. Therefore, some sort of mechanism is required to evaluate the number of UEs receiving a multicast session in RRC_INACTIVE state in a cell.

- Observation 11: A gNB that uses a delivery mode that enables the reception of a multicast MBS session in a cell by the UEs in RRC_INACTIVE state needs to estimate periodically the number of UEs in the cell that are receiving a multicast session in RRC_INACTIVE state in order to be able to adapt to the changing audience size.

Proposal 17: A procedure should be introduced to count the number of UEs in RRC_INACTIVE with active multicast session.

For the gNB to be able to count the number of UEs receiving a particular MBS session in RRC_INACTIVE state, either the UEs need to reconnect to the network each time for counting purposes, or an efficient mechanism shall be developed to enable resource/power efficient periodic counting. In our view, the latter is needed, and that mechanism would require UEs to transmit information in RRC_INACTIVE state (e.g., using RACH resources). Therefore, RAN2 should send an LS to RAN1 to develop the needed functionality and corresponding requirements that RAN2 sees needed for such a mechanism.

In practice the gNB would need to have sufficient information to decide multicast delivery mode, i.e., basically the gNB would need to know if there is significant population of UEs requiring multicast delivery in RRC_INACTIVE mode, but there is no need to know exact number of UEs.

Proposal 18: RAN2 sends an LS to RAN1 to design a counting mechanism for the UEs in RRC_INACTIVE state receiving a particular multicast session.

## 4.3 QoS for UEs Receiving a Multicast Session in RRC_INACTIVE State

It is to be noted that, due to lack of feedback in the RRC_INACTIVE state (e.g., no HARQ feedback), the gNB cannot monitor/estimate the QoS of the multicast session for UEs in RRC_INACTIVE state. RAN2 should consider how can UE/Network recover from the scenario that UE in RRC_INACTIVE state does not receive multicast session with sufficient QoS.

Therefore, RAN2 agreed the following in the last meeting:

UE may trigger RRC connection resumption if the reception quality of the multicast data is below a configured threshold, FFS how to specify the threshold/reception quality.

In our view, the decision to configure a threshold and reception quality should be left to network implementation. However, one thing to be considered is that such threshold should not lead the UE to always to move to RRC_CONNECTED without making reselection to a new cell. Thus there should be some control in the network that in some scenario if QoS of the service gets bad UE needs resume the connection e.g. some services may require very high QoS. But in most scenarios the services are such that QoS tolerates easily any interruptions caused by reselections and then one should not require UE to resume the connection.

At the main motivation to have a such a threshold defined is to ensure appropriate QoS service is achieved best way is to utilize e.g. BLER as a method to trigger for connection resumption. Using radio based method could be alternative but it would not work if the network changes coding of the traffic changes.Therefore, metrics for such threshold should be QoS measurements, rather han radio measurements.

Observation 12: QoS threshold for connection resumption should not lead the RRC_INACTIVE UE to reconnect before each cell reselection, as otherwise, RAN2 agreement on UE mobility without reconnections for reception of multicast service in RRC_INACTIVE state would be violated.

Proposal 19: The configured threshold(s) for controlling UE trigger for RRC connection resumption is QoS threshold(s), rather than radio measurements, whose value is left to network implementation. FFS details on configuration.

## 4.4 Beam Management for Multicast Sessions

- For Rel-17 MBS multicast, since all the UEs receiving the multicast are in RRC_CONNECTED state, the network can transmit the PTM transmission only to those beams under which there are multicast UEs. When some UEs are released to RRC_INACTIVE, the network does not anymore know under which beams there are multicast UEs and therefore, the MBS multicast has to be transmitted to all the beams of the cell. RAN2 therefore agreed that the multicast transmission in RRC_INACTIVE is performed via beam sweeping based on SSB index like broadcast MBS (i.e., beam information is not needed in DCI.)

When the multicast PTM transmission is performed via beam sweeping, it is important that the UEs receiving the transmission know the mapping between a given beam and certain PDCCH monitoring occasions. This is especially beneficial from UE power consumption point of view as the UE need not monitor all the PDCCH monitoring occasions to receive the MBS multicast transmission. Therefore, it would be beneficial to indicate to both RRC_INACTIVE and RRC_CONNECTED UEs when beam sweeping is applied as well as the mapping between the beams and the PDCCH monitoring occasions.

- Proposal 20: For power saving purposes, indicate to both RRC_INACTIVE and RRC_CONNECTED MBS multicast UEs receiving the PTM transmission when beam sweeping  is applied as well as the mapping pattern between the beams and the PDCCH monitoring occasions.

# 5	Conclusion

This document has made the following observations:

Observation 1: Rel. 18 RRC_INACTIVE UEs are unaware whether PDCP COUNT/SN synchronization is applied in the network among the gNBs. Therefore, UE in RRC_INACTIVE state would not know whether it can assume PDCP COUNT/SN continuation when selecting a new cell for camping.

Observation 2: The UE in RRC_INACTIVE state is unaware which MRB in the source cell maps to which MRB in the target cell. Although PDCP COUNT/SN synchronization is applied across gNBs, the UE cannot achieve service continuity without such knowledge.

Observation 3: Group paging would be there when the session is activated for Rel-17 UEs that can be in RRC_INACTIVE/IDLE state, and such group paging messages can also be obtained by Rel-18 UEs in RRC_INACTIVE state.

Observation 4: If enhancements to the group paging message are not made, a Rel-18 UE that receives the group paging, which is aimed for getting Rel-17 UEs to RRC_CONNECTED for multicast delivery, would always need to transition to RRC_CONNECTED state - even if the gNB desires the UE to stay in RRC_INACTIVE for multicast delivery.

Observation 5:  gNB based on information from core and from some other internal information may desire to get only some of the Rel-18 UEs, e.g., special UEs, to RRC_CONNECTED from RRC_INACTIVE, when the session gets activated.

Observation 6: A UE in RRC_INACTIVE state may be out of the service area of the multicast session initially, but then may go into a cell within the service area. This UE may have missed the group paging performed by the gNB for session activation.

Observation 7: If enhanced group paging is used for deactivation, a Rel-17 UE will also be able to receive such paging, as the gNB may have released the Rel-17 UE to RRC_INACTIVE once the session gets deactivated or when there is lack of data coming from the core for a while.

Observation 8: A session activation status in SIBx or MCCH  solves the problem of missed group paging due to UE being out of the service area, the ambiguity mentioned above after cell reselection and the problem of indication of session deactivation to the UEs.

- Observation 9: Rel-17 multicast delivery mode provides significant benefits to the UE compared to reception of multicast in RRC_INACTIVE state, as it offers HARQ feedback that can feed MCS selection and enable retransmissions.

- Observation 10: If the reception of a multicast session by the UEs in RRC_INACTIVE is enabled, the gNB should transmit the data in all beams, as the gNB would not be aware under which beam the UE is located, which may decrease spectral efficiency compared to Rel-17 mechanism where the gNB is aware of the location of the UE (and not transmit data multiple times in different beams).

- Observation 11: A gNB that uses a delivery mode that enables the reception of a multicast MBS session in a cell by the UEs in RRC_INACTIVE state needs to estimate periodically the number of UEs in the cell that are receiving a multicast session in RRC_INACTIVE state in order to be able to adapt to the changing audience size.

Observation 12: QoS threshold for connection resumption should not lead the RRC_INACTIVE UE to reconnect before each cell reselection, as otherwise, RAN2 agreement on UE mobility without reconnections for reception of multicast service in RRC_INACTIVE state would be violated.

And proposed the following:

Proposal 1a: In Rel-18, for multicast reception by UEs in RRC_INACTIVE state:

Proposal 1b: The UE in RRC_INACTIVE state can immediately start receiving the multicast data, e.g., when going from RRC_CONNECTED to RRC_INACTIVE state, after initial cell selection to a cell or after reselecting a cell for which UE has been provided configuration

Proposal 1c: The UE still monitors SIB/MCCH for possible changes in multicast configuration and when UE does not have configuration via suspendConfig.

Proposal 2: SIB/MCCH can be used to indicate Multicast MCCH configuration and PTM configuration of the intra-gNB neighbor cells, as long as SIB/MCCH size stays in reasonable limits. FFS exact information provided in SIB/MCCH messages.

Proposal 3: A UE in RRC_INACTIVE state that joined successfully via NAS signalling to an MBS session shall monitor SIB/MCCH for that particular TMGI, irrespective of what UE receives in RRC release with suspendConfig.

Proposal 4: A new MCCH-RNTI is specified to receive the new MCCH for multicast.

Proposal 5: RAN2/3 further studies the mechanisms and the details to provide necessary enhancements for the needed mobility and state transitions, where the UE is in RRC_CONNECTED state receiving a multicast service in a source cell and to be handed over to a target cell that can serve the UE in RRC_INACTIVE state.

Proposal 6: For multicast service continuity, the UE in RRC_INACTIVE state uses FSAI-based frequency prioritization mechanism of Rel-17 broadcast.

Proposal 7: RAN2 sends an LS to SA2 regarding the feasibility of provisioning of FSAIs also for multicast services.

Proposal 8: Neighbor cell list indicates the services that are provided to the UEs in RRC_INACTIVE state in the neighbor cells.

Proposal 9: An LS is sent to RAN3 to define the signalling for information exchange on multicast delivery to UEs in RRC_INACTIVE state between neighbor gNBs.

Proposal 10: gNB to include information indicating that PDCP COUNT/SN is synchronized among some gNBs. It can include the list of neighbor gNBs/cells where the PDCP is synchronized.  This informing can take place via an indication to the UEs that is broadcast over MCCH channel or sent to individual UEs within an RRC release message with suspendConfig.

Proposal 11: To provide the service continuity when PDCP COUNT/SN synchronization is applied across cells, mapping of Logical Channel IDs (LCIDs) in the source and neighbor cells can be indicated to the UE. As LCID has 1-to-1 mapping with MRBs, the UE can map the source cell’s MRBs to the target cell’s MRBs to achieve service continuity.

Proposal 12: RAN2 to discuss whether such information should be exchanged over the Xn interface among gNBs. Information exchange can include the list of QFIs (QoS flow identifiers) which maps to LCID as an identifier for this LCID.

Proposal 13: If the PDCP COUNT/SN synchronization is not applied among the gNBs, the RRC_INACTIVE UEs can follow Rel. 17 MBS broadcast principle for achieving the service continuity.

Proposal 14: Group paging message is enhanced with a flag to enable RRC_INACTIVE UEs to stay in RRC_INACTIVE state, in case the gNB is to provide the multicast session with the delivery mode that enables the reception of a multicast session in a cell by the UEs in RRC_INACTIVE state.

Proposal 15: Special UEs, e.g., the ones preferred by 5GC to be served in RRC_CONNECTED for a multicast session, can be configured when UEs are released to RRC_INACTIVE state, e.g., to always come to RRC_CONNECTED state in case of session activation.

Proposal 16: An indication is provided in SIBx (or MCCH), indicating whether a service is active or deactive.

Proposal 17: A procedure should be introduced to count the number of UEs in RRC_INACTIVE with active multicast session.

Proposal 18: RAN2 sends an LS to RAN1 to design a counting mechanism for the UEs in RRC_INACTIVE state receiving a particular multicast session.

Proposal 19: The configured threshold(s) for controlling UE trigger for RRC connection resumption is QoS threshold(s), rather than radio measurements, whose value is left to network implementation. FFS details on configuration.

- Proposal 20: For power saving purposes, indicate to both RRC_INACTIVE and RRC_CONNECTED MBS multicast UEs receiving the PTM transmission when beam sweeping  is applied as well as the mapping pattern between the beams and the PDCCH monitoring occasions.

# References

- RP-221458, “Revised WID: Enhancements of NR Multicast and Broadcast Services”.

- R2-2208707, “Report from MBS breakout session”, RAN2#119-e.

- R2-2210807, “Report from MBS breakout session”, RAN2#119bis-e.

- R2-2213007, “Report from MBS breakout session”, RAN2#120.

- R2-2301907 “Report from MBS breakout session”, RAN2#121.

- R2-2304207 “Report from MBS breakout session”, RAN2#121bis-e