3GPP TSG-RAN WG2 Meeting #122	R2-2306378

Incheon, Korea, 22nd - 26th May, 2023

Agenda Item:	7.9.2

Source: 	MediaTek Inc.

Title:  	Discussion on L2 U2U Relay

Document for:	Discussion, decision

- Introduction

In the last RAN2 meeting, some FFS issues for L2 U2U Relay are as following.

Discovery and (re)selection part

- For relay UE selection, the remote UE uses SL-RSRP measurements towards peer remote UE to trigger relay UE selection when there is data transmission on direct link.

- For relay UE reselection, the remote UE uses SL-RSRP measurements towards the relay UE to trigger relay UE reselection when there is data transmission on the indirect link.

- In both cases, it is left to remote UE implementation whether to use SL-RSRP or SD-RSRP for relay (re)selection trigger evaluation in case of no data transmission.

- FFS if there need to be different configured thresholds for SL-RSRP and SD-RSRP.

- Each Remote UE can trigger Relay reselection based at least on current hop quality.

Layer-2 specific U2U relay

- FFS if multiplexing of different destinations in the same RLC channel is supported.

- An ID mappable to the destination remote UE is needed in the first hop (Tx remote UE to relay), at least in case multiplexing of different destinations in the same RLC channel is supported.

- An ID mappable to the source remote UE is needed in the second hop (relay to Rx remote UE).

- FFS if the IDs are different (e.g., source and destination UE IDs) or common (e.g., a local ID for the pair).

- FFS whether both UE IDs are included in the header or the relay UE does a mapping.

This contribution discusses the FFS parts from last meeting [1] for U2U Relay common part and Layer-2 specific part.

# 2	Discussion

## 2.1	Relay (Re)Selection

In common part of L2 U2U Relay, the FFS point is whether different configured thresholds for SL-RSRP and SD-RSRP is needed or not. Based on latest feedback from RAN1 (R1-2304211) and RAN4 (R4-2306366), they identify that there will be power issue for SL-RSRP and SD-RSRP. Therefore, if

Proposal 1: Support different configured thresholds for SL-RSRP and SD-RSRP.

## 2.2	Layer-2 specific U2U relay

QoS split

Originally, the QoS framework is designed from end-to-end point of view. In U2N relay, gNB is responsible for QoS handling between relay UE and gNB, and between relay UE and remote UE. However, the situation for U2U case, there is no gNB involvement, who to handle QoS become a new problem/topic to be discussed in U2U relay case. Among three devices, 1) source remote UE, 2) U2U relay UE, 3) destination remote UE, it is straightforward that U2U relay to handle the QoS, because only U2U relay UE is aware of both link conditions. Source remote UE is not aware of link between U2U relay UE and destination remote UE, it can not make decision for link between U2U relay UE and destination remote UE. And vice versa, destination remote UE can’t do that for U2U relay UE and source remote UE.

Proposal 2: Relay UE is responsible for QoS split in L2 U2U relay.

UE ID in SRAP Header

In previous meeting, we had agreed that E2E Radio Bearer ID should be included in the adaptation layer in first and second PC5 hop, but leave 5 options on the table to be further discussed.

- Option 1: Target remote UE ID (layer-2 ID) in first hop and source remote UE ID (layer-2 ID) in second hop.

- Option 2: Target remote UE ID (local ID) in first hop and source remote UE ID (local ID) in second hop.

- Option 3: Both source remote UE ID (layer-2 ID) and target remote UE ID (layer-2 ID) included in each hop.

- Option 4: Both source remote UE ID (local ID) and target remote UE ID (local ID) included in each hop.

- Option 5: A common ID for a pair between source UD and target remote UE included in each hop.

In Rel-17, the L2-ID is carried within SRAP layer. Therefore, we can adopt the same mechanism for Rel-18. Some discussion about whether both soure UE ID and destination UE ID should be included or only single UE ID is enough, we acknowledge the issues raised in [2]. Unlike U2N Relay, the local ID is assigned by gNB, there is no ID collision issue. However, in U2U relay, gNB is not necessary presented in the network.

Therefore, ID collision issue might happen. Take above figure as an example, if Relay UE J and K are responsible to assigne local ID for remote UE I and L, respectively. It is possible that Relay UE J and K assign the same local ID number to remote UE I and L. In this case, Relay UE J might be able to distinguish data from I and need to forward to K or the data is from source UE L via Relay UE K to destination J. Therefore, we see some problems when local ID is used.

Proposal 3: Support Option 3 (Both source remote UE ID (layer-2 ID) and target remote UE ID (layer-2 ID)) included in each hop.

SA2 authorization LS (R2-2302442)

In last meeting, we postpone replying SA2 authorization LS again. Two questions in the LS are listed below:

- Question 1: Whether the "5G ProSe authorised" information needs to be enhanced to include the authorization information for UE-to-UE Relay operation?

- Question 2: If the answer to Q1 is yes, which bullet(s) need to be included?

For question 1, we think legacy authorization mechanism can be reused, therefore, no further enhancement is needed.

Proposal 4: Legacy authorization mechanim can be reused.

Proposal 5: If proposal 4 is agreed, send reply LS back to SA2 and Cc RAN3.

# 3	Conclusion

We have the following proposals:

Proposal 1: Support different configured thresholds for SL-RSRP and SD-RSRP.

Proposal 2: Relay UE is responsible for QoS split in L2 U2U relay.

Proposal 3: Support Option 3 (Both source remote UE ID (layer-2 ID) and target remote UE ID (layer-2 ID)) included in each hop.

Proposal 4: Legacy authorization mechanim can be reused.

Proposal 5: If proposal 4 is agreed, send reply LS back to SA2 and Cc RAN3.

# 4	References

[1]	R2-2301904, Report from session on positioning and sidelink relay, Session Chair (MediaTek)

[2]	R2-2300134, Discussion on U2U Relay, OPPO