3GPP TSG-RAN WG2 #122	Tdoc R2-2306379

Incheon, KR, May 22nd – 26th, 2023

Agenda item:	7.18.3

Source: 	Ericsson

Title:  	Handling BWP restrictions in MT-SDT

Document for:	Discussion and Decision

# Introduction

In the WID [1] for MT-SDT it is stated that the objective is

“Specify the support for paging-triggered SDT (MT-SDT) [RAN2, RAN3]

- MT-SDT triggering mechanism for UEs in RRC_INACTIVE, supporting RACH procedure based and CG-SDT procedure based UL response;

MT-SDT procedure for initial DL data reception and subsequent UL/DL data transmissions in RRC_INACTIVE.”

This contribution primarily discusses the scheduling restrictions owing to bandwidth part constraints in MT-SDT. We further propose some solutions to enhance the transmission capability for MT-SDT.

# Discussion

## BWP Restriction in MT-SDT

Rel-17 agreements restrict small data transmissions over initial Bandwidth part (BWP); the reason being to render SDT procedure as close to legacy initial access procedure as possible.

For UL transmissions initiated as part of MO-SDT, this assigns frequency resources across the entire initial UL BWP. However, for DL transmission, initial BWP operation restricts the frequency domain resource allocation to the size of CORESET 0. In specification, this is detailed as shown below.

For first DL MT-SDT data, the frequency domain resource assignment is provided in TS 38.212 [3] as follows.

The following information is transmitted by means of the DCI format 1_0 with CRC scrambled by TC-RNTI: - Identifier for DCI formats – 1 bit - The value of this bit field is always set to 1, indicating a DL DCI format - Frequency domain resource assignment – bits - is the size of CORESET 0

The same restriction is applicable for subsequent transmissions as well. In TS 38.212, this is provided as follows.

The following information is transmitted by means of the DCI format 1_0 with CRC scrambled by C-RNTI or CS-RNTI or MCS-C-RNTI: - Identifier for DCI formats – 1 bits - The value of this bit field is always set to 1, indicating a DL DCI format - Frequency domain resource assignment – bits where is given by clause 7.3.1.0 Where clause 7.3.1.0 is defined as follows: Determine DCI format 1_0 monitored in a common search space according to clause 7.3.1.2.1 where is given by - the size of CORESET 0 if CORESET 0 is configured for the cell; and - the size of initial DL bandwidth part if CORESET 0 is not configured for the cell.

According to TS 38.331 [2], the UE applies this restriction until after the reception of RRCSetup/RRCResume/RRCReestablishment when it switches to Connected mode.

Therefore, we make the following observation.

MT-SDT operation over initial BWP restricts DL MT-SDT transmissions to the bandwidth of CORESET 0 in RRC Inactive state.

For 30kHz subcarrier spacing, this means the scheduler can allocate a maximum of up to 48 PRBs within CORESET 0 [4]. If no DL quality indication is introduced in MT-SDT, use of MCS 0 can be expected by the gNB to ensure cell edge coverage. This implies that the restriction of 48 PRBs together with MCS 0 translates to a maximum transport block size (TBS) of roughly 1320 bits.

MT-SDT BWP restriction constrains TBS to roughly 1320 bits at MCS 0. For large data volume, smaller TBS translate to higher number of subsequent transmissions.

Since higher number of subsequent transmissions considerably reduce the energy efficiency of SDT, RAN2 should discuss solutions to handle the BWP restriction in MT-SDT. There are different options to handle this, as listed below.

Option 1: Remove CORESET 0 restriction for DL BWP for UEs from/after first DL MT-SDT data

This option involves considerable changes to RAN 1 specifications: especially in TS 38.212 and TS 38.214, and some minor modifications to TS 38.331. In addition, this option is not backwards compatible.

Option 2: Introduce Channel Quality Indication (CQI) in Message 3 or MsgA

An indicator on the channel quality from the UE allows the scheduler to adapt to better modulation and coding rates. Even with a restriction to 48 PRBs, this allows for considerably larger TB sizes. The details of this option is discussed in section. 2.2.

Option 3: Optimisations to reduce subsequent transmission overhead

This option also aims for no changes to CORESET 0 restriction but considers optimisations to reduce the overhead involved in scheduling a higher number of subsequent transmissions. In this regard, we propose Multi-TB support to be introduced for MT-SDT. This is discussed in detail in Sec. 2.3.

If no standard changes are done, it would require that MT-SDT data volumes to be substantially lower than that for MO-SDT (MO-SDT allowed DVT to be as high as 96000 bytes). There is no formal DL DVT and the choice of performing MT-SDT or resuming to connected mode is handled by the gNB implementation. We think that the restriction reduces the usefulness of the MT-SDT procedure since the gNB will move the UE to connected mode at smaller data volumes than would be needed if some enhancements described in options 1 - 3 are made.

Choose from among the following options to handle BWP restriction in MT-SDT

- Option 1: Remove CORESET 0 restriction for DL BWP for UEs from/after first DL MT-SDT data

- Option 2: Introduce Channel Quality Indication (CQI) in Message 3 or Msg A

- Option 3: Optimisations to reduce subsequent transmission overhead

## CQI reporting in Msg3

In LTE, Bandwidth reduced low complexity (BL) UEs or UEs in enhanced coverage may report Downlink Channel Quality Report (DCQR) in Msg3 of the random access procedure. Without any CQI reporting, the gNB always has to adapt the transmission to reach cell edge. This is not efficient since for the majority of UEs not at the cell edge, larger TBs could be scheduled with sufficient reliablity, especially with the constraint on 48 PRBs remaining for MT-SDT operation within CORESET 0. Additionally, this would be beneficial also in case of MT-SDT to enable better gNB control, e.g. to decide whether to do MT-SDT or go to RRC CONNECTED mode.

Introduction of a DCQR for MT-SDT would enable link adaptation for MT-SDT DL data transmission.

In LTE the DCQR is implemented as a MAC CE. For MT-SDT a similar MAC CE could be defined and multiplexed into Msg3, MsgA or into the initial CG-SDT transmission.

Introduce a DCQR MAC CE which can be multiplexed into Msg3, MsgA or the initial CG-SDT transmission in the MT-SDT procedure.

The exact details in terms of reference signals to measure should be left to RAN1. We also believe that RAN1 should be consulted regarding the feasibility to introduce CQI reporting in Msg3 and therefore we propose:

Send an LS to RAN1 to ask about the feasibility to introduce CQI reporting in Msg3 for MT-SDT.

## Multi-TB to reduce subsequent transmission overhead

Subsequent transmissions in MT-SDT can belong to one of the two categories:

Case 1- Where due to radio conditions or other limitations, the current data within the gNB/UE cannot be transmitted in one shot and is therefore split into multiple transport blocks.

Case 2- Where new data arrives while SDT procedure is ongoing.

In case 1, the gNB or the UE knows the size of the buffer beforehand. Despite this, supporting subsequent transmissions in DL/UL requires the gNB to schedule multiple PDSCH/PUSCH transmissions using separate DCIs, increasing the signalling overhead as shown in Figure 2.

Figure 2; Despite knowing the buffer contents in advance, gNB might have to schedule multiple PDSCH transmissions as shown. The same could be applicable in UL as well.

Transmissions where the existing buffer contents cannot be emptied in one shot results in subsequent transmissions where e.g. the gNB is forced to send multiple PDCCH to schedule each PDSCH/PUSCH transmissions individually.

A possible optimisation to reduce the number of PDCCH transmissions in such a case is to introduce multi-TB support, as with NB-IoT and LTE-M. Multi-TB feature enables scheduling multiple PDSCH (or PUSCH) transmissions with a single DCI as shown in Figure 3.

Figure 3; Multi TB scheduling

Discuss whether to introduce multi-TB support to reduce the number DCIs for scheduling multiple PDSCH transmissions as part of subsequent DL transmissions.

In case 2 (where data arrives in the buffer after the initial transmission) the buffer contents are not known in advance. Therefore, the only plausible option is to schedule the subsequent transmissions dynamically as in Rel-17 MO-SDT. For DL, the gNB can schedule these as separate PDSCH transmissions by scheduling them over DCI. In UL, the transmissions can be scheduled based on the UE’s Buffer Status Report (BSR), as in Rel-17 MO-SDT.

For subsequent DL transmissions to transmit the new incoming data during an ongoing MT-SDT procedure, the gNB can schedule the PDSCH transmissions as in MO-SDT.

For subsequent UL transmissions to transmit the new incoming data during an ongoing MT-SDT procedure, follow the dynamic scheduling based on the contents of BSR as in MO-SDT.

# Conclusion

In the previous sections we made the following observations:

Observation 1	MT-SDT operation over initial BWP restricts DL MT-SDT transmissions to the bandwidth of CORESET 0 in RRC Inactive state.

Observation 2	MT-SDT BWP restriction constrains TBS to roughly 1320 bits at MCS 0. For large data volume, smaller TBS translate to higher number of subsequent transmissions.

Observation 3	Introduction of a DCQR for MT-SDT would enable link adaptation for MT-SDT DL data transmission.

Observation 4	Transmissions where the existing buffer contents cannot be emptied in one shot results in subsequent transmissions where e.g. the gNB is forced to send multiple PDCCH to schedule each PDSCH/PUSCH transmissions individually.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Choose from among the following options to handle BWP restriction in MT-SDT

Option 1: Remove CORESET 0 restriction for DL BWP for UEs from/after first DL MT-SDT data

Option 2: Introduce Channel Quality Indication (CQI) in Message 3 or Msg A

Option 3: Optimisations to reduce subsequent transmission overhead

Proposal 2	Introduce a DCQR MAC CE which can be multiplexed into Msg3, MsgA or the initial CG-SDT transmission in the MT-SDT procedure.

Proposal 3	Send an LS to RAN1 to ask about the feasibility to introduce CQI reporting in Msg3 for MT-SDT.

Proposal 4	Discuss whether to introduce multi-TB support to reduce the number DCIs for scheduling multiple PDSCH transmissions as part of subsequent DL transmissions.

Proposal 5	For subsequent DL transmissions to transmit the new incoming data during an ongoing MT-SDT procedure, the gNB can schedule the PDSCH transmissions as in MO-SDT.

Proposal 6	For subsequent UL transmissions to transmit the new incoming data during an ongoing MT-SDT procedure, follow the dynamic scheduling based on the contents of BSR as in MO-SDT.

# References

3GPP RP-222993. “New WI: Mobile Terminated-Small Data Transmission (MT-SDT) for NR”

3GPP 38.331, 17.2.0, NR; Radio Resource Control (RRC); Protocol specification

- [3]	3GPP 38.212, 17.2.0, NR; Multiplexing and channel coding

- [4]	3GPP 38.214, NR; Physical layer procedures for data