# 3GPP TSG RAN WG2 Meeting #122                       	    R2-2306387

# Incheon, KR, May 22 – 26, 2023

Agenda item:	7.2.1

Source:	Qualcomm Incorporated

Title:	Support of Multiple Target UEs for Sidelink Positioning (draft response LS to R2-2302448 (S2-2303837))

Document for:	Discussion and Decision

# Introduction

In R2-2301938_S2-2303837, SA2 have identified sidelink ranging and positioning use cases involving multiple target UEs for the same positioning session [1]. This contribution discusses Sidelink Ranging and Positioning support for single target UE in a positioning session and for multiple target UEs in a positioning session and proposes a reply LS to SA2 in response to their inquiry on this subject.

1. Overall Description: SA2 would like to inform RAN2 that during the development of the Ranging/SL Positioning support (as in TS 23.586), use cases had been identified where multiple Target UEs are involved in the same positioning session. In these cases, it is desirable to allow the Ranging/SL Positioning control to support multiple target UEs, e.g. with broadcast or groupcast support. SA2 understand that RAN2 is currently defining the Ranging/SL Positioning Protocol (the SLPP in RAN2’s context). Therefore, SA2 would like to ask RAN2 whether the SLPP would support multiple Target UEs in the same signalling session, and whether there is possibility of signalling the positioning results of multiple Target UEs in the same message. 2. Actions: To RAN2 WG: ACTION: SA2 would like to ask RAN2 to provide feedback on the support of multiple Target UEs in the same Ranging/SL Positioning session.

SA2 have provided further motivation for group positioning in R2-2300079_S2-2301786, noting V2X, MCX and Museum Tours as group positioning use cases [2].

Regarding the questions from RAN2 on the use cases for groupcast/broadcast signalling for SL positioning, SA2 would like to point out that for the V2X use cases, for example those documented in TR 38.845, there are use cases of involving Road Side Unit (RSU) and multiple vehicles, or multiple vehicles in a platoon, which would benefit from groupcast/broadcast signalling support. Additionally, the MCX service that relies on group communication would also benefit from groupcast/broadcast signalling support. In addition, for Ranging based services, as documented in TR 22.855, there are multiple use cases that involve groups of UEs performing Ranging/Sidelink positioning services, e.g. Museum Tour, etc.

In this contribution the implications on SLPP signaling of sidelink positioning support for a single target UE or for of multiple target UEs is addressed.

# Discussion

RAN2 has reached the following agreements regarding range/position calculation by one UE on behalf of other, multiple UEs (“centralized” or UE-assisted), and range/position calculation by multiple participating UEs (UE-based) [3], [4].

Agreement (RAN2 #119bis-e) RAN2 agrees to support at least unicast SLPP/RSPP "centralized" operation in the sense used in R2-2210911, i.e., operation where one UE performs range and/or position calculations based on measurement/location information relating to itself and/or other UEs. Agreement (RAN2 #120) From RAN2 perspective, if it is determined to support group positioning, it is feasible to perform at least ranging with the estimate calculation at multiple UEs.

RAN2 has also reached the following agreement regarding distribution of SLPP signaling to multiple target UEs [4].

Agreement (RAN2 #120) RAN2 confirm that from RAN2 perspective, it is feasible to send at least the following positioning signaling for groupcast/broadcast (in addition to unicast): - SL positioning capability - SL positioning assistance data Location information is not excluded and can be further considered in normative work.

Regarding group positioning, RAN2 has reached the following working assumptions [6].

WA: RAN2 understand that group positioning is to acquire location estimates of multiple target UEs (absolute positioning) or multiple UE pairs (Ranging/relative positioning) per LCS request, in line with the guidance already received from SA2. WA: At least part of the group management for group positioning is performed at upper/application layer.

On the general sequence of possible SLPP procedure steps, RAN2 agreed the following as baseline [5].

With respect to the overall signaling procedure for PC5-only positioning (including at least IC and OOC; FFS if there are differences for PC), it is proposed to agree that the sidelink positioning procedure comprises the following series of steps as a baseline, between the LMF/positioning server UE/NG-RAN/candidate Anchor UE(s) and Target UE(s): 1. Triggering event 2. Sidelink positioning capability exchange 3. Sidelink positioning assistance data transfer 4. SL Positioning Request Location Information 5. Measurement of SL-PRS 6. Location calculation 7. SL Positioning Provide Location Information Some steps may have dependencies on SA2 and can be revisited in this light. The order is subject to further discussion. FFS if discovery and selection of anchor UEs and/or server UE are part of the positioning layer in RAN2 scope. LS to SA2 to ask for confirmation and guidance on the SA2 aspects.

Additional agreements related to the SLPP signalling procedures were made at RAN2#121bis-e [6].

Anchor UE and target UE roles can be shown in the sidelink positioning procedures in stage 2. Server UE can be further discussed at least for the case that the server UE is separate from the target and anchor. Discovery procedure is included in the sidelink positioning procedure at least for out of coverage scenario.

In the following, example SLPP procedural steps and call flows aligned with above agreements are illustrated for UE-assisted (centralized) sidelink ranging and positioning and for UE-based (distributed) sidelink ranging and positioning. These flows demonstrate sidelink ranging and positioning for a single target UE and for multiple target UEs and show that the same SLPP messages and message sequence can be used whether a sidelink positioning SLPP session involves single target UE or multiple target UEs.  Specifically, it is shown that the SLPP call flow can be independent of the number of participating UEs and the number of target UEs.

## UE-assisted (centralized) Ranging and Positioning: Single and Multiple Target UEs

### UE-assisted sidelink positioning: Single Target UE

Figure 1 illustrates a UE-assisted sidelink positioning session for a single Target UE, where a server UE (UE-A) initiates the SLPP session and performs position calculation based on SL-PRS measurements received from the single Target UE and the anchor UEs. Following UE discovery, the SLPP message flow consists of session establishment, capability and assistance data exchange, location information transfer and session termination.

Figure 1: UE-assisted (centralized) Sidelink-only Positioning - Single Target UE

- The Application layer in UE-A and other UEs may perform UE discovery, determine and verify required services and perform group management (e.g. create a group containing UEs A, B and E, F, G). The Application layer in UE-A also verifies that all of the UEs support SLPP. The Application layer in UE-A then requests sidelink ranging and positioning results for the single Target UE-B from the SLPP layer in UE-A, and provides the SLPP layer in UE-A the application layer IDs and layer 2 IDs for each of the UEs-A to E, and may provide an application layer group ID and a layer 2 group ID for the group of UEs A to E.

- NOTE: The exact API between Application Layer and SLPP layer can be regarded as implementation dependent.

- The initiating UE (Server UE-A) establishes an SLPP Session to make all participating UEs aware of each other.

- The initiating UE (Server UE-A) transmits an SLPP Request Capabilities message to the single Target UE (UE-B) and the anchor UEs (UE-E, UE-F, UE-G).

- The SLPP Request Capabilities message may be sent by initiating UE (Server UE-A) to the single Target UE (UE-B) and to the anchor UEs (UE-E, UE-F, UE-G) via unicast, groupcast or broadcast, depending on the specific scenario.

- The single Target UE (UE-B) and the anchor UEs (UE-E, UE-F, UE-G) respond to the initiating UE (Server UE-A) with an SLPP Provide Capabilities message.

- The SLPP Provide Capabilities message may be sent by the single Target UE (UE-B) and by the anchor UEs (UE-E, UE-F, UE-G) to the initiating UE (Server UE-A) via unicast, groupcast or broadcast, depending on the specific scenario.

- The SLPP Session may be modified based on the capability information from Step 3 (e.g., removal of UEs with incompatible capability such as SL positioning method, etc.).

- The initiating UE (Server UE-A) transmits an SLPP Provide Assistance Data message to the single Target UE (UE-B) and the anchor UEs (UE-E, UE-F, UE-G).

- The SLPP Provide Assistance Data message may be sent by initiating UE (Server UE-A) to the single Target UE (UE-B) and to the anchor UEs (UE-E, UE-F, UE-G) via unicast, groupcast or broadcast, depending on the specific scenario.

- The SLPP Provide Assistance Data message includes the SL-PRS configuration for each of the UEs in the session. After receipt of the assistance data (or at an indicated starting time included in the message), each UE in the session transmits its respective SL-PRS. The duration of SL-PRS transmission may be controlled by a timer provided in the message or may continue until the SLPP session is terminated at Step 11.

- The initiating UE (Server UE-A) transmits an SLPP Request Location Information message to the single Target UE (UE-B) and/or to the anchor UEs (UE-E, UE-F, UE-G), dependent on the desired positioning method.

- The SLPP Request Location Information message may be sent by initiating UE (Server UE-A) to the single Target UE (UE-B) and to the anchor UEs (UE-E, UE-F, UE-G) via unicast, groupcast or broadcast, depending on the specific scenario.

- The SLPP Request Location Information message indicates the type of location information needed and associated QoS (e.g., desired accuracy, response time, etc.). The message also includes an indication that range/position calculation will be performed by the initiating UE (Server UE-A).

- Each participating UE performs the requested SL-PRS transmissions and measurements.

- Note: The Server UE-A may participate in the sidelink positioning session through transmission and measurement of SL-PRS (for example also taking the role of an anchor UE) or may participate only in managing the session and performing and disseminating position calculations.

- The single Target UE (UE-B) and/or the anchor UEs (UE-E, UE-F, UE-G) respond to the initiating UE (Server UE-A) with an SLPP Provide Location Information message.

- The SLPP Provide Location Information message may be sent by the single Target UE (UE-B) and by the anchor UEs (UE-E, UE-F, UE-G) to the initiating UE (Server UE-A) via unicast, groupcast or broadcast, depending on the specific scenario.

- The SLPP Provide Location Information message may be sent upon completion of the transmission and measurements or periodically or after the response time received at Step 6 expired and may also include an indication of whether other UEs (the initiating Server UE-A in this case) is requested to provide the location result (e.g., calculated range and/or position) back to this UE. SL-PRS transmission and SL-PRS measurement may be performed periodically, based on the configuration received in the SLPP Provide Assistance Data message.

- The initiating UE (Server UE-A) performs the range/position computations using the location results received at Step 8 and the location results UE A has obtained at Step 7.

- If requested at Step 8 to distribute location results, the initiating UE (Server UE-A) may distribute the location results by transmitting an SLPP Provide Location Information message to the single Target UE-B.

- The SLPP Provide Location Information message may be sent by initiating UE (Server UE-A) to the single Target UE (UE-B) via unicast, groupcast or broadcast, depending on the specific scenario.

- The SLPP session may be terminated.

### UE-assisted sidelink positioning: Multiple Target UEs

Figure 2 illustrates a UE-assisted sidelink positioning session for multiple Target UEs, where a server UE (UE-A) initiates the SLPP session and performs position calculation based on SL-PRS measurements received from multiple Target UEs and the anchor UEs. The SLPP message flow is identical to the flow shown in Figure 1 for UE-assisted sidelink positioning for a single Target UE. Following UE discovery, the UEs perform session establishment, capability and assistance data exchange, location information transfer and session termination.

Figure 2: UE-assisted (centralized) Sidelink-only Positioning - Multiple Target UEs

- The Application layer in UE-A and other UEs may perform UE discovery, determine and verify required services and perform group management (e.g., create a group containing UEs A to G). The Application layer in UE-A also verifies that all of the UEs support SLPP. The Application layer in UE-A then requests sidelink ranging and positioning results for the multiple Target UEs (UE-B, UE-C, UE-D) from the SLPP layer in UE-A, and provides the SLPP layer in UE-A the application layer IDs and layer 2 IDs for each of the UEs-A to E, and may provide an application layer group ID and a layer 2 group ID for the group of UEs A to E.

- NOTE: The exact API between Application Layer and SLPP layer can be regarded as implementation dependent.

- The initiating UE (Server UE-A) establishes an SLPP Session to make all participating UEs aware of each other.

- The initiating UE (Server UE-A) transmits an SLPP Request Capabilities message to the multiple Target UEs (UE-B, UE-C, UE-D) and the anchor UEs (UE-E, UE-F, UE-G).

- The SLPP Request Capabilities message may be sent by initiating UE (Server UE-A) to the multiple Target UEs (UE-B, UE-C, UE-D) and to the anchor UEs (UE-E, UE-F, UE-G) via unicast, groupcast or broadcast, depending on the specific scenario.

- The Multiple Target UEs (UE-B, UE-C, UE-D) and the anchor UEs (UE-E, UE-F, UE-G) respond to the initiating UE (Server UE-A) with an SLPP Provide Capabilities message.

- The SLPP Provide Capabilities message may be sent by the multiple Target UEs (UE-B, UE-C, UE-D) and by the anchor UEs (UE-E, UE-F, UE-G) to the initiating UE (Server UE-A) via unicast, groupcast or broadcast, depending on the specific scenario.

- The SLPP Session may be modified based on the capability information from Step 3 (e.g., removal of UEs with incompatible capability such as SL positioning method, etc.).

- The initiating UE (Server UE-A) transmits an SLPP Provide Assistance Data message to the multiple Target UEs (UE-B, UE-C, UE-D) and to the anchor UEs (UE-E, UE-F, UE-G).

- The SLPP Provide Assistance Data message may be sent by initiating UE (Server UE-A) to the multiple Target UEs (UE-B, UE-C, UE-D) and to the anchor UEs (UE-E, UE-F, UE-G) via unicast, groupcast or broadcast, depending on the specific scenario.

- The SLPP Provide Assistance Data message includes the SL-PRS configuration for each of the UEs participating in the SLPP session. After receipt of the assistance data (or at an indicated starting time included in the message), each UE in the session transmits its respective SL-PRS. The duration of SL-PRS transmission may be controlled by a timer provided in the message or may continue until the SLPP session is terminated at Step 11.

- The initiating UE (Server UE-A) transmits an SLPP Request Location Information message to the multiple Target UEs (UE-B, UE-C, UE-D) and/or to the anchor UEs (UE-E, UE-F, UE-G), dependent on the desired positioning method.

- The SLPP Request Location Information message may be sent by initiating UE (Server UE-A) to the multiple Target UEs (UE-B, UE-C, UE-D) and to the anchor UEs (UE-E, UE-F, UE-G) via unicast, groupcast or broadcast, depending on the specific scenario.

- The SLPP Request Location Information message indicates the type of location information needed and associated QoS (e.g., desired accuracy, response time, etc.). The message also includes an indication that range/position calculation will be performed by the initiating UE (Server UE-A).

- Each participating UE performs the requested SL-PRS transmissions and measurements.

- Note: The Server UE-A may participate in the sidelink positioning session through transmission and measurement of SL-PRS (for example also taking the role of an anchor UE) or may participate only in managing the session and performing and disseminating position calculations.

- The Multiple Target UEs (UE-B, UE-C, UE-D) and the anchor UEs (UE-E, UE-F, UE-G) respond to the initiating UE (Server UE-A) with an SLPP Provide Location Information message.

- The SLPP Provide Location Information message may be sent by the multiple Target UEs (UE-B, UE-C, UE-D) and by the anchor UEs (UE-E, UE-F, UE-G) to the initiating UE (Server UE-A) via unicast, groupcast or broadcast, depending on the specific scenario.

- The SLPP Provide Location Information message may be sent upon completion of the transmission and measurements or periodically or after the response time received at Step 6 expired and may also include an indication of whether other UEs (the initiating Server UE-A in this case) is requested to provide the location result (e.g., calculated range and/or position) back to this UE. SL-PRS transmission and SL-PRS measurement may be performed periodically, based on the configuration received in the SLPP Provide Assistance Data message.

- The initiating UE (Server UE-A) performs the range/position computations using the location results received at Step 8 and the location results UE B has obtained at Step 7.

- If requested at Step 8 to distribute location results, the initiating UE (Server UE-A) may distribute the location results by transmitting an SLPP Provide Location Information message to the multiple Target UEs (UE-B, UE-C, UE-D).

- The SLPP Provide Location Information message may be sent by initiating UE (Server UE-A) to the multiple Target UEs (UE-B, UE-C, UE-D) via unicast, groupcast or broadcast, depending on the specific scenario.

- The SLPP session may be terminated.

From the above call flow it is evident that the SLPP messages and message call flow for a UE-assisted sidelink positioning session for multiple Target UEs is identical to the SLPP messages and message call flow for a UE-assisted sidelink positioning session for a single Target UE.

Observation 1: UE-assisted Sidelink Positioning SLPP sessions use the same SLPP messages and SLPP message call flow regardless of whether the session is for a single Target UE or for multiple Target UEs.

If an SLPP positioning session is constrained to support only a single Target UE, an application-layer request to perform sidelink positioning for multiple Target UEs will require as many SLPP positioning sessions as there are target UEs. Each separate session will require its own separate capability negotiation, assistance data exchange, SL-PRS configuration, SL-PRS transmissions and location information transfer, resulting in far less efficient, temporally elongated sidelink positioning transaction than would be the case if an SLPP session supports multiple target UEs.

Observation 2: If an SLPP session does not support multiple target UEs, an application-layer request to position multiple UEs will require as many SLPP sessions as there are target UEs. Each SLPP session requiring its own capability negotiation, assistance data exchange, SL-PRS configuration, SL-PRS transmission and location information exchange. The resulting positioning session will be inefficient in resource utilization and require a longer time to complete compared to a session supporting multiple target UEs.

### UE-assisted sidelink ranging: Single Target UE and Multiple Target UEs

Figure 3 illustrates UE-assisted sidelink ranging for a single Target UE and for multiple Target UEs, where a server UE (UE-A) performs range calculation based on SL-PRS measurements received from the single Target UE or from the multiple Target UEs. The SLPP message flow is the same for ranging to a single Target UE and for ranging to Multiple Target UEs, and is identical to the flow shown in Figure 1 and Figure 2 for UE-assisted sidelink positioning for a single Target UE and multiple Target UEs, respectively. Following UE discovery, the UEs perform session establishment, capability and assistance data exchange, location information transfer and session termination.

Figure 3: UE-assisted (centralized) Sidelink-only Ranging - Left: Single Target UE. Right: Multiple Target UEs

- The Application layer in UE-A and other UEs may perform UE discovery, determine and verify required services and perform group management (e.g., create a group containing UE A and UE B or a group containing UEs A-D). The Application layer in UE-A also verifies that all of the UEs support SLPP. The Application layer in UE-A then requests sidelink ranging and positioning results for the single Target UE-B from the SLPP layer in UE-A, and provides the SLPP layer in UE-A the application layer IDs and layer 2 IDs for each of the UEs-A to E, and may provide an application layer group ID and a layer 2 group ID for the group of UEs A to E.

- NOTE: The exact API between Application Layer and SLPP layer can be regarded as implementation dependent.

- The initiating UE (Server UE-A) establishes an SLPP Session to make all participating UEs aware of each other.

- The initiating UE (Server UE-A) transmits an SLPP Request Capabilities message:

- Single Target UE: The SLPP Request Capabilities message is transmitted by the initiating UE (Server UE-A) to the single Target UE-B.

- Multiple Target UEs: The SLPP Request Capabilities message is transmitted by the initiating UE (Server UE-A) to the multiple Target UEs (UE-B, UE-C, UE-D).

- The SLPP Request Capabilities message may be sent by initiating UE (Server UE-A) to the single Target UE (UE-B) or to the multiple Target UEs (UE-B, UE-C, UE-D) via unicast, groupcast or broadcast, depending on the specific scenario.

- SLPP Provide Capabilities:

- Single Target UE: The Single Target UE (UE-B) responds to the initiating UE (Server UE-A) with an SLPP Provide Capabilities message.

- Multiple Target UEs: The Multiple Target UEs (UE-B, UE-C, UE-D) respond to the initiating UE (Server UE-A) with an SLPP Provide Capabilities message.

- The SLPP Provide Capabilities message may be sent by the single Target UE (UE-B) to the initiating UE (Server UE-A) or by the multiple Target UEs (UE-B, UE-C, UE-D) to the initiating UE (Server UE-A) via unicast, groupcast or broadcast, depending on the specific scenario.

- The SLPP Session may be modified based on the capability information from Step 3 (e.g., removal of UEs with incompatible capability such as SL positioning method, anchor capability, etc.).

- The initiating UE (Server UE-A) transmits an SLPP Provide Assistance Data message:

- Single Target UE: The SLPP Provide Assistance Data message is transmitted by the initiating UE (Server UE-A) to the Target UE-B.

- Multiple Target UEs: The SLPP Provide Assistance Data message is transmitted by the initiating UE (Server UE-A) to the multiple Target UEs established at Step 1 or 4 (UE-B, UE-C, UE-D).

- The SLPP Provide Assistance message may be sent by initiating UE (Server UE-A) to the single Target UE (UE-B) or to the multiple Target UEs (UE-B, UE-C, UE-D) via unicast, groupcast or broadcast, depending on the specific scenario.

- The SLPP Provide Assistance Data message includes the SL-PRS configuration for each of the UEs participating in the SLPP session. After receipt of the assistance data (or at an indicated starting time included in the message), each UE in the session transmits its respective SL-PRS. The duration of SL-PRS transmission may be controlled by a timer provided in the message or may continue until the SLPP session is terminated at Step 11.

- The initiating UE (Server UE-A) transmits an SLPP Request Location Information message:

- Single Target UE: The SLPP Request Location Information message is transmitted by the initiating UE (Server UE-A) to the Target UE-B.

- Multiple Target UEs: The SLPP Request Location Information message is transmitted by the initiating UE (Server UE-A) to the multiple Target UEs (UE-B, UE-C, UE-D).

- The SLPP Request Location Information message may be sent by initiating UE (Server UE-A) to the single Target UE (UE-B) or multiple Target UEs (UE-B, UE-C, UE-D) via unicast, groupcast or broadcast, depending on the specific scenario.

- The SLPP Request Location Information message indicates the type of location information needed and associated QoS (e.g., desired accuracy, response time, etc.). The message also includes an indication that range/position calculation will be performed by the initiating UE (Server UE-A).

- Each participating UE performs the requested SL-PRS transmissions and measurements.

- SLPP Provide Location Information:

- Single Target UE: The Single Target UE (UE-B) responds to the initiating UE (Server UE-A) with an SLPP Provide Location Information message.

- Multiple Target UEs: The multiple Target UEs (UE-B, UE-C, UE-D) respond to the initiating UE (Server UE-A) with an SLPP Provide Location Information message.

- The SLPP Provide Location Information message may be sent by the single Target UE (UE-B) to the initiating UE (Server UE-A) or by the multiple Target UEs (UE-B, UE-C, UE-D) to the initiating UE (Server UE-A) via unicast, groupcast or broadcast, depending on the specific scenario.

- The SLPP Provide Location Information message may be sent upon completion of the transmission and measurements or periodically or after the response time received at Step 6 expired and may also include an indication of whether other UEs (the initiating Server UE-A in this case) is requested to provide the location result (e.g., calculated range and/or position) back to this UE. SL-PRS transmission and SL-PRS measurement may be performed periodically, based on the configuration received in the SLPP Provide Assistance Data message.

- The initiating UE (Server UE-A) performs the range/position computations using the location results received at Step 8 and the location results UE B has obtained at Step 7.

- If requested at Step 8 to distribute location results, the initiating UE (Server UE-A) may distribute the location results by transmitting an SLPP Provide Location Information message:

- Single Target UE: The SLPP Provide Location Information message is transmitted by the initiating UE (Server UE-A) to the single Target UE-B.

- Multiple Target UEs: The SLPP Provide Location Information message is transmitted by the initiating UE (Server UE-A) to the multiple Target UEs (UE-B, UE-C, UE-D).

- The SLPP Provide Location Information message may be sent by initiating UE (Server UE-A) to the single Target UE (UE-B) or to the multiple Target UEs (UE-B, UE-C, UE-D) via unicast, groupcast or broadcast, depending on the specific scenario.

- The SLPP session may be terminated.

The above call flow demonstrates that the SLPP messages and message call flow for an SLPP UE-assisted sidelink ranging session for multiple Target UEs is identical to the SLPP messages and message call flow for an SLPP UE-assisted sidelink ranging session for a single Target UE. Furthermore, the SLPP messages and message call flow for an SLPP UE-assisted sidelink ranging session for a single or for multiple target UEs is identical to the SLPP messages and message flow for an SLPP UE-assisted sidelink positioning session for a single or for multiple target UEs.

Observation 3: UE-assisted Sidelink Ranging SLPP sessions for a single Target UE and UE-assisted Sidelink Ranging SLPP sessions for multiple Target UEs use the same SLPP messages and SLPP message call flow.

Observation 4: Regardless of whether an SLPP session is for a single target UE or for multiple target UEs, the same SLPP messages and SLPP message call flow are used for UE-assisted Sidelink Ranging SLPP sessions and UE-assisted Sidelink Positioning SLPP sessions.

As in UE-assisted sidelink positioning, for UE-assisted sidelink ranging, if an SLPP ranging session is constrained to support only a single Target UE, an application-layer request to perform sidelink ranging for multiple Target UEs will require as many SLPP ranging sessions as there are target UEs, with each separate session requiring its own separate capability negotiation, assistance data exchange, SL-PRS configuration, SL-PRS transmissions and location information transfer. The resulting multiple ranging sessions are far less efficient, and incur greater latency to determine the multiple ranges than would be the case if an SLPP ranging session supports multiple target UEs.

Observation 5: If an SLPP session does not support multiple target UEs, an application-layer request to range multiple UEs will require as many SLPP sessions as there are target UEs, with each SLPP session including its own capability negotiation, assistance data exchange, SL-PRS configuration, SL-PRS transmission and location information exchange, resulting in a resource-inefficient, temporally elongated sidelink ranging session.

## UE-based (distributed) Ranging and Positioning: Single and Multiple Target UEs

### UE-based sidelink positioning: Single Target UE

Figure 4 illustrates a UE-based sidelink positioning session for a single Target UE, where a server UE (UE-A) initiates the SLPP session and subsequent position calculation is performed by the participating UEs based on SL-PRS measurements received from the single Target UE and the anchor UEs. Following UE discovery, the SLPP message flow consists of session establishment, capability and assistance data exchange, location information transfer and session termination.

Figure 4: UE-based (distributed) Sidelink-only Positioning - Single Target UE

0.-5. Same as in Figure 1.

- The initiating UE (Server UE-A) transmits an SLPP Request Location Information message to the Single Target UE-B and/or to the anchor UEs (UE-E, UE-F, UE-G), dependent on the desired positioning method.

- The SLPP Request Location Information message may be sent by initiating UE (Server UE-A) to the single Target UE (UE-B) and to the anchor UEs (UE-E, UE-F, UE-G) via unicast, groupcast or broadcast, depending on the specific scenario.

- The SLPP Request Location Information message indicates the type of location information needed and associated QoS (e.g., desired accuracy, response time, etc.). The message also includes an indication that range/position calculation should be performed by each of the participating UEs (the initiating Server UE-A and the Target UE(s)).

- Each participating UE performs the requested SL-PRS transmissions and measurements.

- Note: The Server UE-A may participate in the sidelink positioning session through transmission and measurement of SL-PRS (for example also taking the role of an anchor UE) or may participate only in managing the session and performing and disseminating position calculations.

- The Single Target UE (UE-B), the initiating UE (Server UE-A) and the anchor UEs (UE-E, UE-F, UE-G) transmit an SLPP Provide Location Information message.

- The SLPP Provide Location Information message may be transmitted by the Single Target UE (UE-B), the initiating UE (Server UE-A) and the anchor UEs (UE-E, UE-F, UE-G) via unicast, groupcast or broadcast, depending on the specific scenario.

- The SLPP Provide Location Information message may be sent upon completion of the transmission and measurements or periodically or after the response time received at Step 6 expired and may also include an indication of whether other UEs are requested to provide the location result (e.g., calculated range and/or position) back to the sending UE. SL-PRS transmission and SL-PRS measurement may be performed periodically, based on the configuration received in the SLPP Provide Assistance Data message.

- Each participating UE performs the range/position computations using the location results received at Step 8 and its own location results obtained at Step 7.

- If requested at Step 8, each UE may distribute its location results to the other UEs in this session by transmitting an SLPP Provide Location Information message.

- The SLPP Provide Location Information message may be sent by the Single Target UE (UE-B), the initiating UE (Server UE-A) and the anchor UEs (UE-E, UE-F, UE-G) via unicast, groupcast or broadcast, depending on the specific scenario.

- The SLPP session may be terminated.

### UE-based sidelink positioning: Multiple Target UEs

Figure 5 illustrates a UE-based sidelink positioning session for a multiple Target UEs, where a server UE (UE-A) initiates the SLPP session and subsequent position calculation is performed by the participating UEs based on SL-PRS measurements received from the multiple Target UEs and the anchor UEs. Following UE discovery, the SLPP message flow consists of session establishment, capability and assistance data exchange, location information transfer and session termination.

Figure 5: UE-based (distributed) Sidelink-only Positioning - Multiple Target UEs

0.-5. Same as in Figure 2.

- The initiating UE (Server UE-A) transmits an SLPP Request Location Information message to the Multiple Target UEs (UE-B, UE-C, UE-D) and/or to the anchor UEs (UE-E, UE-F, UE-G), dependent on the desired positioning method.

- The SLPP Request Location Information message may be sent by initiating UE (Server UE-A) to the Multiple Target UEs (UE-B, UE-C, UE-D) and the anchor UEs (UE-E, UE-F, UE-G) via unicast, groupcast or broadcast, depending on the specific scenario.

- The SLPP Request Location Information message indicates the type of location information needed and associated QoS (e.g., desired accuracy, response time, etc.). The message also includes an indication that range/position calculation should be performed by each of the participating UEs (the initiating Server UE-A and the Target UE(s)).

- Each participating UE performs the requested SL-PRS transmissions and measurements.

- Note: The Server UE-A may participate in the sidelink positioning session through transmission and measurement of SL-PRS (for example also taking the role of an anchor UE) or may participate only in managing the session and performing and disseminating position calculations.

- The Multiple Target UEs (UE-B, UE-C, UE-D), the initiating UE (Server UE-A) and the anchor UEs (UE-E, UE-F, UE-G) transmit an SLPP Provide Location Information message.

- The SLPP Provide Location Information message may be transmitted by the Multiple Target UEs (UE-B, UE-C, UE-D), the initiating UE (Server UE-A) and the anchor UEs (UE-E, UE-F, UE-G) via unicast, groupcast or broadcast, depending on the specific scenario.

- The SLPP Provide Location Information message may be sent upon completion of the transmission and measurements or periodically or after the response time received at Step 6 expired and may also include an indication of whether other UEs are requested to provide the location result (e.g., calculated range and/or position) back to the sending UE. SL-PRS transmission and SL-PRS measurement may be performed periodically, based on the configuration received in the SLPP Provide Assistance Data message.

- Each participating UE performs the range/position computations using the location results received at Step 8 and its own location results obtained at Step 7.

- If requested at Step 8, each UE may distribute its location results to the other UEs in this session by transmitting an SLPP Provide Location Information message.

- The SLPP Provide Location Information message may be sent by the multiple Target UEs (UE-B, UE-C, UE-D), the initiating UE (Server UE-A) and the anchor UEs (UE-E, UE-F, UE-G) via unicast, groupcast or broadcast, depending on the specific scenario.

- The SLPP session may be terminated.

From the above call flow it is apparent the SLPP messages and message call flow for a UE-based sidelink positioning session for multiple Target UEs is identical to the SLPP messages and message call flow for a UE-based sidelink positioning session for a single Target UE. Furthermore, both UE-based and UE-assisted sidelink positioning use the same SLPP messages and SLPP message call flows regardless of whether the session is for a single target UE or for multiple target UEs.

Observation 6: UE-based Sidelink Positioning SLPP sessions use the same SLPP messages and SLPP message call flow regardless of whether the session is for a single Target UE or for multiple Target UEs.

Observation 7: SLPP sidelink positioning sessions (UE-assisted or UE-based) use the same SLPP messages and SLPP message call flow regardless of whether the SLPP session is for a single target UE or multiple Target UEs.

### UE-based sidelink ranging: Single Target UE and Multiple Target UEs

Figure 6 illustrates UE-based sidelink ranging for a single Target UE and for multiple Target UEs, where a server UE (UE-A) initiates the SLPP session and subsequent range calculation is performed by the participating UEs based on SL-PRS measurements received from the single Target UE or multiple Target UEs. The SLPP message flow is the same for ranging to a single Target UE and for ranging to Multiple Target UEs, and is identical to the flow shown in Figure 4 and Figure 5 for UE-based sidelink positioning for a single Target UE and multiple Target UEs, respectively. Following UE discovery, the UEs perform session establishment, capability and assistance data exchange, location information transfer and session termination.

Figure 6: UE-based (distributed) Sidelink-only Ranging - Left: Single Target UE. Right: Multiple Target UEs

0.-5. Same as in Figure 3.

- The initiating UE (Server UE-A) transmits an SLPP Request Location Information message:

- Single Target UE: The SLPP Request Location Information message is transmitted by the initiating UE (Server UE-A) to the Target UE-B.

- Multiple Target UEs: The SLPP Request Location Information message is transmitted by the initiating UE (Server UE-A) to the multiple Target UEs (UE-B, UE-C, UE-D).

- The SLPP Request Location Information message may be sent by initiating UE (Server UE-A) to the single Target UE (UE-B) or multiple Target UEs (UE-B, UE-C, UE-D) via unicast, groupcast or broadcast, depending on the specific scenario.

- The SLPP Request Location Information message indicates the type of location information needed and associated QoS (e.g., desired accuracy, response time, etc.). The message also includes an indication that range/position calculation should be performed by each of the participating UEs (the initiating Server UE-A and the Target UE(s)).

- Each participating UE performs the requested SL-PRS transmissions and measurements.

- SLPP Provide Location Information:

- Single Target UE: The Single Target UE (UE-B) and the initiating UE (Server UE-A) transmit an SLPP Provide Location Information message.

- Multiple Target UEs: The multiple Target UEs (UE-B, UE-C, UE-D) and the initiating UE (Server UE-A) transmit an SLPP Provide Location Information message.

- The SLPP Provide Location Information message may be sent by the single Target UE (UE-B) to the initiating UE (Server UE-A) or by the multiple Target UEs (UE-B, UE-C, UE-D) to the initiating UE (Server UE-A) via unicast, groupcast or broadcast, depending on the specific scenario.

- The SLPP Provide Location Information message may be sent upon completion of the transmission and measurements or periodically or after the response time received at Step 6 expired and may also include an indication of whether other UEs (the initiating Server UE-A in this case) is requested to provide the location result (e.g., calculated range and/or position) back to this UE. SL-PRS transmission and SL-PRS measurement may be performed periodically, based on the configuration received in the SLPP Provide Assistance Data message.

- Each participating UE performs the range/position computations using the location results received at Step 8 and its own location results obtained at Step 7.

- If requested at Step 8, each UE may distribute its location results to the other UEs in this session by transmitting an SLPP Provide Location Information message.

- Single Target UE: The SLPP Provide Location Information message is transmitted by the initiating UE (Server UE-A) and the single Target UE-B.

- Multiple Target UEs: The SLPP Provide Location Information message is transmitted by the initiating UE (Server UE-A) and the multiple Target UEs (UE-B, UE-C, UE-D).

- The SLPP Provide Location Information message may be sent by initiating UE (Server UE-A) to the single Target UE (UE-B) or by the multiple Target UEs (UE-B, UE-C, UE-D) via unicast, groupcast or broadcast, depending on the specific scenario.

- The SLPP session may be terminated.

The preceding set of SLPP session call flows for UE-based and UE-assisted sidelink positioning and ranging clearly demonstrate that the SLPP messages and message sequence is the same regardless of whether an SLPP session is established to range/position a single target UE, or an SLPP session is established to range/position a set of multiple target UEs. There is no difference in SLPP signaling between SLPP sessions for a single target UE or for multiple target UEs. Moreover, enforcing an SLPP session to support only a single target UE is unnecessary and will require any ranging/positioning session involving more than one target UE to invoke multiple SLPP sessions. The resulting performance will be inefficient in use of over-the-air resources and extend the time required to obtain the range and/or position results.

Observation 8: The same SLPP messages and the same SLPP message sequence is used regardless of whether an SLPP session is established for ranging and/or positioning a single target UE, or an SLPP session is established for ranging and/or positioning a set of multiple target UEs.

Observation 9: Invoking multiple SLPP sessions to range and/or position a set of multiple target UEs consumes more over-the-air resources and requires a longer period of time to obtain range/position results for all the UEs than involving a single SLPP session including the set of multiple target UEs.

Sidelink ranging/positioning for a group of UEs (a set of multiple target UEs) is applicable to the use cases defined in the WID [7], including IIoT, commercial, Public Safety and V2X. In particular, the V2X use case can involve large groups of UEs which may engage in sidelink ranging/positioning. In their LS documents to RAN2 ([1], [2]), SA2 has further made clear the applicability of group positioning use cases, including that “it is desirable to allow the Ranging/SL Positioning control to support multiple target UEs”. As the analysis in this paper demonstrates the same SLPP messages and SLPP message sequence applies to a single target UE and to multiple target UEs, RAN2 incurs no additional burden in terms of specification development by supporting multiple target UEs. To satisfying the WID-defined use cases and comply with SA2’s recommendations, RAN2’s SLPP design should support multiple target UEs.

Proposal 1:	SLPP supports a single target UE or multiple target UEs within the same SLPP Ranging/SL Positioning session.

Proposal 2:	Share the reply LS proposed in the Annex with SA2.

# 3	Summary

Observation 1: UE-assisted Sidelink Positioning SLPP sessions use the same SLPP messages and SLPP message call flow regardless of whether the session is for a single Target UE or for multiple Target UEs.

Observation 2: If an SLPP session does not support multiple target UEs, an application-layer request to position multiple UEs will require as many SLPP sessions as there are target UEs. Each SLPP session requiring its own capability negotiation, assistance data exchange, SL-PRS configuration, SL-PRS transmission and location information exchange. The resulting positioning session will be inefficient in resource utilization and require a longer time to complete compared to a session supporting multiple target UEs.

Observation 3: UE-assisted Sidelink Ranging SLPP sessions for a single Target UE and UE-assisted Sidelink Ranging SLPP sessions for multiple Target UEs use the same SLPP messages and SLPP message call flow.

Observation 4: Regardless of whether an SLPP session is for a single target UE or for multiple target UEs, the same SLPP messages and SLPP message call flow are used for UE-assisted Sidelink Ranging SLPP sessions and UE-assisted Sidelink Positioning SLPP sessions.

Observation 5: If an SLPP session does not support multiple target UEs, an application-layer request to range multiple UEs will require as many SLPP sessions as there are target UEs, with each SLPP session including its own capability negotiation, assistance data exchange, SL-PRS configuration, SL-PRS transmission and location information exchange, resulting in a resource-inefficient, temporally elongated sidelink ranging session.

Observation 6: UE-based Sidelink Positioning SLPP sessions use the same SLPP messages and SLPP message call flow regardless of whether the session is for a single Target UE or for multiple Target UEs.

Observation 7: SLPP sidelink positioning sessions (UE-assisted or UE-based) use the same SLPP messages and SLPP message call flow regardless of whether the SLPP session is for a single target UE or multiple Target UEs.

Observation 8: The same SLPP messages and the same SLPP message sequence is used regardless of whether an SLPP session is established for ranging and/or positioning a single target UE, or an SLPP session is established for ranging and/or positioning a set of multiple target UEs.

Observation 9: Invoking multiple SLPP sessions to range and/or position a set of multiple target UEs consumes more over-the-air resources and requires a longer period of time to obtain range/position results for all the UEs than involving a single SLPP session including the set of multiple target UEs.

Proposal 1:	SLPP supports a single target UE or multiple target UEs within the same SLPP Ranging/SL Positioning session.

Proposal 2:	Share the reply LS proposed in the Annex with SA2.

# 4	References

- R2-2301938_S2-2303837, R2-2302448, "LS on support of multiple Target UEs", SA2.

- R2-2300079_S2-2301786, “LS on SL positioning groupcast and broadcast (R2-2213142/ S2-2300048)”

- R2-221084, “Report from session on positioning and sidelink relay”, RAN2 #119bis-e

- R2-2213004, “Report from session on positioning and sidelink relay”, RAN2 #120

- R2-2301904, “Report from session on positioning and sidelink relay”, RAN2 #121

- R2-2304204, “Report from session on positioning and sidelink relay”, RAN2 #121bis-e

- RP-230328, “Revised WID on Expanded and Improved NR Positioning”

# Annex A: Draft Reply LS on support of multiple Target UEs

3GPP TSG RAN WG2 Meeting #122	 R2-2xxxxxx

Incheon, KR, May 22 – 26, 2023

Title:	[draft] Reply LS on support of multiple Target UEs

Response to:	R2-2301938_S2-2303837

Release:	Rel-18

Work Items:	NR_pos_enh2, Ranging_SL

Source:	Qualcomm Incorporated (to be RAN2)

To:	SA2

CC:	RAN1

Contact Person:

Name:	Dan Vassilovski

E-mail Address:	dvassilo@qti.qualcomm.com

1. Overall Description:

RAN2 would like to thank SA2 for the LS (R2-2301938_S2-2303837) on support of multiple Target UEs.  SA2 poses the following question in their LS.

1. Overall Description: SA2 would like to inform RAN2 that during the development of the Ranging/SL Positioning support (as in TS 23.586), use cases had been identified where multiple Target UEs are involved in the same positioning session. In these cases, it is desirable to allow the Ranging/SL Positioning control to support multiple target UEs, e.g. with broadcast or groupcast support. SA2 understand that RAN2 is currently defining the Ranging/SL Positioning Protocol (the SLPP in RAN2’s context). Therefore, SA2 would like to ask RAN2 whether the SLPP would support multiple Target UEs in the same signalling session, and whether there is possibility of signalling the positioning results of multiple Target UEs in the same message. 2. Actions: To RAN2 WG: ACTION: SA2 would like to ask RAN2 to provide feedback on the support of multiple Target UEs in the same Ranging/SL Positioning session.

RAN2 has discussed the issue of multiple target UEs and has concluded that the Sidelink Positioning Protocol (SLPP) can support multiple target UEs within the same Ranging/SL Positioning session.

2. Actions:

To SA2:

ACTION: RAN2 kindly ask SA2 to take above information into consideration.

3. Date of Next RAN WG2 Meetings:

TSG-RAN2 Meeting #122	22-26 May 2023		Incheon, KR

TSG-RAN2 Meeting #123	21-25 August 2023		Toulouse, FR