3GPP TSG-RAN WG2 Meeting #122	R2-2306397

Incheon, Korea, May 22-26, 2023

Agenda Item:	7.14.3

Source:	CATT

Title:	Discussion on Rel-17 leftover issues for QoE

Document for:	Discussion and Decision

# 1	Introduction

In RAN2#121 meeting [1] and RAN3#119bis meeting [2], the R17 leftover issues were discussed and the following agreements were achieved.

RAN2#121 agreements: - 2: Introduce the QoS flows ID information in the RVQoE reporting from the UE. - RAN2 thinks (based on view from majority of companies) buffer level threshold-based triggering of RVQoE reporting by either APP layer or AS layer is feasible, but RAN2 prefers APP layer triggering. RAN2 will send an LS to SA4 to ask whether SA4 can make required specifications changes in Rel-18. RAN3#119bis agreements: Radio-related event triggers for RVQoE reporting is not supported in Rel-18. If a UE is configured with periodic RVQoE reporting that automatically starts at the beginning of the?application?session?or immediately upon reception of RVQoE configuration, it cannot be configured with a threshold-based trigger at the same time. Discuss whether threshold-based buffer level reporting starts: i) when buffer level is greater than a threshold or ii) when buffer level is below a threshold or iii) when buffer level is between two thresholds. RAN3 should discuss how the UE should send the RVQoE reports after the threshold is met, e.g., the following options: Option 1: Just once (after receiving this RVQoE report, gNB might reconfigure this threshold value to get additional reports) Option 2: Periodically based on a gNB configured reporting periodicity Option 3: A certain number of times based on gNB configured report amount

Based on the RAN2’s and RAN3’s progress, there are still several FFS needed to be discussed further in RAN2. In this contribution, we provide some analysis on the remaining issues.

# 2 Discussions

## 2.1 QoE value

For QoE value, it is used to estimate the quality of RAN visible QoE. It reflects QoE from the subjective angle and can reduce the signalling size. But according to the discussion in RAN3, there is still no consensus on how to define the RV QoE value. So the LS is sent to SA4 to ask whether the QoE value can be defined. According to the reply LS from SA4[3], SA4 think that in principle it is possible to define RN QoE value, but it is not feasible at this stage as the absence of necessary standards. So SA4 wishes to defer a specific answer to this question at this stage. So we think we can wait for the progress in RAN3 and SA4 and then we can discuss how to support the RV QoE value transfer from RRC view.

Proposal 1: Wait for the RAN3’s progress on RV QoE value and then RAN2 discusses how to support the value transfer from RRC view.

## 2.2 RV QoE trigger event

For RAN visible QoE trigger event, both threshold-based triggers and event-based triggers were discussed in RAN3. In RAN3#118 meeting [4], it was agreed that introduce buffer level as a threshold-based trigger for RV QoE reporting and do not introduce the threshold-based trigger for reporting playout delay for media startup.

Turn the WA to agreement: Introduce buffer level as a threshold-based trigger for RVQoE reporting. Do not introduce the threshold-based trigger for reporting playout delay for media startup.

In RAN2#121 meeting, an LS was sent to SA4 to ask whether APP layer can be responsible to trigger the buffer level threshold-based RV QoE reporting. According to the SA4’s reply[5], SA4 confirm application layer triggering of buffer level threshold-based RV QoE reporting can be supported in Rel-18 based on the corresponding QoE configuration received from the AS layer.

Observation 1: SA4 confirm that APP layer triggers the buffer level threshold-based RV QoE reporting.

As RV QoE report is mainly used by gNB to optimize the network immediately, the buffer level threshold can be configured by gNB based on the network’s need. In current spec, the RV QoE configuration is send to UE AS layer, AS layer should forward the related RV QoE parameters including measConfigAppLayerId, ran-VisiblePeriodicity, numberOfBufferLevelEntries, reportPlayoutDelayForMediaStartup to APP layer. Similar as other RV QoE parameters, the buffer level threshold should be included in ran-visibleParameters IE sent to UE AS layer and then AS layer forward the buffer level threshold to APP layer. Then APP layer can trigger the RV QoE reporting based on the buffer level threshold. After UE AS layer receives the RV QoE report from APP layer, UE sends the RV QoE report to gNB.

Proposal 2: The buffer level threshold should be included in ran-visibleParameters IE to be sent to UE AS layer and then AS layer forwards it to APP layer.

In last RAN3 meeting, there is still no consensus on the trigger condition of threshold based buffer level reporting. When UE detects the buffer level it greater than a threshold or when the buffer level is below a threshold or the buffer level is between two thresholds, UE APP layer will trigger the RV QoE reporting.

Discuss whether threshold-based buffer level reporting starts: i) when buffer level is greater than a threshold or ii) when buffer level is below a threshold or iii) when buffer level is between two thresholds.

So it is still not clear whether one buffer level threshold or two different buffer level thresholds will be introduced. We can wait for RAN3’s progress on the trigger condition of RV QoE reporting.

Proposal 3: Whether to introduce one or two buffer level thresholds can wait for RAN3’s progress.

## 2.3 The enhancement on RAN Overload scenario

When NG-RAN is overload, QoE measurement may be paused. On the contrary, if overload is relieved, QoE measurement can be resumed. But whether we will introduce the prioritization mechanism for RAN overload scenario is still discussed in both RAN2 and RAN3. Some agreements are achieved in RAN2 [6] as follow.

- 7: RAN2 to postpone the discussion of the QoE reporting enhancement for overload scenario to the next meeting (based on the progress of RAN3). - 8: FFS on whether to send the priority information 1) UE and gNB or 2) only to gNB - 9: To wait for RAN3 decision on granularity of priority.

According to RAN3’s LS [7], when assistance information for handling of QoE reporting upon overload is sent to the RAN, it is sent together with QoE measurement configuration. RAN3 need to further discuss what the assistance information is and an LS [8] is sent to SA5 to ask whether OAM can provide priority to gNB. But from RAN3’s perspective, there is no need to send assistance information to UE and RAN2 can provide feedback if necessary. So we can wait for SA5 and RAN3’s progress on the priority information.

Proposal 4: Whether the priority can be send to UE can wait for SA5 and RAN3’s progress.

If it is agreed that the assistance information can include priority, we think we can give feedback to RAN3 that the priority information is useful to UE. As in RAN2, there are still some companies that see the benefits if the priority information can be send to UE. So we think we can discuss the necessity of sending priority information to UE in RAN2. If we agree that UE need the priority information from gNB, we can send the LS to RAN3.

We also think it is beneficial for UE to get priority information from gNB. As in current specification, multiple QMC jobs may be inactivated for a UE when NG-RAN overload. If the NG-RAN wants to pause some QMC jobs, it can only randomly select some QMC jobs without any assistant information. It is not reasonable for NG-RAN to pause important QMC while keep less important QMC jobs. Besides, when the NG-RAN is overload, the UE AS layer is responsible to store the RV QoE report. It is possible that the RAN overload is not resolved until the memory size is full. In this case, the UE may discard the stored QoE report which is important for RAN. The priority information sent to UE can help UE discard the QoE report with less importance.

Besides, when overload is relieved, there also may be many UEs which store QoE measurement reports during the overload period. If many UEs send QoE measurement reports to network at the same time, it may lead to overload again. So it is beneficial to send the QoE report with higher priority first to avoid the overload occurs again.

Proposal 5: If the priority information can be provided by OAM, it is beneficial to send the priority information to UE.

# 3 Conclusions

According to the above discussion, the following observation and proposals are made:

Observation 1: SA4 confirm that APP layer triggers the buffer level threshold-based RV QoE reporting.

Proposal 1: Wait for the RAN3’s progress on RV QoE value and then RAN2 discusses how to support the value transfer from RRC view.

Proposal 2: The buffer level threshold should be included in ran-visibleParameters IE to be sent to UE AS layer and then AS layer forward it to APP layer.

Proposal 3: Whether introduce one or two buffer level thresholds can wait for RAN3’s progress.

Proposal 4: Whether the priority can be send to UE can wait for SA5 and RAN3’s progress.

Proposal 5: If the priority information can be provided by OAM, it is beneficial to send the priority information to UE.

# 4 References

- Report of 3GPP TSG RAN2 meeting #121

- Draft Report of 3GPP TSG RAN3 meeting #119bis-e

- R2-2300085  Reply LS to RAN3 on RAN visible QoE value

- Draft Report of 3GPP TSG RAN3 meeting #118-e

- R2-2304658 Reply LS on buffer level threshold-based RV QoE reporting

- R2-2211101  Report of 3GPP TSG RAN WG2 meeting #119bis-e, Online

- R2-2302425 LS on assistance information for handling of QoE reporting upon RAN overload

- R2-2304625 LS on the feasibility of introducing assistance information for handling of QoE reporting during RAN overload