3GPP TSG RAN WG2 #122			R2-2306403

Meeting, May 22nd – 26th, 2023

Agenda Item:	7.3.2

Source:	BT Plc, KDDI

Title:	Discussion on cell DTX/DRX mechanisms - configuration and behaviour

Document for:	Discussion/Decision

# Introduction

During RAN2#121-bis-e [1], the following agreements has been made for cell DTX/DRX mechanism.

Agreements

- A periodic cell DTX/DRX configuration is explicitly signalled to the UEs.

- A periodic cell DTX/DRX pattern is configured by UE specific RRC signalling.

- The Cell DTX/DRX configuration contains at least: periodicity, start slot/offset, on duration.

- As a baseline Cell DTX/DRX is activated/deactivated implicitly by RRC signalling, i.e. activated immediately once configured by RRC and deactivated once the RRC configuration is released.

- From RAN2 point of view, majority companies see a benefit with L1 signalling for Cell DTX/DRX activation/deactivation, send a LS to RAN1 (email 308) with our preference and ask about feasibility and design details.   Ask about feasibility and reliability of using L1 signaling.  Clarify that the question is about activation/deactivation copy the agreement from last meeting that we are focusing on single configuration.  Extract a few key benefits of dynamic signaling from email discussion and online discussions

- As baseline, UE doesn’t monitor SPS occasions during Cell DTX non-active period. As baseline, gNB is assumed to be not transmitting PDSCH to that UE on such SPS occasions during the Cell DTX non-active period

- As baseline, UE does not transmit on CG occasions during Cell DRX non-active periods

- As baseline, UE does not transmit SR occasions overlapping with Cell DRX non-active periods, e.g. SR transmissions are dropped during the non-active period

FFS: whether we will allow to configure the UE per SR configuration with whether SR can be transmitted during Cell DRX non-active period to support high priority traffic

- (for the SRs that will be dropped) If SR is not to be transmitted on an PUCCH occasion during Cell DRX non-active time, the UE keep the SR pending, i.e., the UE delays the SR transmission till the Cell DRX active period without triggering RACH.  For the FFS case there may be some exceptions.

- The understanding for the gNB scheduling behaviour for new transmissions during Cell DTX non-active period is that the gNB does not schedule UE-specific dynamic grants/assignments, even if the UE is in C-DRX Active Time.   UE doesn’t monitor PDCCH for dynamic grants/assignments for new transmissions during Cell DTX non-active period, even if the UE is in C-DRX Active time.   FFS how to deal with any exceptions (e.g. SR if agreed and RACH).

FFS how to deal with retransmissions

In this Tdoc, we would like to focus the discussion on agreements 4, 8, 9 and 10.

# Discussion

## Cell DTX/DRX activated/deactivated implicitly by RRC signalling

During RAN2#121-bis-e meeting, it has been discussed how the UE will activate and deactivate the cell DTX/DRX pattern configured by RRC signalling. As a result, the following baseline has been agreed “As a baseline Cell DTX/DRX is activated/deactivated implicitly by RRC signalling, i.e. activated immediately once configured by RRC and deactivated once the RRC configuration is released”.

- Current baseline indicates that as soon as the UE receives the cell DTX/DRX configuration, it will be activated. To deactivate the configuration, network will send a RRC Reconfiguration message without cell DTX/DRX configuration

In general, there are two options to activate the cell DTX/DRX on the UE: before cell starts its DTX/DRX mode or after it is really activated by the network. With current baseline, if the UE receives the RRC Reconfiguration message with the cell DTX/DRX pattern before it is truly activated on the cell, site capacity will be negatively impacted since UE will apply a cell discontinuous pattern when it is not activated on the network yet.

Therefore, to avoid such impact with agreement #4, network is limited to send the cell DTX/DRX activation indication after it is effectively activated.

- Agreement #4 limits the network options to activate the cell DTX/DRX mode and force it to send the RRC Reconfiguration message after the cell DTX/DRX configuration is effectively activated on the cell

With current implicit configuration baseline, network can only send its DTX/DRX configuration during the cell DTX active period and receive the acknowledgment during its cell DRX active period. In case the reconfiguration message with the cell DTX/DRX configuration fails, network cannot ensure the network – UE alignment. For TDD bands, this problem can be magnified due to limited UL subframes. Consequently, network KPIs may be negatively impacted, and UE may waste its energy trying to receive and send during the cell DTX and cell DRX non-active periods respectively.

Apart from that, a cell that want to activate its cell DTX/DRX NES solution, cannot send in advance the configuration to UEs. As reflected in agreement #5, majority of companies envision a benefit with L1 signalling for cell DTX/DRX activation/deactivation. One of the benefits is that cell can configure UEs in advance and the activation is done via L1 signalling.

- L1 signalling will benefit from an explicit cell DTX/DRX activation/deactivation framework

- RAN2 to modify current implicit cell DTX/DRX activation/deactivation baseline to explicit configuration

L1 (if feasible) or RRC signalling can be used to activate/deactivate cell DTX/DRX configuration on UEs if configuration is explicitly configured.

- If explicit cell DTX/DRX configuration is agreed, RAN2 to discuss how cell DTX/DRX explicit activation/deactivation is performed to avoid/mitigate signalling failures and call drops

## SR transmitted during Cell DRX non-active period

Agreement #8 has an FFS “whether we will allow to configure the UE per SR configuration with whether SR can be transmitted during Cell DRX non-active period to support high priority traffic”.

It is important to ensure that critical traffic, e.g., emergency calls, can be delivered on a cell even it has activated its NES DTX/DRX mode. There are two main ways to achieve such behaviour:

- Case a: allow UE to transmit SR during cell DRX non-active periods if it has critical traffic

- Case b: network temporarily deactivates cell DTX/DRX configuration on that specific UE if its critical traffic cannot be served with cell DTX/DRX activated

|  | Network impact | UE impact |
| --- | --- | --- |
| Case a | - NW needs to know in advance that SR is for critical traffic - New timers to adjust the cell DTX/DRX pattern and critical traffic characteristics | - Additional NW configuration for critical traffic |
| Case b | - Vendor specific solutions required to save power | - None |

From a standardization point of view, case a requires defining critical traffic for SR exception transmission, e.g. emergency calls, MCPTT or mission critical video. From testing and validation perspective, case a requires an extra effort to ensure UE only sends SR for specified critical traffic when several traffics are configured.

- Case a, support of SR transmitted during Cell DRX non-active period requires to explicitly define all critical traffics

Even it is feasible to define critical traffics, case a is an unrealistic approach to capture the critical traffic definition for SR exception on standards because each operator may want to define its own critical traffic, gamming, VoNR, XR, etc.

Using case a, it is not possible to ensure a common cell DTX/DRX pattern to all UEs can be used as it was agreed in RAN2#121 “Pattern configuration for cell DRX/DTX is common for Rel-18 UEs in the cell”. It is likelihood that critical traffic requires specific cell DTX/DRX patterns, so to support a single UE critical traffic requires that all others UEs in the cell must be reconfigured with the new cell DTX/DRX pattern.

- A consequence of case a is that it may force to reconfigure all UEs in the cell at the cost to increase the network energy consumption

Considering case b has no standardization impact, it seems straightforward to deactivate the cell DTX/DRX configuration on a specific UE if critical traffic is identified.

- RAN2 to agree that for traffic which SR needs to be transmitted during cell DRX non-active period, network can individually deactivate the UE cell DTX/DRX configuration.

As we mention before, each operator may have a different understanding of critical traffic. Therefore, each operator has to have the capability to configured it based on its requirements.

- It is up to network configuration which traffic is considered critical to initiate the UE specific cell DTX/DRX deactivation

Agreement #9 left for FFS the exceptions if SR is drop due to cell DRX is in non-active period. Our understanding is that all non-critical traffic will be drop therefore, the exceptions referred on the FFS are limited to critical traffic. Based on Proposal 3, there is no need to consider any exception, so we propose to confirm agreement #9 without FFS.

- RAN2 to confirm that there are no exceptions to “if SR is not to be transmitted on an PUCCH occasion during Cell DRX non-active time, the UE keep the SR pending, i.e., the UE delays the SR transmission till the Cell DRX active period without triggering RACH”

Note that even the agreement #8 captures high priority traffic, we believe the common understanding is they refer to critical traffic. High priority traffic can be understood as GBR traffic.

## Cell DTX/DRX exceptions

Agreement #10 has two different pending studies: FFS how to deal with any exceptions (e.g. SR if agreed and RACH) and FFS how to deal with retransmissions.

As we have stated above, we do not envision the need for SR exceptions including no PUCCH resources for SR and SR failure.

There are different occasions where UE triggers a RACH being in connected mode: handover, RLF, on demand SI and beam recovery.

Handovers should be covered by agreement done in RAN2#121 [2], “Rel-18 NES capable CONNECTED UE(s) can perform RACH and receive SIBs in non-active duration of cell DTX and/or DRX (i.e., same behavior for cell DTX and cell DRX).  No further enhancements for CBRA and CFRA will be pursued”.

- No further enhancements for CBRA and CFRA will be pursued by RAN2

For RLF, on demand SI and beam recovery, UE can wait to initiate the CFRA until the cell DRX active period.

- RAN2 to agree as baseline, there are no RACH exceptions to cell DRX transmission during non-active periods for RLF, on demand SI and beam recovery

# Conclusion

During the TDoc, we have made the following observations

Observation 1	Current baseline indicates that as soon as the UE receives the cell DTX/DRX configuration, it will be activated. To deactivate the configuration, network will send a RRC Reconfiguration message without cell DTX/DRX configuration

Observation 2	Agreement #4 limits the network options to activate the cell DTX/DRX mode and force it to send the RRC Reconfiguration message after the cell DTX/DRX configuration is effectively activated on the cell

Observation 3	L1 signalling will benefit from an explicit cell DTX/DRX activation/deactivation framework

Observation 4	Case a, support of SR transmitted during Cell DRX non-active period requires to explicitly define all critical traffics

Observation 5	A consequence of case a is that it may force to reconfigure all UEs in the cell at the cost to increase the network energy consumption

Observation 6	No further enhancements for CBRA and CFRA will be pursued by RAN2

We propose the following:

Proposal 1	RAN2 to modify current implicit cell DTX/DRX activation/deactivation baseline to explicit configuration

Proposal 2	If explicit cell DTX/DRX configuration is agreed, RAN2 to discuss how cell DTX/DRX explicit activation/deactivation is performed to avoid/mitigate signalling failures and call drops

Proposal 3	RAN2 to agree that for traffic which SR needs to be transmitted during cell DRX non-active period, network can individually deactivate the UE cell DTX/DRX configuration.

Proposal 4	It is up to network configuration which traffic is considered critical to initiate the UE specific cell DTX/DRX deactivation

Proposal 5	RAN2 to confirm that there are no exceptions to “if SR is not to be transmitted on an PUCCH occasion during Cell DRX non-active time, the UE keep the SR pending, i.e., the UE delays the SR transmission till the Cell DRX active period without triggering RACH”

Proposal 6	RAN2 to agree as baseline, there are no RACH exceptions to cell DRX transmission during non-active periods for RLF, on demand SI and beam recovery

# References

- RAN2#121-bis-e Chair notes, R2-2304203

- RAN2#121 Chair notes, R2-2302401