3GPP TSG RAN WG2 #122			R2-2306406

Meeting, May 22nd – 26th, 2023

Agenda Item:	7.3.4

Source:	BT Plc

Title:	Identify NES capable UEs by network

Document for:	Discussion/Decision

# Introduction

From RAN2#121 meeting, it was agreed [1]

Agreements - RAN2 confirms that non-NES UEs can access to NES cells if NES solution is backwards compatible […] 1. There will be no impact to RACH, paging, and SIBs in idle/inactive for both gNB and Rel-18 and legacy UEs 2. Rel-18 NES capable CONNECTED UE(s) can perform RACH and receive SIBs in non-active duration of cell DTX and/or DRX (i.e., same behavior for cell DTX and cell DRX). No further enhancements for CBRA and CFRA will be pursued. 3. Pattern configuration for cell DRX/DTX is common for Rel-18 UEs in the cell. FFS whether we have DTX UE specific inactivity timer . FFS on configuration signaling and stage 3. 4. Confirm study item agreement that we can have separate DTX and DRX configuration. We will focus on designing DTX/DRX for at least single configuration. FFS whether multiple configuration of cell DTX or DRX will be supported.

In RAN2#121bis-e [2], it was agreed

Agreements - A periodic cell DTX/DRX configuration is explicitly signalled to the UEs. - A periodic cell DTX/DRX pattern is configured by UE specific RRC signalling. - The Cell DTX/DRX configuration contains at least: periodicity, start slot/offset, on duration. - As a baseline Cell DTX/DRX is activated/deactivated implicitly by RRC signalling, i.e. activated immediately once configured by RRC and deactivated once the RRC configuration is released. - From RAN2 point of view, majority companies see a benefit with L1 signalling for Cell DTX/DRX activation/deactivation, send a LS to RAN1 (email 308) with our preference and ask about feasibility and design details. Ask about feasibility and reliability of using L1 signaling. Clarify that the question is about activation/deactivation copy the agreement from last meeting that we are focusing on single configuration. Extract a few key benefits of dynamic signaling from email discussion and online discussions - As baseline, UE doesn’t monitor SPS occasions during Cell DTX non-active period. As baseline, gNB is assumed to be not transmitting PDSCH to that UE on such SPS occasions during the Cell DTX non-active period - As baseline, UE does not transmit on CG occasions during Cell DRX non-active periods - As baseline, UE does not transmit SR occasions overlapping with Cell DRX non-active periods, e.g. SR transmissions are dropped during the non-active period FFS: whether we will allow to configure the UE per SR configuration with whether SR can be transmitted during Cell DRX non-active period to to support high priority traffic - (for the SRs that will be dropped) If SR is not to be transmitted on an PUCCH occasion during Cell DRX non-active time, the UE keep the SR pending, i.e., the UE delays the SR transmission till the Cell DRX active period without triggering RACH. For the FFS case there may be some exceptions. - The understanding for the gNB scheduling behaviour for new transmissions during Cell DTX non-active period is that the gNB does not schedule UE-specific dynamic grants/assignments, even if the UE is in C-DRX Active Time. UE doesn’t monitor PDCCH for dynamic grants/assignments for new transmissions during Cell DTX non-active period, even if the UE is in C-DRX Active time. FFS how to deal with any exceptions (e.g. SR if agreed and RACH). FFS how to deal with retransmissions

During RAN1##112bis-e [3], the following agreements were made

Agreement For a CSI report config with L sub-configuration(s), support a framework that enables a UE to report N CSI(s) in one reporting instance where the N CSI(s) are associated with N sub-configuration(s) from L (where 1≤N≤L) and each CSI corresponds to one sub-configuration. - For discussion purpose, N=1 refers to single-CSI while N>1 refers to multi-CSI. - For Semi-persistent/Aperiodic CSI reporting, support gNB trigger/indicate/activate report of N≤L CSIs where N>=1 - The maximum value of N and L are subject to UE capability - Further study how to address/minimize additional UE complexity The following bullet was not agreed due to objection from Apple and vivo - For Periodic CSI reporting, at least the case of N=L is supported where N>=1 Agreement - For CSI feedback with CSI overhead/report payload reduction, further study whether/how to report a common value and/or a differential and/or joint coded value across same CSI quantity of different sub-configurations/adaptation patterns, at least for the following - CRI - RI - PMI - CQI - FFS: L1-RSRP - Other (new) report quantity, if any - Further study whether/how it is feasible/possible for the UE to skip the evaluations of some sub-configurations/adaptation patterns to reduce the burden at the UE Agreement For R18 NES, only legacy port configuration values (N1, N2) or (Ng, N1, N2) are supported. - FFS: Whether/what restriction for A1-1-revised and A-1-2-revised w.r.t number of ports Agreement For Semi-persistent/Aperiodic CSI reporting with 1≤N≤L, study what enhancements to the current DCI and MAC-CE mechanisms are needed for gNB triggering/indication/activation of the N CSI(s) in a reporting instance, where the N CSI(s) are associated with N sub-configuration(s) from L in a report config. Agreement From RAN1 point of view, Rel-18 UE supporting cell DTX does not expect to receive and/or process the following signals/channels from the gNB, during non-active periods of cell DTX. The list of signals/channels may be updated based on RAN2/RAN4 input and other signals/channels are not precluded from further discussions. - Periodic/Semi-persistent CSI-RS configured in CSI report configuration in CSI-ReportConfig with reportQuantity including RI (for CSI reporting) - FFS: - PDCCH in USS - UE behavior for retransmission - if some specific RNTI scrambled PDCCH in USS will be excluded from cell DTX operation - PDCCH in Type-3 CSS - UE behavior for retransmission - if some specific RNTI scrambled PDCCH in Type-3 CSS will be excluded from cell DTX operation - PRS - CSI-RS configured by measObjectNR (for RRM) - CSI-RS associated with RadioLinkMonitoringConfig and BeamFailureDectection (for RLM and BFD) - Periodic CSI-RS configured with trs-Info ‘true’ (for tracking) - Periodic/Semi-persistent CSI-RS (for BM) - FFS on how to differentiate (if needed) with other CSI-RS used for CSI reports for BM - FFS: Whether the same or different UE behavior is applicable with or without C-DRX - FFS: Whether the list of impacted signals/channels can be configurable - FFS: Whether there will be exception case(s) for UE receiving and/or processing listed signals/channels during non-active periods of DTX - FFS: RAN1 to consider impact on system if the channels/signals are not transmitted during non-active period Agreement Study L1 signalling for enhancing cell DTX/DRX including activation/deactivation for a single configuration which will have the following characteristics: - PDCCH based signaling - FFS: Whether enhancing legacy DCI or introducing new DCI - FFS: DCI content - FFS: Whether L1 signaling is UE specific DCI or group common DCI - FFS: Timer or validity duration based activation/deactivation of cell DTX/DRX - FFS: whether to specify a reference time for activation/deactivation of cell DTX/DRX - FFS: If multiple Cell DTX/DRX patterns are to be supported - FFS on detailed UE behavior upon reception of L1 signaling at least including application delay - FFS how to guarantee reliability of the L1 signaling - FFS whether the L1 signal can be monitored in non-active periods. Agreement From RAN1 point of view, Rel-18 UE supporting cell DRX is not expected to transmit the following signals/channels to the gNB during non-active periods of cell DRX. The list of signals/channels may be updated based on RAN2/RAN4 input and other signals/channels are not precluded from further discussions. - Periodic/Semi-persistent CSI report - Periodic/Semi-persistent SRS - FFS: SRS for positioning - FFS: - HARQ feedback for SPS PDSCH - FFS whether there will be exception case(s) for UE transmitting listed signals/channels during non-active periods of DRX - FFS Whether the listed signals/channels can be configurable by gNB - FFS: Whether the same or different UE behavior is applicable with or without C-DRX - FFS: RAN1 to consider impact on system if the channels/signals are not transmitted during non-active period Agreement For PDDCH monitoring, further work on Rel-18 NES in RAN1 is to follow the RAN2 agreement below: 10. The understanding for the gNB scheduling behaviour for new transmissions during Cell DTX non-active period is that the gNB does not schedule UE-specific dynamic grants/assignments, even if the UE is in C-DRX Active Time. UE doesn’t monitor PDCCH for dynamic grants/assignments for new transmissions during Cell DTX non-active period, even if the UE is in C-DRX Active time. FFS how to deal with any exceptions (e.g. SR if agreed and RACH).

With that agreement in place, this paper shows the importance for the network to identify NES capable UEs.

# Discussion

## Identification of NES capable UEs

Technical Report TR 38.864 [4] captures the impact on non-NES capable UEs for each of the technique considered during the study phase. For Rel-18 NES techniques that may support legacy and non-NES UEs, some of them may have an impact on UE performance pending of final specification.

Currently, there is no formal definition of a NES cell. For a clear understanding, in this paper we assume NES cell definition proposed in [5] during RAN2#121, “a NES cell refers to a cell that has activated at least one of the NES technique(s)”.

- In this paper, a NES cell refers to a cell that has activated at least one NES technique

The ambiguity of the NES cell definition needs to be solve.

- RAN2 to agree that a NES cell refers to a cell that has activated at least one NES technique

From [1], it is confirmed that non-NES capable UEs, including legacy UEs, can access to NES cell if the NES solution supports non-NES UEs by no specs impact like adaptation of spatial elements or by network implementation like cell DTX/DRX. For cell DTX/DRX solution, agreements #6, #7, #8 and #9 captured in [2] confirm that even cell DTX/DRX may support non-NES capable UEs, their power consumption will by higher than NES UEs.

- Non-NES capable UEs power consumption will be higher than NES UEs on cells where a NES solutions are activated

Agreements made by RAN1 [3], defines new CSI-RS reports. For a coordination between network and UE, it is necessary that network identifies NES UEs. For cell DTX/DRX, RAN1 has also agreed that UE will not receive a set of signals channels but that cannot be achieved unless network knows the NES UE capabilities.

- RAN1 agreements reach in [3] requires that UE informs the network about its NES capabilities

Figure 1 captures a scenarios where a single site has multiple cells. This is the typical deployment in train stations and crowded locations. Usually, operators prioritize one frequency over others for UEs to camp and the load is balanced one UEs moves to connected mode. In Figure 1, prioritized cell for camping is cell_2.

Figure 1 site engineered for capacity

In above scenario, it will be expected that all cells deactivate their NES capabilities during peak hours to cope with high volumes of traffic but during off-peak hours, NES techniques will be activated in as many cells as possible.

Figure 2 captures a snapshot of the above site during an off-peak period of time. In that specific snapshot, two cells are switched off, cell_5 and cell_4; two cells have their NES technique(s) activated, cell_3 and cell_2; and one cell does not activate its NES capabilities, cell_1. Based on the definition provided in observation 1, cell_1 is engineered on purpose to serve all non-NES capable UEs in the coverage area of the site.

Figure 2 multicarrier site engineered for capacity

In RAN2#121 meeting [1], as a concecuence of a note for cell DTX/DRX solution “Note: The impact to IDLE/INACTIVE UEs due to the above enhancement should be avoided” [6], it was agreed that “There will be no impact to RACH, paging, and SIBs in idle/inactive for both gNB and Rel-18 and legacy UEs”. Consequencly, cell_2, that it is engineered as the cell for UE camping, should support the attachements of NES UEs and non-NES capable UEs from idle mode. That means, cell_2 will be used as camping cell for NES UEs and non-NES capable UEs.

To avoid a severe impacts on non-NES capable UEs in cell_2, it will immediately handover these devices to cell_1. But in order to do that, network needs to know which NES solutions are supported by UEs.

- It is beneficial for initial attachment that cell knows the NES solutions that the UE is capable of

From agreements #1 and #2 of [2], UE will be specifically signalled with cell DTX/DRX patterns but unless UE reports its NES capabilities, e.g., cell DTX/DRX, NES configuration signalling will be send to every single UE.

- Unless network knows if UE supports NES cell DTX/DRX, the cell DTX/DRX configuration pattern needs to be sent to all UEs even they do not support the feature

For mobility, it is also beneficial that network knows UE NES capabilities. In an intra-frequency handover target cell may apply a NES solution, e.g., cell DTX/DRX, while the source cell does not. If network does not know UE NES capabilities, the handover will follow legacy procedure and the non-NES capable UE may end on a NES cell that does not support non-NES capable UEs. From study item phase, it was concluded that cell DTX/DRX may support non-NES capable UEs via vendor implementation but it is not mandated and it cannot.

- RAN2 to agree that UE reports to network its NES capabilities, i.e., cell DTX/DRX UE capabilities, during UE capabilities exchange process

## Cell barring for non-NES capable UEs

In section 2.1Figure 2, it is presented a real use case where a NES capable cell has to handover a non-NES capable UE immediately after its initial attachement. It is important to notice that even the handover is performed immediately, the process by itself is ineffient for both, the network and the UE. To avoid it, an option is that NES capable cells have the functionality to bar non-NES UEs.

As part of Rel-17 RedCap WI, it was defined an additional barring mechanism that only applies to RedCap UEs. Similar approach can be used now for non-NES capable UEs.

- Similar to Rel-17 RedCap, in NES WI it is feasible to create an additional mechanism to dynamically bar non-NES capable UEs and accept NES UEs

From UE side, an additional barring mechanism adds an extra complexity to NES capable UEs that will require to check if new barring information is present as described in [7].

- An additional barring mechanism on NES cells force NES UEs to do an extra validation to access the cell

An additional barring mechanism will enhance the case presented above where a non-NES capable UE first attaches to a NES cell and it is handovered to a different cell immediately. But it adds an extra complexity to NES UEs that will be forced to check additional barring conditions.

- RAN2 to discuss if a new barring mechanism to bar non-NES capable UEs is specified in Rel-18 based on pros and cons

# Conclusion

During the TDoc, we have made the following observations

Observation 1	In this paper, a NES cell refers to a cell that has activated at least one NES technique

Observation 2	Non-NES capable UEs power consumption will be higher than NES UEs on cells where a NES solutions are activated

Observation 3	RAN1 agreements reach in [3] requires that UE informs the network about its NES capabilities

Observation 4	It is beneficial for initial attachment that cell knows the NES solutions that the UE is capable of

Observation 5	Unless network knows if UE supports NES cell DTX/DRX, the cell DTX/DRX configuration pattern needs to be sent to all UEs even they do not support the feature

Observation 6	Similar to Rel-17 RedCap, in NES WI it is feasible to create an additional mechanism to dynamically bar non-NES capable UEs and accept NES UEs

Observation 7	An additional barring mechanism on NES cells force NES UEs to do an extra validation to access the cell

We propose the following:

Proposal 1	RAN2 to agree that a NES cell refers to a cell that has activated at least one NES technique

Proposal 2	RAN2 to agree that UE reports to network its NES capabilities, i.e., cell DTX/DRX UE capabilities, during UE capabilities exchange process

Proposal 3	RAN2 to discuss if a new barring mechanism to bar non-NES capable UEs is specified in Rel-18 based on pros and cons

# References

- R2-2302401, Chair notes RAN2#121bis-e

- R2-2304203, Chair notes RAN2#121

- Draft_Minutes_report_RAN1#112b-e_v010

- 3GPP TR 38.864 V18.1.0

- R2-2300231, Discussion on cell selection/reselection for NES	Huawei, HiSilicon

- RP-223540, Network Energy Saving WI definition

- R2-2302915, Barring legacy UEs for NES Cells, Qualcomm Incorporated, T-Mobile US