3GPP TSG-RAN WG2 Meeting#122		       					      R2-2306415

Incheon, KR, May 22nd – 26th , 2023

Source: 			ZTE Corporation, Sanechips

Title: 	Further Discussion On the Purpose Driven Data Collection

Agenda item:		7.16.2.2

Document for: 	Discussion and Decision

- Introduction

In RAN2#121BIS meeting, RAN2 has determined to add extra 3 columns on top of the endorsed data collection table:

Extend the previously endorsed table with 3 columns: Inference, Monitoring and Training, and explain in free text the applicability of the data collection method to the LCM purpose and the use case(s).

Not only the LCM purpose, but also the use case shall be taken into account if there is any progress is made in RAN1 according to the companies’ view in [1]. Fortunately, in RAN1#112bis meeting, there are some progresses on data collection of AI based BM has been made, according to which, RAN2 evaluation of the candidate framework for the AI based beam management can be facilitated.

In this contribution, our intention is to share our views on the LCM purpose and/or use case driven data collection in light of the conclusion achieved in the latest RAN1 and RAN2 meeting, and attempt to fill the information into the extra three columns of the endorsed table (e.g. inference, monitoring, and training ).

- Discussion

Overview

In RAN2#121bis emeeting, it is determined that the applicability of the data collection framework shall be evaluated per LCM purpose for each use case. In our understanding, to evaluate the applicability of the data collection framework, it is nothing but delay and reporting message capacity evaluation. For the delay evaluation, it means the how much delay will be caused for one LCM purpose in one use case by applying the data collection framework, whether the delay can fulfil the requirement of LCM purpose for each use case.. As for the data size factor, it means whether the capacity of the reporting message is able to accommodate the collected data of the LCM purpose for each use case.

To evaluate the applicability of the data collection framework to the LCM purpose for each use case, Intuitively, there are two main factors shall be evaluated:

- Delay evaluation for each LCM purpose in each use case.

- Collected data size for each LCM purpose in each use case.

In our companion paper [2], we express our idea that the potential delay is subject to the logical entity/functionality the LCM functionality mapping to, that is, the collected data should be forwarded from the endpoint of the data collection framework to the logical entity/functionality at NW side.

The delay caused by forwarding the data from the endpoint of the data collection framework to the logical entity the LCM functionality reside in shall be taken into account in the delay evaluation for each LCM purpose in each use case..

Regarding the second factor - the collected data size for each LCM purpose in each use case. It is mainly to evaluate whether the reporting message of the data collection framework can accommodate the collected data for each LCM purpose in each use case. However, so far as now, RAN1 only obtain some initial progress on the data collection of AI based beam management, please see below:

Agreement Regarding data collection for NW-side AI/ML model, study the following options (including the combination of options) for the contents of collected data, - Opt.1: M1 L1-RSRPs (corresponding to M1 beams) with the indication of beams (beam pairs) based on the measurement corresponding to a beam set, where M1 can be larger than 4, if applicable - FFS: the range of M1 - Opt.2: M2 L1-RSRPs (corresponding to M2 beams) based on the measurement corresponding to a beam set, where M2 can be larger than 4, if applicable - FFS: the range of M2 - Opt.3: M3 beam (beam pair) indices based on the measurement corresponding to a beam set, where M3 can be larger than 4, if applicable - FFS: the range of M3 - FFS: How to select the M1/M2/M3 beam(s) or beam pair(s) - Note: Overhead, UE complexity and power consumption should be considered for the above options

However, it is still not stable yet. Hence, the second factor evaluation shall be waiting for more information from RAN1.

From RAN 2 perspective, the delay evaluation can be discussed if the functionality mapping within the NW is settled. The collected data size request for each LCM functionality shall be waiting for more input from RAN1.

Consider RAN2 have no experience to evaluate the delay caused by data forwarding through different interface, in RAN2 paper, we simply to quantify the delay caused data forwarding via multiple interfaces within the NW side with the number of the involved interfaces.

Model inference

In our companion paper [2], the logical entity/function the model inference resides in is proposed as below:

- For AI based beam management, CSI feedback enhancement, the gNB or DU in gNB split case is proposed to place the functionality of model inference.

- For AI based Positioning with LMF sided model, the LMF is proposed to place the functionality of the model inference.

For model inference functionality in the use case of AI based positioning with LMF sided model, the delay caused by data forwarding from the endpoint of each data collection framework to the logical entity/functionality is shown as:

- 1: Logged MDT: The interface between LMF and TCE

- 2: Immediate MDT: The interface between LMF and TCE

- 3: L3 measurements: NG-C interface, NL-1 interface

- 4: L1 measurements: F1 interface in gNB split case, NG-C interface, NL-1 interface

- 5: UAI: NG-C interface, NL-1 interface

- 6: Early measurements : NG-C interface, NL-1 interface

- 7: LPP: No interface involved

For AI based positioning with LMF sided model, the following contents shall be included for model inference functionality corresponding to each data collection framework.

  - Logged MDT: The interface between LMF and TCE

  - Immediate MDT: The interface between LMF and TCE

  - L3 measurements: NG-C interface, NL-1 interface

  - L1 measurements: F1 interface in gNB split case, NG-C interface, NL-1 interface

  - UAI: NG-C interface, NL-1 interface

  - Early measurements : NG-C interface, NL-1 interface

  - LPP: No interface involved

For model inference functionality in the use case of AI based beam management , CSI, so the delay caused by data forwarding from the endpoint of each data collection framework to the logical entity/functionality is shown as:

- 1: Logged MDT: the private interface between OAM and gNB/DU.

- 2: Immediate MDT: the private interface between OAM and gNB/DU.

- 3: L3 measurements: no additional interface in the gNB non-split case, F1 interface in gNB split case.

- 4: L1 measurements: no additional interface is involved in the both gNB split case and non-split case.

- 5: UAI: no additional interface is involved in the case of the gNB non-split case, F1 interface in the case of the gNB split case.

- 6: Early measurements : no additional interface is involved in gNB non-split case, F1 interface in gNB split case.

- 7: LPP: Not applicable.

For AI based beam management and AI based CSI enhancement, the following contents for model inference functionality shall be included corresponding to each data collection framework:

- Logged MDT: at least one additional private interface is involved

- Immediate MDT: at least one additional private interface is involved

- L3 measurement: no additional interface is involved in the gNB non-split case, F1 interface is involved in gNB split case.

- L1 measurement: no additional interface is involved for both gNB non-split and gNB split case.

- UAI: no additional interface is involved in the gNB non-split case, F1 interface is involved in gNB split case.

- Early measurements: no additional interface is involved in the gNB non-split case, F1 interface is involved in gNB split case.

- LPP: Not applicable.

Model Training/Update

In our companion paper [2], the logical entity/functions for model training are proposed as below:

- For AI based beam management, CSI feedback enhancement, the gNB in gNB non-split case, CU in gNB split case, or OAM is proposed to place the functionality of the model training, specifically, the OAM is dedicatedly for off-line training.

- For AI based Positioning with LMF sided model, the LMF or NWDAF is proposed to place the functionality of the model training.

For model offline training functionality in the use case of AI based positioning with LMF sided model, the delay caused by data forwarding from the endpoint of each data collection framework to the logical entity/functionality is shown as:

- 1: Logged MDT: the interface between TCE and NWDAF, or the interface between TCE and LMF

- 2: Immediate MDT: the interface between TCE and NWDAF, or the interface between TCE and LMF

- 3: L3 measurements: NG-C interface, NL1 interface if the model training reside in LMF; NG-C interface, interface between AMF and NWDAF

- 4: L1 measurements: F1 interface in gNB split case, and NG-C interface, NL1 interface if the model training reside in LMF; F1 interface in gNB split case, and NG-C interface, interface between AMF and NWDAF

- 5: UAI: NG-C interface, NL1 interface if the model training reside in LMF; NG-C interface, interface between AMF and NWDAF

- 6: Early measurements : NG-C interface, NL1 interface if the model training reside in LMF; NG-C interface, interface between AMF and NWDAF

- 7: LPP: no involved interface if model training reside in LMF; interface between LMF and NWDAF if model training resides in LMF.

For AI based positioning with LMF sided model, the following contents shall be included for model training functionality corresponding to each data collection framework:

  - Logged MDT: the interface between TCE and NWDAF, or the interface between TCE and LMF

  - Immediate MDT: the interface between TCE and NWDAF, or the interface between TCE and LMF

  - L3 measurements: NG-C interface, NL1 interface if the model training reside in LMF; NG-C interface, interface between AMF and NWDAF

  - L1 measurements: F1 interface in gNB split case, and NG-C interface, NL1 interface if the model training reside in LMF; F1 interface in gNB split case, and NG-C interface, interface between AMF and NWDAF

  - UAI: NG-C interface, NL1 interface if the model training reside in LMF; NG-C interface, interface between AMF and NWDAF

  - Early measurements : NG-C interface, NL1 interface if the model training reside in LMF; NG-C interface, interface between AMF and NWDAF

  - LPP: no involved interface if model training reside in LMF; interface between LMF and NWDAF if model training resides in LMF.

For model offline training functionality in the use case of AI based CSI and BM, the delay caused by data forwarding from the endpoint of each data collection framework to the logical entity/functionality is shown as:

- 1: Logged MDT: No interface is involved. (OAM)

- 2: Immediate MDT: No interface is involved. (OAM)

- 3: L3 measurements: one private interface between OAM and gNB/CU

- 4: L1 measurements: one private interface between OAM and gNB/DU

- 5: UAI: one private interface between OAM and gNB/CU

- 6: Early measurements : one private interface between OAM and gNB/CU

- 7: LPP: NL1 interface, one private interface between OAM and gNB/CU

For AI based beam management and CSI, the following contents shall be included for model training functionality corresponding to each data collection framework:

	1: Logged MDT: No interface is involved. (OAM)

	2: Immediate MDT: No interface is involved. (OAM)

	3: L3 measurements: one private interface between OAM and gNB/CU

	4: L1 measurements: one private interface between OAM and gNB/DU

	5: UAI: one private interface between OAM and gNB/CU

	6: Early measurements : one private interface between OAM and gNB/CU

	7: LPP: NL1 interface, one private interface between OAM and gNB/CU

Model Monitoring

For model monitoring, according to the latest agreement about it:

Agreement Study at least the following metrics/methods for AI/ML model monitoring in lifecycle management per use case: Monitoring based on inference accuracy, including metrics related to intermediate KPIs Monitoring based on system performance, including metrics related to system peformance KPIs Other monitoring solutions, at least following 2 options. Monitoring based on data distribution Input-based: e.g., Monitoring the validity of the AI/ML input, e.g., out-of-distribution detection, drift detection of input data, or something simple like checking SNR, delay spread, etc. Output-based: e.g., drift detection of output data Monitoring based on applicable condition Note: Model monitoring metric calculation may be done at NW or UE

It can be seen that the model monitoring so far is not stable, the functionality mapping of the model monitoring cannot be done until RAN1 have achieved enough progress on it.

The model monitoring study is still ongoing in RAN 1 discussion, according to the latest agreement in RAN1, there are 4 solutions for AI model monitoring on the table and each solution may cause a different understandings on the selection of the framework for data collection.

- Conclusion and proposals

In this contribution, we discussed the model transfer for AI for PHY with the following observations and proposals:

Obsevation 1.	To evaluate the applicability of the data collection framework to the LCM purpose for each use case, Intuitively, there are two main factors shall be evaluated:

	Delay evaluation for each LCM purpose in each use case.

	Collected data size for each LCM purpose in each use case.

Obsevation 2.	The delay caused by forwarding the data from the endpoint of the data collection framework to the logical entity the LCM functionality reside in shall be taken into account in the delay evaluation for each LCM purpose in each use case..

Obsevation 3.	From RAN 2 perspective, the delay evaluation can be discussed if the functionality mapping within the NW is settled. The collected data size request for each LCM functionality shall be waiting for more input from RAN1.

Proposal 1.	For AI based positioning with LMF sided model, the following contents shall be included for model inference functionality corresponding to each data collection framework.

1.	Logged MDT: The interface between LMF and TCE

2.	Immediate MDT: The interface between LMF and TCE

3.	L3 measurements: NG-C interface, NL-1 interface

4.	L1 measurements: F1 interface in gNB split case, NG-C interface, NL-1 interface

5.	UAI: NG-C interface, NL-1 interface

6.	Early measurements : NG-C interface, NL-1 interface

7.	LPP: No interface involved

Proposal 2.	For AI based beam management and AI based CSI enhancement, the following contents for model inference functionality shall be included corresponding to each data collection framework:

	Logged MDT: at least one additional private interface is involved

	Immediate MDT: at least one additional private interface is involved

	L3 measurement: no additional interface is involved in the gNB non-split case, F1 interface is involved in gNB split case.

	L1 measurement: no additional interface is involved for both gNB non-split and gNB split case.

	UAI: no additional interface is involved in the gNB non-split case, F1 interface is involved in gNB split case.

	Early measurements: no additional interface is involved in the gNB non-split case, F1 interface is involved in gNB split case.

	LPP: Not applicable.

Proposal 3.	For AI based positioning with LMF sided model, the following contents shall be included for model training functionality corresponding to each data collection framework:

1.	Logged MDT: the interface between TCE and NWDAF, or the interface between TCE and LMF

2.	Immediate MDT: the interface between TCE and NWDAF, or the interface between TCE and LMF

3.	L3 measurements: NG-C interface, NL1 interface if the model training reside in LMF; NG-C interface, interface between AMF and NWDAF

4.	L1 measurements: F1 interface in gNB split case, and NG-C interface, NL1 interface if the model training reside in LMF; F1 interface in gNB split case, and NG-C interface, interface between AMF and NWDAF

5.	UAI: NG-C interface, NL1 interface if the model training reside in LMF; NG-C interface, interface between AMF and NWDAF

6.	Early measurements : NG-C interface, NL1 interface if the model training reside in LMF; NG-C interface, interface between AMF and NWDAF

7.	LPP: no involved interface if model training reside in LMF; interface between LMF and NWDAF if model training resides in LMF.

Proposal 4.	For AI based beam management and CSI, the following contents shall be included for model training functionality corresponding to each data collection framework:

Obsevation 4.	The model monitoring study is still ongoing in RAN 1 discussion, according to the latest agreement in RAN1, there are 4 solutions for AI model monitoring on the table and each solution may cause a different understandings on the selection of the framework for data collection.

- References

- R2-2304541 	Report of [AT121bis-e][024][AIML18] on Data Collection Table  (Nokia)	Nokia

- R2-2306414 Discussion on Functionality Mapping within NW,  ZTE Corporation, Sanechips