3GPP TSG-RAN WG2 Meeting#122		       						R2-2306419

Incheon, SK, May 22nd – 26rd, 2023

Source: 			ZTE Corporation, Sanechips

Title: 	Considerations On early RACH for LTM

Agenda item:		7.4.2.1

Document for: 	Discussion and Decision

- Introduction

In RAN2#121bis meeting, the early RACH is initially discussed in RAN 2, and the following agreements are achieved:

- From RAN2 perspective, to enable shared preamble resource among multiple UEs, it is beneficial that the information that identifies the allocated CFRA resource (i.e., SS/PBCH index, RACH occasion, and Random Access Preamble index) can be indicated in the PDCCH order (as legacy intra-cell PDCCH order).

- RRC RACH configuration for early TA acquisition (e.g., including whether RAR needs to be received) is specific per target cell and is signalled separately (separate IEs) from the candidate cell configuration (the part that need to be applied at cell switch).

- R2 assumes that Early TA RACH option 3 (with RAR from candidate cell) is not needed in Rel-18.

Meanwhile, RAN 1 also have made some progress on the early TA in the parallel RAN1 meeting. To our understanding, the majority work shall be done in RAN1 such as: RRC configuration for early RACH, RA-ResponseWindow design in the case of RAR is present in inter-DU case, preamble transmission power, etc. From RAN2 perspective, we can just focus on the procedure part and TA processing when UE obtaining TA from RAR, The intention of this contribution is to share our views on the procedure part and TA processing

- Discussion

In the parallel WG meetings of RAN1 and RAN2, The supported early RACH type in R18 LTM has been confirmed:

- Type 1 early RACH: RACH with RAR present on serving cell.

- Type 2 early RACH: RACH without RAR present

It is concluded that both early RACH with RAR present on serving cell (i.e. type 1 early RACH) and early RACH without RAR (i.e. type 2 early RACH) is supported.

Type 1 early RACH

In the CA mode, the PDCCH ordered RACH for SCell is supported, that is, preamble is sent on an SCell and then the RAR is received in the PCell which is generally as same as the type 1 early RACH (e.g. preamble is sent on a candidate cell and the corresponding RAR is sent on the serving cell), which can be shown as below:

- STEP 1: Random Access Resource Selection according to the indication from the received PDCCH order.

- STEP 2: Monitoring for a PDCCH transmission for RAR within the configured ra-ResponseWindow

- STEP 3a: If RAR is successfully received and decoded, Processing the TA value and UL grant present in the RAR.

- STEP 3b: Otherwise, consider the type 1 early RACH is failed.

For designing the type 1 early RACH procedure (i.e. early RACH with RAR) in RAN2 spec, the general procedure of legacy PDCCH ordered RACH for SCell can be considered as the baseline.

  - STEP 1: Random Access Resource Selection according to the indication from the received PDCCH order.

  - STEP 2: Monitoring for a PDCCH transmission for RAR within the configured ra-ResponseWindow

  - STEP 3a: If RAR is successfully received and decoded, Processing the TA value and UL grant present in the RAR.

  - STEP 3b: Otherwise, consider the type 1 early RACH is failed.

In step 3a of proposal 1, the UL grant of RAR is for SCell in the case of PDCCH ordered RACH for SCell. However, in the case of the type 1 early RACH, UE have not yet formally connected to the candidate cell, so the UL grant in the RAR must not be available for the candidate cell. In this sense, there are two alternatives for the UL grant present in the RAR:

- Alternative 1: Nothing, UE should ignore the UL grant received in the RAR of type 1 early RACH.

- Alternative 2: UL grant for the serving cell, UE will process the UL grant for the serving cell where the RAR is received.

Compare between two alternatives, the second alternative seems better since, through the PUSCH reception, the UL data arrival at UE or triggered BSR MAC CE can be sent to NW as soon as the early RACH is finished which is benefit for the URLLC service, which also can explicit to indicate the NW the TA has been successfully received.

The UL grant contained in the RAR is for the serving cell where the RAR is received

In step 3a of proposal 1, the TA processing and/or management, RAN 1 have achieved the following agreement:

Agreement For PDCCH ordered RACH mechanism in R18 LTM, when reception of RAR is configured, - the UE stores(remembers/maintains/handles) a TA for at least one candidate cell - storing(remembering/maintaining/handling) corresponding TAs for more than one candidate cell is up to UE capability - detailed number of candidate cell is up to UE capability

According to the agreement, it implies that the TA of candidate cell obtained from the early RACH is no longer regulated based on the TAG.

According to RAN 1 agreements, the TA of candidate cell obtained from the early RACH is  regulated/preserved per candidate cell rather than TAG.

Basically, it is more or less aligned with the RAN 2’s current understanding: From RAN 2 point of view, the tag-Id for a candidate cell is mostly like to be configured in the candidate cell configuration which is following the legacy configuration structure, that is, the tag-Id is configured in the servingCellConfig. Considering RAN 2 have confirmed that it is not mandatory to UE perform the early decoding and/or early validity check for the candidate cell configuration, that means, UE may have no idea about the TAG information (i.e. tag-Id) for a candidate cell when obtaining the TA value from the early RACH on such candidate cell.

- The UE may perform early decoding and early validity check. FFS whether Early validity check triggers early re-establishment. FFS the possible timing, FFS subset of cells, FFS if need to specify anything or just up to UE impl, FFS if other signalling to notify network is needed.

Hence, UE perform the TA processing and management for a candidate cell before LTM is per candidate cell rather than per TAG.

TA processing and management for a candidate cell before execution of the cell switch is per candidate cell rather than per TAG.

- In step 3b, the failure handling of the type 1 early RACH. In the legacy RACH failure handling, UE should report the failure to the higher layer to trigger radio link failure. However , to my understanding, the early RACH failure is not regarding the current radio link, it does not make sense to follow the legacy behavior.

If type 1 early RACH is failed , there is no need to indicate the random access problem to upper layer.

Type 2 early RACH

In the previous RAN 1 meeting, RAN 1 have concluded the following agreements as indicated in observation 3:

It is concluded that the autonomous retransmission of Preamble is not allowed regardless of the value of PreambleTransMax.

It implies that one PDCCH order only to trigger preamble transmission once. Consider there is no RAR is present in the type 2 early RACH, we understand, the RACH should be considered as successfully completed once the preamble is sent over the indicated PRACH occasion.

Type 2 early RACH (i.e. early RACH without RAR) is considered as successfully completed once the preamble is sent over the indicated PRACH occasion.

- Conclusion and proposals

In this contribution, we discussed the early RACH across DUs contents with the following observations and proposals:

Obsevation 1.	It is concluded that both early RACH with RAR present on serving cell (i.e. type 1 early RACH) and early RACH without RAR (i.e. type 2 early RACH) is supported.

Proposal 1.	For designing the type 1 early RACH procedure (i.e. early RACH with RAR) in RAN2 spec, the general procedure of legacy PDCCH ordered RACH for SCell can be considered as the baseline.

a)	STEP 1: Random Access Resource Selection according to the indication from the received PDCCH order.

b)	STEP 2: Monitoring for a PDCCH transmission for RAR within the configured ra-ResponseWindow

c)	STEP 3a: If RAR is successfully received and decoded, Processing the TA value and UL grant present in the RAR.

d)	STEP 3b: Otherwise, consider the type 1 early RACH is failed.

Proposal 2.	The UL grant contained in the RAR is for the serving cell where the RAR is received

Obsevation 2.	According to RAN 1 agreements, the TA of candidate cell obtained from the early RACH is  regulated/preserved per candidate cell rather than TAG.

Proposal 3.	TA processing and management for a candidate cell before execution of the cell switch is per candidate cell rather than per TAG.

In step 3b, the failure handling of the type 1 early RACH. In the legacy RACH failure handling, UE should report the failure to the higher layer to trigger radio link failure. However , to my understanding, the early RACH failure is not regarding the current radio link, it does not make sense to follow the legacy behavior.

Proposal 4.	If type 1 early RACH is failed , there is no need to indicate the random access problem to upper layer.

Obsevation 3.	It is concluded that the autonomous retransmission of Preamble is not allowed regardless of the value of PreambleTransMax.

Proposal 5.	Type 2 early RACH (i.e. early RACH without RAR) is considered as successfully completed once the preamble is sent over the indicated PRACH occasion.

- References

[1] R2_121BIS Chair Notes  MTK

[