3GPP TSG-RAN WG2 Meeting #122	 R2-2306427

Incheon, Korea, 22nd - 26th May, 2023

Agenda Item:	7.9.2

Source:	Xiaomi

Title:	U2U Relay UE discovery / (re)selection, SRAP, QoS Handling

Document for:	Discussion and Decision

# Introduction

L2 and L3 U2U relay is to be supported in REL18. In this contribution, we continue to discuss the common parts of L2/L3 U2U relay, i.e. relay UE discovery, relay UE selection and relay UE reselection.

In addition this contribution also addresses the topics of SRAP configuration and QoS management for U2U Relay configuration.

# Common parts L2/L3 Relay UE

In general the baseline for U2U NR sidelink relaying is based on the scenario and assumptions captured in TR38.836 as agreed in the RAN WID [1]. The following discusses details regarding the discovery message transmission, AS conditions for a Relay UE acting as Relay, Relay UE selection AS criteria, Relay UE reselection and discovery message content relating to access stratum.

## 2.1 U2U Relay Discovery Transmission

RAN2 have made the following agreements in respect to common selection/reselection behaviour [2], [3] and [4] in regards to progress on the U2U Sidelink Relay objective of this work item.

RAN2#119 bis-e

- SL-RSRP and SD-RSRP can be used for relay selection/reselection criteria.  FFS when each of the two quantities are used and whether to re-use the criteria in Rel17.

- Relay selection triggers include at least

1) Upper layer trigger;

2) PC5 signal strength conditions. RAN2 further discuss details for trigger 2).

- Relay reselection triggers include at least

1) Upper layer trigger;

2) PC5-RLF detection at the remote UE;

3) PC5-RLF indication received from the relay;

4) PC5 signal strength conditions;

5) PC5 link release message from relay to remote.

RAN2 further discuss details for trigger 4), potentially including T400 expiry.  FFS if some of the conditions could be indicated to upper layer instead of directly causing reselection.

- Discovery message transmission at the remote UE is conditioned on at least upper layer indication.

RAN2#120

- UE-to-UE relay selection can be triggered based on the PC5 RSRP (FFS SL-RSRP or SD-RSRP) of the direct link falling below a threshold.  FFS which remote UE (or both) can trigger relay selection.  FFS the relationship between selection and discovery.

- UE-to-UE relay reselection can be triggered based on the PC5 RSRP (FFS SL-RSRP or SD-RSRP) between a remote UE and the relay UE falling below a threshold.  FFS which remote UE (or both) can trigger relay reselection.  FFS if/how the second hop between the relay UE and the peer UE is considered.

RAN2#121

- For relay UE selection, the remote UE uses SL-RSRP measurements towards peer remote UE to trigger relay UE selection when there is data transmission on direct link.

- For relay UE reselection, the remote UE uses SL-RSRP measurements towards the relay UE to trigger relay UE reselection when there is data transmission on the indirect link.

- In both cases, it is left to remote UE implementation whether to use SL-RSRP or SD-RSRP for relay (re)selection trigger evaluation in case of no data transmission.

- FFS if there need to be different configured thresholds for SL-RSRP and SD-RSRP.

- Each Remote UE can trigger Relay reselection based at least on current hop quality.

RAN2#121bis

- For the integrated-discovery case, the relay UE forwards the discovery message for DCR message with integrated Discovery case only if the PC5 RSRP between the relay UE and the source remote UE is above a threshold.

- For Model-B discovery, after receiving a discovery message from a relay UE, a target remote UE transmits the discovery response message only if the PC5 RSRP between the target remote UE and the relay UE is above a configured threshold.  FFS if there is separate impact for this agreement from the relay selection functionality.

- Each remote UE (source or destination) can trigger relay selection based on the direct link quality.  FFS interaction between discovery and selection.

It is clear that in order to perform Relay UE selection requires a candidate Relay UE hence the selection step is preceded by a Relay UE discovery step, assuming one has not already been completed. For a Relay UE to be identified as a candidate during discovery the Relay Service Code is exchanged by the NAS layer in the Discovery solicitation/announcement message [5]. This identifies the Relay UE can support the service required by the Remote UE. The key being whether a candidate Relay UE has been identified at the point Relay UE selection is triggered. When the AS measurement or detection of PC5-RLF is detected on the direct link between a Remote UE and peer Remote UE this trigger Relay UE selection. In the case where no candidate Relay UE has been established this AS trigger is reported to the higher layer to trigger a U2U Relay UE discovery, before a Relay UE can be selected.

Proposal 1: The Remote UE triggers Relay UE selection on detection of PC5-RLF or when the PC5 RSRP falls below a threshold.

Proposal 2: When Relay UE selection is triggered and no Relay UE candidate exists the Remote UE AS indicates the Relay UE selection trigger to the upper layer, and the upper layer may trigger Relay UE Discovery. (Whether discovery is triggered if the Remote UE has one or more Relay UE candidates at the AS trigger of selection is a matter for UE implementation.)

- In both cases, it is left to remote UE implementation whether to use SL-RSRP or SD-RSRP for relay (re)selection trigger evaluation in case of no data transmission.

- FFS if there need to be different configured thresholds for SL-RSRP and SD-RSRP.

The SL-RSRP or SD-RSRP measurement of the direct link should be below a threshold to trigger the initiation of the Relay UE selection/ (discovery) process.

The selection of SL-RSRP or SD-RSRP measurement type has a dependency to ongoing data transmission as well as UE implementation choice, but the determination step is reliant on a threshold for the respective measurement type in order for the Remote UE to make the determination.

In addition when Relay selection is triggered, and for the case where a new candidate Relay UE is to be evaluated before it can be selected the Remote UE obtains an SD-RSRP measurement for the candidate Relay UE. This SD-RSRP measurement is then assessed and must exceed a threshold to determine its suitability criteria as a candidate before selection.

Related to the above FFS, we have received reply LS’s in [6] and [7] to RAN2s question in R2-2302234. [7] indicates, “the measured SD-RSRP and SL-RSRP between two UEs cannot be used relative to each other or relative to same reference threshold.”. So from these responses no direct comparison can be made between the measurement types nor is a comparison using a single threshold for both measurements reliable.

Proposal 3: RAN2 uses different thresholds for SL-RSRP and SD-RSRP measurements. If the UE uses both RSRP measurement types then it is left to UE implementation to select between them.

In addition RAN2 are invited to consider whether the threshold(s) for SL-RSRP and SD-RSRP are also different for triggering of Relay UE selection and Relay UE reselection.

Proposal 4: RAN2 consider the threshold(s) for SL-RSRP and SD-RSRP measurements are set different for triggering of Relay UE selection compared to the SL-RSRP and SD-RSRP measurements to trigger Relay UE reselection.

In addition to the potential numerous different thresholds and their values for use in executing the identified evaluations, RAN2 may also consider whether these thresholds need to be signalled or can be preconfigured. It is noted that support for a Remote UE out-of-coverage is required so preconfigured threshold(s) is proposed, whether signalled values are also needed may need further consideration.

Proposal 5: RSRP Thresholds to trigger Relay UE selection and Relay UE reselection are preconfigured. The additional need for configurable thresholds is FFS.

## 2.2 AS criteria to act as U2U Relay UE

In Rel-17 U2N relay, a minimum and a maximum threshold are defined for a Relay UE to determine whether it can be a Relay UE or not. This is to ensure the Relay UE is neither too close to nor too far away from the network to guarantee both the coverage and the communication quality.

When it comes to U2U relaying, according to SA2 U2U relay discovery both Model A and Model B are supported. For Model A, the U2U Relay UE broadcasts an announcement message which may include the user information of Remote UEs that the Relay UE has or can make contact with. For Model B, the U2U Relay UE broadcasts the discovery solicitation message and the discovery response message to the target/source remote UEs respectively.

Similar as U2N relaying, in order to guarantee the coverage and quality of the PC5 link it makes sense for U2U relaying to define some AS criteria in order to determine whether a UE can serve as a Relay UE or not and separately for both Model A and Model B. Considering the different operating schemes, different principles may need to be introduced for the different models.

For Model A, since the candidate Relay UE is to broadcast the announcement message, the SL-RSRP/SD-RSRP between the relay UE and at least a certain number of remote UE should be larger than a minimum threshold to ensure the transmission quality for each hop to those remote UEs.

Proposal 6: For Model A, a UE can act as candidate relay UE when the SL-RSRP/SD-RSPR between this UE and a configured number of remote UEs is above the minimum threshold.

Whereas for Model B, since upon reception of the discovery message, Relay UE is able to derive the L2 ID information of both the source remote UE and target remote UE, the SL-RSRP/SD-RSRP of each hop should be considered separately, i.e., the SL-RSRP/SD-RSRP between the source/target remote UE and relay UE is above the minimum threshold. At R2#121bis,

R2#121bis - AGREEMENT

For Model-B discovery, after receiving a discovery message from a relay UE, a target remote UE transmits the discovery response message only if the PC5 RSRP between the target remote UE and the relay UE is above a configured threshold.  FFS if there is separate impact for this agreement from the relay selection functionality.

This agreement indicates that for Model B discovery the candidate Relay UE can only act as a U2U Relay if the PC5 RSRP between the candidate Relay UE and the target Remote UE is above a minimum configured threshold.

However this agreement does not consider for Model B discovery the behaviour for the source Remote UE. It also does not consider whether the threshold is pre-configured or configured by a network.

Proposal 7: For Model-B discovery, on receiving a discovery response message from a relay UE, a source Remote UE considers only Relay UE(s) with a PC5 RSRP above a minimum threshold can act as candidate U2U Relay UE. FFS regarding how to set the minimum threshold e.g. signalled or preconfigured.

Also this agreement from R2#121bis does not consider the Relay UE selection by the target Remote UE or the source Remote UE, in particular if more than one candidate U2U Relay UE exists based on the received discovery messages and the PC5 RSRPs to the candidate U2U Relay UEs being above a minimum threshold.

.

## 2.3 Relay UE Selection

RAN2 has previously made the following agreements regarding triggering of Relay UE selection [3], [4].

RAN2#119bis-e

Proposal 7.1a:    Relay selection triggers include at least 1) Upper layer trigger; 2) PC5 signal strength conditions.  RAN2 further discuss details for trigger 2).

RAN2#120

UE-to-UE relay selection can be triggered based on the PC5 RSRP (FFS SL-RSRP or SD-RSRP) of the direct link falling below a threshold.  FFS which remote UE (or both) can trigger relay selection.  FFS the relationship between selection and discovery.

And in section 2.1 we already considered the Remote UE triggering Relay UE selection procedure, when using a direct link between 2 UEs following detection of the link PC5-RLF or when the PC5 RSRP falls below a threshold. The next step is the selection of Relay UE candidates.

So for Relay UE selection the SA2 TR [5] captures a number of scenarios and proposed solutions relating to Relay UE selection both for source Remote UE and target Remote UE, which also depend on the discovery model used.

Generally for both discovery models in REL17 U2N relay, the U2N relay UE whose PC5 signal strength is above the threshold is considered for relay selection. A similar approach is proposed for U2U relay UE selection.

Proposal 8: Candidate U2U Relay UE is considered to meet the AS criteria for selection if its PC5 link quality measurement is above a configured threshold. FFS regarding how to set the Threshold e.g. signalled or preconfigured.

For Model B discovery a source Remote UE cannot be sure of the ability for any single Relay UE to be able to reliably reach a target Remote UE, subsequently the target Remote UE may potentially receive multiple solicitation messages, each from different Relay UEs and each originating from the same source Remote UE. How the target Remote UE responds to these multiple received messages has been considered by SA2.

in 23.700-33 V1.1.0 [9], 6.1.1 … It may exist a situation where multiple UE-to-UE relays can be used to reach the target UE or the target UE may also directly receive the Direct Communication Request or Solicitation message from the source UE. The target UE may choose which one to reply according to e.g. signal strength, local policy (e.g. traffic load of the UE-to-UE relays), Relay Service Code if there is any or operator policies (e.g. always prefer direct communication or only use some specific UE-to-UE relays). Whereas, in 23.304 [4], the U2U Relay Model B discovery procedure states, “If the discoveree 5G ProSe End UE (UE-2) receives multiple UE-to-UE Relay Discovery Solicitation messages from different 5G ProSe UE-to-UE Relays, it may choose to respond or not to a 5G ProSe UE-to-UE Relay (e.g. based on the PC5 signal strength of each message received).

Let us consider that the target Remote UE receives one or more solicitation messages from one or more candidate Relay UEs where the PC5 RSRP is above a configured minimum threshold. i.e. considered as meeting the AS selection criteria.

The target Remote UE following the reception of one or more solicitation messages may reply to all, a subset, only one or maybe none of those candidate Relay UEs where a PC5 RSRP is above the configured minimum threshold.

Proposal 9: For Model B discovery the target Remote UE receiving one or more U2U Relay Discovery Solicitation messages from one or more candidate U2U Relay UEs, considers these Relay UEs have met the AS criteria for selection when the PC5 RSRP between the candidate Relay UE(s) and the target Remote UE is above a configured threshold. FFS regarding how to set the configured threshold e.g. signalled or preconfigured.

So for example, if the target Remote UE responds to all received U2U Relay Discovery Solicitation messages from candidate U2U Relay UE(s) where the PC5 RSRP between the candidate Relay UE(s) and the target Remote UE are above a configured or pre-configured minimum threshold, e.g. the U2U Relay UE meets the AS criteria for selection. This leaves the Relay UE selection to the source Remote UE.

Alternatively, receiving one or more U2U Relay Discovery Solicitation messages from candidate U2U Relay UE(s) where the PC5 RSRP between the candidate Relay UE(s) and the target Remote UE are above a configured or pre-configured minimum threshold, e.g. the U2U Relay UE(s) meet the AS criteria for selection, then the target Remote UE may select a subset or one of these U2U Relay UEs.

The selection of one or a subset of candidate U2U Relay UEs may consider assistance parameters explicitly signalled or determined implicitly for the candidate U2U Relay UE(s). Some of these assistance parameters could relate to Non-Access Stratum information and some relate to Access Stratum information including (but not limited to) the following examples.

- PC5 Signal strength

- Local policy/ preference

- Traffic load of potential U2U Relay UE

- Relay Service Code

- Operator policies (direct over U2U relay)

- Specific Relay UE or Relay UEs for communication

RAN2 should resolve on the use of which Access Stratum parameters to use in the selection process.

Proposal 10: For Model B discovery, the target Remote UE receiving one or more U2U Relay Discovery Solicitation messages from candidate U2U Relay UE(s) with a PC5 RSRP above a minimum threshold selects candidate Relay UEs via UE implementation in order to send a discovery response message(s) to. The UE implementation may consider assistance information provided via AS or NAS during the selection process.

The target Remote UE sends to the selected candidate U2U Relay UE(s) a U2U Relay Discovery Response message. And the candidate U2U Relay UE(s) then send a Discovery Response message including the User Info ID of the U2U Relay UE and ID of the target Remote UE (User Info ID of the discoveree 5G ProSe End UE [4]) to the source Remote UE.

In the case of the source Remote UE receiving multiple U2U Relay Discovery Response messages from multiple candidate Relay UEs where the PC5 RSRP is above a configured threshold, then the source Remote UE performs U2U Relay UE selection.

Proposal 11: For Model B discovery for a source Remote UE receiving one or more 5G ProSe UE-to-UE Relay Discovery Response messages from one or more candidate U2U Relay UEs, for which the PC5 RSRP between the candidate Relay UE(s) and the source Remote UE is above a configured threshold. The U2U Relay UE selection is left to source Remote UE implementation, although the UE may consider assistance information provided via AS or NAS during the selection process.

For Model A discovery, the source Remote UE selects the relay UE from amongst potential candidate Relay UEs if it receives multiple announcement messages which include the user information of the target remote UE. A similar principle from REL17 U2N Relay selection can be reused, i.e., the SL-RSRP/SD-RSRP between the source remote UE and the U2U relay UE is above the minimum threshold. If there are multiple relay UE satisfying the condition, it is up to source remote UE’s implementation to select one relay UE.

Proposal 12: For Model A, source Remote UE selects a Relay UE for which the SL-RSRP/SD-RSRP is above the minimum threshold.

Proposal 13: A source Remote UE receiving multiple discovery messages from multiple Relay UEs with a PC5 link quality measurement above the PC5 link quality threshold, may rank these Relay UEs. The Relay UE ranking may be according to their respective PC5 link quality measurement, and forwarded to the source Remote UE higher layer for Relay UE selection.

In addition to the PC5 channel quality measurement as an Access Stratum selection criteria the Remote UE may also consider the respective Relay UEs’ load as a factor for Relay UE selection. If multiple U2U Relay UEs fulfil the AS and NAS requirements as viable candidate Relay UEs, a simple solution is to select the U2U Relay UE with the lowest load. If there are multiple lowest load relay UEs, the remote UE may use a ranking of Relay UE candidates based on Relay UE load and/or additionally a ranking of their respective PC5 channel quality measurement to select the Relay UE or may make a Relay UE selection based on other parameters.

Proposal 14: If multiple U2U Relay UEs fulfil the AS and NAS selection requirement, the Remote UE selects the U2U relay UE with the lowest load.

## 2.4 Relay UE Reselection

The following agreements relating Relay UE reselection triggers was made at R2#120 [2].

UE-to-UE relay reselection can be triggered based on the PC5 RSRP (FFS SL-RSRP or SD-RSRP) between a remote UE and the relay UE falling below a threshold.

FFS which remote UE (or both) can trigger relay reselection.

FFS if/how the second hop between the relay UE and the peer UE is considered.

Proposal 15: RAN2 does not agree T400 as a new relay reselection trigger because it is already considered when determining PC5 RLF to trigger relay reselection.

Proposal 16 (modified): When the remote UE receives PC5-RLF indication from the U2U relay UE, it would inform upper layers and rely on upper layers to trigger relay reselection (or not).  FFS if there would be any constraints on the remote UE implementation behaviour to keep or release the PC5 link with the relay UE.

In addition TS 23.304 captured U2U L2 Relay UE reselection as follows.

### 6.7.4	5G ProSe UE-to-UE Relay reselection

#### 6.7.4.1	General

After being connected to the 5G ProSe UE-to-UE Relay, the 5G ProSe End UEs keep performing the measurement of the signal strength of PC5 unicast link with the 5G ProSe UE-to-UE Relay for relay reselection.

The UE-to-UE Relay discovery procedures defined in clause 6.3.2.4 are used for 5G ProSe UE-to-UE Relay reselection to discovery available 5G ProSe UE-to-UE Relays or after the 5G ProSe End UEs have negotiated which UE-to-UE Relay to reselect.

In the negotiated UE-to-UE Relay reselection, one 5G ProSe End UE initiates the UE-to-UE Relay reselection procedure, and the 5G ProSe End UEs can negotiate a new 5G ProSe UE-to-UE Relay using the existing connection.

In addition to the above triggers and further to the discussion above regarding Relay UE selection criteria the Remote UE may also consider indication by the Relay UE of the Relay UE’s load as a trigger for Relay UE reselection. The Relay UE can trigger the indication on the Relay load reaching a threshold, where the actual Relay UE loading may be causing delay to one or more of the traffic channels currently supported by the Relay UE. The resulting action by the Remote UE may be to trigger a reconfiguration or to trigger a Relay UE reselection. An indication including an indication of the load enables the Remote UE to make an informed choice based on its understanding of the actual traffic conditions.

Proposal 15: Relay UE triggers an indication to the Remote UE when it’s loading reaches a threshold. The Indication includes information relating to the relative loading of the Relay UE. FFS how to measure the actual load and the corresponding load indication. The remote UE performs relay UE reselection following receipt of the load indication from the Relay UE.

Alternatively and additionally the U2U Relay UE may explicitly indicate e.g. via a PC5-S link release message, that the Remote UE should trigger a Relay UE reselection. The resulting behaviour for the Remote UE may vary depending on the explicit indication cause sent by the Relay UE. In one example this may be a notably different behaviour to a RLF indication (e.g. may apply specifically for the second hop) wherein the ongoing service may be maintained up to the point where the Remote UE successfully reselects an alternative Relay UE e.g. during the negotiated U2U Relay reselection.

Proposal 16: The Relay UE sends an explicit indication to trigger the Relay UE reselection by the Remote UE. In one example the indication may include an indication regarding the status of the second hop. Additionally or differently the indication may enable the Remote UE to continue with the existing sidelink service until U2U Relay UE reselection is completed.

## 2.5 Discovery Message

Regarding the content carried in U2U relay discovery message particularly to aid Relay UE selection where multiple Relay UEs are candidates to establish a U2U relay link, SA2 TR [5], proposed to consider traffic load as well as a number of other factors of the relay UE during relay UE selection as following.

6.1.1 … When a UE-to-UE relay receives a Direct Communication Request or a Solicitation message with the relay_indication set, then it shall decide whether to forward the message (i.e. modify the message and broadcast it in its proximity), according to e.g. Relay Service Code if there is any, Application ID, authorization policy (e.g. relay for specific ProSe Service), the current traffic load of the relay, the radio conditions between the source UE and the relay UE, etc. It may exist a situation where multiple UE-to-UE relays can be used to reach the target UE or the target UE may also directly receive the Direct Communication Request or Solicitation message from the source UE. The target UE may choose which one to reply according to e.g. signal strength, local policy (e.g. traffic load of the UE-to-UE relays), Relay Service Code if there is any or operator policies (e.g. always prefer direct communication or only use some specific UE-to-UE relays).

Therefore, traffic load should be included in or with the discovery message. RAN2 can further discuss how to define the load of the relay UE. One straight forward way to define the load is based on relay UE’s CBR result.

Proposal 17: Load information of U2U relay UE can be included in or with the discovery message for example in the AS information transmitted with the U2U relay discovery message, FFS on how to determine the load information.

# SRAP Design

detailed design on the SRAP has been discussed over the last couple of meetings and the following agreements were achieved [3].

Agreements:

FFS if multiplexing of different destinations in the same RLC channel is supported.

An ID mappable to the destination remote UE is needed in the first hop (Tx remote UE to relay), at least in case multiplexing of different destinations in the same RLC channel is supported.

An ID mappable to the source remote UE is needed in the second hop (relay to Rx remote UE).

FFS if the IDs are different (e.g., source and destination UE IDs) or common (e.g., a local ID for the pair).

FFS whether both UE IDs are included in the header or the relay UE does a mapping.

R2#121bis

•	Multiplexing of different destinations in the same RLC channel of the first hop is supported.

•	RAN2 confirms that multiplexing of the different bearers from the different source remote UEs into the same RLC channel in the second hop is supported.

Whether to use separate source and destination UE IDs or common, i.e., a local ID for the pair is discussed. Actually both solutions can work but we slightly prefer separate IDs which is more future proof for multiple hop. In addition, if a common ID solution is adopted, it should be the relay UE to allocate and configure this common ID to both source and destination remote UE in order to ensure the uniqueness for each pair via this relay UE. If there are multiple pairs via this relay UE, the configuration signalling will consume more bits compared with different ID solution since the association between a pair and local ID should be indicated while for the other solution only the association between the L2 ID and local ID is needed.

Proposal 18: The IDs mappable to the source and destination remote UE are different IDs.

Regarding whether both UE IDs are included in the header or the relay UE does a mapping. We think to reduce the complexity of relay UE and to have a common design on the SRAP header on both hops, also considering to support multiple hop in the future release, it is better to have both UE IDs in the SRAP header.

Proposal 19: Include both source and destination UE IDs in the SRAP header.

# QoS handling

According to the latest TS 23.304, the QoS negotiation of L2 U2U is specified as follows

For a 5G ProSe Layer-2 End UE connecting with another 5G ProSe Layer-2 End UE(s) via 5G ProSe Layer-2 UE-to-UE Relay, the source 5G ProSe Layer-2 End UE and the target 5G ProSe Layer-2 End UE negotiate the end-to-end QoS for the traffic transmission between source 5G ProSe Layer-2 End UE and target 5G ProSe Layer-2 End UE. Editor's note: It is FFS whether and how to perform QoS enforcement for first hop PC5 interface (between the source 5G ProSe Layer-2 End UE and 5G ProSe Layer-2 UE-to-UE Relay) and second hop PC5 interface (between the 5G ProSe Layer-2 UE-to-UE Relay and the target 5G ProSe Layer-2 End UE).

And subsequently we have received at this meeting an LS from SA2 (R2-2304652), where SA2 confirm that handling is required to enforce the QoS for the first hop and the second hop, as the current signalling is transparent to the Relay UE. And they ask RAN2 to define an AS solution to address this for L2 U2U Relay.

In R17 U2N relay, both relay UE and remote UE are in connected state and E2E QoS split is totally up to gNB implementation. However, in R18 U2U scenario, since we have agreed to simplify the gNB involvement as compared to the U2N case, it is much more reasonable to rely on the UE itself to perform the QoS split especially when the remote/relay UEs are under different cells and/or RRC states. In this case, two candidate solutions can be considered.

- Alternation 1: Source remote UE is in charge of QoS split

For this option, it is the source remote UE to determine the E2E QoS parameters between the source remote UE and the target remote UE and perform QoS spit on both hops. To assist the source remote UE on QoS split, e.g., PDB split between two hops, some assistance information from the relay UE on the second hop is needed, e.g., CBR, measurement report, transmission latency etc. After determining the detailed configuration of both links, the source remote UE sends configurations on both hops to the relay UE and the relay UE can further transmits the configuration on the second hop to the target remote UE.

- Alternation 2: Relay UE is in charge of QoS split

For this option, it is still the source remote UE to determine the E2E QoS parameters between the source remote UE and the target remote UE. However, the relay UE is in a centralized control on the QoS split on both hops since the relay UE can know the status of two links at the same time. In this case, some information from the source remote UE is needed, e.g., E2E QoS parameters of different bearers including PDB, PQI etc. After determining the QoS configuration for each hop, the relay UE transmits the configuration of the first hop to the source remote UE while the configuration of the second hop to the target remote UE.

Proposal 20: Rely on the UE itself to perform QoS split. FFS source remote UE or relay UE.

# RRC signalling

After an E2E bearer has been established via a U2U Relay, we are likely to have a cause to send an RRCReconfigurationSidelink message to the peer PC5-RRC at the Remote UE. When we do this in a non-Relay case we start T400  as a precaution. When considering the transmission is now over two hops clearly this normal performance for peer RRC message exchange will be impacted. We propose RAN2 consider how to address this impact to T400.

Figure 5.8.9.1.1-1: Sidelink RRC reconfiguration, successful

The UE shall set the contents of RRCReconfigurationSidelink message as follows:

…

1>	start timer T400 for the destination;

Proposal 21: RAN2 consider how to handle T400, when RRCReconfigurationSidelink is sent over a U2U Relay between peer End Remote UEs

# Conclusion

Based on the discussion in sections 2, 3 and 4 we have following proposals.

Discovery Transmission

Proposal 1: The Remote UE triggers Relay UE selection on detection of PC5-RLF or when the PC5 RSRP falls below a threshold.

Proposal 2: When Relay UE selection is triggered and no Relay UE candidate exists the Remote UE AS indicates the Relay UE selection trigger to the upper layer, and the upper layer may trigger Relay UE Discovery. (Whether discovery is triggered if the Remote UE has one or more Relay UE candidates at the AS trigger of selection is a matter for UE implementation.)

Proposal 3: RAN2 uses different thresholds for SL-RSRP and SD-RSRP measurements. If the UE uses both RSRP measurement types then it is left to UE implementation to select between them.

Proposal 4: RAN2 consider the threshold(s) for SL-RSRP and SD-RSRP measurements are set different for triggering of Relay UE selection compared to the SL-RSRP and SD-RSRP measurements to trigger Relay UE reselection.

Proposal 5: RSRP Thresholds to trigger Relay UE selection and Relay UE reselection are preconfigured. The additional need for configurable thresholds is FFS.

AS Criteria to Act As a U2U Relay UE

Proposal 6: For Model A, a UE can act as candidate relay UE when the SL-RSRP/SD-RSPR between this UE and a configured number of remote UEs is above the minimum threshold.

Proposal 7: For Model-B discovery, on receiving a discovery response message from a relay UE, a source Remote UE considers only Relay UE(s) with a PC5 RSRP above a minimum threshold can act as candidate U2U Relay UE. FFS regarding how to set the minimum threshold e.g. signalled or preconfigured.

Relay UE Selection

Proposal 8: Candidate U2U Relay UE is considered to meet the AS criteria for selection if its PC5 link quality measurement is above a configured threshold. FFS regarding how to set the Threshold e.g. signalled or preconfigured.

Proposal 9: For Model B discovery the target Remote UE receiving one or more U2U Relay Discovery Solicitation messages from one or more candidate U2U Relay UEs, considers these Relay UEs have met the AS criteria for selection when the PC5 RSRP between the candidate Relay UE(s) and the target Remote UE is above a configured threshold. FFS regarding how to set the configured threshold e.g. signalled or preconfigured.

Proposal 10: For Model B discovery, the target Remote UE receiving one or more U2U Relay Discovery Solicitation messages from candidate U2U Relay UE(s) with a PC5 RSRP above a minimum threshold selects candidate Relay UEs via UE implementation in order to send a discovery response message(s) to. The UE implementation may consider assistance information provided via AS or NAS during the selection process.

Proposal 11: For Model B discovery for a source Remote UE receiving one or more 5G ProSe UE-to-UE Relay Discovery Response messages from one or more candidate U2U Relay UEs, for which the PC5 RSRP between the candidate Relay UE(s) and the source Remote UE is above a configured threshold. The U2U Relay UE selection is left to source Remote UE implementation, although the UE may consider assistance information provided via AS or NAS during the selection process.

Proposal 12: For Model A, source Remote UE selects a Relay UE for which the SL-RSRP/SD-RSRP is above the minimum threshold.

Proposal 13: A source Remote UE receiving multiple discovery messages from multiple Relay UEs with a PC5 link quality measurement above the PC5 link quality threshold, may rank these Relay UEs. The Relay UE ranking may be according to their respective PC5 link quality measurement, and forwarded to the source Remote UE higher layer for Relay UE selection.

Proposal 14: If multiple U2U Relay UEs fulfil the AS and NAS selection requirement, the Remote UE selects the U2U relay UE with the lowest load.

Relay UE Reselection

Proposal 15: Relay UE triggers an indication to the Remote UE when it’s loading reaches a threshold. The Indication includes information relating to the relative loading of the Relay UE. FFS how to measure the actual load and the corresponding load indication. The remote UE performs relay UE reselection following receipt of the load indication from the Relay UE.

Proposal 16: The Relay UE sends an explicit indication to trigger the Relay UE reselection by the Remote UE. In one example the indication may include an indication regarding the status of the second hop. Additionally or differently the indication may enable the Remote UE to continue with the existing sidelink service until Relay UE reselection is completed.

Discovery Message

Proposal 17: Load information of U2U relay UE can be included in or with the discovery message for example in the AS information transmitted with the U2U relay discovery message, FFS on how to determine the load information.

SRAP Design

Proposal 18: The IDs mappable to the source and destination remote UE are different IDs.

Proposal 19: Include both source and destination UE IDs in the SRAP header.

QoS Handling

Proposal 20: Rely on the UE itself to perform QoS split. FFS source remote UE or relay UE.

RRC Signalling

Proposal 21: RAN2 consider how to handle T400, when RRCReconfigurationSidelink is sent over a U2U Relay between peer End Remote UEs

# Reference

[1] 	RP-223501	Revised WID on NR sidelink relay enhancements, LG Electronics

[2]	RP-222933	Status Report to TSG NR sidelink relay enhancements, TSG RAN meeting #98

[3]	RP-230701	Status Report to TSG NR sidelink relay enhancements, TSG RAN meeting #99

[4]	Draft_R2_121bis_meeting_report_v2

[5]	TS 23.304 V18.1.0 Proximity based Services (ProSe) in the 5G System (5GS) (Release 18)

[6]	R2-2304617 Reply LS on comparison of SL-RSRP and SD-RSRP measurements (R1-2304211)

[7]	R2-2304637 LS on Comparison of SL-RSRP and SD-RSRP measurements (R4-2306366)

[8]	TS 38.331 v18.2.0	NR; Radio Resource Control (RRC) protocol specification (Release 18)

[9]	TR 23.700-33 V1.1.0 (2022-10)  Study on system enhancement for Proximity based Services (ProSe) in the 5G System (5GS); Phase 2 (Release 18)

[10]	R2-2213121	Summary of agenda item 8.9.2 on UE-to-UE relay	vivo