3GPP TSG-RAN WG2 #122	R2-2306435

Incheon, South Korea, May 22 – 26, 2023

Agenda Item:	7.1.3

Source:	Ericsson

Title:	Remaining issues for NCR

Document for:	Discussion, Decision

# 1	Introduction

From RAN2#121bis-e, the following was left to be discussed during the present meeting:

Proposal 4: (11/14) If needed, beam monitoring for backhaul link when NCR-MT in RRC_INACTIVE state can be done by implementation. E///: not against, we have question about what “implementation” means QCOM: what happens in NCR-FWD selects a new beam while in RRC_INACTIVE? ZTE: it is up to the network to send UE to RRC_INACTIVE, and the network should be aware of the situation (e.g. whether the beam can change) and in that case it can keep the UE in RRC_CONNECTED. QCOM: can we agree that NCR-FWD is off in this case E///: We would like to make sure that the beam is not changed during INACTIVE without the network to know - Can be discussed in the next meeting based on contributions. Proposal 11: The handling of OAM configured allowed cell list and forbidden cell list is already captured by RAN3 in Stage 2 TS 38.300 spec. No need to specify it in RAN2 spec. Intel: we have a concern about not capturing UE behaviour in RAN2 specs ZTE: the configuration is provided by OAM, and usually we do not capture OAM-related operation in 38.304 - Can be discussed in the next meeting

In this contribution we address these issues.

# 2	Discussion

## 2.1	Beam monitoring for backhaul link when in RRC_INACTIVE

During RAN2#121bis-e, two options were discussed to address backhaul link monitoring when the NCR-MT is in RRC_INACTIVE:

- Option 1: the NCR-MT in RRC_INACTIVE state may perform backhaul beam monitoring.

- Option 2: the NCR-MT may not perform backhaul beam monitoring, so the “responsibility” could lie on the gNB.

As observed in RAN2#121bis-e’s offline (see R2-2304420), there was a clear majority to support Option 2, as Option 1 appears to be an optimization and as this solution, arguably, comes with a greater specification workload/impact with only one meeting remaining.

However, according to our understanding both options are up to the UE and network implementation and there may be no need to specifying anything. So, to reduce conflicting debates, we would be fine to agree to the original proposal discussed during last meeting’s CB session.

Agree that if needed, beam monitoring for backhaul link when NCR-MT in RRC_INACTIVE state can be done by implementation.

## 2.2	OAM configured allowed and forbidden cell lists

The following Proposals were treated with lower priority during RAN2#121bis-e’s offline (see R2-2304420):

Proposal 8: Cells in forbidden cell list (if configured) are considered as barred for NCR-MT. Cells not in allowed cell list (if configured) are considered as barred for NCR-MT. Proposal 9: NCR-MT prioritization of cells for reselection is based on configured allowed and/or forbidden cell lists. [Rapp-ZTE]: P8 and P9 are both related to the “allowed/forbidden cell list” agreed in RAN3. Based on RAN3’s conclusion, the cell list may be configured via OAM. We understand technically, P8/P9 are aligned with RAN3’s intention when introducing the “allowed/forbidden cell list”, however, we think there is no need capture it explicitly in stage 3 as they are all OAM initiated and controlled behaviours.

As captured by the email discussion’s Rapporteur, we also agree there is no need to capture the above in Stage 3. And since RAN3 already captures the followings in Stage 2 specification (TS 38.300):

“A NCR may be configured with a list of allowed gNB cell(s) that the NCR-MT is allowed to connect with, and/or a list of forbidden gNB cell(s) that the NCR-MT is not allowed to connect with.”

Then we believe that RAN2 can agree to the original proposal discussed online.

Since already captured by RAN3 in TS 38.300, RAN2 agree that there is no need to further specify the handling of OAM-configured allowed and forbidden cell lists spec.

# 3	Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	Agree that if needed, beam monitoring for backhaul link when NCR-MT in RRC_INACTIVE state can be done by implementation.

Proposal 2	Since already captured by RAN3 in TS 38.300, RAN2 agree that there is no need to further specify the handling of OAM-configured allowed and forbidden cell lists spec.

# 4	References

R2-2304209, “Report from NC Repeater breakout session”, Session chair (Apple), RAN2#121bis-e, April 2023

R2-2304420	, “Summary of [AT121bis-e][704][NCR] NCR RRC running CR”, ZTE, RAN2#121bis-e, April 2023