3GPP TSG-RAN WG2 Meeting #122	R2-2306436

Incheon, South Korea, 22nd – 26th May 2023

Agenda Item:	7.1.3

Source:	Ericsson

Title:	Discussion on transitioning from IDLE to CONNECTED

Document for:	Discussion, Decision

# 1	Introduction

During RAN2#121 the following was agreed:

- The network should be able to send NCR-MT to RRC_IDLE

This topic was also discussed during the last RAN2#121-bis-e meeting, but no consensus has been reached and the topic was postponed:

Proposal 1:	In Rel-18, do not define “wake-up timer” IE in RRCRelease message, if needed, it can be done via OAM (no specification impact)

Samsung: we would like to ensure network control in a simple manner, NAS can trigger service request or registration request and we think we need to send LS to CT1

Intel: NCR-WRD is off when in RRC_IDLE, so the original intention is no longer valid and the only purpose is to bring it back to RRC_CONNECTED. We think there are multiple implementation-specific methods to achieve that. No time to send LS to CT1.

E///: the main motivation against the timer is NAS impact, but the impact appears to be minor. If NAS impact is large we would agree for an OAM solution.

HW: the biggest issue is not NAS impact, but the motivation to have such timer.

vivo: the timer should be handled in AS, and then indicate when the timer expires to NAS. OAM is static and cannot be used in this case, so we prefer gNB control.

Nokia: in same cases it may be handled by OAM; RAN3 agreed that OAM is supported.

CATT: whether interoperability is the key issue under NCR scope?

ZTE: both solutions can work

To be continued in the next meeting

Hence, in this contribution we address this issue and propose a solution.

# 2	Discussion

In legacy UE-Network operation, when the network decides to send a UE in RRC_IDLE, this UE will transition back to RRC_CONNECTED mainly for two reasons:

- the UE has UL data to send to the network and thus will initiate an RRC setup procedure, or

- the UE is paged by the network (e.g., because the network has DL data to send to the UE) and this will trigger in turn the UE to initiate an RRC setup procedure.

Nevertheless, the NCR-MT has no traffic itself since it only supports control plane traffic, and it simply relays data from/to the gNB to/from the UE.

So, given the fact that for a UE in RRC_IDLE only CN-initiated paging is supported, this means that the AMF will never initiate a paging procedure for NCR-MT in RRC_IDLE.

Since the NCR does not have its own traffic, a CN-initiate paging for NCR-MT in RRC_IDLE will never happen.

There are then different solutions to overcome this issue. The following can be applied:

- When sending the NCR-MT in RRC_IDLE, a new timer is included in the RRCRelease message so that the NCR-MT is aware of when it should initiate the RRC setup procedure. Eventually, the timer may be also broadcasted in system information for those cases in which the NCR-MT will go to RRC_IDLE without any explicit signalling from the network.

- The NG-RAN node that has sent the NCR-MT in RRC_IDLE sends a request to core network (e.g., AMF) to initiate a CN-initiate paging procedure with the NCR-MT.

- The NCR-MT is pre-configured (e.g., by OAM) to know when it needs to be in RRC_CONNECTED.

According to the listed options, Option 2 requires a new signalling over the NG interface (which is not in the scope of the WID). Also, the NG-RAN that requests the paging to the core network is the last node to have the NCR-MT in RRC_CONNECTED, but it may not be the same node later. This brings further complexity to this solution.

Introducing a new signalling over the NG interface to request the core network to page the NCR-MT is not desirable, as it is complex and needs the involvement of SA and CT (which are not part of the WID).

Regarding Option 3, this solution provides the less flexibility since OAM is not done to handle dynamic changes to the configuration. For this release this solution may be fine since the NCR is considered anyway to be a static node, but it would be difficult to scale this solution in scenarios where the NCR(-MT) is moving.

Also, RAN2 agreed that OAM traffic goes via DRB(s) to the NCR-MT, and when the NCR-MT is in RRC_IDLE there is no active DRB. Therefore, this solution may require further standardization efforts to be adopted. Further, the feasibility of this solution would require input from RAN3 and SA5.

The NCR-MT may be pre-configured to know when it needs to be in RRC_CONNECTED (e.g., via OAM) but this solution implies no flexibility and is also difficult to scale in case the NCR-MT is mobile.

Finally, one of the main arguments on having the timer handled by OAM, was that this requires some heavy involvement from CT1. This is indeed true, but the impact on TS 24.501 does not seems to be substantial. Considering that legacy procedure already allow the RRC establishment procedure to be triggered by upper layer at the UE, the only impact that CT1 would need to capture in TS 24.501 is a new triggering condition related to an NCR-MT.

If the RRC wake-up is agreed, the impact on TS 24.501 is very limited.

--------------------------------------- TS 24.501 ---------------------------------------------------

## 5.6	5GMM connection management procedures

### 5.6.1	Service request procedure

#### 5.6.1.1	General

[Text omitted]…

The UE shall invoke the service request procedure when:

a)	the UE, in 5GMM-IDLE mode over 3GPP access, receives a paging request from the network;

NOTE 3:	As an implementation option, the MUSIM UE is allowed to not invoke service request to respond to paging based on the information available in the paging message, e.g. voice service indication.

b)	the UE, in 5GMM-CONNECTED mode over 3GPP access, receives a notification from the network with access type indicating non-3GPP access;

c)	the UE, in 5GMM-IDLE mode over 3GPP access, has uplink signalling pending (except in case i);

d)	the UE, in 5GMM-IDLE mode over 3GPP access, has uplink user data pending (except in case j);

e)	the UE, in 5GMM-CONNECTED mode or in 5GMM-CONNECTED mode with RRC inactive indication, has user data pending due to no user-plane resources established for PDU session(s) used for user data transport;

f)	the UE in 5GMM-IDLE mode over non-3GPP access, with T3346 not active or upon expiry of T3346, receives or has already received an indication from the lower layers of non-3GPP access, that the access stratum connection is established between the UE and the network;

g)	the UE, in 5GMM-IDLE mode over 3GPP access, receives a notification from the network with access type indicating 3GPP access when the UE is in 5GMM-CONNECTED mode over non-3GPP access;

h)	the UE, in 5GMM-IDLE, 5GMM-CONNECTED mode over 3GPP access, or 5GMM-CONNECTED mode with RRC inactive indication, receives a request from the upper layers to perform emergency services fallback and performs emergency services fallback as specified in subclause 4.13.4.2 of 3GPP TS 23.502 [9];

i)	the UE, in 5GMM-CONNECTED mode over 3GPP access or in 5GMM-CONNECTED mode with RRC inactive indication, receives a fallback indication from the lower layers (see subclauses 5.3.1.2 and 5.3.1.4) and the UE has a pending NAS procedure other than a registration, service request, or de-registration procedure;

j)	the UE, in 5GMM-CONNECTED mode over 3GPP access or in 5GMM-CONNECTED mode with RRC inactive indication, receives a fallback indication from the lower layers (see subclauses 5.3.1.2 and 5.3.1.4) and the UE has pending uplink user data for PDU session(s) with user-plane resources already established but no pending NAS procedure;

k)	the UE, in 5GMM-CONNECTED mode and has a NAS signalling connection only, is using 5GS services with control plane CIoT 5GS optimization and has pending user data to be sent via user-plane resources;

l)	the UE in 5GMM-IDLE mode over 3GPP access has to request resources for V2X communication over PC5 (see 3GPP TS 23.287 [6C]);

m)	the network supports the paging restriction, and the MUSIM UE in 5GMM-IDLE mode is requesting the network to remove the paging restriction;

n)	the UE in 5GMM-IDLE mode over 3GPP access

-	has to request resources for 5G ProSe direct discovery over PC5 or 5G ProSe direct communication over PC5 (see 3GPP TS 23.304 [6E]); or

-	acts as a 5G ProSe layer-2 UE-to-network relay UE and receives a trigger from lower layers to establish the NAS signalling connection (see 3GPP TS 23.304 [6E]);

o)	the network supports the N1 NAS signalling connection release, the MUSIM UE,

-	is in 5GMM-CONNECTED mode, requests the network to release the NAS signalling connection and, if the network supports the paging restriction, optionally includes paging restriction;

-	is in 5GMM-CONNECTED mode with RRC inactive indication, requests the network to release the NAS signalling connection and, if the network supports the paging restriction, optionally includes paging restriction; or

-	is in 5GMM-CONNECTED mode with RRC inactive indication, rejects the RAN paging, requests the network to release the NAS signalling connection and, if the network supports the paging restriction, optionally includes paging restriction; or

p)	the network supports the reject paging request, the MUSIM UE in 5GMM-IDLE mode when responding to paging rejects the paging request from the network, requests the network to release the NAS signalling connection and, if the network supports the paging restriction, optionally includes paging restriction.

q)	the UE supporting the reconnection to the network due to RAN timing synchronization status change has been requested to reconnect to the network upon receiving an indication of a change in the RAN timing synchronization status (see subclauses 5.4.4.2, 5.5.1.2.4, and 5.5.1.3.4) and the UE in 5GMM-IDLE mode receives an indication of a change in the RAN timing synchronization status.

r)	the UE operating as an NCR-node, in 5GMM-IDLE mode, receives an indication from lower layer;

[Text omitted]…

----------------------------------------------------------------------------------------------------------

From the above, Option 1 seems to be the most suitable for the scope of this WI. Indeed, it just requires introducing a timer that, upon its expiry, indicates to the NCR-MT that it should initiate the RRC setup procedure.

A new timer is introduced in the RRCRelease message to indicate to the NCR-MT when it should transition to RRC_CONNECTED.

Then what would remain is to define the behaviour when the timer has not yet expired, and the NCR-MT reselects a cell while in IDLE. On this matter, we believe that the NCR-MT should follow what is captured in the following agreement:

- After cell reselection, the NCR-MT to resume so that it can receive side-control configuration from the new gNB (can be done by network configuration using existing specifications). [...]

Hence, for this case, the NCR-MT shall stop the timer and transit to CONNECTED state, as the old’s gNB configuration (or for example the original intention for this NCR-MT to be in IDLE) does not apply any longer.

The NCR-MT shall stop the timer and transit to CONNECTED state when it performs cell reselection in RRC_IDLE state.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	Since the NCR does not have its own traffic, a CN-initiate paging for NCR-MT in RRC_IDLE will never happen.

Observation 2	Introducing a new signalling over the NG interface to request the core network to page the NCR-MT is not desirable, as it is complex and needs the involvement of SA and CT (which are not part of the WID).

Observation 3	The NCR-MT may be pre-configured to know when it needs to be in RRC_CONNECTED (e.g., via OAM) but this solution implies no flexibility and is also difficult to scale in case the NCR-MT is mobile.

Observation 4	If the RRC wake-up is agreed, the impact on TS 24.501 is very limited.

Based on the discussion in the previous sections we propose the following:

Proposal 1	A new timer is introduced in the RRCRelease message to indicate to the NCR-MT when it should transition to RRC_CONNECTED.

Proposal 2	The NCR-MT shall stop the timer and transit to CONNECTED state when it performs cell reselection in RRC_IDLE state.

# 4	References

RP-230175, “Revised WID on NR network-controlled repeaters”, RAN#99, Rotterdam, Netherlands, March 2023

R2-2301909, “Report from NC Repeater breakout session”, Session Chair (Apple), RAN2#121, February 2023, Athens, Greece

R2-2304209, “Report from NC Repeater breakout session”, Session chair (Apple), RAN2#121bis-e, April 2023

R2-2304420	, “Summary of [AT121bis-e][704][NCR] NCR RRC running CR”, ZTE, RAN2#121bis-e, April 2023