3GPP TSG-RAN WG2 Meeting #122	R2-2306441

Incheon, South Korea, May 22 – 26, 2023

Agenda Item:	7.25.3

Source:	Ericsson

Title:	Discussion for NPN Rel-18

Document for:	Discussion, Decision

# 1	Introduction

RAN#99 agreed the WI for eNPN in Rel-18 (see RP-230788), with the following description of the objectives for RAN2:

The objective of this work item is to perform normative work for RAN functionalities required to support 5GS functions of the SA2 WID “Enhanced support of Non-Public Networks Phase 2”, with the following features being of RAN relevance: - Support for enhanced mobility by enabling support for idle and connected mode mobility between SNPNs without new network selection [RAN3, RAN2] - Support for non-3GPP access for SNPN [RAN3]

While during RAN2#121bis-e the following was agreed:

- RAN2 assumes that the following need to be addressed For TS 38.304, the impact to RAN2 is on the following to extend equivalent PLMN to also SNPN: - Equivalent SNPN list definition - NAS interactions with AS for equivalent SNPN - Suitable cell definition while operating in SNPN access mode - Suitability check - Intra-frequency Reselection Indication For TS 38.331, to extend the cell barring for IAB to also if the selected SNPN is equivalent SNPN.

This document describes the necessary changes to TS 38.300, TS 38.304 and 38.331.

# 2	Discussion

## 2.1	Support of equivalent SNPNs

Support of equivalent SNPNs follows the same principles as support of equivalent PLMNs.

Note, that SA2 decided that (for Rel-18) mobility between SNPNs and PLMNs is not supported, i.e., it is not allowed to include a mix of equivalent PLMNs and equivalent SNPNs in the Mobility Restriction List. As this is sufficiently specified in TS 23.501 [2] clause 5.18.2a, it is recommended to not duplicate the statements in RAN TSs.

Selection of a target PLMN at handover is specified in TS 38.300 and that specification text is replicated for specifying the same behaviour for equivalent SNPNs.

As for DC, current TS 38.300 [3] clause 16.6.1 specifies that:

NR-NR Dual Connectivity within a single SNPN is supported.

With eSNPNs, it should be straight forward to remove this restriction and allow NR-NR DC within the eSNPNs. It is proposed to not remove the statement but rather leave an explicit statement as follows:

NR-NR Dual Connectivity within the SNPNs identified within the mobility restrictions is supported.

On stage 3, TS 38.304, and since mobility between SNPNs and PLMNs is not supported, the changes then involve:

- updating the general description of RRC_IDLE and RRC_INACTIVE states,

- the criteria to categorize a cell, and

- the impacts of the latter matters towards the cell selection and reselection processes.

Then finally, for TS 38.331, as per the agreement reached during RAN2#121bis-e, the change would involve extending the cell barring for IAB if the selected SNPN is also an equivalent SNPN.

### 2.1.1	Way forward

Specification work along the agreements made in SA2 seem to be straight forward and are already reflected in the understanding and agreements reached in RAN2#121bis-e. We thus propose the following.

Review the provided (draft) CR material in R2-2306442, R2-2306443, and R2-2306444. If agreeable, endorse them.

# 3	Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	Review the provided (draft) CR material in R2-2306442, R2-2306443, and R2-2306444. If agreeable, endorse them.

# 4	References

RP-230788, “New WID on Further Enhancement of Private Network Support for NG-RAN”, China Telecom, RAN #99, Rotterdam, Netherlands, March 2023

3GPP TS 23.501 v18.1.0, “System architecture for the 5G System (5GS); Stage 2”.

3GPP TS 38.300 v17.4.0, “NR; NR and NG-RAN Overall Description; Stage 2”.