3GPP TSG-RAN WG2 #122	R2-2306450

Incheon, South Korea, May 22nd – 26th 2023

Agenda Item:	7.13.5

Source:	Ericsson

Title:	Enhancements of SON reports for NR-U

Document for:	Discussion, Decision

# 1	Introduction

In this paper, we discuss required enhancements that can be introduced in the SON/MDT framework to optimize the NR-U system in particular enhancements needed for RA-Report, RLF report and the SHR.

This topic was discussed in RAN2#121-bis during the email discussion in [1].

# 2	Discussion

Related to NR-U, the following agreements were taken so far:

Agreements from RAN2#119-bis-e:

1	The UE will log information of multiple RA procedures related to consistent LBT failures. FFS details.

Agreements:

1	Introduce a new raPurpose in the RA-Report to indicate that the RA was initiated following a “consistent LBT failures” in the SpCell.

2	RAN2 agree to log kind of “the number of LBT failures” in the RA report.

LBT failure is the failure to access the channel before transmission.

The definition of “the number of LBT failures” should be clarified.

FFS how to log the number of LBT failures in the RA report.

Agreements from RAN2#121:

1: 	Log the last successful RA procedure related information in the RA report. Only some information to be logged for multiple successive RA procedures failed due to LBT issue. FFS what information.

In the following, we further discuss the possible enhancements in the RA-Report, RLF report as well as SHR, taking into account the agreements above.

### 2.1 Enhancements to the RA-InformationCommon

Multiple RA procedures (performed as part of LBT recovery) may belong to multiple BWPs, hence it would be beneficial to include in the RA-Report the multiple RA information associated the random access attempts performed in the various BWPs, i.e. a list of RA-InformationCommon IEs.

Since there are concerns on the amount of information that may need to be conveyed in the RA-Report, it was agreed in RAN2#121 that the UE should log only the complete information in the RA-InformationCommon for the last successful RA procedure in the RA-Report. On the other hand, for the other BWPs in which the UE performed RA, some limited amount of information will be logged.
We believe that the same principles should be applied also the RA-InformationCommon included in the RLF-Report.

In case the UE experiences consistent LBT failures in multiple BWPs of the PCell prior the RLF/HOF, the UE logs in the RLF the entire RA-InformationCommon associated to the random access attempts performed in the last BWP, and some limited information for the other BWPs in which the UE experienced consistent LBT failures prior the RLF/HOF.

More specifically, for the other BWPs the UE should log at least a minimum amount of information to allow the network to identify the BWPs. Hence, we believe that at least the locationAndBandwidth, and the subcarrierSpacing should be included, whereas the absoluteFrequencyPointA should be common for all the BWPs for a given UE.

For each BWP in which the UE experienced the consistent LBT failure, the UE includes in the RA-InformationCommon at least the locationAndBandwidth, and the subcarrierSpacing of the BWP.

It seems natural for the UE to include the information associated to the various BWPs in which consistent LBT failures were experienced in the chronological order of operation, so that the network can retrieve the order in which the UE performed the RA procedures, and in particular which was the first BWP in which the UE was operating when it first got consistent LBT failures. If the first BWP is not included, the NW will not know which was the first BWP in which the UE was operating when it experienced the first consistent LBT failure, because in that BWP the UE may have not performed any RA. Hence it is important to log in the RA-Report some info associated to this first BWP (the locationAndBandwidth and the subcarrierSpacing would be sufficient we believe). No need to log non-PRACH information.

The UE includes in the RA-Report the BWP information (proposed in Proposal 2) associated to the first BWP in which the UE was operating when it first experienced the consistent LBT failures.

The various BWPs in which the UE experienced the consistent LBT failures prior the RLF/HOF/successful RA, are logged in the RA-InformationCommon in chronological order.

#### 2.1.1 How to represent the number of LBT failures in the RA-Report

In the email discussion [1], it was discussed whether a random access attempt which was blocked by LBT should be represented in the perRAAttemptInfoList within the perRAInfoList.

There is a large majority of companies that prefer logging in the “per RA attempt info list” for the last BWP, only the random access attempts for which the LBT was successful, i.e. the random access attempts blocked by LBT are not included as separate entries in the “per RA attempt info list”.

Only the preamble transmission attempts for which LBT was successful are included in the “per RA attempt info list” for a given beam of the last BWP in which the UE performed random access.

As highlighted in the email discussion [1], the above Proposal 5 may lead to an issue that RAN2 should take into account when designing the content of the “per RA attempt info list”. 
In particular, we would like to highlight that in legacy the entries of the perRAAttemptInfoList are included in chronological order of attempts. That is an important legacy feature in NR, since that allows the network to further analyze the random access problems, e.g. the network can estimate when the UE has performed the power ramping and can estimate whether the UE would have reached the max transmission power or not, so that the network can optimize the RA parameters (e.g. the preamble target power, the power ramping steps).
However, if the above Proposal 5 is agreed, the chronological order of RA attempts in the perRAAttemptInfoList will not be maintained since the failed random access attempts will not be included in that list.

To better explain the consequence of the above Proposal 5, let´s consider the following TS 38.321 excerpt:

From TS 38.321: The MAC entity shall, for each Random Access Preamble: 1> if PREAMBLE_TRANSMISSION_COUNTER is greater than one; and 1> if the notification of suspending power ramping counter has not been received from lower layers; and 1> if LBT failure indication was not received from lower layers for the last Random Access Preamble transmission; and 1> if SSB or CSI-RS selected is not changed from the selection in the last Random Access Preamble transmission: 2> increment PREAMBLE_POWER_RAMPING_COUNTER by 1.

From the above TS 38.321 excerpt, it is clear that the UE does not increment the preamble power ramping counter if the random access attempt was blocked by LBT, or if the UE changes the SSB/CSI-RS resource.

Let´s now consider, the following example in which the UE performs RA in SSB1 and SSB2, in which some preamble transmissions passed the LBT (in green below) and some did not (in red below):

Figure 1

In SSB1, the UE ramped the power twice, i.e. once after the 1st attempt, i.e. for the 2nd attempt, and once after the 3rd attempt, i.e. for the 4th failed attempt. After another failed attempt, then the UE switches to SSB2 and it keeps the same power used for the 4th attempt. However, if the UE does not log any information related to the failed RA attempts before the beam switch in the perRAAttemptInfoList, and just indicate that there were two successful attempts in SSB1 and one attempt in SSB2, then the network will believe that power used in the 1st attempt in SSB2, is the same power used for the 3rd attempt in SSB1, which is incorrect. 
In other words, there will be a gap between the actual UE behaviour, and what the network can perceive from the “perRAAttemptInfoList”. This gap is visualized in the figure below:

Figure 2

If Proposal 5 is agreed, and no information is included “in the per RA info list” related to random access attempts blocked by LBT before a beam switch, then the network may not be able to properly assess the power ramping steps performed by the UE (as depicted in Figure 2).

In our view, to solve the above issue, it would be enough to include for the last successful random access attempt included in the “per RA attempt info list” before the beam switch, a flag indicating whether any random access attempt was blocked by LBT after this successful preamble transmission.

For the last successful random access attempt included in the “per RA attempt info list” before the beam switch, the UE includes a flag indicating whether any random access attempt was blocked by LBT after this successful preamble transmission.

Additionally, it should be discussed how to represent the number of number of preamble transmission attempts blocked by LBT. From the outcome of the email discussion in [1], we foresee three possible alternatives:

- The number of preamble transmissions blocked by LBT are represented by a parameter that counts the overall number of preamble transmissions blocked by LBT per RA procedure

- A flag is introduced for each successful random access attempt included in the perRAInfoList indicating whether there was at least a failure before the success random access attempt.

- The UE indicates for each successful attempt included in the perRAInfoList, the number of subsequent LBT failures that occurred before this successful attempt

We have a preference for option a) or c), since for option b) it is used just a single flag in each entry of the perRAAttemptInfoList which may not provide a good understanding of how many failures really occurred, and hence of the severity of the LBT issues. Moreover, we note that Option c) has the clear advantage that it allows the network to determine whether the LBT problems were due to burst-type of interference which may imply the recurrent presence of an interferer at some time. Option a) does not enable this possibility since it is just a counter of the number of failures.

For the logging of the number of LBT failures for the last BWP, RAN2 selects one of the two following options:

  - The number of preamble transmissions blocked by LBT are represented by a parameter that counts the overall number of preamble transmissions blocked by LBT per RA procedure

  - The UE indicates for each successful attempt included in the “per RA info list”, the number of subsequent LBT failures that occurred before this successful attempt

Related to the other BWPs where the UE performed random access, it is proposed in Proposal 1 and Proposal 2 to just log a minimum set of information, rather than the entire RA-InformationCommon including the perRAAttemptInfoList. Besides the locationAndBandwidth, and the subcarrierSpacing, in our view it is beneficial that the UE also includes the number of LBT failures experienced in those other BWPs in which consistent LBT failure was declared. One may claim that this value would be the same as the consistent LBT failure counter (lbt-FailureInstanceMaxCount), however we note that the lbt-FailureInstanceMaxCount may be reset during the RA, e.g. upon lbt-FailureDetectionTimer expiry, hence the failures may be more than the configured lbt-FailureInstanceMaxCount.

For each BWP of the PCell, except the last BWP, in which the UE experienced consistent LBT failures, the UE logs the number of LBT failures experienced in the BWP during the RA procedure.

#### 2.1.2 How to aid the energy detection configuration

The energy detection (ED) configuration, i.e. energyDetectionConfig, is provided to the UE as part of the channel access configuration for a serving cell. The ED configuration consists of a maximum ED threshold and of an ED threshold offset. A proper ED configuration is crucial for the UE performances in the unlicensed spectrum, because that impacts the likelihood of the UE to access the NR-U channel and at the same time it influences the coexistence of different devices in the unlicensed spectrum.

In fact, according to the TS 37.213, the UE compares detected power with the energy detection threshold (EDT) to access the channel.

- A channel access procedure is a procedure based on sensing that evaluates the availability of a channel for performing transmissions. The basic unit for sensing is a sensing slot with a duration Tsl=9us. The sensing slot duration Tsl is considered to be idle if an eNB/gNB or a UE senses the channel during the sensing slot duration, and determines that the detected power for at least 4us within the sensing slot duration is less than energy detection threshold XThresh. Otherwise, the sensing slot duration Tsl is considered to be busy.

As mentioned above UE performs channel access based on sensing that evaluates the availability of a channel for performing transmission. UE measures the detected power and determines if the channel is busy or free based on ED threshold. If the ED threshold is set to a too low value, then there it might be more difficult for a 3GPP UE to access the unlicensed spectrum in this cell, i.e. more LBT failures will be experienced, whereas if that is set to a too high value, the interference in the unlicensed spectrum may increase potentially impacting a fair coexistence with other devices.

A proper energy detection threshold configuration is crucial to ensure a fair coexistence of devices in the unlicensed spectrum, especially for the random access.

However, the network does not know the power detected by the UE. In our opinion, the information of the detected power and the applied EDT used by the UE at the time of LBT issue when executing a random-access procedure would be highly beneficial to pinpoint whether a random-access related issue was due a sub-optimal ED configuration used by the UE at the time of random-access procedure.

RAN2 includes some information per RA procedure to aid the network to properly configure the energy detection configuration.

In order to address Proposal 9, since the detected power perceived by the UE is checked against the ED configuration, it seems that one straightforward approach is for the UE to log the average detected power during an RA procedure.

Such measurements enable the RAN node to understand the dynamic of the channel and accordingly set the optimal EDT threshold based on the reported measurements. As discussed in some papers at RAN2#121 and in the email discussion [1], it is noted that the ED threshold applied by the UE may be different from the maximum ED threshold configured by the network. Including the information on the actual applied EDT may be beneficial to properly set the maximum ED threshold.

The UE includes the following information per RA procedure:

  - The UE includes the average detected power per RA procedure

  - The average applied EDT value per RA procedure.

In case Proposal 10 is not agreeable, we believe that at least the UE could log information on whether the detected power is higher than the configured maximum ED threshold. In fact, an LBT failure may be due either to a too little value of the applied ED threshold, or to a too little value of the configured maximum ED threshold. For the former case, the network cannot do much, since the ED value applied by the UE may depend on the UE implementation. On the other hand, for the latter case, the LBT failure may depend on the network implementation, and the network may then decide to adjust the configuration of the maximum ED threshold (e.g. if too many LBT failures are experienced due to detected power higher than the maximum configured ED threshold).

An LBT failure may depend on the detected power being higher than the applied ED threshold and lower than the maximum configured ED threshold, or on the detected power being only higher than the maximum configured ED threshold.

Hence, if Proposal 10 is not agreeable, RAN2 could consider the following solution to address the problem in Proposal 3.

If Proposal 10 is not agreeable, the UE indicates whether the average detected power per RA procedure was higher than the configured maximum ED threshold (maxEnergyDetectionThreshold).

Besides the detected power and the average applied EDT value, the network may also need to know the specific ED configuration provided to the UE since that can be configured in a UE-specific fashion and the network may not have this information available at the time of the RA information reporting.

The UE includes in the RA procedure information on the configured maximum ED threshold (maxEnergyDetectionThreshold).

### 2.2 Enhancements to the RLF-Report

#### 2.2.1 RSSI reporting

Related to RLF-Report, RAN2 should first address the LS R3-225241 [2] in which RAN3 indicates to RAN2 that the “RLF Report needs to be enhanced by adding the latest measured RSSI….”.
From RRC perspective, this would imply adding in the RLF-Report the measured RSSI as part of the measurement results of the last serving cell. For the case of HOF, the UE should at least include the measured RRSI of the source cell, and the RSSI of the target cell. We note that according to specification, the UE performs the RSSI and channel occupancy measurements on a frequency indicated in the associated measObject only if the measRSSI-ReportConfig is configured in the associated reportConfig. Hence, the UE should provide RSSI measurements for a certain cell only if the UE is configured to perform RSSI measurements on the corresponding frequency.

The UE includes in the RLF-Report, for the case of RLF, the latest measured RSSI as part of the measurement results of the NR-U channel of the last serving cell, if the UE is configured with measRSSI-ReportConfig for the associated frequency.

The UE includes in the RLF-Report, for the case of HOF, the latest measured RSSI of the NR-U channel of the source cell, and the latest measured RSSI of the NR-U channel of the target cell, if the UE is configured with measRSSI-ReportConfig for the associated frequency.

#### 2.2.2 RA-InformationCommon reporting in the RLF-Report

In certain scenarios the UE may include in the RLF-Report the RA-InformationCommon. Currently, this happens in case the RLF is due to random access problems and in case of HOF.

In case of HOF, as said, the UE already includes RA-InformationCommon. Hence, the new information associated to LBT failures and to the BWPs in which the RA procedures were attempted, that are proposed to be included in RA-InformationCommon in the previous section, will be enough for the network to analyze the RA issues.

In case of HOF, the UE includes the RA-InformationCommon in the RLF-Report (as in legacy) which will include NR-U related information as proposed in previous proposals.

For the case of RLF however, as previously mentioned, the UE includes the RA-InformationCommon only in case of RLF due to random access problems. However, in case of UL consistent LBT failures experienced just before the Radio Link failure, the UE may have executed multiple random-access procedures in different BWPs of the PCell, if the UE is configured with LBT recovery configuration. The information associated to all these random access procedures initiated just before the Radio Link failure while UL consistent LBT failures were triggered and not cancelled, will be lost if we follow the current specification. We believe that at least some RA information associated to those failed random access procedures may be of benefit for the network.

At the moment of RLF, if the UE had consistent UL LBT failures triggered in one or more BWPs at MAC layer, the RLF-Report includes the RA-InformationCommon in the RLF-Report for the random-access procedures that were initiated in one or more BWPs before the RLF.

We also observe that it would be useful for the network to identify the BWP in which the first consistent LBT failure is experienced. If the UE includes only the RA-InformationCommon, then the network will get to know the BWPs in which the UE attempted RA after consistent LBT failure, but the RA-InformationCommon will not contain information related to the BWP in which the UE detected the first consistent LBT failure, which may be triggered when the UE was performing non-RA related transmissions, e.g. PUSCH/PUCCH. Since the consistent LBT failure is detected per UL BWP by counting LBT failure indications, for all UL transmissions, then we believe that information associated to the initial BWP, i.e. the locationAndBandwidth, and the subcarrierSpacing, should be included in the RLF-Report.

The information related to the BWP in which the UE detected the first consistent LBT failure will be missing if the first detected LBT failure is not associated to performing RA procedure.

The UE includes in the RLF-Report the locationAndBandwidth, and the subcarrierSpacing of the BWP, in which the UE experienced the first consistent UL LBT failure.

#### 2.2.3 DL LBT issue information in the RLF-Report for HOF

The reason for HOF is the expiry of T304. In the unlicensed spectrum this might be due to UL LBT issues experiencing while performing RACH towards the target cell, or by potential DL problems at the gNB. In fact, according to RAN4 specification, before starting a PRACH transmission the UE will suffer an interruption time, e.g. for searching the target cell (if that is unknown), or for fine time tracking and acquiring full timing information of the target cell. This interruption time is thus impacted by the lack of SMTC occasion detected at the UE side, due to LBT issues at the gNB side.

In particular, according to the TS 38.133 (Section 6.1B), the UE is enabled to detect lack of DL reference signal transmission while performing HO.

In the requirements of clause 6.1B.1, the term SMTC occasion not available at the UE refers to when the SMTC contains SSBs configured by gNB in a cell on a carrier frequency subject to CCA, but the first two successive candidate SSB positions for the same SSB index within the discovery burst transmission window are not available at the UE due to DL CCA failures at gNB during the corresponding detection or time tracking period; otherwise the SMTC occasion is considered as available at the UE.

According to RAN4 specification (TS 38.133), the unavailability of SMTC occasions impact the HO interruption time and hence it can potentially contribute to the HOF.

The unavailability of the SMTC occasion may also affect the RLF in some cases. For example, in case of RLF cause due to T310 or T312 expiry, a possible unavailability of the SMTC occasions may obviously contribute to the expiry of those timers.

This issue is also under discussion in RAN3. In particular, RAN3 agreed the following:

From RAN3 chair notes: FFS on whether to enable a UE-based solution or a NW-based solution to inform the source gNB of a handover about DL LBT issues occurring at the target gNB, during a handover execution.

Knowing whether the UE was detecting unavailability of the SMTC occasions would be very valuable for the network to evaluate the impact of DL issues in the RLF/HOF. In particular, this information would allow the network to avoid misclassifications of too-early HO or HO to wrong cell. For example, let´s consider the case in which the NW has problems in transmitting SSBs, then the UE fails during the HO execution, and finally the UE reestablishes in the same source cell. This would be interpreted by the network as a too-early HO, and accordingly it may tune the mobility parameters. However, the HO failure was not due to wrong mobility configuration at network side, rather that it was due to LBT problems experienced by the network (which may just temporarily occurred during the HO). The same issue may occur in case of RLF, e.g. the UE may experience an RLF due to DL LBT issues at the network (which may lead for example to the expiry of the T310 or T312), and then the network may classify this HO as too-late HO, whereas in fact the main source of the RLF was DL LBT issues experienced in the gNB.

Information on unavailability of the SMTC occasions would be important to avoid misclassification of RLF/HOF, e.g. the network may classify an HOF as too-early HO and then tune mobility parameters, whereas the main problem during the HO was the DL LBT issues experienced by the gNB.

One may claim that the target cell may provide information on DL LBT problems to the source cell, however, if the RLF-Report does not contain any information on this regard, then it will not be possible for the network to correlate the content of the RLF-Report with such DL LBT failures provided by the target cell. As a consequence, the network may include in the above HO/RLF misclassification.

The UE indicates in the RLF-Report whether the UE detected unavailable SMTC occasions while T304/T310/T312 was running right before the RLF/HOF.

In case companies are not sure whether the UE is really capable to detect unavailability of SMTC occasion, we propose sending an LS to RAN4 asking for clarifications.

If Proposal 18 cannot be agreed, RAN2 to send an LS to RAN4, asking if the UE is capable to detect unavailability of the SMTC occasions during HO to a target NR-U cell, and while connected to a NR-U cell (LS available in the Annex).

#### 2.2.4 On the inclusion of lbt-FailureRecoveryConfig in the RLF-Report

An LS [2] was sent to RAN3 asking the possibilities of keeping the LBT failure recovery configuration at the network side. In the reply [3] to LS, RAN3 has replied that although one similar mechanism exists in RAN3, it would be practically minimal to store such information as LBT recovery procedure for a long time since that limits the network functionality. This is shown on the following excerpt of RAN3 LS [2].

RAN3 observes though, that the above mechanism was designed with intention to retrieve UE context or the configuration information if the UE attempts reconnection and reports the failure right after connection failure; if the failure information is fetched from the UE hours after the failure, then the likelihood that the source and the last serving node can retrieve the needed information depends on RAN implementation and is practically minimal (this depends on RAN implementation, e.g., how long the gNB stores the UE context or how long that allocated C-RNTI is not reused by the RAN).

According to RAN3, a network-based solution to store LBT-Recovery config would be limiting network functionality and thus less practical in particular when re-establishment procedure fails.

In order to minimize the UE impact and the size of the RLF-Report, we propose a UE-based solution where UE can store the LBT-Recovery configuration information in the RLF report only in case of re-establishment procedure failure after the RLF.

UE logs the lbt-FailureRecoveryConfig in the RLF-Report, only in case of re-establishment procedure failure (i.e., when UE performs transition to RRC_IDLE state).

### 2.3 Enhancements to the SHR

For the SHR, one immediate enhancement that RAN2 could consider is whether to introduce for the NR-U system some new triggering conditions for the SHR generation. For example, the HO may be successful but during the HO the UE may have experienced some LBT problems, e.g. UL consistent LBT failures were detected, or the number of UL LBT failures were higher than a certain threshold. Logging this information may be beneficial for the network to optimize the HO, and avoid problems during the HO.

Introduce “the number of UL LBT failure prior to successfully completion of the HO” as a new SHR triggering conditions for NR-U.

Similar to the RLF case, in the current specification the UE logs the RA information in the SHR, only when the SHR is triggered due to T304 timer value becoming larger than a certain threshold. We note also for this case, that the UE may experience LBT failures or even trigger random access in multiple BWPs due to consistent UL LBT failures just before successfully completing the HO. The information associated to LBT failures during RA and all the possible random access procedures initiated just before the successful HO completion, may be of interest.

In addition, similar to the RLF report, we believe addition of the non-available DL SSB at the target cell may help the network to understand the impact of DL LBT in the UE HO procedure and in particular on the HO interruption time.

SHR includes information associated to the random access procedure performed at HO, similar to RA- and RLF-Report, i.e. the number of UL LBT failures during HO (depending on the outcome of Proposal 7), and the information on the multiple BWPs in which consistent UL LBT failures was triggered (as per Proposal 2) prior to successful HO completion.

UE includes in the SHR the number of unavailable SMTC occasions detected during the HO.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	If Proposal 5 is agreed, and no information is included “in the per RA info list” related to random access attempts blocked by LBT before a beam switch, then the network may not be able to properly assess the power ramping steps performed by the UE (as depicted in Figure 2).

Observation 2	A proper energy detection threshold configuration is crucial to ensure a fair coexistence of devices in the unlicensed spectrum, especially for the random access.

Observation 3	An LBT failure may depend on the detected power being higher than the applied ED threshold and lower than the maximum configured ED threshold, or on the detected power being only higher than the maximum configured ED threshold.

Observation 4	The information related to the BWP in which the UE detected the first consistent LBT failure will be missing if the first detected LBT failure is not associated to performing RA procedure.

Observation 5	According to RAN4 specification (TS 38.133), the unavailability of SMTC occasions impact the HO interruption time and hence it can potentially contribute to the HOF.

Observation 6	Information on unavailability of the SMTC occasions would be important to avoid misclassification of RLF/HOF, e.g. the network may classify an HOF as too-early HO and then tune mobility parameters, whereas the main problem during the HO was the DL LBT issues experienced by the gNB.

Observation 7	According to RAN3, a network-based solution to store LBT-Recovery config would be limiting network functionality and thus less practical in particular when re-establishment procedure fails.

Based on the discussion in the previous sections we propose the following:

Proposal 1	In case the UE experiences consistent LBT failures in multiple BWPs of the PCell prior the RLF/HOF, the UE logs in the RLF the entire RA-InformationCommon associated to the random access attempts performed in the last BWP, and some limited information for the other BWPs in which the UE experienced consistent LBT failures prior the RLF/HOF.

Proposal 2	For each BWP in which the UE experienced the consistent LBT failure, the UE includes in the RA-InformationCommon at least the locationAndBandwidth, and the subcarrierSpacing of the BWP.

Proposal 3	The UE includes in the RA-Report the BWP information (proposed in Proposal 2) associated to the first BWP in which the UE was operating when it first experienced the consistent LBT failures.

Proposal 4	The various BWPs in which the UE experienced the consistent LBT failures prior the RLF/HOF/successful RA, are logged in the RA-InformationCommon in chronological order.

Proposal 5	Only the preamble transmission attempts for which LBT was successful are included in the “per RA attempt info list” for a given beam of the last BWP in which the UE performed random access.

Proposal 6	For the last successful random access attempt included in the “per RA attempt info list” before the beam switch, the UE includes a flag indicating whether any random access attempt was blocked by LBT after this successful preamble transmission.

Proposal 7	For the logging of the number of LBT failures for the last BWP, RAN2 selects one of the two following options:

a.	The number of preamble transmissions blocked by LBT are represented by a parameter that counts the overall number of preamble transmissions blocked by LBT per RA procedure

b.	The UE indicates for each successful attempt included in the “per RA info list”, the number of subsequent LBT failures that occurred before this successful attempt

Proposal 8	For each BWP of the PCell, except the last BWP, in which the UE experienced consistent LBT failures, the UE logs the number of LBT failures experienced in the BWP during the RA procedure.

Proposal 9	RAN2 includes some information per RA procedure to aid the network to properly configure the energy detection configuration.

Proposal 10	The UE includes the following information per RA procedure:

a.	The UE includes the average detected power per RA procedure

b.	The average applied EDT value per RA procedure.

Proposal 11	If Proposal 10 is not agreeable, the UE indicates whether the average detected power per RA procedure was higher than the configured maximum ED threshold (maxEnergyDetectionThreshold).

Proposal 12	The UE includes in the RA procedure information on the configured maximum ED threshold (maxEnergyDetectionThreshold).

Proposal 13	The UE includes in the RLF-Report, for the case of RLF, the latest measured RSSI as part of the measurement results of the NR-U channel of the last serving cell, if the UE is configured with measRSSI-ReportConfig for the associated frequency.

Proposal 14	The UE includes in the RLF-Report, for the case of HOF, the latest measured RSSI of the NR-U channel of the source cell, and the latest measured RSSI of the NR-U channel of the target cell, if the UE is configured with measRSSI-ReportConfig for the associated frequency.

Proposal 15	In case of HOF, the UE includes the RA-InformationCommon in the RLF-Report (as in legacy) which will include NR-U related information as proposed in previous proposals.

Proposal 16	At the moment of RLF, if the UE had consistent UL LBT failures triggered in one or more BWPs at MAC layer, the RLF-Report includes the RA-InformationCommon in the RLF-Report for the random-access procedures that were initiated in one or more BWPs before the RLF.

Proposal 17	The UE includes in the RLF-Report the locationAndBandwidth, and the subcarrierSpacing of the BWP, in which the UE experienced the first consistent UL LBT failure.

Proposal 18	The UE indicates in the RLF-Report whether the UE detected unavailable SMTC occasions while T304/T310/T312 was running right before the RLF/HOF.

Proposal 19	If Proposal 18 cannot be agreed, RAN2 to send an LS to RAN4, asking if the UE is capable to detect unavailability of the SMTC occasions during HO to a target NR-U cell, and while connected to a NR-U cell (LS available in the Annex).

Proposal 20	UE logs the lbt-FailureRecoveryConfig in the RLF-Report, only in case of re-establishment procedure failure (i.e., when UE performs transition to RRC_IDLE state).

Proposal 21	Introduce “the number of UL LBT failure prior to successfully completion of the HO” as a new SHR triggering conditions for NR-U.

Proposal 22	SHR includes information associated to the random access procedure performed at HO, similar to RA- and RLF-Report, i.e. the number of UL LBT failures during HO (depending on the outcome of Proposal 7), and the information on the multiple BWPs in which consistent UL LBT failures was triggered (as per Proposal 2) prior to successful HO completion.

Proposal 23	UE includes in the SHR the number of unavailable SMTC occasions detected during the HO.

# 4	References

- R2-2304200, Summary of AI 7.13.5 SON for NR-U (Ericsson), RAN2#121-bis-e

- R3-225241, LS on NR-U support for MRO, 3GPP TSG-RAN WG2 #117-e

# 5	Annex – LS to RAN4

3GPP TSG-RAN WG2 Meeting #122		TDoc R2-23xxxxx

Incheon, South Korea, 2023-05-22 – 2023-05-26

Title:	[DRAFT] LS on unavailability of SMTC occasions in NR-U

Release:	Rel.18

Work Item:	NR_ENDC_SON_MDT_enh2-Core

Source:	RAN2

To:	RAN4

Cc:	RAN3

Contact person:	Marco Belleschi (Ericsson)

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# 1	Overall description

As part of the Rel.18 WI on “Further enhancement of data collection for SON MDT in NR and EN-DC”, RAN2 is discussing the possibility for the UE to include in the RLF-Report, which is generated due to an RLF or HO failure, information on the unavailability of the SMTC occasions experienced during HO or just before the RLF.

RAN2 would like to ask RAN4 clarifications on whether the UE is capable of detecting the unavailability of the SMTC occasions during an HO to a target NR-U cell, or while connected to an NR-U cell.

# 2	Actions

To RAN4

ACTION: 	RAN2 respectfully asks RAN4 to clarify whether the UE is capable of detecting the unavailability of the SMTC occasions during an HO to target NR-U cell, or while connected to an NR-U cell.

# 3	Dates of next TSG-RAN WG2 meetings

RAN2#123		21 – 25 August, 2023	Toulouse, FR

RAN2#123-bis	09 – 13 October 2023	Xiemen, China