3GPP TSG-RAN WG2 #122	R2-2306451

Incheon, South Korea, May 22nd – 26th 2023

Agenda Item:	7.16.2.2

Source:	Ericsson

Title:	Data collection for AI/ML

Document for:	Discussion, Decision

# 1	Introduction

This is the fifth meeting on which RAN2 discusses the “AI/ML for NR Air Interface” SI (see the approved SID in RP-213599, and the revised version in RP-221348).

This is the second meeting though on which we have a sub–Agenda Item solely intended to discuss data collection aspects.

From previous meeting agreements and related discussions, in this paper we provide our views on what RAN2 should focus on when it comes to collection of data for the AIML for PHY SI, while also sharing our thoughts with respect to the previously endorsed table in R2-2302286.

# 2	Discussion

During RAN2#121 and RAN2#121-bis, RAN2 spent time analysing the existing signalling methods for the UE to provide the network with certain performed measurements. In this paper, we further analyse those methods, and we try to describe how those can be adopted (if possible) in the context of AIML.

In particular, the following agreements were taken in RAN2#121-bis:

Extend the previously endorsed table with 3 columns: Inference, Monitoring and Training, and explain in free text the applicability of the data collection method to the LCM purpose and the use case(s). P1: RAN2 to understand/determine/capture requirements of data collection for the LCM functionalities and document the results. FFS on the exact presentation format. Expect RAN1 to provide some related information. P2: RAN2 to capture the analysis (see P1 above) separately for the use-cases, i.e., CSI feedback enhancement, beam management and positioning enhancement. FFS how we do the formatting/presentation of the results. P3: Study the applicability (and limitations) of each identified data collection framework for each of the identified LCM purposes, i.e., inference, monitoring and (offline) training. FFS how we do the formatting/presentation of the results. P4: With more progress on architectural discussion, consider the suitability of each identified data collection framework for the termination points and mapping with the location of LCM purposes/functions (inference, monitoring, (offline) training) - - Model sidedness (UE side, NW side, two sided) FFS - - Use case mapping FFS P5: RAN2 to modify the previously endorsed table by adding 3 additional columns: inference; monitoring and (offline) training. Whether to, and how to further restructure the table is FFS.

## 2.1	Main data collection purposes

As agreed in our last WG meeting, RAN2 may study data collection for:

  - training,

  - performance monitoring, and

  - model inference.

However, we would like to stress that for the time being, RAN2 should especially focus on training and performance monitoring.

On the one hand, let us note that the model inference process (i.e., the process of making predictions or decisions) will depend on the input data defined by RAN1. And in principle, RAN2 do not have much of a saying on this matter. So, data collection for inference should first be handled by RAN1.

On the other hand, and while we believe that studying data collection for a functionality’s operation could very much be needed from a RAN2 standpoint, this analysis should come in due time according to the use cases’ progress.

RAN1 is responsible for handling the model inference process which depends on input data that RAN2 has little influence over.

RAN2 to now focus on data collection for training and (model/functionality) performance monitoring. If necessary, data collection for model inference could be addressed upon receiving further information from RAN1.

The three different RAN1-agreed use cases and their respective sub use cases are listed below:

- CSI feedback enhancement

  - Spatial-frequency domain CSI compression using two-sided AI model

  - Time domain CSI prediction using UE-sided model

- Beam Management (BM) enhancement

  - Spatial-domain DL beam prediction, with UE-sided or NW-sided AI model

  - Temporal DL beam prediction, with UE-sided or NW-sided AI model

- Positioning accuracy enhancement

  - Direct AI/ML positioning (i.e., the AI/ML model is directly producing the UE location as output)

  - Assisted AI/ML positioning (i.e., the AI/ML model is producing an existing or new measurement report that is used to estimate the UE location using legacy positioning methods)

  - For the above 2 points (i.e., direct/assisted AI/ML positioning), RAN1 have captured the following (sub)cases:

    - Case 1: UE-based positioning with UE-sided model, direct AI/ML or AI/ML assisted positioning

    - Case 2a: UE-assisted/LMF-based positioning with UE-sided model, AI/ML assisted positioning

    - Case 2b: UE-assisted/LMF-based positioning with LMF-sided model, direct AI/ML positioning

    - Case 3a: NG-RAN node assisted positioning with gNB-sided model, AI/ML assisted positioning

    - Case 3b: NG-RAN node assisted positioning with LMF-sided model, direct AI/ML positioning

In RAN2#121, the following was endorsed:

Proposal 1 RAN2 to simultaneously focus on studying data collection solutions for both NW- and UE-sided AIML models, including assistance signalling and (dataset) reporting from the concerning entity.

To properly assess whether the existing data collection frameworks for the AIML use cases listed above, as mentioned in at the beginning, we believe that RAN2 should first discuss what are the expectations for training and performance monitoring.

Even if discussing requirements may be hard without clearer RAN1 guideline, we believe that RAN2 can still roughly evaluate, for example, what might be the expected latency requirements of the data report, the expected information overhead that needs to be transferred, the periodicity of the reporting, etc.

In the following, we provide our analysis for each AIML data collection entity taking into account the sidedness of the model and the location of the model.

## 2.2	Requirements for NW-sided models

### 2.2.1	Training

In line with the following RAN2#121bis-e agreement:

R2 will deprioritize aspects of on-line/real-time training for the whole SI (unless R1 identifies that it is needed for one of the studied use cases).

for NW-sided model training, the intention is to enable the UE to report long term performance measurements/evaluation. Unlike performance monitoring, the training process may not require timely delivery of data to the network since the purpose of training is to enable the network to adjust parameters in the long run. Hence, in our view, for training the UE would be required to log certain information for some time and transmit it to the network when required. As a consequence, the amount of data that may be transmitted to the network may be relatively large and span measurement/predictions that the UE may have performed in a large time interval.

For NW-sided model training, latency requirements should not represent a concern. RAN2 then focuses on data information overhead when evaluating the corresponding data collection framework.

### 2.2.2	Performance monitoring

The objective of NW-sided performance monitoring is to allow the network to assess the performances of its own models or functionalities, e.g., by evaluating whether a certain configuration provided to the UE based on NW model predictions is beneficial enough considering, for example, the UE’s actual channel measurements.

Unlike training, which involves model/functionality development, the goal of performance monitoring is to evaluate if a model/functionality is working as expected. Hence, any signalling over the air interface used for performance monitoring should take into account the latency, because such signalling can be used by the network to change some configurations/operations, and the risk for potential AIML performance degradation should be minimized. Therefore, the reporting time interval for performance monitoring may need to be shorter, with potential latency concerns, and event- triggering reporting, while the volume of data per report may not be a significant factor.

For NW-sided performance monitoring, RAN2 focuses on the latency requirements when evaluating the corresponding data collection framework.

## 2.3	Requirements for UE-sided models

### 2.3.1	Training

The objective of UE model training is to assist the UE to train or enforce its AIML-enabled functionalities, e.g., via information/reports transmitted by the network to the UE.

And while the requirements for UE-sided model training should be similar to the NW-sided cases (e.g., relaxed latency), we believe that the main challenge would be to figure out what could be the information that the UE may need, while keeping the signalling framework’s complexity within acceptable boundaries.

Some information useful for training may be inferred by the UE from legacy configurations provided by the network, e.g., via broadcast or dedicated to specific UEs. While some other information may need to be signalled explicitly by the network for the sake of training. For example, the UE can use a legacy configuration to train a model, and if it needs more samples to optimize/enhance the current model, then it may provide an indication to the network with the desired configuration (e.g., via the UE Assistance Information).

Even though, it is hard to properly assess what information the UE requires to train models, we can assume that similarly to the NW-sided model training, also for UE-sided training latency requirements can be relaxed

For UE-sided model training, latency requirements should not represent a concern. RAN2 then to focus on overhead of information reporting from the UE to the NW, and on signalling configuration from NW to UE.

### 2.3.2	Performance monitoring

We believe that the UE can independently monitor its own system/link level performance (e.g., BLER of a hypothetical PDCCH transmission, number of beam failures, number of beam switches) for its beam prediction feature. By comparing these statistics to target system KPI(s), the UE can detect or predict potential performance degradation.

For UE-sided performance monitoring at UE side, RAN2 assumes that the UE can perform much of the monitoring on its own, so no need for RAN2 to focus on requirements.

However, for improved UE performance, the UE may also report measurements/events that were performed as a consequence of a certain model being used at the UE. Although the network may not know which model the UE is using, the UE can report to the network information on whether a certain AIML functionality is working properly, e.g.:

- with a certain accuracy, or

- if it is no longer applicable (i.e., it does not fulfil anymore certain requirements)

Indeed, the UE may be requested by the network to report model monitoring performance results and also declare (non)applicability of AIML functionality due to KPIs performance degradation.

This way, the network can evaluate the performance of an AIML functionality operated by the UE. If necessary, the network can de-configure the AIML function in the UE or provide the UE with a different configuration to improve the model prediction (e.g., provide a fallback configuration).

For UE-sided performance monitoring at network side, latency requirements should be taken into account, as well as potential signalling overhead for the UE to report information about UE-operated AIML functionality (e.g. its applicability/whether the functionality it is working or not, its related model performance monitoring results).

## 2.4	Two-sided models

For two-sided models, a tight coordination between UE and network may be required. To limit the scope of the discussion at this point and since this topic requires more RAN1 discussion, we propose to first focus on the one-sided models.

At this stage, RAN2 prioritizes the discussion on NW/UE-sided models over the two-sided model.

## 2.5	Analysis of identified data collection frameworks

During RAN2#121, RAN2 endorsed in R2-2302286 a table identifying potential data collection frameworks. The table serves as a starting point and could be modified according to the SI progress.

The frameworks identified by RAN2 for possible data collection used are listed in the follow:

- Logged MDT

- Immediate MDT

- L3 measurements

- L1 measurement (CSI reporting)

- UAI

- Early measurements

- LPP

### 2.5.1	NW-sided models

#### 2.5.1.1	Training

For training of NW-sided models, the suitability of the above legacy frameworks or new frameworks very much depend on the location of the training function. Hence, before discussing the suitability of the various data collection frameworks, RAN2 should first discuss the location of the training function of NW-sided models.

We note that this discussion was already taken by RAN3 in their study item. In the RAN3 TR 37.817, it is assumed that the training function of NW-sided models can be gNB-centric or OAM-centric.

From RAN3 TR 37.817: ### 5.1.2 Solutions and standard impacts #### 5.1.2.1 Locations for AI/ML Model Training and AI/ML Model Inference The following solutions can be considered for supporting AI/ML-based network energy saving: - AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB. - AI/ML Model Training and AI/ML Model Inference are both located in the gNB. Note: gNB is also allowed to continue model training based on AI/ML model trained in the OAM In case of CU-DU split architecture, the following solutions are possible: - AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU. - AI/ML Model Training and Model Inference are both located in the gNB-CU.

In TR 37.817, RAN3 considers that the AI/ML model training can be located in the OAM or in the gNB.

Given the scope of the Rel-18 RAN SI and the above RAN3 assumptions, it seems straightforward to assume that the data collection for CSI/beam management should be at least RAN-centric, whereas for positioning the LMF can also be involved (as agreed by RAN1). Hence, RAN2 should evaluate the suitability of the existing data collection frameworks (and of any other potentially new collection framework), when applied to a gNB-centric data collection. The gNB-centric data collection implies that the gNB initiates and terminates the data collection.

For training of NW-sided models, RAN2 prioritizes discussion on the suitability of data collection frameworks for gNB-centric data collection.

For training of NW-sided models, the gNB-centric data collection implies that the gNB configures the UE to transfer data and initiates/terminates a data transferring session.

Related to an OAM-centric data collection, it is not clear yet in RAN1, whereas there are use cases for physical layer parameters collection at the OAM. Nevertheless, in the study item, RAN2 can also evaluate the suitability of data collection frameworks for the case of OAM-centric data collection.

For training of NW-sided models, RAN2 evaluates the suitability of data collection frameworks for OAM-centric data collection.

For training of NW-sided models, the OAM-centric data collection implies that the OAM initiates and terminates the data collection.

Regarding the possibility of considering CN-centric or OTT centric data collection for NW-sided models, it is not clear what would be the advantage. Based on what we discuss in Section 2.5.3	"Data Collection from the UE Application” for EVEX, the objectives of the AIML-based frameworks in our RAN study item are purely targeting RAN optimizations and intended to improve (RAN) performances at the gNB and at the UE radio layers. So, it is not clear why the CN should be involved.

The CN can indeed collect data via the EVEX framework (e.g. by subscribing the application function), but today this framework does not consider the reporting of the data collection for CSI feedback enhancement, or for beam management enhancements, nor for positioning. Rather, it is just for data analytics of higher layer performances (as discussed in Section 2.5.3	Data Collection from the UE Application).

Hence, work would be required in SA groups. Additionally, there is currently no interface for the CN to transfer models to the gNB and receive feedback from the gNB, e.g., on the basis of gNB performance monitoring. Therefore, it is not clear how the interaction between CN and gNB will be possible.

For training of NW-sided models, RAN2 down prioritizes discussion on CN-centric or OTT-centric data collection.

From the discussion and expected requirements discussed above, we could draw inspiration from logged MDT’s framework for NW-sided model training. In fact, in logged MDT, the UE logs certain measurements while being in RRC_IDLE/INACTIVE and it temporarily stores those measurements until the network requests them. Since a great amount of data may have been collected by the UE, the UE can also signal to the network whether there are additional RRC segments available containing MDT info. This way the network can keep requesting the UE to transmit the remaining information.

For the other legacy signalling frameworks currently specified, only a single measurement/sample can be collected and transferred to the network, which may be a limitation for model training.

However, logged MDT can only be enabled for idle/inactive UEs, which is obviously a limitation for the AIML use cases. Additionally, logged MDT (as well as immediate MDT) is not terminated at the gNB. But the configuration is controlled by the OAM and subject to CN signalling. So, the collected data is terminated in the Trace Collection Entity (TCE). Hence, that there might be more suitable for the OAM-centric data collection, rather than for the gNB-centric data collection.

For OAM-centric data collection for NW-sided models, RAN2 considers enhancements to logged MDT, such as logging measurements in RRC Connected mode.

On the other hand, for gNB-centric data collection for NW-sided model, for the reasons mentioned above, some parts of MDT can be reused for the purposes of NW-sided model training, but the framework is not really suitable for our scope.

For gNB-centric data collection for NW-sided model, the data collection framework may incorporate some parts of logged MDT. However, MDT as such has certain properties that make it not suitable.

For gNB-centric data collection for NW-sided model, RAN2 to study a L3 data collection framework that allows the UE to measure and store a set of measurements (details up to RAN1) to be reported to the gNB upon request, and in multiple RRC segments.

#### 2.5.1.2	Performance monitoring

For performance monitoring of NW-sided models, the NW can in principle use off-the-shelf techniques to evaluate the performance of its own models, e.g., legacy L1/L3 measurements. For example, the network can monitor system/link level performance (e.g., throughput, number of beam failures, number of beam switches) of the users whose transmissions/receptions are scheduled/configured based on the NW-sided beam prediction model output. By comparing the system/link level performance statistics to a target system KPI(s), the NW can detect or predict potential system/link performance degradation, thereby, triggering LCM actions like fallback and, subsequently, perform error cause analysis if needed.

Whether there is the need to improve the current L1 reporting or L3 RRC measurements for the sake of NW-sided model monitoring should be however studied in RAN1. For example, the UE may need to be configured to collect data within a time window and then report measurement samples in the measured period, i.e., somewhat like the data collection mechanism for training, but with a different time scale.

For NW-sided performance monitoring, RAN2 waits for RAN1 input on the need to enhance the L1 reporting configuration or the L3 RRC measurement configuration and reporting.

### 2.5.2	UE-sided models

#### 2.5.2.1	Training

Similar to the discussion for the NW-sided models, also for the UE-sided models, the suitability of the legacy frameworks, listed in the beginning of Section 2.5, or any new potential framework very much depend on the location of the training function.

In our view, for UE-sided models, the natural assumption seems that the AIML training function resides in the UE and/or in an OTT server. The UE can for example perform data collection based on radio configurations and measurements to enrich a model locally stored in the UE, and/or transfer this information to an OTT server which performs the training of more general AIML models which are based on inputs received from different UEs.

An alternative option is to consider NW-centric approaches for AIML training of UE-sided models (e.g. gNB centric or CN-centric). However, the advantages of those approaches over the UE/OTT-centric approaches are not clear. The training of UE-sided models depends on radio configurations and measurements of the radio environment taken by the UE. Hence, it is not clear why the training should occur in the network. That would imply the transferring of the data collected by the UE to the network, which would then be in charge of training the models for many different UEs. Besides the large computational complexity at the network side, that would imply the need to specify the data that the UE should collect for UE-sided models. Additionally, that would also imply the need to specify a model transfer procedure from the NW (gNB/CN) to the UE, which in turn may require a very large standardization effort.

For UE-sided model training, RAN2 to consider UE-centric data collection performed OTT, i.e. no impact on RAN2 protocols.

About data to be collected, it is not clear yet which information the UE needs to acquire to train UE-sided models. That should be discussed, if needed, by RAN1, but that can also be left to the UE implementation and no specification work might be needed. 
However, irrespective of whether the specific data that the UE should collect for UE-sided training are specified or not, the UE may need at some point certain specific radio configurations, in order to collect data to further train the model. At a first glance, this task could be accomplished by the UE Assistance Information framework (UAI), but the suitability of UAI also depends on RAN1 progress.

For UE-sided model training, RAN2 considers (subject to RAN1 progress), the UE Assistance Information framework as a tool to aid the training at the UE.

#### 2.5.2.2	Performance monitoring

As discussed in Section 2.3.2, for UE-sided performance monitoring at the UE, the UE can independently monitor its own system/link level performance and it can detect or predict potential performance degradation. So RAN2 does not need to focus on data collection for UE-sided performance monitoring at the UE.

However, the UE may also report measurements/events that were performed as a consequence of a certain model being used at the UE. Although the network may not know which model the UE is using, the UE can report to the network information on whether a certain AIML functionality is working properly.

For UE-sided performance monitoring, RAN2 focuses on data collection for UE-sided performance monitoring at network side. No Uu signalling impact is expected for UE-sided performance monitoring at UE side.

The UE may report performance results of model monitoring to the NW, e.g. accuracy performances, as well as the (non)applicability of a certain AIML functionality. 
For this, a mixture of a layer-1, or layer-2, or layer-3 mechanism might be needed. Which specific layer to adopt may depend on the type of information, and on the amount of information to be reported, as well as on RAN1 discussion..

For UE-sided performance monitoring at NW side, RAN2 to focus on impacts in layer-2, or layer-3 (possibly including some layer-1 related measurements) for reporting of the outcome of performance monitoring (e.g. performance monitoring results, (non)applicability of AIML functionality). Layer-1 details are left to RAN1.

### 2.5.3	Data Collection from the UE Application

During RAN2#121 and RAN2#121bis-e, it was highlighted that SA2 has also developed a framework for data collection from the UE application. Whether that could be somehow applicable to the RAN uses cases addressed in Rel-18 SI should be discussed.

TS 23.288 envisages a set of high-level procedures by which data is collected by a Network Data Analytics Function (NWDAF) from UE Application(s) via an intermediary Application Function (AF). As described in TS 23.288, this protocol implies that the NWDAF interacts with an AF to trigger the data collection from UE Application(s) as an input for analytics generation and ML model training. In particular, the NDWAF can subscribe to the AF for UE data collection using a subscribing procedure to request certain information from the UE application. The information that can be subscribed and, hence, transferred by the UE to the NWDAF as part of this framework are described in TS 23.502 (see e.g., clause 5.2.19.2), and include for example “UE Mobility information”, “Service Experience information”, “User Data Congestion information”, etc.

Given this analysis, it appears that this data collection framework is not RAN-centric, since the configuration, and the initiation of the data collection is performed outside the RAN, as well as the data collection itself is terminated outside the RAN. Besides, the use cases currently addressed in this framework are certainly not RAN-centric, as the cases studied in RAN1 (especially the CSI and beam management use case).

How to make the data collection controllable by gNB and terminated in the gNB (where the model may reside) should be studied and addressed if this framework is used for the RAN-centric data collection.

We also note, from what is specified in TS 23.288, that user consent for analytics may be required when using this framework (similarly to MDT). The data exposed by the UE in this framework is for example: the mobility information, the service experience information, the QoS monitoring, etc., which may be sensitive privacy-wise.

On the other hand, the objectives of the AIML-based frameworks in our RAN study item are purely targeting RAN optimizations and intended to improve (RAN) performances at the gNB and at the UE radio layers.

Given the above analysis, the “Data Collection from the UE Application” may come with some limitations/assumptions that are not suitable for the RAN use cases that RAN1/2 is considering in Rel-18. If some of the RAN-centric data used for NW-sided model training needs to be exposed to the OAM, that can be studied in a later stage in conjunction with SA5/SA2, for example by enhancing the event exposure service if needed.

The framework of “data collection from the UE application” developed by SA2 is not RAN-centric from an architectural point of view, and it comes with a set of assumptions (e.g., on the use cases) which may not fit the AIML scenarios studied in RAN1/2 in Rel-18.

The current framework of “data collection from the UE application” designed in SA2/SA4 is not suitable for the RAN-centric use cases studied in the Rel-18 RAN SI.

## 2.6	Summary

To summarize the discussion in the above sections, we provide in the Annexes summary tables.

The table in the Annex summarises the expected requirements on a data collection framework to fulfil a certain LCM function.

For CSI/beam management use cases, RAN2 to agree to Table 1 in Annex A about data collection framework requirements to fulfil the different LCM functions.

The table in the Annex summarises the suitability of the various existing legacy frameworks, taking into account the sidedness of the model, and the entity terminating/initiating the data collection.

For CSI/beam management use cases, RAN2 to agree to Table 2 in Annex B which maps LCM functions to the various existing data collection frameworks considering; the sidedness of the model, and the entity terminating/initiating the data collection.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	RAN1 is responsible for handling the model inference process which depends on input data that RAN2 has little influence over.

Observation 2	In TR 37.817, RAN3 considers that the AI/ML model training can be located in the OAM or in the gNB.

Observation 3	For gNB-centric data collection for NW-sided model, the data collection framework may incorporate some parts of logged MDT. However, MDT as such has certain properties that make it not suitable.

Observation 4	The framework of “data collection from the UE application” developed by SA2 is not RAN-centric from an architectural point of view, and it comes with a set of assumptions (e.g., on the use cases) which may not fit the AIML scenarios studied in RAN1/2 in Rel-18.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 to now focus on data collection for training and (model/functionality) performance monitoring. If necessary, data collection for model inference could be addressed upon receiving further information from RAN1.

Proposal 2	For NW-sided model training, latency requirements should not represent a concern. RAN2 then focuses on data information overhead when evaluating the corresponding data collection framework.

Proposal 3	For NW-sided performance monitoring, RAN2 focuses on the latency requirements when evaluating the corresponding data collection framework.

Proposal 4	For UE-sided model training, latency requirements should not represent a concern. RAN2 then to focus on overhead of information reporting from the UE to the NW, and on signalling configuration from NW to UE.

Proposal 5	For UE-sided performance monitoring at UE side, RAN2 assumes that the UE can perform much of the monitoring on its own, so no need for RAN2 to focus on requirements.

Proposal 6	For UE-sided performance monitoring at network side, latency requirements should be taken into account, as well as potential signalling overhead for the UE to report information about UE-operated AIML functionality (e.g. its applicability/whether the functionality it is working or not, its related model performance monitoring results).

Proposal 7	At this stage, RAN2 prioritizes the discussion on NW/UE-sided models over the two-sided model.

Proposal 8	For training of NW-sided models, RAN2 prioritizes discussion on the suitability of data collection frameworks for gNB-centric data collection.

Proposal 9	For training of NW-sided models, the gNB-centric data collection implies that the gNB configures the UE to transfer data and initiates/terminates a data transferring session.

Proposal 10	For training of NW-sided models, RAN2 evaluates the suitability of data collection frameworks for OAM-centric data collection.

Proposal 11	For training of NW-sided models, the OAM-centric data collection implies that the OAM initiates and terminates the data collection.

Proposal 12	For training of NW-sided models, RAN2 down prioritizes discussion on CN-centric or OTT-centric data collection.

Proposal 13	For OAM-centric data collection for NW-sided models, RAN2 considers enhancements to logged MDT, such as logging measurements in RRC Connected mode.

Proposal 14	For gNB-centric data collection for NW-sided model, RAN2 to study a L3 data collection framework that allows the UE to measure and store a set of measurements (details up to RAN1) to be reported to the gNB upon request, and in multiple RRC segments.

Proposal 15	For NW-sided performance monitoring, RAN2 waits for RAN1 input on the need to enhance the L1 reporting configuration or the L3 RRC measurement configuration and reporting.

Proposal 16	For UE-sided model training, RAN2 to consider UE-centric data collection performed OTT, i.e. no impact on RAN2 protocols.

Proposal 17	For UE-sided model training, RAN2 considers (subject to RAN1 progress), the UE Assistance Information framework as a tool to aid the training at the UE.

Proposal 18	For UE-sided performance monitoring, RAN2 focuses on data collection for UE-sided performance monitoring at network side. No Uu signalling impact is expected for UE-sided performance monitoring at UE side.

Proposal 19	For UE-sided performance monitoring at NW side, RAN2 to focus on impacts in layer-2, or layer-3 (possibly including some layer-1 related measurements) for reporting of the outcome of performance monitoring (e.g. performance monitoring results, (non)applicability of AIML functionality). Layer-1 details are left to RAN1.

Proposal 20	The current framework of “data collection from the UE application” designed in SA2/SA4 is not suitable for the RAN-centric use cases studied in the Rel-18 RAN SI.

Proposal 21	For CSI/beam management use cases, RAN2 to agree to Table 1 in Annex A about data collection framework requirements to fulfil the different LCM functions.

Proposal 22	For CSI/beam management use cases, RAN2 to agree to Table 2 in Annex B which maps LCM functions to the various existing data collection frameworks considering; the sidedness of the model, and the entity terminating/initiating the data collection.

# 4	References

RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, TSG RAN, RAN#94-e, Dec 2021

RP-221348, “Revised SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, TSG RAN, RAN#96, Budapest, Hungary, June 2022

R2-2302286, “Summary of [AT121][025]: Progress table of analyzing data collection framework”, Apple, RAN2#121, Athens, February 2023

R1-2302063, “Session notes for 9.2 (Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface)	“, Ad-hoc Chair (CMCC), RAN1#112, Athens, February 2023

R1-2301868, “Final Summary of General Aspects of AIML Framework”, Qualcomm, RAN1#112, Athens, February 2023

# 5	Annex A – Data collection requirements

| Model sidedness | Model Training | Model Performance monitoring |
| --- | --- | --- |
| NW-sided models | High volume of data to be transmitted by the UE. No stringent latency requirements. For gNB-centric data collection, data should be available to the gNB, and it should be possible for the gNB to configure the UE to transfer data and initiate/terminate the collection session For OAM-centric data collection , data should be available to the OAM, and it should be possible for the OAM to initiate/terminate the collection session | No stringent requirements on the volume of data to be transmitted. Potentially stringent latency requirements. |
| UE-sided models | High volume of data to be transmitted over user plane. No stringent latency requirements. | For model performance monitoring at the UE, the UE can perform it on its own. For model performance monitoring at the gNB/OAM (e.g. KPIs reporting, (non)applicability of AIML functionality): - No stringent requirements on the volume of data to be transmitted - Potentially stringent latency requirements |
| Two-sided models | Note: not prioritized at this point, wait for RAN1 progress. |  |

Table 1: Requirements on data collection for the different LCM functions.

# 6	Annex B – Analysis of suitability of the legacy data collection frameworks

|  | Model Training |  |  | Model Performance monitoring |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | NW-sided |  | UE-sided (only UE-centric) | NW-sided |  | UE-sided |  |
|  | gNB-centric data collection | OAM-centric data collection |  | gNB-centric data collection | OAM-centric data collection | Monitoring at UE | Monitoring at gNB |
| Logged MDT | Not suitable: in MDT the initiation of the reporting session and the data collection termination point is in the TCE/OAM. | Suitable, provided that enhancements are at least introduced for logging in RRC-Connected mode. | Not suitable: The training is performed by the UE (possibly with the aid of OTT server). No need to report trained results to the OAM for the UE-sided training. | Not suitable: performance monitoring has stringent latency requirements. Moreover, in MDT the initiation of the reporting session and the data collection termination point is in the TCE/OAM. Additionally, logged MDT measurements are not done in RRC Connected state. | Not suitable: performance monitoring has stringent latency requirements. | Not suitable: in MDT the initiation of the reporting session and the data collection termination point is in the TCE/OAM. | Not suitable: performance monitoring has stringent latency requirements. Moreover, in MDT the initiation of the reporting session and the data collection termination point is in the TCE/OAM. |
| Immediate MDT | Not suitable: in MDT the initiation of the reporting session and the data collection termination point is in the TCE/OAM. | Suitable: in Immediate MDT, the initiation of the reporting session and the data collection termination point is in the TCE/OAM, and it works in RRC_Connected mode. Some enhancements may be potentially needed related to large data volume reporting | Not suitable: The training is performed by the UE (possibly with the aid of OTT server). No need to report trained results to the OAM for the UE-sided training. | Not suitable: in MDT the initiation of the reporting session and the data collection termination point is in the TCE/OAM. | Suitable: in Immediate MDT, the initiation of the reporting session and the data collection termination point is in the TCE/OAM, and it works in RRC_Connected mode. | Not suitable: in MDT the initiation of the reporting session and the data collection termination point is in the TCE/OAM. | Not suitable: in MDT the initiation of the reporting session and the data collection termination point is in the TCE/OAM. |
| L3 Measurements | Suitable provided that enhancements are introduced for the reporting to the gNB of multiple samples of measurements which are stored by the UE during measurement period. | Not suitable: L3 measurements are configured by the gNB and reported to the gNB. | Not suitable: The training is performed by the UE (possibly with the aid of OTT server). No need to report trained results to the gNB for the UE-sided training. | Suitable: up to RAN1 to discuss what enhancements are needed. | Not suitable: L3 measurements are configured by the gNB and reported to the gNB. | Not suitable: if the monitoring occurs at the UE, L3 measurements are not suitable because they are configured by the gNB and reported to the gNB. | Suitable: L3 measurements can be used by the UE to report its model/function applicability conditions, performance monitoring results. FFS in RAN1/2 what should be reported. |
| L1 Measurement (CSI reporting) | Not suitable: there are no stringent latency requirements for training, and data overhead can be an issue. | Not suitable: L1 measurements are configured by the gNB and reported to the gNB. | Not suitable: The training is performed by the UE (possibly with the aid of OTT server). No need to report trained results to the gNB for the UE-sided training. | Suitable: up to RAN1 to discuss what enhancements are needed. | Not suitable: L1 measurements are configured by the gNB and reported to the gNB. | Not suitable: if the monitoring occurs at the UE, L1 measurements are not suitable because they are configured by the gNB and reported to the gNB. | Not suitable: For the reporting of model/fucntion applicability conditions, accuracy performance, or desired configurations, L3 measurements or UAI seem more sutiable from RAN2 point of view. This assumption can be revisited based on RAN1 progress. |
| UAI | Not suitable: in legacy the information reported in the UAI and the triggering conditions for the reporting, are typically left to the UE implementation. Hence, it is not clear what would be the gain of UAI versus L3 measurements. | Not suitable:UAI are configured by the gNB and reported to the gNB. | Suitable: The UE can use UAI to request to the gNB some desired configurations in order to train the UE model. | Not suitable: in legacy the information reported in the UAI and the triggering conditions for the reporting, are typically left to the UE implementation. Hence, it is not clear what would be the gain of UAI versus L3/L1 measurements. | Not suitable:UAI are configured by the gNB and reported to the gNB. | Not suitable: if the monitoring occurs at the UE, UAI is not applicable because it is reported to the gNB. | Suitable: UAI can be used by the UE to report its model/function applicability conditions, performance monitoring results. FFS in RAN1/2 what should be reported. However, since L3 measurements can also be used for the same purpose, RAN2 should discuss whether L3 measurements or UAI is more suitable. |
| Early Measurements | Not suitable: in legacy the early measurements are collected while in IDLE/INACTIVE mode. So it is not clear what would be the gain of Early Measurements versus L3 measurements | Not suitable: early measurements are configured by the gNB and reported to the gNB. | Not suitable: The training is performed by the UE (possibly with the aid of OTT server). No need to report trained results to the gNB for the UE-sided training. | Not suitable: in legacy the early measurements are collected while in IDLE/INACTIVE mode, whereas performance monitoring of NW-sided models should be based on UE operations while in connected mode. | Not suitable: early measurements are configured by the gNB and reported to the gNB. | Not suitable: if the monitoring occurs at the UE, early measurements are not suitable because they are configured by the gNB and reported to the gNB. | Not suitable: in legacy the early measurements are collected while in IDLE/INACTIVE mode. So it is not clear what would be the gain of Early Measurements versus L3 measurements reporting or UAI. |
| LPP | N/A: LPP is not applicble to CSI and beam management use cases | N/A: LPP is not applicble to CSI and beam management use cases | N/A: LPP is not applicble to CSI and beam management use cases | N/A: LPP is not applicble to CSI and beam management use cases. | N/A: LPP is not applicble to CSI and beam management use cases | N/A: LPP is not applicble to CSI and beam management use cases | N/A: LPP is not applicble to CSI and beam management use cases |

Table 2: Suitability analysis of legacy data collection frameworks for LCM functions for CSI and beam management use case.