3GPP TSG-RAN WG2 Meeting #122	R2-2306453

Incheon, South Korea, 22nd – 26th May 2023	Revision of R2-2304134

Agenda item:	7.7.4.2

Source: 	Sequans Communications

Title: 	NTN-NTN handover enhancements

Document for:	Discussion and Decision

# Introduction

The Rel-18 NR NTN enhancements WID includes the following objectives:

4.1.4 NTN-TN and NTN-NTN mobility and service continuity enhancements This work considers existing methods from NR TN as well as outcome of Rel-17 NR NTN WI outcome as baseline for NTN-TN mobility. - Specify NTN-TN and NTN-NTN measurement/mobility and service continuity enhancements [RAN2,RAN3,RAN4] - […] - Specify NTN-NTN handover enhancement for RRC_CONNECTED UEs in the quasi-earth-fixed cell and earth-moving cell to reduce the signalling overhead. [RAN2, RAN3] - […]

In the past meetings, several solutions have been proposed. In this contribution, we detail our view on some of these solutions, and introduce new possible schemes.

# Discussion

As clearly expressed in the WID, the main goal of the sub-objective is to reduce the signaling overhead during handover procedure. This is linked to the fact that with earth moving and quasi earth-fixed scenarios, independently of UE mobility, the serving cell would regularly change (intra-satellite or inter-satellite cell change) implying handover of all connected UEs. In addition, feeder link change would also result in handover of all connected UEs.

The overhead issue is twofold:

- during handover preparation time, need to send HO command (possibly within CHO configuration) to all UEs in the cell

- during handover execution, random access performed by all UEs in the cell

## Group HO command/indication

The group HO command approach tries to reduce the overhead of sending similar HO commands to lots of UE in the cell by using instead a group HO command that would be addressed to a group of UEs. This is however difficult to achieve as only part of the HO configuration is common (e.g., the cell specific parameters). Most of resource configuration would depend on UE capabilities, expected QoS, etc or would require UE specific values (C-RNTI etc).

A group HO command could have more traction if all UEs would need to be configured for HO at the same time. However, the HO command signaling can already be performed well in advanced through time-based CHO (at least for quasi earth-fixed cells), which means that the corresponding signaling load can already be smoothed out.

Observation 1: “Group HO command” appears complex and less required given time-based CHO availability

A variant was proposed in which each UE would be pre-configured with HO command (e.g. through legacy CHO), and a group HO indication message would be used to trigger the execution of the handover. However, it is difficult to understand how this helps reducing the overhead since in the Rel-17 time-based CHO, there is no need of such triggering message. One rationale invoked is about “cases when CHO conditions would not have been met”, which seems a completely different issue (and it is also not clear why a HO should be forced if the conditions are not met).

Observation 2: “Group HO indication” adds overhead compared to time-based CHO

Proposal 1: “Group HO command/indication” is not pursued unless real benefits are demonstrated

## Common signalling of HO common information

### General

As agreed at RAN2#119bis-e, RAN2 can further consider whether some information in the handover command that can be common to all UEs, can be delivered to UEs in common signaling and if there is real benefit (in terms of signaling overhead reduction) in this.

In terms of overhead reduction, there is a tradeoff when using common signaling (CS). We consider a simplistic model of delivering the common information:

- Dedicated CHO: X PRBs in average per HO command, n HO commands/s => n*X PRBs/s used in average.

- CS: Y PRBs per CS occurrence, m CS occurrences/s => m*Y PRBs/s used in average, + additional signaling for group configuration

In general, Y would be larger than X as CS link adaptation cannot be optimized (compared to dedicated case). Y would need to correspond at least to the maximum of X in the group. In case CS is ensured by broadcast, Y would correspond to cell edge conditions so that all UEs in the cell would be able to decode the common information.

Regarding m, this also depends how CS is implemented. In case CS is ensured by group signaling, it would be around n/group size. In case CS is ensured by broadcast, m would need to be large enough to ensure connected UEs always have an opportunity to acquire it before handover, but small enough to not compromise the expected signaling gain.

Finally, in case of group signaling, additional signaling for group configuration should also be taken into account.

Note that this also assumes the use of delta signaling to deliver the common information. In legacy implementation, the corresponding signaling (in HO) would already use the existing delta signaling of RRC configuration (yielding above X PRBs resource in average). With CS implementation, delta signaling would need to be implemented in a similar way (yielding above Y PRBs resources in average).

Observation 3: Overhead reduction of “Common signaling” solution depends on multiple parameters and would need further analysis

In addition, we believe that an important aspect of CS is how the NW can ensure that the CS transmission was effectively received by the UE / what happens if for some reason this CS transmission was not received at the time of handover execution.

Regarding what happens if the CS transmission was not received at the time of handover, a solution would be that the UE fallbacks to reading the SIB information directly in the target cell (in a similar way as what we propose in section 2.3). That would however increase the handover interruption for such UE.

Regarding how to ensure CS transmission was received, assuming broadcast is used, there is no feedback. A CS transmission acquisition feedback message could be sent in UL (that might be limited to UEs with QoS constraints, e.g. voice call), but it doesn’t seem attractive as this would increase UL overhead (especially generating bursts of synchronized UL traffic).

Proposal 2: “Common signaling” should demonstrate real benefits and address the case where the “common signaling’ is not received at the time of HO execution

### NTN config

As detailed in our companion contribution, the “NTN config IE” is a cell specific parameter in the HO command that should be handled separately, since there is already the possibility to broadcast it in the source cell.

From AIP CR R2-2303923, our understanding is that Rel-17 specification already allows the NW to the omit NTN-config IE in the HO/CHO message. The UE will either rely on the target NTN-config IE broadcasted in the source cell, or it will reacquire SIB19 at the time of HO execution.

Proposal 3: Confirm that the NW may already omit NTN-config IE in HO/CHO and broadcast it in the source cell SIB19 (as a neighbor NTN-config)

## Omission of HO common information

An alternative which was not yet discussed would be to omit including the HO common information, while not sending it through common signalling in the source cell.

The main candidate for HO common information is the ServingCellConfigCommon IE, which contains parameters which a UE would typically acquire from SSB, MIB or SIBs when accessing the cell from IDLE. A possible scheme would then be for the NW to omit those parameters (the PCI might be the only one that would still need to be transmitted), while indicating the UE that it should acquire those parameters in the target cell. This incurs further handover interruption; however, this could be acceptable depending of the QoS / traffic pattern of the UE. For instance, one could expect that it is not a problem for delay tolerant traffic

Based on the knowledge of the QoS / traffic pattern, the NW could:

- keep including HO common information in the HO message (Rel-17 behavior), e.g. for UEs with on-going voice call

- omit HO common information in the HO message, e.g. for UEs with sporadic data transfer.

Given this would apply to UE with sporadic data transfer, it may be also possible for such UE to acquire the HO common information by reading MIB / SIB1 while in the source cell (during source/target overlap time), e.g. by reusing a mechanism similar to CGI reading.

A main advantage of this scheme is that this there is clear gains in terms of signalling overhead (with a compromise on handover interruption time, but only for UEs that would tolerate it).

Observation 4: Omission of HO common information, without common signaling, enables clear reductions of signaling overhead at the expense of increased handover interruption time

Proposal 4: Consider further “QoS-based omission of HO common information / SIB acquisition in target cell”

## Reusing PCI after satellite changes

The aim of this scheme would be to avoid handover procedure. We detail our view in our companion contribution [1].

## Multiple future NTN cells configuration / Chain of CHOs

As confirmed by RAN2, “at least for the moving cell case the next serving cells can be largely predicted in NTN (at least for UEs not at the cell edge)”. It was then proposed that a UE can be provided with CHO configurations for multiple future NTN cells.

A possible simple implementation is to provide the future CHO candidate configurations similarly as other CHO candidates, while preventing removing those candidates upon CHO execution.

Whether this helps reducing the overhead is unclear to us, as this is just moving CHO signalling of (potential) target cell N+2 that would have been sent in (potential) target cell N+1 to the current serving cell N. System wise, there is no signalling reduction (except for the reduction linked to multiplexing the configurations into the same message, that we consider neglectable and not worth the added complexity). Conversely, if the connection ends in cell N before the CHO was required, this would actually lead to additional signalling overhead. Similarly, the more the prediction in the future, the less the accuracy of the prediction and the uselessness of the provided CHO candidate configuration, given e.g. potential UE movement.

Observation 5: “Multiple future NTN cells configuration” is likely to increase signaling overhead

This solution also relies on the NW preparing in advance the configuration for future cells. Any update on the configuration of a cell M (due to specific conditions in that cells, which were not initially planned) might likely result in cancelling the configuration of cells M+1, 2, … as delta configuration is used. That is to say, the flexibility of RRC configuration is impacted. Moreover, as the main signaling overhead benefit is linked to using only one RRC configuration message in cell N instead of several smaller RRC configurations in cells N, N+1, …, having to use RRC reconfiguration in future cells N+1, … would kill the benefit of the feature.

Observation 6: “Multiple future NTN cells configuration” reduce RRC configuration flexibility

Proposal 5: “Multiple future NTN cells configuration” is not pursued unless clear gains are shown

## Random time-based CHO

In Rel-17, the serving cell stop time t-service is broadcasted in SIB19 to help cell reselection. This could be leveraged in connected as well. In the quasi-earth fixed case, for time-based CHO, the NW has to configure t1-Threshold-r17 / duration-r17 for all UEs.

In order to smooth out the handovers, the NW has to configure different time periods for each UE, within the overlapping time of source and target cells.

Instead, the overlapping time could be broadcasted (cell stop time t-service is already broadcasted, so only a cell start time offset for the target cell would be required). Then, the UEs could be configured to perform time-based CHO at a random time during the overlapping time.

Proposal 6: Consider further “Random time-based CHO” to leverage broadcasted overlapping time between source and target cells

# Conclusion

In this contribution, we make the following observations and proposals:

Observation 1: “Group HO command” appears complex and less required given time-based CHO availability

Observation 2: “Group HO indication” adds overhead compared to time-based CHO

Proposal 1: “Group HO command/indication” is not pursued unless real benefits are demonstrated

Observation 3: Overhead reduction of “Common signaling” solution depends on multiple parameters and would need further analysis

Proposal 2: “Common signaling” should demonstrate real benefits and address the case where the “common signaling’ is not received at the time of HO execution

Proposal 3: Confirm that the NW may already omit NTN-config IE in HO/CHO and broadcast it in the source cell SIB19 (as a neighbor NTN-config)

Observation 4: Omission of HO common information, without common signaling, enables clear reductions of signaling overhead at the expense of increased handover interruption time

Proposal 4: Consider further “QoS-based omission of HO common information / SIB acquisition in target cell”

Observation 5: “Multiple future NTN cells configuration” is likely to increase signaling overhead

Observation 6: “Multiple future NTN cells configuration” reduce RRC configuration flexibility

Proposal 5: “Multiple future NTN cells configuration” is not pursued unless clear gains are shown

Proposal 6: Consider further “Random time-based CHO” to leverage broadcasted overlapping time between source and target cells

# References

- R2-2306517, ““Unchanged PCI” solution vs “PCI change only” solution”, Sequans Communications