3GPP TSG-RAN WG2 Meeting #122						  R2-2306482

Online, 22nd May– 26th May, 2023

Agenda item	: 7.22.2

Source		: Samsung

Title		: On impact to UE IDLE/INACTIVE Procedure to support LP-WUR

Document for	: Discussion & Decision

# 1	Introduction

In release 18, proposed study item for low-power Wake-up Signal and Receiver for NR [1], the following RAN2 agreements were made in R2-121bis-e,

Aim to do every Q: Collect RAN2 text proposals in a single document during the following meeting(s) and send the document to RAN1 as the input to the TR 38.869.

Confirm that we follow R1 and include RRC idle/inactive/connected.

Ultra-deep-sleep = R2 understands for now that this is a power saving state (introduced by R1) to denote a state when the Main Receiver (MR) may sleep/turn off.

In scope: Use LPWUS with Idle / Inactive UE camping with reception of paging and other necessary transmissions (from serving cell), reusing if possible/reasonable concepts from earlier releases, where the LPWUS either wakes the UE to receive by MR, or it conveys information by itself, or both.

This contribution discusses aspects related to impact on the Layer 2 procedures to support low-power wake-up receiver, and in particular the IDLE/INACTIVE mode operations.

# 2	Discussion

The UE is IDLE/INACTIVE mode performs operations such as monitoring for Paging message (both for RAN/CN/Group paging), short message indication, neighbor cell measurement for mobility, receive system information. During these operations, one of the major factor contributing to power consumption is the reception of PDCCH as the UE is required to perform intensive computation to monitor/decode the DCI in the Control Channel. The Duty Cycle based DRX mechanism makes sure that the UE does not have to monitor the PDCCH constantly and only on those occasions which the network is expected to schedule a DCI for the UE. Previous releases have worked on reducing this power consumption further by introducing features such as Paging-Early Indication, Search Space Switching, reducing the time required to wake up from sleep by reusing connected mode reference signals for syncing.

In release 18, a new approach is being studied which involves a Low Power Wake Up receiver (LP-WUR). The Low Power Wake up Receiver is a separate hardware specifically monitoring a wake up signal (WUS) from the network and consuming relatively lower power compared to Main Radio (MR). When the LP-WUR is in use, the MR can be in OFF state or a Deep Sleep state which consumes less power than when active. When the LP-WUR detects the configured Wake up signal, the MR is indicated to move to ON state and perform the required operations. Thus resulting in lesser power consumption at the UE. However, the introduction of LP-WUR will have impact to the legacy layer 2 procedures at the UE side and needs to be studied.

## 2.1. Impact on Paging and Short message Reception:

Monitoring and reception of paging message is one of the functions of the UE in IDLE/INACTIVE state. Currently the UE monitors paging message based on the configured DRX cycle during the paging occasions and once the UE receives a paging message addressed to the UE ID, the UE initiates the connection establishment/resumption procedure. When LP-WUR is used for monitoring the Wake up indication from the network, the MR of the UE could be moved to sleep state or turned off completely. When the WUR receives a wake up indication from the network, the UE can handle the wake up indication from the network in one of the following ways,

Option 1: 	Indicates to MR about the WUS reception and MR Initiates a connection establishment/resumption procedure

Option 2a: 	Indicates to MR about the WUS reception and MR initiates paging monitoring in the next valid paging occasion. This will be analogous to paging early indication (PEI) where the Rel-17 PEI is indicated via DCI and here it is indicated in via a wake up signal and received by the WUR.

Observation 1: The paging monitoring/reception procedure will be impacted when LP-WUR is introduced

Similar to receiving a Paging message in IDLE/INACTIVE mode, the network indicates to the UE the change in the broadcasted system information or in case of Public warnings such as ETWS/CMAS notification via short message through DCI. When a WUR is used and MR is turned off or not monitoring for short messages, there should be way for Network to provide these indications to the UE.

Observation 2: The short message reception procedure is impacted when LP-WUR is used

Based on above observations, there is a clear need to study and specify the ways to handle the paging and short message reception procedure when a WUR is introduced.

Proposal 1: RAN2 to study the impact on the paging reception procedure when a WUR is used.

Proposal 2: RAN2 to study the impact on short message reception procedure when WUR is used.

## 2.2. Handling of synchronization issues:

When the UE MR moves from sleep state to ON state during transition from DRX Sleep period to Active period, the UE uses the SSBs and Reference signals broadcasted by the network to estimate the channel characteristics and also to obtain DL synchronization. RAN1 WG has made the working assumption [1] that the ramp up time taken for MR to move from OFF state or ultra-deep sleep state to ON state is between 100ms – 400ms which does not include the time required for sync. If the UE were to rely only upon the MR for acquiring sync with the network, it will induce higher latency. Thus, there is a need to study if it is feasible to use the wake up signal for perform sync operation and its impact on Layer 2 specification if any.

Proposal 3: RAN2 to study the impact to Layer 2 specification if WUS were to be used for synchronization.

## 2.3. Scenarios to activate WUR:

If the UE supports the use of WUR, there is a need to study when the UE can make use of the WUR. There can be various cases when the UE starts using the WUR such as when the network sends an indication to use WUR, upon satisfying certain condition such as measured signal level, upon having low mobility condition for certain period of time etc.

Proposal 4: RAN2 to study the triggers/conditions to use LP-WUR for monitoring wake up indication.

# 3	Conclusion

In section 2, the following observations are made:

Observation 1: The paging monitoring/reception procedure will be impacted when LP-WUR is introduced

Observation 2: The short message reception procedure is impacted when LP-WUR is used

Based on the above observations, the following proposals are made,

Proposal 1: RAN2 to study the impact on the paging reception procedure when a WUR is used.

Proposal 2: RAN2 to study the impact on short message reception procedure when WUR is used.

Proposal 3: RAN2 to study the impact to Layer 2 specification if WUS were to be used for synchronization.

Proposal 4: RAN2 to study the triggers/conditions to use LP-WUR for monitoring wake up indication.

# 4	Reference

[1] 3GPP TR 38.869 v0.1.0 Study on low-power wake up signal and receiver for NR