3GPP TSG-RAN WG2 Meeting #122	R2-2306484

Incheon, Korea, 22nd May – 26th May, 2023

Agenda item:	7.6.3.1

Source:	Samsung

Title:	On enhancements for neighbour cell measurements

WID/SID:	IoT_NTN_enh-Core

Document for:	Discussion and Decision

# Introduction

In RAN#94-e the WID for Rel-18 IoT NTN was agreed [1] and in RAN#98-e it was updated [2]. The updated objectives of the WID is the following:

4.1.1 IoT-NTN Performance Enhancements in Rel-18 to address remaining issues from Rel-17 This work considers Rel-17 IoT-NTN as baseline as well as Rel-17 NR-NTN outcome and the further IoT-NTN performance enhancements objectives are listed below: - Disabling of HARQ feedback to mitigate impact of HARQ stalling on UE data rates [RAN1,RAN2] - Study and specify needed improved GNSS operations for a new position fix for UE pre-compensation during long connection times and for reduced power consumption. Simultaneous GNSS and NTN NB-IoT/eMTC operation is not assumed. [RAN1, RAN2] - NOTE: The need for RAN4 Core requirements for this objective will be identified after the conclusion on the need for improvements. 4.1.2 Mobility enhancements The following mobility enhancements objectives are listed. - Support of neighbour cell measurements and corresponding measurement triggering before RLF, using Rel-17 (TN) NB-IoT, eMTC as a baseline. [RAN2] - Support signalling in system information of neighbour cell ephemeris, for eMTC and NB-IoT [RAN2] - Re-use the solutions introduced in Rel-17 NR NTN for mobility enhancements for eMTC, with minimum necessary changes to adapt them to eMTC [RAN2] - Define UE RRM core requirements for the above mobility enhancement features [RAN4]. 4.1.3 Further enhancement to discontinuous coverage - Study and specify, if needed, mobility management enhancements and power saving enhancements for discontinuous coverage, taking into account the conclusions from the SA2 study FS_5GSAT_Ph2. [RAN2, RAN3].

In this contribution we discuss issues related to measurements of neighbouring cells, signaling of neighbour cell ephemeris for IoT NTN and RLF procedures.

# Discussion

- In Rel-17 IoT NTN was introduced with minimal enhancements for mobility purposes. The idea was that mobility enhancements would not be crucial for a first release due to the use case being to deliver small amounts of IoT data. In Rel-18 the use case is for an IoT UE to be connected for a longer time. Regarding performing neighbour cell measurements, there are two objectives considered that are different. One is supporting conditions for performing neighbour cell measurements as those introduced for Rel-17 NB-IoT, the other is supporting the actual signalling as required by RAN4 for neighbour cell measurements in NTN.

## Neighbour cell assistance signaling

- In RAN2#120 [3] the following was agreed in RAN2 for Rel-17 IoT NTN:

Agreements:

- RAN2 doesn’t introduce neighbour cell ephemeris in Rel-17 IoT-NTN, neither for eMTC not for NB-IoT. RAN2 agrees to support this in Rel-18, with details FFS.

- In RAN2#121bis-e the following was agreed in RAN2 on neighbour cell assistance signaling:

Agreements:

- New SIBxx is introduced to broadcast the neighbor cell/satellite information.

Agreements via email – from offline 114:

- Common TA parameters are broadcast as assistance information for neighbor cell measurements.

- Kmac is broadcast as neighbor cell assistance information.

- For moving cell, the UE can derive the trajectory of serving cell with rough accuracy based on serving satellite ephemeris and epochTime, with the assumption that the serving cell reference location broadcast by the network is the one at Epoch time (like in NR-NTN)

Agreements online:

- Introduce satellite ID for the satellite in a list in new SIB-xx. FFS on the details of the new IE

- For eMTC NTN, for fixed cell, location-based measurement initiation can also be used in RRC_IDLE for cell re-selection purposes (like in NR-NTN)

- For eMTC NTN, for moving cell, location-based measurement initiation can also be used in RRC_IDLE for cell re-selection purposes (like in NR-NTN). FFS whether to consider solution that does not require UE to update the GNSS for this same as in connected mode

- SIB3 is extended to include the reference location and distanceThresh

Proposal 9          (13/14) For fixed cell, same mechanism of location-based connected mode measurement initiation can also be used in RRC_IDLE (like in NR-NTN).

Proposal 10       (12/14) For moving cell, same mechanism of location-based connected mode measurement initiation can also be used in RRC_IDLE. FFS whether to consider solution that does not require UE to update the GNSS for this same as in connected mode.

Proposal 2          (Proposed way forward) A single validity duration is assumed for the new SIBxx including all neighbor satellites information. FFS whether it is defined as implicit (serving satellite validity duration) or explicit in SIBxx or both.

Proposal 3          (10/13) For fixed cell, the cell start time of the neighbor cell is broadcast.

Proposal 6          (8/13) for moving cell, reference location and distance threshold of neighbor cell are broadcast.

Proposal 13       (9/14) For cell (re)selection in earth-moving system, time-based measurement initiation is used to address feeder-link switch case.

- Satellite ID was agreed to be introduced in order to compress the signalling, as the new SIBxx only contains neighbour assistance information per satellite rather than per cell. This is motivated by the fact that there will be multiple cells per satellite. Thus the question is how this is tied to other frequencies. It was mentioned that the satellite indication is included in SIB5, but it is unclear what exactly this means.

- One way to link a neighbour satellite assistance information element via the satelliteId is to include the satelliteId in the information about neighbour cell frequencies in the related SIBs. Suitable information elements include:

- Intra frequency Neighbour cell Info (IntraFreqNeighCellInfo) in SIB4

- Inter frequency carrier frequency info (InterFreqCarrierFreqInfo) in SIB5.

- Thus we propose that satelliteId is included the above information elements, either as a parallel list or in the actual information element:

Proposal 1: satelliteId is introduced in/alongside (as a parallel list) information elements IntraFreqNeighCellInfo in SIB4(-NB) or InterFreqCarrierFreqInfo in SIB5(-NB).

- In the e-mail discussion there were several agreements on new content for the new SIB or for each individual neighbour satellite assistance information element. One crucial component of that is the validity duration. In NR NTN it was agreed that there will be validity durations for each neighbour satellite information element, the UE may then by implementation decide how to re-acquire SIB19 according to the validity duration. In IoT NTN this cannot work as the UE is not able to read system information in RRC connected mode. Thus the validity duration needs to be handled differently. In the e-mail discussions the following was suggested as a way forward:

Proposal 2          (Proposed way forward) A single validity duration is assumed for the new SIBxx including all neighbor satellites information. FFS whether it is defined as implicit (serving satellite validity duration) or explicit in SIBxx or both.

- This is likely a good way forward to ensure that the neighbour cell assistance information is kept valid. But here we explore the two suggested options a bit further and their implications. If there is a single validity duration for all of neighbour cell assistance information elements, this may mean that we need to define a new timer for the validity duration as well as the timer for when to read SIBxx, similar to SIB31. If it is implicit, i.e being the same as the serving satellite validity duration, then we assume that SIBxx is acquired at the same time as SIB31. This means that no new timers need to be introduced for SIBxx and several other considerations are made more simple. Thus we propose that RAN2 agrees that implicit validity duration is defined, i.e the UE reads re-acquires new SIBxx during T318 according to T317.

- Observation 1: Explicit validity duration for new SIBxx may require new timers specifically for SIBxx and implicit validity duration may not require new timers.

Proposal 2: Implicit validity duration, same as serving satellite validity, is assumed and UE acquires SIBxx when it acquires SIB31 during T318.

For the target cell neighbour cell ephemeris for eMTC, there are also some other considerations needed. In IoT NTN Rel-17, the target cell ephemeris is sent with SIB31 as part of RRCConnectionReconfiguration along with the handover command (mobilityControlInfo) so that IoT NTN UE does not have to read any SIB in connected mode except as indicated by T317. This is because the UE should be configured with all system information required for operating in that cell, so that UE does not need to acquire any system information when having connected. This implies that the new SIB (if agreed) should be included in the handover command to the UE. If the target cell neighbour cell ephemeris is included in a handover command, it is likely that the handover command may become prohibitively large.

Options can be that a UE acquires the target cell neighbour cell ephemeris for measurements by reading the system information after having connected to the target cell or via dedicated signalling. But we think that RAN2 should discuss this.

Proposal 3: RAN2 to discuss how UE acquires target cell neighbour cell ephemeris during handovers.

## Neighbour cell measurements triggering

While it is clear that neighbour cell measurement enhancements shall be supported for NB-IoT, whether it is supported for eMTC is still an open issue. In RAN2#121 the issue was further discussed without a conclusion [4]. In the table 1 below, we consider the arguments for and against introducing the measurements regardless of whether this is considered to be supported by the wording in the WID.

Table 1. Pros and cons of measurements based on NTN-conditions.

|  | Reasons for introducing the enhancements | Arguments for | Arguments against |
| --- | --- | --- | --- |
| NB-IoT | Triggering neighbour cell measurements when needed in non-terrestrial network to make RLF-based mobility more efficient | - The current neighbour cell measurement criteria is unlikely to function well in a non-terrestrial network - There are no connected mode neighbour cell measurements in NB-IoT NTN | - With traffic model of small packets it is likely that mobility may not be needed in most cases - Neighbour cell measurements may take a lot of time |
| eMTC | Power savings as UE does not have to perform neighbour cell measurements in some cases. Compared to Rel-17, which has no standard-supported neighbour cell measurements, this may bridge the gap to rel-18 with neighbour cell measurements. | - Triggering neighbour cell measurements with specific conditions can support energy savings | - eMTC connected mode mobility is not based on RLF, but rather handovers - Connected mode measurements are configured dedicatedly, thus the network may only configure neighbour cell measurements when deemed needed |

One further benefit of introducing the measurement triggering for eMTC is that the power saving of performing measurements can be reduced. This is especially important as neighbour cell measurements are not standard-supported in Rel-17. This means that the if neighbour cell measurements are introduced by acquiring neighbouring cell ephemeris for Rel-18, then there is a risk that the power consumption compared to Rel-17 will be a lot larger.

Observation 2: Trigger-based neighbour cell measurements can bridge the increased power consumption in Rel-18 compared to Rel-17.

Proposal 4: Introduce the same measurement triggers as for NB-IoT for eMTC, but for power saving purposes.

For an NB-IoT UE to perform neighbour cell measurements will be a rather time-consuming process involving performing lengthy measurements that in some cases may take a very long time. The UE may also have to acquire neighbour cell ephemeris and synchronize to the moving neighbouring cells to measure them. In many use cases, performing these measurements will not be necessary as the most common IoT use case is to deliver very small packets. Thus to enable power savings, it should be possible for devices to not perform these actions in an NTN network even if the conditions as RAN2 are currently discussing are fulfilled. Thus we propose that RAN2 should consider other conditions for not considering to perform neighbour cell measurements.

Proposal 5: RAN2 to consider use case-based / traffic-based conditions for not performing neighbour cell measurements.

It was also discussed last meeting whether the reference location, distance threshold and cell stop time should be introduced for neighbour cell. However, for it is not clear how this is to be used. For the neighbour cell measurement triggering, the conditions as currently discussed all depend on the serving cell. We thus think that to keep the measurement triggering simple, that these are not included in the algorithm. This we propose:

Proposal 6: Reference location and distance threshold for moving cell and cell stop time of neighbouring cell are not used for neighbour cell measurement triggering and thus not broadcasted.

## RLF procedures

- The main mechanism for performing connected mode mobility for NB-IoT is through RLF and subsequently the RRC re-establishment procedures. The RRC re-establishment procedures allow for a UE to re-establish its connection to a cell, that may be the same cell or a new cell. During the re-establishment procedure, if the an eNB has the UE context, then the UE does not need to be reconfigured. If an eNB does not have the UE context then there are two alternatives, either the eNB replies with an RRCSetup or the eNB may request the UE context from the eNB that the UE was connected to in the previously. This procedure will be greatly useful for the RLF procedures in an NB-IoT Non-Terrestrial Network.  All together this can be seen in Figure 1.

Figure 1. RRC Re-establishment procedures.

- The enhancements that are currently being discussed with regards to the neighbour cell measurements are targeted at improving the RLF procedures in NB-IoT NTN. In order to improve these procedures, we think that the handling of the UE context can also be improved. One such enhancement is that the UE Context is sent in advance from an eNB that is aware of the potential future eNBs that a UE will connect to after an RLF is triggered. Thus we propose that RAN2 discuss how to improve NB-IoT RLF procedures by sending the UE context in advance:

Proposal 7: RAN2 to discuss how to improve NB-IoT RLF procedures by sending UE context in advance.

# Conclusion

In this contribution we discussed issues related to enhancements for neighbour cell measurements. We have the following observations and proposals:

- Observation 1: Explicit validity duration for new SIBxx may require new timers specifically for SIBxx and implicit validity duration may not require new timers.

Observation 2: Trigger-based neighbour cell measurements can bridge the increased power consumption in Rel-18 compared to Rel-17.

Proposal 1: satelliteId is introduced in/alongside (as a parallel list) information elements IntraFreqNeighCellInfo in SIB4(-NB) or InterFreqCarrierFreqInfo in SIB5(-NB).

Proposal 2: Implicit validity duration, same as serving satellite validity, is assumed, i.e UE acquires SIBxx when it acquires SIB31 during T318.

Proposal 3: RAN2 to discuss how UE acquires target cell neighbour cell ephemeris during handovers.

Proposal 4: Introduce the same measurement triggers as for NB-IoT for eMTC, but for power saving purposes.

Proposal 5: RAN2 to consider use case-based / traffic-based conditions for not performing neighbour cell measurements.

Proposal 6: Reference location and distance threshold for moving cell and cell stop time of neighbouring cell are not used for neighbour cell measurement triggering and thus not broadcasted.

Proposal 7: RAN2 to discuss how to improve NB-IoT RLF procedures by sending UE context in advance.

# Reference

RP-213596, New WID on IoT NTN enhancements, Mediatek Inc., RAN#94-e, December 2021

RP-223519, Revised WID on IoT NTN enhancements, Mediatek Inc., RAN#98-e, December 2022

RAN#120-e Chairman’s notes.

RAN#121-e Chairman’s notes.