3GPP TSG-RAN WG2 Meeting #122	R2-2306487

Incheon, Korea, 22nd May – 26th May, 2023

Agenda item:	7.1.3

Source:	Samsung

Title:	Further considerations on NCR procedures

WID/SID:	NR_NetCon_Repeater

Document for:	Discussion and Decision

# Introduction

In RAN#94-e the SID for Network-Controlled Repeaters was agreed [1] and the latest WID updates were approved in RAN#98-e [2]. The updated objectives of the WID are the following:

The objectives of NR NCR WI follow the recommendations defined in TR 38.867 and will focus on scenarios and assumption listed below: - Network-controlled repeaters are inband RF repeaters used for extension of network coverage on FR1 and FR2 bands based on the NCR model in TR38.867 - For only single hop stationary network-controlled repeaters - The NCR is transparent to the UE. - Network-controlled repeater can maintain the gNB-repeater link and repeater-UE link simultaneously With these considerations, NR NCR supports the following features: Specify the signalling and behavior of the following side control information for controlling the NCR-Fwd [RAN1, RAN2] - Beamforming - UL-DL TDD operation - ON-OFF information Specify control plane signalling and procedures [RAN2, RAN1] - The configuration of signalling for side control information indication - NOTE: Down-selection of solutions in section 7.2 of TR 38.867 is needed Specify the solution of network-controlled repeater management (i.e., the identification and authorization/validation of NCR) [RAN3, RAN2] - NOTE: Down-selection of solutions in section 8 of TR 38.867 is needed taking into account the feedback of other working groups (i.e., SA3 and SA5). From a security point of view, the feasibility of NCR validation procedure in solution 1 and the feasibility of solution 2 will be decided by SA3.The selected solution shall provide inter-vendor interoperability. Study the RRM functions to be supported and specify the RRM requirements of NCR-MT if necessary [RAN2, RAN4] Study and specify the RF and EMC requirements of NCR if necessary [RAN4] Note: The existing requirements defined in RAN4 can be reused if applicable. Note: The work in RAN4 for beam related is expected to start on FR2 first.

In this contribution we discuss some further issues on NCR procedures.

# Discussion

- In this contribution we discuss how to enable a NCR to have consistent behaviour for the UE to operate in RRC idle mode.

## RRC idle-related considerations

### Wakeup-timer

Regardless of whether an NCR-MT can operate in RRC idle or not, it was agreed that the NCR-MT needs to be able to be released to RRC idle:

- The network should be able to send NCR-MT to RRC_IDLE

This is needed for a number of network-operation reasons, such the case where an gNB is down for maintenance or similar. Thus the question is how the NCR-MT remains reachable in RRC idle. The method that is mainly discussed is to use a so-called wakeup timer whereby an NCR-MT wakes up regularly. This is a rather crude method, but a workable one, since the important part is that the UE remains reachable.

During the previous discussions, there were two alternatives discussed to realize this wakeup timer:

1) OAM solution, where the wakeup timer is configured via OAM,

2) the wakeup timer is configured via RRC Release

While the OAM solution could in theory be used, there are some technical issues with it. The main issue is that while the OAM may configure the wakeup timer, the gNB may not be aware of when the OAM has configured the timer. This means that there is a risk that an NCR is released when the OAM has yet to configure or update the wakeup timer.

Observation 1: If configured via OAM, the donor gNB may not know when the wakeup timer has been (re-)configured, leading to the problem the donor gNB not being aware of when the NCR-MT may be released or not. Solving this issue requires extra implementation considerations.

Another argument that was mentioned was that OAM is preferred because setting the RRC wakeup timer might not be easy since it is a tradeoff between reachability and overhead of waking up. However, this issue exists for both the OAM and the solution of providing the wakeup timer via RRC Release.

Observation 2: If using RRC or OAM to configure wakeup timer, it will still have same tradeoff between reachability and overhead.

Furthermore, agreeing to the wakeup timer does not prevent a network implementer from using the OAM solution, if OAM is the preferred solution.

Observation 3: Even if wakeup timer is introduced, this does not prevent the implementation of the OAM solution.

Given the above, we think that the solution to this problem is that the wakeup timer is configured via RRC Release. For triggering the NCR-MT to connect to the cell, we think the simplest solution is to trigger an indication to NAS that in turn triggers a connection establishment. This allows for all of the existing procedures, such as changing the identifiers and other related procedures to remain as normal. Triggering NAS to establish a connection can be done by defining an indication to upper layers, and then a specific case is added to when the NCR-MT shall establish a connection in NAS for instance using Service Request procedure.

Proposal 1: Wakeup timer is configured via RRC Release. Upon timer expiry, it is indicated to NAS to trigger establishment.

Proposal 2: Send LS to CT1 to implement establishment upon indication from lower layers due to expiry of wakeup timer.

To highlight how this wakeup timer can be implemented in RRC, we have an example in Section 6.1.

Proposal 3: Adopt the TP in Section 6.1.

## RRC inactive-related considerations

### RRC Resume

A UE can leave RRC inactive by performing the RRC Resume procedures. Triggering the RRC Resume procedures can be initiated by 1) initiated by the UE due to uplink data in the buffers, 2) initiated by the network, through RAN-paging, 3) initiated by the UE due to expiry of T380 or 4) due to RNAU due to camping on new RANAC. For an NCR-MT, the 1-3) procedures are valid use cases of bring an NCR-MT to RRC connected mode, for instance to change NCR-Fwd configuration. For 4), it has been agreed that the NCR-Fwd will be turned OFF if a UE camps on a new cell.

When an NCR-MT resumes as initiated by 1-3), i.e in the case where the NCR-MT resumes to the same cell as released to, the NCR-Fwd configuration shall be resumed. This means that the periodic forwarding configuration shall continue forwarding if configured, and the other NCR-Fwd configurations should be resumed so that they can be controlled by MAC CE/DCI or alternatively be changed by RRC.

Observation 4: When NCR-MT resumes in the same cell as released, the NCR-Fwd configuration should be resumed, meaning that periodic forwarding continues and MAC CE/DCI can control semi persistent and aperiodic forwarding configurations. No specification changes are needed to ensure this.

One issue that still remains is how to resume the NCR-Fwd configuration when RRC resume is triggered due to camping on a new cell, i.e due to camping on a new RNA. When a UE resumes due to camping on a new RNA, it is possible for the UE to resume as if resuming on an old cell. The UE indicates its i-RNTI to the new cell which is used by the network to retrieve the UE context and thus allow for the full RRC to be resumed at the UE. This procedure can be seen in Figure 1a. For NCR, the question is whether the NCR-Fwd configuration can be resumed in this case.

As RAN2 has already specified that only the periodic forwarding configuration will remain forwarding if ON when released to RRC inactive, and that the NCR-Fwd will be turned OFF when selecting a different cell, we believe that the NCR-Fwd configuration may be resumed if the new gNB succeeds in retrieving the NCR-MT context. If the new donor gNB cannot resume NCR-MT due not finding the NCR-MT context, or if there are any other issues, the new donor gNB can always perform fullConfig in RRCResume or trigger RRCSetup. This procedure is seen in Figure 1b.

Figure 1. a) UE performing RRC resume with another gNB where gNB retrieves RRC context from the old gNB, b) An NCR-MT performing RRC resume and NCR-Fwd configuration is resumed if gNB is able to retrieve NCR-MT context.

Proposal 4: NCR-Fwd configuration can be resumed after successfully receiving RRC Resume due to camping on new RNA, i.e performing RRC resume to new donor gNB.

When RRC resume is triggered, the RRC resume procedure may fail or be diverted, in other words the UE starts a RRC Resume procedure, but does not finish being resumed in a normal sense. The reasons may be many, but the different cases may be divided up in to:

- T319 expiry

  - This is due to the allowed time to perform RRC resume has expired.

  - When this occurs, the procedures will make the UE go to RRC idle.

- Inability to comply with RRCResume

  - When this occurs, the procedures will make the UE go to RRC idle

- Receiving RRCRelease in response to RRCResumeRequest

  - This is for instance when the RNAU is being performed and the network swiftly sends the UE back to RRC inactive.

- Receiving RRCSetup in response to RRCResumeRequest

  - This can for instance occur if a gNB is unable to retrieve the UE context or any other errors prevent RRC resume from being established. Using this procedure can for instance be a design-choice when the gNB wants to establish a new configuration. When the UE receives the RRCSetup in response to RRCResumeRequest, the UE will discard the UE Inactive AS context and release all resources.

- Receiving RRCReject in response to RRCResumeRequest

  - This can for instance occur when the cell is overloaded or similar. When the RRCReject is received in response to a RRCResumeRequest, the UE can remain in RRC inactive using the waitTime (T302).

In relation to how an NCR acts, the NCR will or is expected to have the following actions:

T319 expiry

When T319 expiry occurs, the NCR-MT in RRC inactive will be going to RRC idle and during the procedure to go to RRC idle in the latest endorsed running RRC CR, the NCR-Fwd will be turned OFF.

Inability to comply with RRCResume

If the NCR-MT is unable to comply with RRCResume after having sent a RRCResumeRequest, then the UE will similarly go to RRC idle, where it is specified that the NCR-MT will off NCR-Fwd.

Observation 5: When T319 expires or NCR-MT is unable to comply with RRCResume, the NCR-Fwd will be turned OFF and NCR-MT goes to RRC idle.

Receiving RRCRelease in response to RRCResumeRequest

In this case the NCR-MT may be sent back to RRC inactive or to RRC idle. In case the NCR-MT is sent back to RRC inactive, the UE inactive AS context will be updated and then NCR-MT is returned back to RRC inactive. For this case the NCR-Fwd configuration will be maintained and NCR-Fwd will remain forwarding if configured to do so. If however the NCR-MT is sent to RRC idle, then the NCR-Fwd will cease any forwarding according to the procedures of going to RRC idle.

Observation 6: When receiving RRCRelease in response to RRCResumeRequest the procedures are clear regarding how to deal with NCR-Fwd.

Receiving RRCSetup in response to RRCResumeRequest

In this case there are specific procedures related to receiving the RRCSetup [3]:

1>	if the RRCSetup is received in response to an RRCResumeRequest or RRCResumeRequest1:

2>	if sdt-MAC-PHY-CG-Config is configured:

3>	instruct the MAC entity to stop the cg-SDT-TimeAlignmentTimer, if it is running;

3>	instruct the MAC entity to start the timeAlignmentTimer associated with the PTAG, if it is not running;

2>	if srs-PosRRC-InactiveConfig is configured:

3>	instruct the MAC entity to stop the inactivePosSRS-TimeAlignmentTimer, if it is running;

2>	discard any stored UE Inactive AS context and suspendConfig;

2>	discard any current AS security context including the KRRCenc key, the KRRCint key, the KUPint key and the KUPenc key;

2>	release radio resources for all established RBs except SRB0 and broadcast MRBs, including release of the RLC entities, of the associated PDCP entities and of SDAP;

2>	release the RRC configuration except for the default L1 parameter values, default MAC Cell Group configuration, CCCH configuration and broadcast MRBs;

2>	indicate to upper layers fallback of the RRC connection;

2>	discard any application layer measurement reports which were not transmitted yet;

2>	inform upper layers about the release of all application layer measurement configurations;

2>	stop timer T380, if running;

In this case, it is not clear whether a NCR-Fwd shall remain forwarding. Thus to ensure that it is clarified, we propose that it is clarified that NCR-Fwd does not continue forwarding.

Proposal 5: Clarify NCR-Fwd does not remain forwarding if RRCSetup is received in response to RRCResumeRequest.

Receiving RRCReject in response to RRCResumeRequest

In this case the NCR-MT has been performing RRC resume, but the donor gNB rejects the RRC resume attempt. In this case the NCR-MT may either remain in RRC inactive to attempt again, or upper layer procedures will be attempting to deal with the problem [3]:

1>	else if RRCReject is received in response to an RRCResumeRequest or an RRCResumeRequest1:

2>	if resume is triggered by upper layers:

3>	inform upper layers about the failure to resume the RRC connection;

2>	if resume is triggered due to an RNA update; or

2>	if resume is triggered for SDT and T380 has expired:

3>	set the variable pendingRNA-Update to true;

2>	discard the current KgNB key, the KRRCenc key, the KRRCint key, the KUPint key and the KUPenc key derived in accordance with 5.3.13.3;

2>	if resume is triggered for SDT:

3>	for SRB2, if it is resumed and for SRB1:

4>	trigger the PDCP entity to perform SDU discard as specified in TS 38.323 [5];

4>	re-establish the RLC entity as specified in TS 38.322 [4];

3>	for each DRB that is not suspended:

4>	indicate PDCP suspend to lower layers;

4>	re-establish the RLC entity as specified in TS 38.322 [4];

2>	suspend SRB1 and the radio bearers configured for SDT, if any;

2>	the procedure ends.

We think that if an NCR-MT is rejected it needs to be discussed how the NCR-Fwd should react. On one hand, RRCReject may signal that there are some issues with the gNB and that the NCR should not continue forwarding. On the other hand, some times the RRCReject may be used in temporary congestion-cases, and the NCR-Fwd should remain ON. We think that RAN2 should discuss this:

Proposal 6: RAN2 to discuss whether NCR-Fwd should remain ON when receiving RRCReject in response to an RRCResumeRequest.

### Cell selection when released to RRC inactive

When an NCR-MT is released to RRC inactive, the NCR-MT will perform several actions. One of these is that only the periodic forwarding resources will be kept in RRC inactive. One important aspect is that cell selection will be performed [3]:

1>	if the RRCRelease includes suspendConfig:

… OMITTED …

2>	indicate the suspension of the RRC connection to upper layers;

2>	if the UE is capable of L2 U2N Remote UE:

3>	enter RRC_INACTIVE, and perform either cell selection as specified in TS 38.304 [20], or relay selection as specified in clause 5.8.15.3, or both;

2>	else:

3>	enter RRC_INACTIVE and perform cell selection as specified in TS 38.304 [20];

In RAN2#121 it was agreed that when an NCR-MT reselects to another cell in RRC inactive, the NCR-MT will stop forwarding. However, from the above, cell selection may be performed when NCR-Fwd is forwarding. In order to ensure that procedures are consistent, there is a need to ensure that if an NCR-MT selects another cell in the cell selection process after entering RRC inactive after being released, the NCR-Fwd is turned OFF. Thus we propose that this is clarified:

Proposal 7: NCR-Fwd is turned OFF if an NCR-MT selects another cell during cell selection after released to RRC inactive.

To capture this, we think that a clarification in 38.304 could be needed to emphasize that NCR-Fwd being turned OFF applies to both cell selection and cell reselection.

Proposal 8: Agree to 38.304 TP on NCR-Fwd during cell selection after NCR-MT is released to RRC inactive.

### Beam-handling in RRC inactive state

In RRC connected mode, the beam handling for backhaul had been decided by RAN1 and the agreements are as:

Agreement The following pre-defined rules are applied to determine the beam for backhaul link: - In the time domain resource with simultaneous downlink reception or uplink transmission in C-link and backhaul link, the beam of backhaul link is the same as the beam of C-link regardless whether there is beam indicated by the dedicated signal for backhaul link. - In the time domain resource without simultaneous downlink reception or uplink transmission in C-link and backhaul link, if the NCR does not support capability with the new signalling for backhaul beam indication or if no beam is indicated for backhaul link by the dedicated signal, - When Rel-15/16 beam indication framework is used for C-link, - The beam determined by QCL assumption for CORESET with the lowest ID and spatial relationship for PUCCH with lowest PUCCH resource ID in the C-link is applied for the DL and UL of backhaul link, respectively. - When Rel-17 beam indication framework (i.e., unified TCI framework) is used for C-link, the indicated unified TCI for C-link DL and UL is applied for the DL and UL of backhaul link, respectively. Otherwise, the beam indicated by the dedicated signalling is applied for backhaul link.

How to handle the beams in inactive state was discussed in RAN2#121bis. The problem mentioned was that while it has been defined how the NCR selects the backhaul beam in RRC connected mode, it has not been defined in RRC inactive. The backhaul beams may become worse and this may affect the forwarding operation.

To solve the above problem, one approach was to have the NCR-Fwd turn OFF and then force the NCR-MT to connect to the cell for an update of the NCR-Fwd configuration, similar to how it was agreed that an NCR-MT will turn OFF NCR-Fwd when the NCR-MT selects another cell.

Our understanding is that an NCR in RRC inactive will be forwarding and backhauling regardless of the backhauling beams as long as the cell selection criteria is fulfilled and the NCR-MT does not select another cell. If for instance a donor gNB detects that the signal strength as received on specific backhauling beam is poor, the donor gNB can RAN-page the NCR-MT can reconfigure the NCR-Fwd.

It was also mentioned that an NCR-MT shall maintain the same beam as is used in RRC connected mode, but this may encounter some difficulties as currently the NCR will flush any beam configuration when being released from RRC connected mode.

In order for the above suggestion to work, then there needs to be definition of beam selection defined in RRC inactive, as currently a UE only selects cells in RRC inactive, and not beams.

Observation 7: With the current operation of NCR in RRC inactive, the NCR will be forwarding and backhauling regardless of the backhauling beams as long as cell (re-)selection criteria is fulfilled for the cell.

It should also be clear that the operation cannot be as optimized as for RRC connected mode as the donor gNB is unable to control the beam operation via MAC CE.

The triggering of going to RRC connected mode and turning OFF NCR-Fwd can for instance be configured by the NCR-MT if a certain configured threshold has been met and then the NCR-MT can connect and report this to the donor gNB.

However, first we think that RAN2 needs to discuss how the NCR beam operation is maintained in RRC inactive to reach a common understanding before discussing any mitigations.

Proposal 9: RAN2 to discuss how NCR beam operation functions in RRC inactive.

# LS to CT1 on UAC

In RAN2#119bis-e, the following was agreed:

NCR-MT should ignore cellBarred, cellReservedForOperatorUse, cellReservedForFutureUse， cellReservedForOtherUse, intraFreqReselection indications and UAC configuration if broadcast in system information.

For IAB, the following is defined for NAS [4]:

------------------ 24.501 V18.2.1 [4] ------------------

4.5	Unified access control

4.5.1	General

When the UE needs to access the 5GS, the UE not operating as an IAB-node (see 3GPP TS 23.501 [8]) and not acting as a 5G ProSe layer-2 UE-to-network relay UE (see 3GPP TS 23.304 [6E]) whose access attempt is triggered by a 5G ProSe layer-2 remote UE, first performs access control checks to determine if the access is allowed. Access control checks shall be performed for the access attempts defined by the following list of events:

NOTE 1:	Although the UE operating as an IAB-node skips the access control checks, the UE operating as an IAB-node determines an access category and one or more access identities for each access attempt in order to derive an RRC establishment cause. In this case the NAS provides the RRC establishment cause but does not provide the access category and the one or more access identities to the lower layers.

------------------ 24.501 V18.2.1 [4] ------------------

Due to the fact that UAC is not applicable to NCR-MT, we believe that the above text needs to be clarified for NCR as well. In order to align the specs we think an LS to CT1 on this is needed.

Proposal 10: Send LS to CT1 on UAC not being applicable for an NCR node.

# Conclusion

In this contribution we discussed issues related to NCR RRC states and procedures.

Observation 1: If configured via OAM, the donor gNB may not know when the wakeup timer has been (re-)configured, leading to the problem the donor gNB not being aware of when the NCR-MT may be released or not. Solving this issue requires extra implementation considerations.

Observation 2: If using RRC or OAM to configure wakeup timer, it will still have same tradeoff between reachability and overhead.

Observation 3: Even if wakeup timer is introduced, this does not prevent the implementation of the OAM solution.

Observation 4: When NCR-MT resumes in the same cell as released, the NCR-Fwd configuration should be resumed, meaning that periodic forwarding continues and MAC CE/DCI can control semi persistent and aperiodic forwarding configurations. No specification changes are needed to ensure this.

Observation 5: When T319 expires or NCR-MT is unable to comply with RRCResume, the NCR-Fwd will be turned OFF and NCR-MT goes to RRC idle.

Observation 6: When receiving RRCRelease in response to RRCResumeRequest the procedures are clear regarding how to deal with NCR-Fwd.

Observation 7: With the current operation of NCR in RRC inactive, the NCR will be forwarding and backhauling regardless of the backhauling beams as long as cell (re-)selection criteria is fulfilled for the cell.

Proposal 1: Wakeup timer is configured via RRC Release. Upon timer expiry, it is indicated to NAS to trigger establishment.

Proposal 2: Send LS to CT1 to implement establishment upon indication from lower layers due to expiry of wakeup timer.

Proposal 3: Adopt the TP in Section 6.1.

Proposal 4: NCR-Fwd configuration can be resumed after successfully receiving RRC Resume due to camping on new RNA, i.e performing RRC resume to new donor gNB.

Proposal 5: Clarify NCR-Fwd does not remain forwarding if RRCSetup is received in response to RRCResumeRequest.

Proposal 6: RAN2 to discuss whether NCR-Fwd should remain ON when receiving RRCReject in response to an RRCResumeRequest.

Proposal 7: NCR-Fwd is turned OFF if an NCR-MT selects another cell during cell selection after released to RRC inactive.

Proposal 8: Agree to 38.304 TP on NCR-Fwd during cell selection after NCR-MT is released to RRC inactive.

Proposal 9: RAN2 to discuss how NCR beam operation functions in RRC inactive.

Proposal 10: Send LS to CT1 on UAC not being applicable for an NCR node.

# Reference

RP-213700, New SI: Study on NR Network-controlled Repeaters, ZTE Corporation, RAN#94-e, December 2021.

RP-223505, Revised WID on NR network-controlled repeaters, ZTE, Sanechips, RAN#98-e, December 2022

3GPP TS 38.331, “Radio Resource Control (RRC) protocol specification”, V17.4.0, March 2023.

3GPP TS 24.501, “Non-Access-Stratum (NAS) protocol for 5G System (5GS)”, V18.2.1, March 2023.

# RRC TP

## Wake-up timer

---------------------------- 38.331 V17.4.0 TP ----------------------------

<OMITTED>

5.3.3.X	T4xx expiry

Upon T4xx expiry, the NCR-MT shall:

1>	indicate the expiry of the wakeup timer to the upper layers

<OMITTED>

5.3.8.3	Reception of the RRCRelease by the UE

The UE shall:

1>	delay the following actions defined in this clause 60 ms from the moment the RRCRelease message was received or optionally when lower layers indicate that the receipt of the RRCRelease message has been successfully acknowledged, whichever is earlier;

1>	stop timer T380, if running;

1>	stop timer T320, if running;

1>	if timer T316 is running;

2>	stop timer T316;

2>	clear the information included in VarRLF-Report, if any;

1>	stop timer T350, if running;

1>	stop timer T346g, if running;

1>	if the AS security is not activated:

2>	ignore any field included in RRCRelease message except waitTime;

2>	perform the actions upon going to RRC_IDLE as specified in 5.3.11 with the release cause 'other' upon which the procedure ends;

1>	if the RRCRelease message includes redirectedCarrierInfo indicating redirection to eutra:

2>	if cnType is included:

3>	after the cell selection, indicate the available CN Type(s) and the received cnType to upper layers;

NOTE 1:	Handling the case if the E-UTRA cell selected after the redirection does not support the core network type specified by the cnType, is up to UE implementation.

2>	if voiceFallbackIndication is included:

3>	consider the RRC connection release was for EPS fallback for IMS voice (see TS 23.502 [43]);

1>	if the RRCRelease message includes the cellReselectionPriorities:

2>	store the cell reselection priority information provided by the cellReselectionPriorities;

2>	if the t320 is included:

3>	start timer T320, with the timer value set according to the value of t320;

1>	else:

2>	apply the cell reselection priority information broadcast in the system information;

1>	if deprioritisationReq is included and the UE supports RRC connection release with deprioritisation:

2>	start or restart timer T325 with the timer value set to the deprioritisationTimer signalled;

2>	store the deprioritisationReq until T325 expiry;

NOTE 1a:	The UE stores the deprioritisation request irrespective of any cell reselection absolute priority assignments (by dedicated or common signalling) and regardless of RRC connections in NR or other RATs unless specified otherwise.

1>	if the RRCRelease includes the measIdleConfig:

2>	if T331 is running:

3> stop timer T331;

3>	perform the actions as specified in 5.7.8.3;

2>	if the measIdleConfig is set to setup:

3>	store the received measIdleDuration in VarMeasIdleConfig;

3>	start timer T331 with the value set to measIdleDuration;

3>	if the measIdleConfig contains measIdleCarrierListNR:

4>	store the received measIdleCarrierListNR in VarMeasIdleConfig;

3>	if the measIdleConfig contains measIdleCarrierListEUTRA:

4>	store the received measIdleCarrierListEUTRA in VarMeasIdleConfig;

3>	if the measIdleConfig contains validityAreaList:

4>	store the received validityAreaList in VarMeasIdleConfig;

1> if t4xx is included:

2> start timer T4XX, with the timer value set to t4xx

1>	if the RRCRelease includes suspendConfig:

2>	reset MAC and release the default MAC Cell Group configuration, if any;

2>	apply the received suspendConfig except the received nextHopChainingCount;

2>	if the sdt-Config is configured:

3>	for each of the DRB in the sdt-DRB-List:

4>	consider the DRB to be configured for SDT;

3>	if sdt-SRB2-Indication is configured:

4>	consider the SRB2 to be configured for SDT;

3>	for each RLC bearer (except those associated with broadcast MRBs) that is not suspended:

4>	re-establish the RLC entity as specified in TS 38.322 [4];

3>	for SRB2 (if it is resumed) and for SRB1:

4>	trigger the PDCP entity to perform SDU discard as specified in TS 38.323 [5];

3>	if sdt-MAC-PHY-CG-Config is configured:

4>	configure the PCell with the configured grant resources for SDT and instruct the MAC entity to start the cg-SDT-TimeAlignmentTimer;

2>	if srs-PosRRC-Inactive is configured:

3>	apply the configuration and instruct MAC to start the inactivePosSRS-TimeAlignmentTimer;

NOTE 1b:	The Network should provide full configuration to UE for SRS for Positioning in RRC_INACTIVE.

2>	remove all the entries within the MCG and the SCG VarConditionalReconfig, if any;

2>	for each measId of the MCG measConfig and for each measId of the SCG measConfig, if configured, if the associated reportConfig has a reportType set to condTriggerConfig:

3>	for the associated reportConfigId:

4>	remove the entry with the matching reportConfigId from the reportConfigList within the VarMeasConfig;

3>	if the associated measObjectId is only associated to a reportConfig with reportType set to condTriggerConfig:

4>	remove the entry with the matching measObjectId from the measObjectList within the VarMeasConfig;

3>	remove the entry with the matching measId from the measIdList within the VarMeasConfig;

2>	if the UE is acting as L2 U2N Remote UE:

3>	if the PC5-RRC connection with the U2N Relay UE is determined to be released:

4>	indicate upper layers to trigger PC5 unicast link release;

3>	else (i.e., maintain the PC5 RRC connection):

4>	establish or re-establish (e.g. via release and add) SL RLC entity for SRB1;

2>	else:

3>	re-establish RLC entities for SRB1;

2>	if the RRCRelease message with suspendConfig was received in response to an RRCResumeRequest or an RRCResumeRequest1:

3>	stop the timer T319 if running;

3>	in the stored UE Inactive AS context:

4>	replace the KgNB and KRRCint keys with the current KgNB and KRRCint keys;

4>	replace the nextHopChainingCount with the value of nextHopChainingCount received in the RRCRelease message;

4>	replace the cellIdentity with the cellIdentity of the cell the UE has received the RRCRelease message;

4>	if the suspendConfig contains the sl-UEIdentityRemote (i.e. the UE is a L2 U2N Remote UE):

5>	replace the C-RNTI with the value of the sl-UEIdentityRemote;

5>	replace the physical cell identity with the value of the sl-PhysCellId in sl-ServingCellInfo contained in the discovery message received from the connected L2 U2N Relay UE;

4> else:

5>	replace the C-RNTI with the C-RNTI used in the cell (see TS 38.321 [3]) the UE has received the RRCRelease message;

5>	replace the physical cell identity with the physical cell identity of the cell the UE has received the RRCRelease message;

3>	replace the nextHopChainingCount with the value associated with the current KgNB;

3>	stop the timer T319a if running and consider SDT procedure is not ongoing;

2>	else:

3>	store in the UE Inactive AS Context the nextHopChainingCount received in the RRCRelease message, the current KgNB and KRRCint keys, the ROHC state, the EHC context(s), the UDC state, the stored QoS flow to DRB mapping rules, the application layer measurement configuration, the C-RNTI used in the source PCell, the cellIdentity and the physical cell identity of the source PCell, the spCellConfigCommon within ReconfigurationWithSync of the NR PSCell (if configured) and all other parameters configured except for:

-	parameters within ReconfigurationWithSync of the PCell;

-	parameters within ReconfigurationWithSync of the NR PSCell, if configured;

-	parameters within MobilityControlInfoSCG of the E-UTRA PSCell, if configured;

-	servingCellConfigCommonSIB;

-	sl-L2RelayUE-Config, if configured;

-	sl-L2RemoteUE-Config, if configured;

NOTE 1c:	suspendConfig is not stored as part of UE Inactive AS Context, except for the fields explicitly specified.

3>	store any previously or subsequently received application layer measurement reports for which no segment, or full message, has been submitted to lower layers for transmission;

NOTE 2:	NR sidelink communication/discovery related configurations and logged measurement configuration are not stored as UE Inactive AS Context, when UE enters RRC_INACTIVE.

2>	suspend all SRB(s) and DRB(s) and multicast MRB(s), except SRB0 and broadcast MRBs;

2>	indicate PDCP suspend to lower layers of all DRBs and multicast MRBs;

2>	release the SRAP entity, if configured;

2>	if the t380 is included:

3>	start timer T380, with the timer value set to t380;

2>	if the RRCRelease message is including the waitTime:

3>	start timer T302 with the value set to the waitTime;

3>	inform upper layers that access barring is applicable for all access categories except categories '0' and '2';

2>	if T390 is running:

3>	stop timer T390 for all access categories;

3>	perform the actions as specified in 5.3.14.4;

2>	indicate the suspension of the RRC connection to upper layers;

2>	if the UE is capable of L2 U2N Remote UE:

3>	enter RRC_INACTIVE, and perform either cell selection as specified in TS 38.304 [20], or relay selection as specified in clause 5.8.15.3, or both;

2>	else:

3>	enter RRC_INACTIVE and perform cell selection as specified in TS 38.304 [20];

1>	else:

2>	perform the actions upon going to RRC_IDLE as specified in 5.3.11, with the release cause 'other'.

NOTE 3:	Whether to release the PC5 unicast link is left to L2 U2N Remote UE's implementation.

NOTE 4:	It is left to UE implementation whether to stop T430, if running, when going to RRC_INACTIVE.

<OMITTED>

–		RRCRelease

The RRCRelease message is used to command the release of an RRC connection or the suspension of the RRC connection.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: Network to UE

RRCRelease message

-- ASN1START

-- TAG-RRCRELEASE-START

RRCRelease ::=                      SEQUENCE {

rrc-TransactionIdentifier           RRC-TransactionIdentifier,

criticalExtensions                  CHOICE {

rrcRelease                          RRCRelease-IEs,

criticalExtensionsFuture            SEQUENCE {}

}

}

RRCRelease-IEs ::=                  SEQUENCE {

redirectedCarrierInfo               RedirectedCarrierInfo                                                       OPTIONAL,   -- Need N

cellReselectionPriorities           CellReselectionPriorities                                                   OPTIONAL,   -- Need R

suspendConfig                       SuspendConfig                                                               OPTIONAL,   -- Need R

deprioritisationReq                 SEQUENCE {

deprioritisationType                ENUMERATED {frequency, nr},

deprioritisationTimer               ENUMERATED {min5, min10, min15, min30}

}                                                                                                               OPTIONAL,   -- Need N

lateNonCriticalExtension                OCTET STRING                                                        OPTIONAL,

nonCriticalExtension                    RRCRelease-v1540-IEs                                                OPTIONAL

}

RRCRelease-v1540-IEs ::=            SEQUENCE {

waitTime                           RejectWaitTime                OPTIONAL, -- Need N

nonCriticalExtension               RRCRelease-v1610-IEs          OPTIONAL

}

RRCRelease-v1610-IEs ::=            SEQUENCE {

voiceFallbackIndication-r16        ENUMERATED {true}                             OPTIONAL, -- Need N

measIdleConfig-r16                 SetupRelease {MeasIdleConfigDedicated-r16}    OPTIONAL, -- Need M

nonCriticalExtension               RRCRelease-v1650-IEs                          OPTIONAL

}

RRCRelease-v1650-IEs ::=            SEQUENCE {

mpsPriorityIndication-r16          ENUMERATED {true}                             OPTIONAL, -- Cond Redirection2

nonCriticalExtension               RRCRelease-v1710-IEs                          OPTIONAL

}

RRCRelease-v1710-IEs ::=            SEQUENCE {

noLastCellUpdate-r17                ENUMERATED {true}                            OPTIONAL,   -- Need S

nonCriticalExtension                SEQUENCE {}                                  OPTIONAL

}

RRCRelease-v18xx-IEs ::=            SEQUENCE {

t4xx-r18                			NCR-WakeupTimer-r18                          OPTIONAL,   -- Need S

nonCriticalExtension                SEQUENCE {}                                  OPTIONAL

}

<OMITTED>

NCR-WakeupTimer-r18 ::=         ENUMERATED { min5, min10, min20, min30, min60, min120, min360, min720}

-- TAG-RRCRELEASE-STOP

-- ASN1STOP

RRCRelease-IEs field descriptions

cellReselectionPriorities Dedicated priorities to be used for cell reselection as specified in TS 38.304 [20]. The maximum number of NR carrier frequencies that the network can configure through FreqPriorityListNR and FreqPriorityListDedicatedSlicing together is eight. If the same frequency is configured in both FreqPriorityListNR and FreqPriorityListDedicatedSlicing, the frequency is only counted once.

cnType Indicate that the UE is redirected to EPC or 5GC.

deprioritisationReq Indicates whether the current frequency or RAT is to be de-prioritised.

deprioritisationTimer Indicates the period for which either the current carrier frequency or NR is deprioritised. Value minN corresponds to N minutes.

measIdleConfig Indicates measurement configuration to be stored and used by the UE while in RRC_IDLE or RRC_INACTIVE.

mpsPriorityIndication Indicates the UE can set the establishment cause to mps-PriorityAccess for a new connection following a redirect to NR. If the target RAT is E-UTRA, see TS 36.331 [10]. The gNB sets the indication only for UEs authorized to receive MPS treatment as indicated by ARP and/or QoS characteristics at the gNB, and it is applicable only for this instance of release with redirection to carrier/RAT included in the redirectedCarrierInfo field in the RRCRelease message.

noLastCellUpdate Presence of the field indicates that the last used cell for PEI shall not be updated. When the field is absent, the PEI-capable UE shall update its last used cell with the current cell. The UE shall not update its last used cell with the current cell if the AS security is not activated.

srs-PosRRC-InactiveConfig SRS for positioning configuration during RRC_INACTIVE state.

suspendConfig Indicates configuration for the RRC_INACTIVE state. The network does not configure suspendConfig when the network redirect the UE to an inter-RAT carrier frequency or if the UE is configured with a DAPS bearer.

t4xx Refers to the NCR wakeup timer that triggers indication to upper layers to wake-up and establish with network. Value min5 corresponds to 5 minutes, value min10 corresponds to 10 minutes and so on.

redirectedCarrierInfo Indicates a carrier frequency (downlink for FDD) and is used to redirect the UE to an NR or an inter-RAT carrier frequency, by means of cell selection at transition to RRC_IDLE or RRC_INACTIVE as specified in TS 38.304 [20]. Based on UE capability, the network may include redirectedCarrierInfo in RRCRelease message with suspendConfig if this message is sent in response to an RRCResumeRequest or an RRCResumeRequest1 which is triggered by the NAS layer (see 5.3.1.4 in TS 24.501 [23]).

voiceFallbackIndication Indicates the RRC release is triggered by EPS fallback for IMS voice as specified in TS 23.502 [43].

<OMITTED>

---------------------------- 38.331 V17.4.0 TP ----------------------------

# 38.304 TP

---------------------------- 38.304 TP based on R2-2304426 ----------------------------

5.2	Cell selection and reselection

5.2.1	Introduction

UE shall perform measurements for cell selection and reselection purposes as specified in TS 38.133 [8].

When evaluating Srxlev and Squal of non-serving cells for reselection evaluation purposes, the UE shall use parameters provided by the serving cell and for the final check on cell selection criterion, the UE shall use parameters provided by the target cell for cell reselection.

The NAS can control the RAT(s) in which the cell selection should be performed, for instance by indicating RAT(s) associated with the selected PLMN, and by maintaining a list of forbidden registration area(s) and a list of equivalent PLMNs. The UE shall select a suitable cell based on RRC_IDLE or RRC_INACTIVE state measurements and cell selection criteria.

In order to expedite the cell selection process, stored information for several RATs, if available, may be used by the UE.

When camped on a cell, the UE shall regularly search for a better cell according to the cell reselection criteria. If a better cell is found, that cell is selected. The change of cell may imply a change of RAT. Details on performance requirements for cell reselection can be found in TS 38.133 [8].

For NCRs, if the NCR-MT in RRC_INACTIVE selects or reselects a cell other than the last serving cell on which side control configuration was received, then the NCR-MT shall indicate to NCR-Fwd to cease forwarding. Besides, if the NCR-MT in RRC_INACTIVE detects no suitable cell, then the NCR-MT shall indicate to NCR-Fwd to cease forwarding.

The NAS is informed if the cell selection and reselection result in changes in the received system information relevant for NAS.

---------------------------- 38.304 TP based on R2-2304426 ----------------------------