3GPP TSG-RAN WG2 Meeting #122	R2-2306491

Incheon, KR, 22nd – 26th May 2023

Source: 			ZTE Corporation, Sanechips

Title: 	Height-dependent measurement configuration

Agenda item:		7.8.2

Document for:		Discussion and Decision

- Introduction

In RAN2#121bis-e meeting [1], following agreements were reached related to height-dependent measurement configuration.

Agreements

- Height-dependent more-than-one configurations is supported on parameter/field level (i.e. different fields/values within the same MO) where different values (or value ranges) of the parameter/field applies to different height or height range.

- For MO configuration parameters: at least the following will have ability to be configured with height-dependent more-than-one configurations/values, each for a specific height region: SSB-ToMeasure. Details on how to specify is FFS.    FFS on UE behavior on L1 and L3 measurement.

- For MR configuration parameters: at least the following will have ability to be configured with height-dependent more-than-one configurations/values, each for a specific height region: Event A4 threshold and numberoftriggeringcells.  Details on how to specify is FFS (i.e. maybe it can be achieved by combination of events).

- When height-dependent more-than-one configurations are provided, UE applies the new value once it moves to new height (or height range) similar to the case of RRC reconfiguration. Need Codes, field descriptions, etc. as in legacy specifications apply

- If a height-specific value is not explicitly configured for certain height, whether to keep using the value that was used or consider the parameter as released (i.e. parameter/value not applicable at this height) should be looked into case by case, and can be clarified by need code, field description, or procedural text as needed.   FFS details

There are still some FFS issues as highlighted above. This contribution provides considerations and proposals on the remaining FFS issues based on the agreements reached in previous meetings.

- Discussion

  - 2.1 On height-dependent SSB-ToMeasure

RAN2 agreed in last meeting that for MO configuration parameters, SSB-ToMeasure have ability to be configured with height-dependent more-than-one values each for a specific height region:

- For MO configuration parameters: at least the following will have ability to be configured with height-dependent more-than-one configurations/values, each for a specific height region: SSB-ToMeasure. Details on how to specify is FFS.    FFS on UE behavior on L1 and L3 measurement.

As to how to specify height-dependent SSB-ToMeasure, the first issue is whether more than one height-region can be configured. Usually maybe two regions are enough. One is above a height threshold, and another is below a height threshold. However, to be more flexible and future proof, it is preferred specification can support more than 2 height regions.

The second issue is how to avoid ping-pong issue as raised in last RAN2 meeting. A hysteresis can be introduced to address this issue. And we think only one value is enough for all height regions.

The third issue is whether delta configuration is support. It is beneficial for reducing signaling size when UAV UE moves between cells if neighboring cells have similar height-dependent SSB-ToMeasure configuration.

The fourth issue is how to ensure there is no overlapping height-regions to avoid confusion in UE side. One possible way is let to network configuration. However, we think it is also possible to ensure this by the definition of height-region. The details can be left to stage 3.

Proposal 1: The height-dependent SSB-ToMeasure is defined by including an ToAddMod list of non-overlapping height-regions and associated SSB-ToMeasure, a ToRemove list, and a hysteresis in MeasObjectNR.

On the UE behavior when UE applies the new value once it moves to new height region, following agreement is reached:

- When height-dependent more-than-one configurations are provided, UE applies the new value once it moves to new height (or height range) similar to the case of RRC reconfiguration. Need Codes, field descriptions, etc. as in legacy specifications apply

According to current specification, the reconfiguration of SSB-ToMeasure will apply following procedure[38.331]:

5.5.2.5 Measurement object addition/modification The UE shall: 1> for each measObjectId included in the received measObjectToAddModList: 2> if an entry with the matching measObjectId exists in the measObjectList within the VarMeasConfig, for this entry: 3> reconfigure the entry with the value received for this measObject, except for the fields cellsToAddModList, excludedCellsToAddModList, allowedCellsToAddModList, cellsToRemoveList, excludedCellsToRemoveList, allowedCellsToRemoveList, tx-PoolMeasToRemoveList, tx-PoolMeasToAddModList, ssb-PositionQCL-CellsToRemoveList, ssb-PositionQCL-CellsToAddModList, cca-CellsToRemoveList, and cca-CellsToAddModList; <skip irrelevant part> 3> for each measId associated with this measObjectId in the measIdList within the VarMeasConfig, if any: 4> remove the measurement reporting entry for this measId from the VarMeasReportList, if included; 4> stop the periodical reporting timer or timer T321 or timer T322, whichever one is running, and reset the associated information (e.g. timeToTrigger) for this measId; <skip irrelevant part>

If autonomous switching of SSB-ToMeasure follow the RRC reconfiguration procedure, the TTT will be reset, periodical reporting timer, T321/T322 will be stopped. The related measurement reporting entry is removed from VarMeasReportList if included.

Some issues and potential impacts need to be addressed:

-	Whether to stop the periodical reporting timer or timer T321/T322.

-	Whether to reset numberOfReportsSent in VarMeasReportList.

-	Whether to reset the timeToTrigger, e.g. considering the impact of reset of timeToTrigger on the measurement reporting delay.

Obaservation 1: If we model the change of SSB-ToMeasure as RRC reconfiguration, following issues need to be addressed:

-	Whether to stop the periodical reporting timer or timer T321/T322.

-	Whether to reset numberOfReportsSent in VarMeasReportList.

-	Whether to reset the timeToTrigger, e.g. considering the impact of reset of timeToTrigger on the measurement reporting delay

We understand the purpose of modeling the UE behavior as RRC reconfiguration is to simplify the specification. However, there is another model to make it even simpler.

In NR specification, the triggering of L3 measurement reporting is based on cell level measurement results. From this point of view, the change of SSB-ToMeasure is similar to the change of the best beams that are used for cell level quality derivation. And the behavior of such change is not specified but is left to UE implementation.

Let’s take TTT as example. When TTT is running while the best beams changes, the cell level quality may also changes. The UE decides whether to stop TTT or not based on the current cell level quality but not based on change of the used beams.

Another example is periodical reporting. When a periodical reporting entry is already included in VarMeasReportList, the change of beams used for cell level quality derivation will not remove the entry, and will not stop periodical reporting timer. The numberOfReportSent is not reset and cellsTriggeredList is updated accordingly by UE implementation.

Observation 2: The UE behavior on the change of beams used for cell level quality derivation is not specified.

From L3 measurement point of view, the change of SSB-ToMeasure is similar to the change of beams used for cell level quality derivation, except that the UE start or stop performing beam level measurement on some beams according to the new SSB-ToMeausre.

Observation 3: From L3 measurement point of view, the change of SSB-ToMeausre is similar to the change of beams used for cell level quality derivation, except for that UE starts or stops performing beam level measurement on some beams when applying new SSB-ToMeasure.

Therefore, if we model the change of SSB-ToMeasure this way, the only UE behavior needs to be captured is that the UE applies the SSB-ToMeasure value when it is in the associated height-region in field description. It is much simpler than RRC reconfiguration model.

Proposal 2: The only impact on L3 measurement is to capture in the SSB-ToMeasure field description that the UE applies the SSB-ToMeasure when it is in associated height-region.

One FFS issue identified in last RAN2 meeting is a following:

- If a height-specific value is not explicitly configured for certain height, whether to keep using the value that was used or consider the parameter as released (i.e. parameter/value not applicable at this height) should be looked into case by case, and can be clarified by need code, field description, or procedural text as needed.   FFS details

We understand that if height-specific SSB-ToMeasure is not explicitly configured for a certain height, which SSB-ToMeasure is applied depends on whether it is defined to support delta configuration.

However, we can firstly determine the basic principle in case the height-specific SSB-ToMeasure is not configured or released by network. There are two options:

- Option 1: legacy SSB-ToMeasure value.

- Option 2: all SSBs.

Per our understanding, when network does not configure or release the height-specific SSB-ToMeasure for a certain height, the UE behavior should be aligned with legacy, i.e. to apply legacy SSB-ToMeasure value. This should be the basic principle.

As to other cases such as caused by need code and condition, e.g. SSB-ToMeasure field is absent while height-region is present, we can discuss in stage 3.

Proposal 3: As a basic principle, if no height-specific SSB-ToMeasure is configured for a specific height region, the legacy SSB-ToMeasure is applied.

With same reasons as introducing height-dependent SSB-ToMeasure for RRC_CONNECTED measurement, we believe it is also beneficial to extend this mechanism to RRM measurement for RRC_IDLE and RRC_INACTIVE state. And it is also beneficial to apply this mechanism to RACH resource selection considering the RACH resource selection is based on the measurement on SSB.

Proposal 4: To extend height-dependent SSB-ToMeasure to RRM measurement for cell re-selection, Idle/Inactive measurements and RACH resource selection.

  - 2.2 On height dependent measurement report configuration

In last RAN2 meeting, it was agreed that:

3.  For MR configuration parameters: at least the following will have ability to be configured with height-dependent more-than-one configurations/values, each for a specific height region: Event A4 threshold and numberoftriggeringcells.  Details on how to specify is FFS (i.e. maybe it can be achieved by combination of events).

Combination of events can reuse existing method as much as possible and legacy measurement behavior of each event will not be changed. With this method, RAN2 don’t need to consider the impact of each height-dependent fields case by case. Further, we can reuse the method which is already introduced in conditional reconfiguration where execution condition can include two measIds for two triggering conditions.

On the other hand, it is more complicate to specify separate height-dependent fields/values in measurement report configuration. Taking numberOfTriggeringCells as example, we need to consider the UE behavior on whether remove current triggered cell list when numberOfTriggeringCells is changed.

Observation 4: combination of events approach is much simpler in terms of specification impact.

During discussion in last RAN2 meeting, some companies had some concern on how to describe height range with combination of events. We think it can be achieved by combining Ax event with H1 and H2 events respectively. For example, to combine combination of event A4 and event H1 with a lower height threshold, and a combination of the same event A4 and event H2 with a higher height threshold. Thus, when UE ascend above the lower height threshold or descend below the higher height threshold, the same event A4 is applied.

Observation 5: Association of event Ax with height range can be achieved by combination of event Ax with event H1 and combination of same event Ax with event H2.

Based on observations above, we propose combination of events is used to achieve height-dependent report configuration.

proposal 5: Height-depend report configuration is achieved by combination of event H1/H2 and events Ax.

- Conclusion and proposals

On height-dependent SSB-ToMeasure

Obaservation 1: If we model the change of SSB-ToMeasure as RRC reconfiguration, following issues need to be addressed:

-	Whether to stop the periodical reporting timer or timer T321/T322.

-	Whether to reset numberOfReportsSent in VarMeasReportList.

-	Whether to reset the timeToTrigger, e.g. considering the impact of reset of timeToTrigger on the measurement reporting delay

Observation 2: The UE behavior on the change of beams used for cell level quality derivation is not specified.

Observation 3: From L3 measurement point of view, the change of SSB-ToMeausre is similar to the change of beams used for cell level quality derivation, except for that UE starts or stops performing beam level measurement on some beams when applying new SSB-ToMeasure.

Proposal 1: The height-dependent SSB-ToMeasure is defined by including an ToAddMod list of non-overlapping height-regions and associated SSB-ToMeasure, a ToRemove list, and a hysteresis in MeasObjectNR.

Proposal 2: The only impact on L3 measurement is to capture in the SSB-ToMeasure field description that the UE applies the SSB-ToMeasure when it is in associated height-region.

Proposal 3: As a basic principle, if no height-specific SSB-ToMeasure is configured for a specific height region, the legacy SSB-ToMeasure is applied.

Proposal 4: To extend height-dependent SSB-ToMeasure to RRM measurement for cell re-selection, Idle/Inactive measurements and RACH resource selection.

On height dependent measurement report configuration

Observation 4: combination of events approach is much simpler in terms of specification impact.

Observation 5: Association of event Ax with height range can be achieved by combination of event Ax with event H1 and combination of same event Ax with event H2.

proposal 5: Height-depend report configuration is achieved by combination of event H1/H2 and events Ax.

- Reference

- RAN2#121-bis chairman notes