3GPP TSG-RAN WG2 Meeting #122						  R2-2306499

Incheon, Korea, May 22 – May 26, 2023

Agenda item:	6.1.3.2

Source:			MediaTek Inc.

Title:	On signaling for the maximum aggregated bandwidth

Document for:	Discussion and Decision

- Background

The RAN4 LS [1] was discussed in RAN2#121bis-e meeting and there was no consensus [2] in RAN2.

Postpone (topic 2/3),

- Allow/recommend companies to discuss offline until RAN2#122:

- UE capability signalling overhead for existing UE implementations.

- Additional UE capability signalling overhead caused by FBG5 and BCS4/5.

- Potential signalling overhead reduction gain in light of above.

- Any other relevant aspect.

FFS whether there is any legacy network implementation for FBG5 or BCS4/5 that must be taken into account in further discussion.

We would like to provide further views on the issue and the proposals from RAN4.

- Discussion

  - The nature of signaling overhead for FR1 CA with BCS4/5

We think that the issue in the RAN4 LS [1] comes from a UE implementation supporting high order FR1 CA band combination regardless of its RF and/or baseband limitation.

The RAN4 configuration tables for CA describe Bandwidth Combination Sets. BCS4 and 5 contain all possible defined channel bandwidths for each band in the combination. While claiming to support BCS4 or 5, current RAN2 signaling has allowed that the bandwidths the UE supports for each band, the maximum bandwidth and/or minimum bandwidth for the band in the band combination are indicated in the UE capabilities. The most important thing is that UE is allowed to decide what CA band combinations with BCS4/5 can be supported considering of its own RF and/or baseband limitation. There is no requirement to force the UE to support the order of CA band combination as high as possible.

Take the FR1 CA in the RAN4 LS [1] as example. The UE needs to decide what CA band combinations consists of NR bands n25, n41 and n66 should be supported. Given that maximum aggregated bandwidth is 140MHz due to RF and/or baseband limitation, the UE ought to support those 2CC band combinations with BCS0, 1 and BCS4/5 in the table 2.1-1 (cited from TS 38.101-1).

Table 2.1-1: NR CA configurations and BCSs defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n25A-n41A | n418, 9 CA_n25A-n41A8 | n25 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n25A-n66A | CA_n25A-n66A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n66 | 5, 10, 15, 20, 30, 40 |  |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41A-n66A | n418,9 CA_n41A-n66A8 | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |

As for the 3CC band combination in the table 2.1-2 (cited from TS 38.101-1), the maximum aggregated bandwidth requirement for supporting BCS0 is 160MHz so that the UE with aggregated bandwidth 140MHz limitation doesn’t have to support it and even BCS4/5.

One could argue that the UE with 140MHz limitation, by contrast, is still allowed to support the 3CC band combination CA_ n25A-n41A-n66A. In RAN2 perspective, we can imagine the reason to do so could only be based on pure capability signaling overhead consideration. On one hand, the UE can report the parent CA band combination (i.e., the superset) only and skip all fallbacks, but on the other hand the signaling overhead issue occurs as stated in the RAN4 LS [1]. We not only want to remind that the UE needs to report the fallbacks with additional capabilities in typical product implementation, but also we would like to further inspect the consequence by doing so, in next clause.

Table 2.1-2: NR CA configurations and BCSs defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier6 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n25A-n41A-n66A | - | n25 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
|  | CA_n25A-n41A CA_n25A-n66A CA_n41A-n66A | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  | CA_n25A-n41A CA_n25A-n66A CA_n41A-n66A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |

- Current specification is flexible enough to allow the UE to avoid the signaling overhead issue. The UE shall not be forced to choose solution from sole two options provided in the RAN4 LS.

- RAN2 to clarify there are still another solution options other than the original ones in the Reply LS.

  - The consequence of supporting higher order CA band combination regardless of RF/baseband limitation

By the same example, given that following configurations, we calculate the approximate data rate for every feature set case in the 3CC CA band combination.

FR1 SCS: 15kHz for FDD; 30kHz for TDD

Total MIMO layers: 8 (This is another typical UE implementation constraint.)

Modulation order: 256QAM

Scaling factor: 1

Overhead: 0.14

The results are shown in the table 2.2-1 below.

Table 2.2-1: 3-bands inter-band NR CA configurations and approximate data rate

| FS# | n25-BW (MHz) | n25-MIMO (Layers) | n41-BW (MHz) | n41-MIMO (Layers) | n66-BW (MHz) | n66-MIMO (Layers) | data rate (Gbps) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 0 | 100 | 4 | 40 | 4 | 3.26 |
| 2 | 5 | 2 | 100 | 4 | 35 | 2 | 2.80 |
| 3 | 10 | 2 | 100 | 4 | 30 | 2 | 2.80 |
| 4 | 12 | 2 | 100 | 4 | 25 | 2 | 2.80 |
| 5 | 20 | 2 | 100 | 4 | 20 | 2 | 2.80 |
| 6 | 25 | 2 | 100 | 4 | 15 | 2 | 2.80 |
| 7 | 30 | 2 | 100 | 4 | 10 | 2 | 2.80 |
| 8 | 35 | 2 | 100 | 4 | 5 | 2 | 2.80 |
| 9 | 40 | 4 | 100 | 4 | 0 | 0 | 3.26 |
| 10 | 5 | 2 | 90 | 4 | 45 | 2 | 2.67 |
| 11 | 10 | 2 | 90 | 4 | 40 | 2 | 2.67 |
| 12 | 15 | 2 | 90 | 4 | 35 | 2 | 2.67 |
| 13 | 20 | 2 | 90 | 4 | 30 | 2 | 2.67 |
| 14 | 25 | 2 | 90 | 4 | 25 | 2 | 2.67 |
| 15 | 30 | 2 | 90 | 4 | 20 | 2 | 2.67 |
| 16 | 35 | 2 | 90 | 4 | 15 | 2 | 2.67 |
| 17 | 40 | 2 | 90 | 4 | 10 | 2 | 2.67 |
| 18 | 45 | 2 | 90 | 4 | 5 | 2 | 2.67 |
| 19 | 15 | 2 | 80 | 4 | 45 | 2 | 2.54 |
| 20 | 20 | 2 | 80 | 4 | 40 | 2 | 2.54 |
| 21 | 25 | 2 | 80 | 4 | 35 | 2 | 2.54 |
| 22 | 30 | 2 | 80 | 4 | 30 | 2 | 2.54 |
| 23 | 35 | 2 | 80 | 4 | 25 | 2 | 2.54 |
| 24 | 40 | 2 | 80 | 4 | 20 | 2 | 2.54 |
| 25 | 45 | 2 | 80 | 4 | 15 | 2 | 2.54 |
| 26 | 25 | 2 | 70 | 4 | 45 | 2 | 2.42 |
| 27 | 30 | 2 | 70 | 4 | 40 | 2 | 2.42 |
| 28 | 35 | 2 | 70 | 4 | 35 | 2 | 2.42 |
| 29 | 40 | 2 | 70 | 4 | 30 | 2 | 2.42 |
| 30 | 45 | 2 | 70 | 4 | 25 | 2 | 2.42 |
| 31 | 35 | 2 | 60 | 4 | 45 | 2 | 2.31 |
| 32 | 40 | 2 | 60 | 4 | 40 | 2 | 2.31 |
| 33 | 45 | 2 | 60 | 4 | 35 | 2 | 2.31 |
| 34 | 50 | 2 | 50 | 4 | 40 | 2 | 2.18 |
| 35 | 45 | 2 | 50 | 4 | 45 | 2 | 2.17 |

The highest data rate can be achieved by 2CC cases while the UE using higher-order MIMO configuration. Even we enlarge the total MIMO layer limitation to 12 layers which makes every CC 4 layers each, the results are the same that 2CC configurations still reach highest data rate though the throughput difference from other 3CC cases is smaller. We also did data rate calculation for the 4CC case in the RAN4 LS with 120MHz aggregated bandwidth limitation and confirmed the same result (2CC cases have higher data rate than the 3CC ones, then the 3CC ones have higher data rate than the 4CC ones).

It is our understanding that the gNB would typically adopt the CA configuration in prioritizing the UE to reach highest theoretical peak data rate. No matter with or without the new signaling for maximum aggregated bandwidth, the gNB would anyway have an approach (such as searching and calculation) for the CA configuration candidate determination. Therefore, the UE to advertise the 2CC band combinations support in the first place is a best practice. In the issue scenario we see no point to advertise 3CC or 4CC band combination capabilities achieving lower CA data rate at the cost of more signaling bits in the air interface.

- There are no substantial benefits to support higher-order CA band combination regardless of UE RF and/or baseband limitations so that it is pointless to introduce the new signaling for maximum aggregated bandwidth.

- RAN2 doesn’t pursue the RAN4 proposals in introducing maximum aggregated bandwidth signaling for FR1 CA.

  - FBG5

We have very similar calculation result and observation for FR2 FBG5 case.

Table 2.3-1	: n260 intra-band CA with FBG5

| NR CA configuration / Bandwidth combination set / Fallback group |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configurations | BW channel (MHz) | BW channel (MHz) | BW channel (MHz) | BW channel (MHz) | BW channel (MHz) | BW channel (MHz) | BW channel (MHz) | BW channel (MHz) | BW channel (MHz) | BW channel (MHz) | BW channel (MHz) | BW channel (MHz) | Maximum aggregated BW (MHz) | BCS | Fallback group |
| CA_n260R2 | CA_n260R2 | 100, 200 | 100, 200 |  |  |  |  |  |  |  |  |  |  | 400 | 0 | 5 |
| CA_n260R3 | CA_n260R2 CA_n260R3 | 100, 200 | 100, 200 | 100, 200 |  |  |  |  |  |  |  |  |  | 600 | 0 |  |
| CA_n260R4 | CA_n260R2 CA_n260R3 CA_n260R4 | 100, 200 | 100, 200 | 100, 200 | 100, 200 |  |  |  |  |  |  |  |  | 800 | 0 |  |
| CA_n260R5 | CA_n260R2 CA_n260R3 CA_n260R4 CA_n260R55 | 100, 200 | 100, 200 | 100, 200 | 100, 200 | 100, 200 |  |  |  |  |  |  |  | 1000 | 0 |  |
| CA_n260R6 | CA_n260R2 CA_n260R3 CA_n260R4 CA_n260R55 | 100, 200 | 100, 200 | 100, 200 | 100, 200 | 100, 200 | 100, 200 |  |  |  |  |  |  | 1200 | 0 |  |
| NOTE 3: In this release of the specification, contiguous DL CA configurations within FR2-2 may only contain multiples of the same channel bandwidth. NOTE 4: In Rel-18 maximum aggregated downlink BW is limited to 1600 MHz. NOTE 5: In Rel-18 maximum aggregated uplink BW is limited to 800 MHz. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

FR2 SCS: 120kHz for TDD

Total aggregated bandwidth: 600MHz

Total MIMO layers: 12

Modulation order: 256QAM

Scaling factor: 0.75

Overhead: 0.18

Table 2.3-2: FR2 intra-band NR CA configurations with FBG5 and approximate data rate

| FBG5 &FS# | CC1-BW | CC1-MIMO | CC2-BW | CC2-MIMO | CC3-BW | CC3-MIMO | CC4-BW | CC4-MIMO | CC5-BW | CC5-MIMO | CC6-BW | CC6-MIMO | data rate (Gbps) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R3 |  |  |  |  |  |  | 200 | 4 | 200 | 4 | 200 | 4 | 7.27 |
| R4-FS1 |  |  |  |  | 100 | 2 | 100 | 2 | 200 | 4 | 200 | 4 | 6.06 |
| R4-FS2 |  |  |  |  | 100 | 2 | 100 | 4 | 200 | 2 | 200 | 4 | 5.45 |
| R4-FS3 |  |  |  |  | 100 | 4 | 100 | 2 | 200 | 2 | 200 | 4 | 5.45 |
| R4-FS4 |  |  |  |  | 100 | 4 | 100 | 4 | 200 | 2 | 200 | 2 | 4.85 |
| R5-FS1 |  |  | 100 | 2 | 100 | 2 | 100 | 2 | 100 | 2 | 200 | 4 | 4.85 |
| R5-FS2 |  |  | 100 | 2 | 100 | 2 | 100 | 2 | 100 | 4 | 200 | 2 | 4.24 |
| R5-FS3 |  |  | 100 | 2 | 100 | 2 | 100 | 4 | 100 | 2 | 200 | 2 | 4.24 |
| R5-FS4 |  |  | 100 | 2 | 100 | 4 | 100 | 2 | 100 | 2 | 200 | 2 | 4.24 |
| R5-FS5 |  |  | 100 | 4 | 100 | 2 | 100 | 2 | 100 | 2 | 200 | 2 | 4.24 |
| R6 | 100 | 2 | 100 | 2 | 100 | 2 | 100 | 2 | 100 | 2 | 100 | 2 | 3.64 |

From the Table 2.3-2, the UE implementation takes more important rule in resolving signaling overhead because some permutations are obviously not beneficial and redundant unless they are required by particular operator requirement in practical deployment.

- RAN2 doesn’t pursue the RAN4 proposals in introducing maximum aggregated bandwidth signaling for FR2 FBG5.

- Summary

In previous sections we made the following observations and proposals:

Observation 1	Current specification is flexible enough to allow the UE to avoid the signaling overhead issue. The UE shall not be forced to choose solution from sole two options provided in the RAN4 LS.

Proposal 1	RAN2 to clarify there are still another solution options other than the original ones in the Reply LS.

Observation 2	There are no substantial benefits to support higher-order CA band combination regardless of UE RF and/or baseband limitations so that it is pointless to introduce the new signaling for maximum aggregated bandwidth.

Proposal 2	RAN2 doesn’t pursue the RAN4 proposals in introducing maximum aggregated bandwidth signaling for FR1 CA.

Proposal 3	RAN2 doesn’t pursue the RAN4 proposals in introducing maximum aggregated bandwidth signaling for FR2 FBG5.

- References

- R2-2302439	LS on UE signalling for the maximum aggregated bandwidth for FR1 CA (R4-2303685; contact: Qualcomm)

- R2-2304444	Summary of [AT121bis-e][011][NR17] UE Caps BW related Corrections (Qualcomm)