3GPP TSG-RAN WG2 Meeting #122	R2-2306513

Incheon, Korea, 22nd – 26th  May, 2023

Agenda Item:	7.25.1

Source:	Huawei, HiSilicon

Title:	Discussion on lower MSD capabilities

Document for:   Discussion and decision

Introduction

In RAN#99, RAN2 was tasked to be responsible for the revised WI objective[1] to enable lower MSD capability for inter-band combination.

Objective Enable lower MSD capability for inter-band CA/EN-DC/DC combinations [RAN4, RAN2] - Continue the study on approaches for a UE to indicate the improved lower MSD performance based on the progress of the study phase and specify the related signalling - Based on the outcome of the signalling study, specify the UE RF requirements to support lower MSD capability

In the latest RAN4 LS[2], more details are provided by RAN4 on this issue. In this contribution, we provide our analysis on the capability signalling design for lower MSD.

Discussion

In current specification, MSD was defined to reflect the sensitivity degradation of the victim band caused by interference from an aggressor band within an inter-band CA/DC combination. For a certain band combination, there could be multiple interference sources with different orders, e.g. harmonic, harmonic mixing, intermodulation and so on. When a configured band combination suffers from a large MSD, the DL reception of the victim band will be impacted heavily by the UL transmission of the aggressor band, which leads to a performance downgrading. In TS 38.101, based on the evaluation results, the worst MSD for band combinations was specified for typical cases, e.g. for certain condition on the UL/DL bandwidths and the uplink power levels. Such information could help the network to configure and schedule CA/DC combinations with better performance.

In Rel-18, according to evaluation during study phase, it was concluded by RAN4 that compared with those MSD specified in the specifications, it is feasible for the UE to improve the MSD with some implementation solutions. To facilitate the network configuration and scheduling, the lower MSD capability should be allowed to be reported to the network. Based on the agreement in RAN4, the lower MSD capability is defined per band combination, and following information are considered to be essential:

- Victim band;

- MSD type (harmonic; harmonic mixing; cross band isolation; IMD) with orders

- MSD indication of corresponding threshold (capability class)

- Power class

- [Aggressor UL and victim DL bandwidth]

From the information above, we can see the actual MSD value (i.e. the MSD threshold/capability class mentioned in the LS) for a band combination is impacted by a lot of things, i.e. MSD source, power class and so on. If there is more than one victim band/MSD source/power class supported by the UE, the lower MSD value will be reported separately for different cases. Even we only consider the default power class (PC3) case, assuming a 3-tuple unit <MSD value, victim band, MSD source> is used to define the lower MSD capability, there could be more than one such unit to signal for a band combination, not to mention that the UE may support more than one power classes (e.g. PC2, PC1.5) for the BC. As a result, this capability signaling will bring a heavy signaling overhead and a storage burden for both the UE and the NW.

Observation 1: It will bring heavy signaling overhead and storage burden when the lower MSD capability is signaled for each supported victim band/MSD source/power class for a band combination.

Based on analysis above, we understand the lower MSD capability design aiming to save the signaling overhead should be the key issue discussed in RAN2.

Proposal 1: RAN2 should discuss the lower MSD capability design aiming to save the signaling overhead.

In our view, considering the lower MSD capability is used for better CA/DC configuration and scheduling for the network, the capability filter requested from the network is helpful to decrease signaling overhead. The lower MSD capability is only reported to the network when there is a requirement from the network. Besides, the filter can additionally include the concerned MSD source, or the concerned MSD threshold requested by the network, and the lower MSD capability should be reported based on the filter from the network.

Proposal 2: Consider lower MSD capability reporting based on the filter requested from the network.

Furthermore, to decrease the signaling overhead, it was agreed in RAN4 that such lower MSD capability is only reported for a two-band combination when the interference to the victim band results from another one aggressor band, or reported for a three-band combination when the interference to the victim band results from dual UL operation from another two bands. It is considered that for the combination with more than 3 bands, there would be the same lower MSD capability for the two bands or three bands within the higher order combination. In this case, for higher order combinations including the same lower order ones, the repeated lower MSD capability can be omitted to save signaling overhead.

However, such kind of capability reporting agreed by RAN4 is not aligned with the legacy fallback principle in RAN2. In current RAN2 spec, the fallback principle only allows the capability of a lower order combination(i.e. fallback BC) implicitly derived from a higher order combination(i.e. parent BC), but it cannot be applied oppositely. Besides, the capability of different parent BCs should be signaled separately. With the current BC reporting structure, the signaling overhead for lower MSD capability is even worse.

Observation 2: The capability reporting mechanism agreed by RAN4 is not aligned with existing fallback principle defined in RAN2.

From RAN2 perspective, to avoid repeated capability reporting for band combinations, there are following ways to enable the signaling design:

- Alt1: The lower MSD capability is reported once for one band combination among the band combinations including the same 2-band and 3-band combinations.

In this way, the lower MSD capability of other band combinations including the same band entries is derived from this band combination. For example, if the BC {bandA+bandB+bandC} is reported with lower MSD capability for bandA+bandB, then the same capability can be omitted in BC {bandA+bandB+bandD}.

- Alt2: Only the band combinations with 2 bands and 3 bands are reported explicitly to indicate the lower MSD capability.

In this way, the lower MSD capability for its parent band combinations are derived from the fallback band combination. For example, if the BC {bandA+bandB} is reported with lower MSD capability, then the same capability can be omitted in BC{bandA+bandB+bandC} and BC{bandA+bandB+bandD}.

- Alt3: Introduce a separate IE to indicate the lower MSD capability for the 2-band and 3-band combinations, outside the defined BandCombination IE.

In this way, the lower MSD capability for all defined band combinations is derived based on the new IE.

For the alternatives above, we think the Alt3 is simple for the NW to determine the lower MSD capability for one BC, without breaking the current fallback principle. Both the Alt1 and the Alt2 will bring new logic to determine the capability for a band combination, which increases the complexity of the network. To avoid unnecessary signaling overhead for a fallback BC, the lower MSD capability can be reported separately in a new IE, outside the defined band combination.

Proposal 3: The lower MSD capability can be reported in a separate field outside the band combination.

Conclusion

In this contribution, we provide the following observations and proposals:

Observation 1: It will bring heavy signaling overhead and storage burden when the lower MSD capability is signaled for each supported victim band/MSD source/power class for a band combination.

Observation 2: The capability reporting mechanism agreed by RAN4 is not aligned with existing fallback principle defined in RAN2.

Proposal 1: RAN2 should discuss the lower MSD capability design aiming to save the signaling overhead.

Proposal 2: Consider lower MSD capability reporting based on the filter requested from the network.

Proposal 3: The lower MSD capability can be reported in a separate field outside the band combination.

References

[1] RP-230753, Revised WID Rel-18 NR_ENDC_RF_FR1_enh2

[2] R4-2306594, draft LS on lower MSD capability.