3GPP TSG-RAN WG2 Meeting #122	R2-2306543

Incheon, Korea, May 22-26, 2023

Source: 	Session Chair (InterDigital)

Title:	Report from Session on NES, UAV, Rel-15-17 UP, Rel-17 Small Data, IIoT/URLLC, and RACH partitioning

Email discussions:

[AT121bis-e][300] Organizational Diana – NES, UAV, UP R15-17 UP/SDT

Scope:

    - Share plans for the meetings and list of ongoing email discussions for the sessions related to Rel-17 URLLC/IIoT, Small data, RA Partitioning, R15-16 UP, Rel-18 UAV and NES

    - Share meetings notes and agreements for review and endorsement

[AT121bis-e][306][R17 UP]  Correction on SDT (Vivo)

Scope: Revision of R2-2304906

Deadline: end of meeting

Intermediate email discussion

[POST121bis-e][308][MT-SDT] 38.300 Running CR (Nokia)

Scope: Running CR

Intended outcome: Version ready for endorsement to be submitted in next meeting

Deadline:  Intermediate email discussion

[POST121bis-e][309][MT-SDT] 38.321 Running CR (Huawei)

Scope: Running CR

Intended outcome: Version ready for endorsement to be submitted in next meeting

Deadline:  intermediate email discussion

[POST121bis-e][310][UAV] 38.300 Running CR (Nokia)

Scope: Running CR

Intended outcome: Version ready for endorsement to be submitted in next meeting

Deadline:  Intermediate email discussion

[POST121bis-e][311][NES] 38.300 Running CR (Ericsson)

Scope: Running CR

Intended outcome: Version ready for endorsement to be submitted in next meeting

Deadline:  Intermediate email discussion

Long email discussion

[POST121bis-e][307][NES] DTX/DRX – alignment, single/multiple configurations, parameter values (Huawei)

Scope: Provide and summarize companies' views on:

- Alignment between Cell DTX/DRX and UE C-DRX

- Single/multiple configurations

- Cell DTX/DRX parameter value range

Intended outcome: Report to the next meeting (with agreeable proposals)

Deadline:  long email discussion

[POST121bis-e][312][MT-SDT] 38.331 Running CR (ZTE)

Scope: Running CR

Intended outcome: Version ready for endorsement to be submitted in next meeting

Deadline:  Iong email discussion

# 5	NR Rel-15 and Rel-16

Essential corrections only.

Tdoc Limitation: 8 tdocs in total for all sub agenda items.

In case a correction need to be reflected in both NR TS and LTE TS, the corrections should be submitted under one single AI (so the NR and LTE correction can be treatee together), the sub-AIs below this

## 5.1	Common

Includes the following WIs and input that doesn’t fit elsewhere.

(NR_newRAT-Core; leading WG: RAN1; REL-15; started: Mar. 17; closed: Jun. 19: WID: RP-191971)

(NR_IAB-Core; leading WG: RAN2; REL-16; started: Dec 18; target Aug 20; WID: RP-200840)

(NR_unlic-Core; leading WG: RAN1; REL-16; started: Dec 18; Closed June 20; WID: RP-192926).

(NR_IIOT-Core; leading WG: RAN2; REL-16; started: Mar 19; Completed: Jun 20; WID: RP-200797)

(NR_UE_pow_sav-Core; leading WG: RAN1; REL-16; started: Mar 19; Completed Jun 20; WID: RP-200494).

(NR_2step_RACH-Core; leading WG: RAN1; REL-16; started: Dec 18; Completed: June 20; WID: RP-200085).

(SRVCC_NR_to_UMTS-Core; leading WG: RAN2; REL-16; started: Dec 18; Completed; Mar 20; WID: RP-190713)

(RACS-RAN-Core, leading WG: RAN2; REL-16; started: Mar 19; completed: Jun 20; WID: RP-191088)

(NG_RAN_PRN-Core; leading WG: RAN3; REL-16; started: Mar 19; completed: June 20; WID: RP-200122)

(NR_eMIMO-Core, leading WG: RAN1; REL-16; started: Jun 18; target; Aug 20; WID: RP-200474;)

(NR_CLI_RIM; leading WG: RAN1; REL-16; started: Dec 18; Completed: Jun 20; WID: RP-191997;)

(NR_L1enh_URLLC-Core, leading WG: RAN1; REL-16; Completed: June 20; WID: RP-191584)

(LTE_NR_DC_CA_enh-Core; leading WG: RAN2; REL-16; started: Jun 18; Target Aug 20; WI RP-200791)

(NR_Mob_enh-Core; leading WG: RAN2; REL-16; started: Jun 18; Completed June 20; WID: RP-192277).

(NR_HST, NR_RRM_enh-Core, NR_RF_FR1, NR_RF_FR2_req_enh, NR_n66_BW, LTE_NR_B41_Bn41_PC29dBm-Core, NR_CSIRS_L3meas,)

(NR TEI16).

LTE mob enh corrections that are common with NR mobility enhancements should be submitted to this AI.

### 5.1.2	User Plane corrections

User Plane corrections will be handled in the User Plane break out session

#### 5.1.2.0 	In-Principle-Agreed CRs

#### 5.1.2.1	MAC

#### 5.1.2.2	RLC PDCP SDAP BAP

#### 5.1.2.3	Other

User plane related corrections that should be handled in User plane break out session.

# 6	NR Rel-17

## 6.1	Common

(NR_MG_enh-Core; leading WG: RAN4; REL-17; WID: RP-211591)

(NR_UDC_enh-Core; leading WG: RAN2; REL-17; WID: RP-211203)

(NG_RAN_PRN_enh-Core; leading WG: RAN3; REL-17; WID: RP-202363)

(NR_IAB_enh-Core; leading WG: RAN2; REL-17; WID: RP-211548)

(NR_UE_pow_sav_enh-Core; leading WG: RAN2; REL-17; WID: RP-212632)

(LTE_NR_DC_enh2-Core; leading WG: RAN2; REL-17; WID: RP-201040)

(LTE_NR_MUSIM-Core; leading WG: RAN2; REL-17; WID: RP-212610)

(NR_Slice -Core; leading WG: RAN2; REL-17; WID: RP-212534)

(NR_QoE-Core; leading WG: RAN3; REL-17; WID: RP-211406)

(NR_ext_to_71GHz-Core; leading WG: RAN1; REL-17; WID: RP-212637)

(NR_cov_enh-Core; leading WG: RAN1; REL-17; WID: RP-211566): non-RACH-indication parts

(NR_redcap-Core; leading WG: RAN1; REL-17; WID: RP-211574)

(NR_feMIMO-Core; leading WG: RAN1; REL-17; WID: RP-212535)

(NR_SmallData_INACTIVE-Core, leading WG: RAN2; REL-17; WID: RP-212594)

(NR_IIOT_URLLC_enh-Core; leading WG: RAN2; REL-17; WID: RP-210854)

PRACH partitioning items

NR TEI17: Corrections are accepted. New TEI17 tech proposal requirements: a) authored by an operator (and preferably co-signed by more), AND: b) resolves a concrete problem in the market for this operator (no new vendor initiated enhancements).

Includes Rel-17 Work Items without specific R2 Agenda Item, e.g. RAN1 and RAN4 led items, SA2 and CT1 led items (was previously “Rel-17 Other”)

Includes aspects that does not fit under the more specific AIs, e.g. multi-WI aspects.

Tdoc Limitation: 10 tdocs

### 6.1.2	User Plane corrections

User Plane Related aspects will be handled in the User Plane break out session. (exception: TEI new proposals if any).

#### 6.1.2.0 	In-Principle-Agreed CRs

R2-2304791	Correction to CG-SDT LCH restriction	Huawei, HiSilicon	CR	Rel-17	38.321	17.4.0	1580	2	F	NR_SmallData_INACTIVE-Core	R2-2304351

=>	the CR is agreed

R2-2305463	Corrections on SDT using NCD-SSB for RedCap	Huawei, HiSilicon	CR	Rel-17	38.321	17.4.0	1584	2	F	NR_redcap-Core	R2-2304443

-	Vivo would like to move this text to SDT section.   Huawei and Ericsson think that this is sufficient.

=>	Understanding is that this note covers both CG and RA SDT

=>	The CR is agreed

R2-2305856	Clarification on RA Resource Selection During CG-SDT	vivo, ZTE Corporation (rapporteur), Sanechips	CR	Rel-17	38.321	17.4.0	1576	2	F	NR_SmallData_INACTIVE-Core	R2-2304446

=>	The CR is agreed

#### 6.1.2.1 	Other

R2-2304727	Correction to RA partition selection for Msg1 based SI request	Samsung Electronics Co., Ltd	CR	Rel-17	38.321	17.4.0	1613	-	F	NR_cov_enh-Core, NR_redcap-Core, NR_SmallData_INACTIVE-Core, NR_slice-Core

-	CATT doesn’t think this is needed.  LG thinks that this is good intention but we should use the same as contention free resource.  Samsung clarifies that in the MAC we use different terminology so that’s why we need the CR.

-	Qualcomm thinks that nothing is broken and the network can configure the UE in a consistent way.    Ericsson and Vivo agrees.  ZTE clarifies that for Redcap we need to initialize the UE to use redcap resources and the intention is correct and they are ok to simplify in line with LG’s comment.

=>	The CR will move to offline to discuss the need for the CR and finalize CR [301]

=>	The CR is revised in R2-2306806

R2-2306806	Correction to RA partition selection for Msg1 based SI request	Samsung Electronics Co., Ltd	CR	Rel-17	38.321	17.4.0	1613	-	F	NR_cov_enh-Core, NR_redcap-Core, NR_SmallData_INACTIVE-Core, NR_slice-Core

-	Huawei thinks we can change the word to ‘neither’

=>	change to “if neither contention-free Random Access Resources nor Random Access Resources for SI request have been provided for this Random Access procedure and one or more of the features including RedCap and/or Slicing and/or SDT and/or MSG3 repetition is applicable for this Random Access procedure”

=>	The CR is agreed in R2-2306814 with the change above

R2-2304906	Correction on SDT with separate initial BWP	vivo, Huawei, HiSilicon, Guangdong Genius	CR	Rel-17	38.321	17.4.0	1616	-	F	NR_redcap-Core, NR_SmallData_INACTIVE-Core	R2-2302660

-	Xiaomi thinks we need more discussion for CG SDT as RAN1 still needs more discussion on what PDCCH UE monitors.  Vivo has the understanding that even CG SDT monitors initial BWP.

-	LG agrees that this is not clearly specified in MAC but it is very clear in the RRC spec and there is no need to specify in both.   Vivo explains that in RRC it is just configuration and in MAC we have UE behavior.

-	Nokia thinks it is obvious that the RRC will configure it right and there is no need to clarify.   Ericsson has the same understanding and reminds us that we added clarifications in the RRC spec and it is very clear.

-	Qualcomm thinks that the network has to monitor the search space that the network configures but it would be good to clarify it in RRC.

-	ZTE agrees is ok with the CR but I heard similar comments.   Huawei thinks that we need to clarify the PDCCH monitoring behaviour that is not clear in RRC.

=>	We will only pursue a clarification related to the second change of the CR.

=>	Continue over email [306]

R2-2305748	Correction on HARQ buffer flush at SCG deactivation	Nokia, Apple, Mediatek, Qualcomm, Nokia Shanghai Bell	CR	Rel-17	38.321	17.4.0	1620	-	F	LTE_NR_DC_enh2-Core

-	Huawei thinks that when SCG is re-activated the network will anyways chose a new TB size so even if the UE misses the grant the UE will realize the TB mismatch and HARQ buffer will be flushed automatically when there is a new tx.

-	Lenovo also thinks that this is a rare conditions and it is the UE missing the first PDCCH transmission but are ok if majority want it.

-	LG clarifies that last meeting most companies thought that the toggling of NDI would solve the problem and most likely the PTAG will be expired and the case is extremely rare.   CATT agrees that this is very very corner case and the network is all knowing and knows the issue may occur when re-activated.

-	Mediatek explains that this is clearly an oversight as we have this for all other cases and we should fix this oversight and the UE should behave the same.

-	Samsung also thinks that there is nothing broken and network can solve it.

-	ZTE agrees with the intention and the impact is on the chipset and it is cleaner to change it.

-	LG thikns that if we agree to have a CR we don’t need to check if PTAG is running.  Nokia thinks that we can remove.

=>	Offline whether if agreable and what release

-	Huawei and Samsung is still not convinced and can compromise with R18

-	LG thinks that this is not a critical CR as it only happens in a particular scenario.

=>	The CR is postponed

R2-2305749	Clarification on unknown, unforeseen and erroneous protocol data during SDT	Nokia, Intel, Mediatek, Nokia Shanghai Bell	CR	Rel-17	38.321	17.4.0	1621	-	F	NR_SmallData_INACTIVE-Core

-	Vivo agrees with the intention but the legacy text has covered this case when the MAC resets.  Ericsson also thought this was clear.

-	LG explains that this text was written vaguely on purpose to cover all the cases and we should avoid listing all the cases.

-	Lenovo thinks this is needed as the bearer is suspended.

-	ZTE thinks that we should keep it general and not highlight SDT specifically.  “When a MAC entity receives a MAC PDU for the MAC entity's C-RNTI or CS-RNTI, or by the configured downlink assignment, containing an LCID or eLCID value which is not configured or LCID/eLCID associated to RBs not resumed, the MAC entity shall at least:

=>	 The CR is revised in R2-2306799

R2-2306799	Clarification on unknown, unforeseen and erroneous protocol data during SDT	Nokia, Intel, Mediatek, Nokia Shanghai Bell	CR	Rel-17	38.321	17.4.0	1621	1	F	NR_SmallData_INACTIVE-Core

=>	The CR is agreed

R2-2304907	Correction on CG-SDT with NCD-SSB measurement	vivo	CR	Rel-17	38.321	17.4.0	1617	-	F	NR_redcap-Core, NR_SmallData_INACTIVE-Core

-	ZTE clarifies that what we agreed last meeting covers both CG and RA SDT.  Huawei agrees

-	Ericsson also thinks it is clear

=>	Company understanding is that the current NOTE is applicable to CG and RA SDT

=>	the CR is not pursued

R2-2306385	Correction to carrier selection for RA-SDT	Langbo	CR	Rel-17	38.321	17.4.0	1628	-	F	NR_SmallData_INACTIVE-Core

-	Mediatek doesn’t see a need for a change  and there is no problem

-	LG thinks that this was discussed extensively in RA partition and it was concluded that there is no issue.   Ericsson agrees

=>	The CR is not pursued

R2-2306495	Correction on Enhanced BFR MAC CE	ZTE Corporation, Sanechips	CR	Rel-17	38.321	17.4.0	1629	-	F	NR_FeMIMO-Core

-	Qualcomm thinks that the current text covers both cases

-	Ericsson also understands that it covers the cases, it says one ore more.   CATT, Huawei and Lenovo agrees with QC and Ericsson.

-	LG agrees with ZTE

-	Nokia thinks that CR is makes and the at least one BFD sets is referring to the case where we have more than on BFD sets and the case covered by CR is missing.

=>	offline discussion [CB 303]

=>	The CR is revised in R2-2306798

R2-2306798	Correction on Enhanced BFR MAC CE	ZTE Corporation, Sanechips	CR	Rel-17	38.321	17.4.0	1629	-	F	NR_FeMIMO-Core

=>	The CR is agreed

# 7	Rel-18

## 7.3	Network energy savings for NR

(Netw_Energy_NR -Core; leading WG: RAN1; REL-18; WID: RP-223540)

Time budget: 1 TU

Tdoc Limitation: 4 tdocs

### 7.3.1	Organizational

LS, workplan, email discussion etc

Expected inputs to next meeting, running CRs for the following: 38.300 [Ericsson], 38.331 [Huawei], 38.321 [InterDigital]

Expected inputs after more agreements are made: 38.304 [Apple] (if needed), 38.306 [Vivo]

R2-2304627	LS on the enhancements to restricting paging in a limited area (R3-232084; contact: Nokia)	RAN3	LS in	Rel-18	Netw_Energy_NR-Core	To:RAN2, SA2

-	Nokia thinks that we should respond that there is no RAN2 impact.  Huawei agrees but thinks we should wait for SA2 respond.

-	Xiaomi is wondering if this solution works at all.  Qualcomm thinks that there are impact to RAN2.

=>	We will for SA2 to respond to RAN3

=>	Noted

R2-2306067	Work plan for NR network energy savings	Huawei, HiSilicon	Work Plan	Rel-18	Netw_Energy_NR

=>	Noted

### 7.3.2	DTX/DRX mechanism

R2-2305081	Support high priority traffic in Cell DTX / DRX	Apple, InterDigital, T-Mobile USA, MediaTek Inc., Intel	discussion	Rel-18	Netw_Energy_NR-Core

Proposal 2: The UE can be configured per SR configuration whether to allow SR transmission associated with high priority LCH(s) during Cell DRX non-active period.

-	Vodafone would like to understand what the action would be from the system.

-	Lenovo asks why we are limiting to just LCH and not SR for MAC CE (e.g. BFR).  Qualcomm thinks that we can allow MAC CE for BFR.   LG asks what type of traffic pattern is assumed.  Apple explain that if it is an emergency call it is not a priority.

Proposal 3: Reuse UE CDRX mechanism when the UE transmits SR during non-active duration of Cell DRX, i.e. the UE keeps monitoring PDCCH when a SR is sent on PUCCH and is pending.

Proposal 4: For the allowed SR during non-active duration of Cell DRX, no spec impact on its transmission behavior is foreseen, i.e. the SR can be retransmitted up to sr-TransMax and RACH is triggered when retransmission number is above the threshold.

=>	Noted

R2-2305335	Discussion on cell DTX-DRX mechanism	vivo	discussion	Rel-18

Proposal 3	RAN2 to confirm: As baseline, UE does not transmit SR occasions overlapping with Cell DRX non-active periods, and SR is not configurable to be transmitted during cell DRX non-active periods.

=>	Noted

R2-2305435	Emergency calls, Voice, Scheduling Requests and RACH	Vodafone 	discussion	Rel-18

Proposal 1: Cell DTX/DRX should not be enabled in the cell where emergency calls are ongoing.

Proposal 2: Cell DTX/DRX should be disabled once the gNB is informed about the emergency call to be setup within RRC connection request/Resume Request.

Proposal 3: There are no specific designs for SR handling due to emergency calls once the UE is in active, but the gNB shall disable the cell DTX/DRX functionality once it is informed about emergency call (in this case from the CN).

=>	Noted

R2-2306330	Discussion on SR transmission during the Cell DRX non-active period	NTT DOCOMO INC.	discussion	Rel-18	Netw_Energy_NR-Core

Propoasal 1	RAN2 to agree that SR exception handling should be configurable for emergency calls.

=>	Noted

R2-2305628	Discussion on cell DTX/DRX	CMCC	discussion	Rel-18	Netw_Energy_NR-Core

Proposal 1: Traffic with high priority can transmit SR during the non-active period of Cell DTX/DRX.

Proposal 3: Inactivity timer is introduced for extend the fixed length on duration for a better transmission assurance. Whether it is only used for a new transmission or a it can also be used for retransmission can be further discussed.

=>	Noted

R2-2305205	Discussion on Cell DTX/DRX	Fujitsu	discussion	Rel-18	Netw_Energy_NR-Core

=>	Noted

R2-2305120	Cell DTX-DRX Mechanism	Qualcomm Incorporated	discussion	Rel-18

Proposal 5: Upon transmitting SR during cell DTX non-active period, RAN2 to consider introducing a short CellDTX_SR timer to allow the UE to decode PDCCH, e.g., for an emergency call SR

Proposal 6: For SR that would be kept pending during cell DRX non-active period, a timer is configured by the gNB to control how long this SR can be kept pending in order to: 1. Avoid an immediate SR-RACH once cell DRX active time starts 2. Limit the amount of time SR can be kept pending.

=>	Noted

R2-2305975	Remaining issues for Cell DTX/DRX	ETRI

Proposal 1: Keep the agreement at the last meeting that no SR is transmitted during Cell DRX non-active period.

Proposal 2: Discuss whether RACH procedure is allowed when urgent uplink transmission is required during Cell DRX non-active period.

=>	Noted

Discussion

-	Xiaomi thinks that public safety should be prioritized over NES.

-	Vodafone thinks that we should discuss all the use cases and explains that the cases of emergency is very rare an domes delay is accepted.   What is not accepted is any kind of quality degradation and we shouldn’t allow an emergency call to have a 300ms duty cycle.

-	Huawei also agrees with Vodafone.  We have been using CS fallback so initial delays have been acceptable.   Oppo agrees

-	NEC thinks that if we allow to turn off the cell and if the UE wants to make the call the UE can use the coverage layer.  ZTE thinks that we don’t need to have SR.

-	CMCC would prefer to make it configurable to allow high priority traffic.   T-mobile thinks that an emergency call should be handled and not allowing SR for such calls is not acceptable.

-	CATT has a suggestion to allow RA access as a fall-back.   AT&T highlights that we have regulatory requirements that we have to meet and enable the calls.

-	Lenovo thinks that everyone agrees that emergency calls are important so the question is do we give the tools to the network to enable faster access or is the initial delay acceptable.

-	Ericsson thinks too that we have the random access and that goes quite far and for the NW you can exit the NES call once you get the request.

-	InterDigital explains that we agreed if we have SR we keep it pending and don’t trigger RA and the question is how long is the period expected to last.  Qualcomm agrees, we disabled the RACH and if we make the RACH fallback configurable we might as well make SR configurable.

-	Qualcomm explains that to use the RACH we need a new trigger.

-	Vodafone would like to ensure that the follow-up actions from network have to ensure that the quality of the emergency call is not degraded.

=>	offline to discuss mechanisms to enable emergency calls and the action from the network.

After offline:

Proposal 2: Once NW recognized there is an emergency call or public safety related service (e.g. MPS/MCS) going to be established, the NW deactivates Cell DTX/DRX (FFS for all UEs or some particular UEs with emergency call of public safety service in the cell). The behavior is captured in stage 2 spec.

-	Nokia thinks that we don’t need to capture when and how the NW deactivates.   Intel also clarifies that there are cases that there are cases that the gNB doesn’t know.

-	Vodafone thinks that it is important to capture in RAN2 the behavior that once recognized the NW should disable the functionality.

-	Qualcomm thinks that there is no RAN2 impact and not sure how we can capture this as this is a NAS procedure.

-	Huawei thinks that we can capture in stage 2 as it is an operator requirement.  LG clarifies that the NW may deactivate depending on the DTX pattern.

-	BT is trying to understand the issue with this proposal.  Intel clarifies that there are cases that gNB isn’t aware so if we want to enable it we need to involve CT1.

-	Ericsson thinks that we anyways still need to discuss whether we disable for some UEs or all UEs in general for the feature and not just related to this.

-	Vodafone wants to ensure that there is no impact to the emergency calls due to this feature.

-	CMCC asks if there is a risks that there will some inter-operability issue.  It should be up to gNB implementation.

R2-2304692	Discussion on Cell DTX/DRX configuration and operation	Xiaomi	discussion	Rel-18

R2-2305082	Discussion on key open issues of Cell DTX / DRX	Apple	discussion	Rel-18	Netw_Energy_NR-Core

PDCCH monitoring for retx

R2-2305321	Further discussion on cell DTX and DRX	ZTE corporation, Sanechips	discussion	Rel-18	Netw_Energy_NR-Core

Proposal 1: UE monitors PDCCH for RAR during Cell DTX non-active time. The ra-ResponseWindow could be started as legacy.

Proposal 2: UE monitors PDCCH for msg4 during Cell DTX non-active time. The ra-ContentionResolutionTimer could be started as legacy.

Proposal 4: UE doesn’t monitor PDCCH for retransmission during Cell DTX non-active time. The drx-RetransmissionTimer DL(UL) could be stopped during Cell DTX non-active time.

=>	Noted

R2-2305651	Remaining issues on DTX/DRX	Nokia, Nokia Shanghai Bell	discussion	Rel-18	Netw_Energy_NR-Core

Proposal 3: retransmission can be scheduled in case it falls out of the Cell DTX active period, i.e., when the DRX retransmission timer is running, the UE should monitor PDCCH regardless of the Cell DTX.

=>	Noted

Discussion:

-	Lenovo thinks that not continuing with the retransmission is acceptable.

-	CATT thinks we should make thinks simple and we shouldn’t touch C-DRX at all and it belongs to gNB to ensure that it doesn’t schedule during non-active periods.  The fraction of time the UE is on active time is a very short period of time and we should not optimize just for this.

-	Apple thinks that this is linked to the high priority traffic discussion.  If we allow high priority traffic we should proceed with Nokia proposal.

-	Ericsson thinks the network should be able to retransmit and the UE monitor PDCCH as per normal C-DRX.

-	Huawei supports ZTE’s proposal to keep it simple.

-	Qualcomm agrees Nokia even though we might making some exceptions.   Interdigital agrees and we should keep C-DRX as is.  Vodafone thinks we should ensure retx should be handle active period and we are taking about background traffic.   What is the problem if we wait for the retx.

-	CMCC thinks that it should be allowed as it is only a small fraction of the time.

-	LG, Samsung  and Oppo think that we shouldn’t monitor the PDCCH for the retx.

-	Xiaomi retx wonders if it is for emergency calls or others.

-	Mediatek has the same understanding as Nokia.

-	NEC thinks that the network should be able to configure the retransmission and maybe an activity timer can be considered.

-	Sony thinks that there is no need to optimize and would like to go with Nokia approach.  ETRI supports the Nokia proposal

-	BT thinks we should clarify whether it is ok to wait for the a bit and which types of traffic we are talking about.

Scheduled DG PUSCH/PDSCH during cell DRX/DTX non-active periods:

R2-2306044	Discussion on DTX/DRX mechanism	LG Electronics Inc.	discussion	Rel-18	Netw_Energy_NR-Core

=>	Noted

R2-2305120	Cell DTX-DRX Mechanism	Qualcomm Incorporated	discussion	Rel-18

Proposal 8: When an UL grant (PUCCH/PUSCH)/DL assignment (PDCCH/PDSCH) is scheduled by the gNB during cell DRX/DTX, respectively, the UE follows the grant assignment. No spec. impact for cell DTX/DRX, i.e., no implicit PHY cancellation at the UE.

=>	Noted

R2-2305925	Cell DTX/DRX mechanism	InterDigital	discussion	Rel-18	Netw_Energy_NR-Core

Proposal 8: 	The UE can stop drx-inactivity timer during the cell DTX non-active period if there are no pending retransmissions on any HARQ process.

-	Nokia thinks that we don’t need to modify the timers, we can just specify that the UE stop monitors.    Interdigital explains that this would avoid handling of each case.  Qualcomm thinks that we also  have to handle CA and groups.  Lenovo also thinks that we will need to discuss the timer later anyways.

Proposal 5. It is up to gNB implementation how to avoid scheduling of PDSCH reception/PUSCH transmission occurring during non-active period of cell DTX/DRX.

-	Interdigital thinks that with the assumption we made on Monday this is no longer applicable.  LG clarifies that that was related to re-tx only.

-	Huawei thinks that this is for PUSCH and PDSCH and we agreed only about PDCCH.

-	Qualcomm thinks that the intention is that if the UE gets something that is dynamically scheduled the UE shouldn’t have to have different behaviour and need to figure out what the DCI is for.   Oppo also thinks that if the DCI is received the UE should respect the grant.  Lenovo also agrees that we follow whatever the gNB does.  Apple has the same view.

-	NEC asks how we would handle the HARQ feedback.  CATT explains that the UE follows the order so the UE would also follow the HARQ feedback.   Huawei explains that it means we follow legacy.

-	NEC explains that the HARQ feedback should be for DL HARQ feedback as the UE doesn’t know when the gNB will schedule.  Nokia explained that anyways we agreed that the UE will monitor during retx timer so it can receive the UL feedback.

=>	Noted

Agreements:

1	UE monitors PDCCH for RAR during Cell DTX non-active time. The ra-ResponseWindow could be started as legacy.

2	UE monitors PDCCH for msg4 during Cell DTX non-active time. The ra-ContentionResolutionTimer could be started as legacy.

3	Working assumption:  When the retransmission timer is running (if C-DRX is configured), the UE is expected to monitor PDCCH, like in legacy.  It is up to the network whether it schedules retransmissions out of the Cell DTX active period, i.e., when the DRX retransmission timer is running, the UE should monitor PDCCH regardless of the Cell DTX.

4	Once gNB recognizes there is an emergency call or public safety related service (e.g. MPS/MCS), the NW should ensure there is no impact to the emergency call (e.g. may deactivate Cell DTX/DRX).  The behavior is captured in stage 2 spec

5	When an DG grant is received, by the gNB during cell DRX/DTX, the UE follows the grant assignment (i.e. like in legacy).  This includes DL HARQ feedback.

Multiple Configuration

R2-2305840	Further aspects on cell DTX/DRX	Ericsson	discussion

Proposal 2	The UE can be provided with at least two Cell DTX/DRX configurations. Only one configuration is active at a time.

R2-2306222	Cell DTX/DRX NES Techniques	III	discussion

Proposal 2: Multiple Cell DTX/DRX configurations should be included in further discussion.

C-DRX and Cell DTX alignment

R2-2305840	Further aspects on cell DTX/DRX	Ericsson	discussion

Proposal 3	A functional coexistence of UE C-DRX and Cell DTX can be ensured by the NW through the appropriate UE C-DRX and Cell DTX configurations, and by specifying the UE behaviour in cases when UE C-DRX active periods occur during Cell DTX non-active periods.

R2-2305651	Remaining issues on DTX/DRX	Nokia, Nokia Shanghai Bell	discussion	Rel-18	Netw_Energy_NR-Core

Proposal 1: alignment of UE’s C-DRX and NW cell DTX is up to NW implementation.

R2-2305389	Discussion on cell DTX and DRX	Huawei, HiSilicon	discussion	Rel-18	Netw_Energy_NR-Core

Proposal 2:	RAN2 discuss and define the related timers of cell DTX/DRX, e.g. celldtx-onDurationTimer and celldrx-onDurationTimer. The start timer formula of the onDurationTimer from UE C-DRX can be reused, i.e. “[(SFN * 10) + subframe number] modulo (cell DTX/DRX Cycle) = celldtx/celldrx StartOffset”

R2-2305870	Alignment between Cell DTX/DRX and C-DRX	CATT, Turkcell	discussion	Rel-18	FS_Netw_Energy_NR

Proposal 1: The Cell DTX/DRX active period can be extended on a UE basis, to follow the UE’s C-DRX Active Time (e.g., extended due to UE activity or ReTx).

Proposal 2: UEs which C-DRX on-duration starts earlier than the Cell DTX/DRX on-duration implicitly use the start-offset of Cell DTX/DRX as the (new) start-offset for their C-DRX after Cell DTX/DRX activation.

DTX-DRX alignement

R2-2305120	Cell DTX-DRX Mechanism	Qualcomm Incorporated	discussion	Rel-18

Proposal 1: Cell DRX is configured as part of Cell DTX configuration and must be, if configured, fully aligned with Cell DTX, i.e., Cell DRX and Cell DTX are both either active or non-active at a time.

HARQ feedback

R2-2305013	Remaining issues for Cell DTX_DRX	Samsung Electronics Co., Ltd	discussion	Rel-18	Netw_Energy_NR-Core

Proposal 2: RAN2 to discuss whether UE transmits HARQ feedback or not if the HARQ feedback occasion overlaps with non active period of Cell DRX.

Proposal 3: if HARQ feedback is not transmitted when HARQ feedback occasion overlaps with non active period of Cell DRX, UE start drx-HARQ-RTT-TimerDL in first symbol after the end of HARQ feedback occasion.

Activation/deactivation

R2-2306403	Discussion on cell DTX/DRX mechanisms - configuration and behaviour	BT plc, KDDI	discussion	Rel-18

Proposal 1	RAN2 to modify current implicit cell DTX/DRX activation/deactivation baseline to explicit configuration

Proposal 2	If explicit cell DTX/DRX configuration is agreed, RAN2 to discuss how cell DTX/DRX explicit activation/deactivation is performed to avoid/mitigate signalling failures and call drops

R2-2306407	Cell DTX and DRX Enhancements	Fraunhofer IIS, Fraunhofer HHI	discussion	Rel-18

Proposal 1:	RAN2 continues to discuss and evaluate dynamic signaling benefits, especially in light of the latest agreements.

Proposal 2:	Cell DTX/DRX supports L1 activation / de-activation. FFS L2 signaling

Proposal 3:	L1/L2 activation of Cell DTX/DRX supports common (group) activation

Proposal 4:	L1/L2 de-activation of Cell DTX/DRX supports both UE specific and common (group) de-activation

Other RA and paging

R2-2305529	Discussion on DTX/DRX mechanism	OPPO	discussion	Rel-18	Netw_Energy_NR

Proposal 7	RAN2 confirms no impact on RACH, paging and SIBs for both legacy UEs and Rel-18 non-NES capable UEs in the CONNECTED mode.

Proposal 8	RAN2 confirms no impact on paging for Rel-18 NES capable UEs in the CONNECTED mode.

R2-2305321	Further discussion on cell DTX and DRX	ZTE corporation, Sanechips	discussion	Rel-18	Netw_Energy_NR-Core

Proposal 1: UE monitors PDCCH for RAR during Cell DTX non-active time. The ra-ResponseWindow could be started as legacy.

Proposal 2: UE monitors PDCCH for msg4 during Cell DTX non-active time. The ra-ContentionResolutionTimer could be started as legacy.

R2-2305853	DL considerations for cell DTX/DRX	NEC Telecom MODUS Ltd.	discussion

R2-2305855	UL considerations for Cell DTX/DRX	NEC Telecom MODUS Ltd.	discussion

R2-2305941	Various alignment aspects	Lenovo	discussion	Netw_Energy_NR-Core

R2-2306074	Considerations on Cell DTX/DRX	KDDI Corporation	discussion

R2-2306500	Reminding issues on stage 2 of the Cell DTX/DRX	MediaTek Inc.	discussion	Rel-18	Netw_Energy_NR-Core

### 7.3.3	SSB-less Scell operation

Contributions on inter-band CA for FR1 and co-located cells

R2-2305083	Discussion on RAN2 work of inter-band SSB-less CA	Apple	discussion	Rel-18	Netw_Energy_NR-Core

Proposal 1: If RAN4 conclude SSB-less SCell for inter-band CA for FR1 and co-located cells is feasible, the signaling of intra-band CA (including RRC change on timing of SSB-less SCell and capability signaling) can be considered as its baseline. Whether other new signaling is required depends on RAN4 input.

Proposal 2: Before RAN4 provide sufficient input, RAN2 do not start discussion on enhancement of L1/ L3 measurement and SCell activation procedures.

=>	Noted

R2-2305775	Discussion on SSB-less SCell operation	CMCC	discussion	Rel-18	Netw_Energy_NR-Core

Proposal 1: RAN2 needs to wait for further RAN4 progress on the scenario feasibility of the inter-band SSB-less SCell requirements.

Proposal 2: If RAN4 concludes it is feasible, RAN2 can further work on the following specification impacts:

-	RRC configuration of the frequency of the SSB to be used for the UE to obtain the timing reference for the inter-band SCell.

-	UE capability reporting to indicate whether UE supports configuration of inter-band SCell that does not transmit SS/PBCH block.

-	Potential impact on beam management, radio link monitor, RRM measurement.

=>	Noted

Discussion

-	Huawei explains that RAN2 has agreed to continue work on scenario 1 and 2a, but there are still concerns about RAN1 involvment.  The RAN2 impacts in CMCC papers are good impacts but RRM is still being discussed in RAN4.

Agreements:

- If RAN4 conclude SSB-less SCell for inter-band CA for FR1 and co-located cells is feasible, the signaling of intra-band CA (including RRC change on timing of SSB-less SCell and capability signaling) can be considered as its baseline. Whether other new signaling is required depends on RAN4 input.

- If RAN4 concludes it is feasible, RAN2 can further work on at leaest the following specification impacts:

-	RRC configuration of the frequency of the SSB to be used for the UE to obtain the timing reference for the inter-band SCell.

-	UE capability reporting to indicate whether UE supports configuration of inter-band SCell that does not transmit SS/PBCH block.

Not treated

R2-2304694	Discussion on inter-band SSB-less Scell	Xiaomi	discussion	Rel-18

R2-2304862	Enhancements of SBB/SIB-less NES solutions	Dell Technologies	discussion	Rel-18

R2-2305250	Discussion on SSB/SIB-less Solutions for NES	Samsung	discussion	Rel-18

R2-2305320	Discussion on SSB-less SCell operation for NES	ZTE corporation, Sanechips	discussion	Rel-18	Netw_Energy_NR-Core

R2-2305336	RAN2 impact on supporting inter-band SSB-less Scell operation	vivo	discussion	Rel-18

R2-2305721	Discuss on SSB-less SCell operation in NES	Lenovo	discussion	Rel-18

R2-2305841	SSB-less Scell operation on inter-band CA for FR1	Ericsson	discussion

R2-2305907	On NES SSB-less SCell operation	Nokia, Nokia Shanghai Bell	discussion	Rel-18	Netw_Energy_NR-Core

R2-2305928	SSB-less Scell operation	InterDigital	discussion	Rel-18	Netw_Energy_NR-Core

R2-2306068	Discussion on SSB-less SCell operation	Huawei, HiSilicon	discussion	Rel-18	Netw_Energy_NR

### 7.3.4	Cell selection/re-selection

Contributions mechanisms to prevent legacy UEs camping on cells adopting the Rel-18 NES mode

R2-2306406	Identify NES capable UEs by network	BT plc	discussion	Rel-18

Proposal 2	RAN2 to agree that UE reports to network its NES capabilities, i.e., cell DTX/DRX UE capabilities, during UE capabilities exchange process

-	Huawei thinks that we can agree that we will have a UE capability and further details are FFS.

Proposal 3	RAN2 to discuss if a new barring mechanism to bar non-NES capable UEs is specified in Rel-18 based on pros and cons

=>	Noted

R2-2305121	Barring legacy UEs for NES Cells	Qualcomm Incorporated, T-Mobile US	discussion	Rel-18

Proposal 1: A Solution to introduce legacy UEs from camping on a cell applying cell DTX/DRX is not pursued in Rel-18. It is up to NW to align legacy UEs to cell DTX/DRX cycles or offloading them to another cell in case of overlapping coverage.

=>	Noted

R2-2305390	Discussion on cell selection/reselection for NES	Huawei, HiSilicon	discussion	Rel-18	Netw_Energy_NR-Core

Proposal 1: Separate camping restrictions for NES-capable and non-NES UEs are needed for ensuring NW NES gains. Which NES technique(s) apply to the new restrictions is left up to NW implementation and can be revisited once the details of NES techniques are specified.

Proposal 2: Introduce a new cellBarred-NES IE to enable separate barring of legacy and NES UEs.

Proposal 3: Introduce new IntraFreqExcludedCellList-NES / InterFreqExcludedCellList-NES IEs enable proper reselection behaviour of legacy and NES UEs.

=>	Noted

Discussions on barring

-	Oppo and Huawei think that we should be able to bar non-NES capable UE.

-	Vodafone asks if we are barring all legacy UEs.  Huawei explains we are talking about one bit and let the network decide whether it uses it or not.   Vodafone would like to have a mechanism that doesn’t bar all non-NES UEs.    Vivo thinks that a mechanism to just prioritize NES cell should be sufficient.

-	Lenovo explains that we should not use MIB barring as MIB barring would disable emergency calls.

-	CMCC agrees that we need to have a mechanism especially for overlapping coverage to avoid performance drop for legacy UEs.   AT&T thinks that we should introduce this mechanism just like other barring mechanisms for other cases.  Samsung agrees.

-	Vodafone is not against barring but doesn’t think that it work.   Interdigital thinks one bit is sufficient and the network can decide anyways.   RAN1 is working on spatial adaptation which will have coverage impact on legacy UEs so we should have a mechanism.  Nokia agrees so we should minimize performance degradation to legacy UEs.   One bit is enough but not sure if we would need a barring per feature.

-	LG doesn’t agree to proposal 3 but is ok with other proposal.  Ericsson would fine with a single bit for barring.

-	Qualcomm is concerned that we would need to define a bit for every NES feature.  Ericsson and Nokia explain that it would be one bit that would include all NES features.   Qualcomm is still concerned.  Ericsson thinks that this is will be a small limitation of the feature.  T-mobile is concerned about having a single bit for all NES feature it means that the coverage layer cannot enjoy a particular NES feature.

R2-2305530	Discussion on cell selection reselection	OPPO	discussion	Rel-18	Netw_Energy_NR

Proposal 1	If needed, cellBarred in MIB is used to block the legacy UE from accessing an NES cell.

Proposal 2	RAN2 introduces a new barring indication in SIB1 to control the NES-capable UEs accessing an NES cell. In detail, once cellBarred in MIB is indicated as Barred, the NES-capable UEs need to further check the new barring indication in SIB1.

Proposal 3	RAN2 confirms a unified control of all NES-capable UEs accessing a certain NES cell, i.e. all NES-capable UEs are uniformly blocked or allowed to access a cell.

=>	Noted

Agreements:

- We will define UE capabilities with signaling.  Details are FFS and will be discussed later during the WI phase.

- Separate camping restrictions for NES-capable and non-NES UEs will be defined.  FFS if it is a single bit or more.

Not treated

R2-2304691	Discussion on UE access control in NES cell	Xiaomi	discussion	Rel-18

R2-2305251	Discussion on Cell Selection and Reselection for NES	Samsung	discussion	Rel-18

R2-2305323	Consideration on preventing legacy UEs camping on NES cell	ZTE corporation, Sanechips	discussion	Rel-18	Netw_Energy_NR-Core

R2-2305337	Discussion on cell selection/re-selection	vivo	discussion	Rel-18

R2-2305455	Definition of NES and barring on cell DTX/DRX cells	Vodafone 	discussion	Rel-18

R2-2305718	Cell selection/re-selection in NES	Lenovo	discussion	Rel-18

R2-2305776	Discussion on cell barring and reselection for NES	CMCC	discussion	Rel-18	Netw_Energy_NR-Core

R2-2305842	NES Cell selection/reselection	Ericsson	discussion

R2-2305858	Procedure for legacy UEs camping on NES cells	NEC Telecom MODUS Ltd.	discussion	R2-2301522

R2-2305871	Consideration on Cell Selection/Re-selection on NES cells	CATT, Turkcell	discussion	Rel-18	FS_Netw_Energy_NR

R2-2305892	Cell Reselection Enhancements Supporting NES	Google Inc.	discussion

R2-2305926	Cell selection and resection for NES	InterDigital	discussion	Rel-18	Netw_Energy_NR-Core

R2-2305974	Legacy UE Handling for NES	ETRI	discussion	R2-2301463

R2-2306059	Considerations on Cell selection/re-selection	KDDI Corporation	discussion

R2-2306276	Access control enhancement for NES 	LG Electronics France	discussion	Netw_Energy_NR-Core

=> Revised in R2-2306538

R2-2306538	Access control enhancement for NES 	LG Electronics France	discussion	Netw_Energy_NR-Core

R2-2306329	Discussion on Cell selection	NTT DOCOMO INC.	discussion	Rel-18	Netw_Energy_NR-Core

R2-2306361	Reselection and Paging handling for NES	Nokia, Nokia Shanghai Bell	discussion	Rel-18	FS_Netw_Energy_NR

R2-2306410	Cell Selection and Re-Selection for NES	Fraunhofer IIS, Fraunhofer HHI	discussion	Rel-18

### 7.3.5	Connected mode mobility

Contributions on CHO procedure enhancement(s) in case source/target cell is in NES mode

R2-2305122	NES Connected mode mobility	Qualcomm Incorporated	discussion	Rel-18

Proposal 4: As a baseline, NES CHO comprises a change in existing CHO configuration conditions to make it harder for UEs to connect or stay connected to a cell in NES mode.

Proposal 5: RAN2 to discuss whether the RRC CHO configuration can be enhanced to include multiple offsets to account for source/target cell being in Normal or NES mode.

Proposal 6: RAN2 to discuss how an L2 “NES trigger” can modify CHO thresholds among the following two baseline options:

•	Option 1: NES CHO trigger informs UE of NES mode of source and target cell(s), UE changes the conditions for CHO according to a pre-configuration of mode dependent A3-A5 offsets.

•	Option 2: NES CHO trigger directly instructs UE to change the condition of an existing CHO configuration.

-	Huawei wants to ensure that we are addressing the same requirements, in DRX we are addressing high QoS cases.

-	Vodafone as if switching cells off is considered here.   Qualcomm explains that first you want the UEs to exist the cell and you can switch off the cell.

-	Apple asks if cell means source or target cell.   Qualcom explains that it is both.  CATT asks how the UE know know that the target cell has changed.

=>	Noted

R2-2305942	CHO Procedure in NES Mode	Lenovo	discussion	Netw_Energy_NR-Core

Proposal 1: If relaxed measurements are configured (and subsequently handover condition is met), UE starts handover execution only when the source cell is about to get into Cell DRX/ DTX sleep.

-	Vodafone asks what is the relaxed measurement.

Proposal 2: The time when the source cell starts (or about to start) Cell DRX/ DTX sleep can be determined by the UE based on the received start offset of Cell DTX/ DRX configuration received in UE dedicated RRC signalling.

Proposal 3: L1 based signaling to activate/ deactivate NES mode for executing conditional handover is not considered.

=>	Noted

R2-2306362	CHO on NES	Nokia, Nokia Shanghai Bell	discussion	Rel-18	FS_Netw_Energy_NR

Proposal 1: Add for events A3, A4 and A5 a additional parameter that indicates that event is triggered only if “NES trigger” is active for the source cell.

-	Nokia explains that you configure a NES A4 specific event.

Proposal 2: “The NES trigger” would be at least for the use case of turning off the cell (whether other triggers are enabled is FFS and need to wait that WI progresses on other aspects of the WI).

=>	Noted

Discussion on CHO threshold configuration or event configuration

-	Samsung thinks that we use configure existing events with NES triggers.  Apple explains that with existing framework that the same event is configured with different threshold.

-	Google thinks that event A4 is the best even to use and even then A4 is only applicable to NTN scenario.   CATT agrees with google and we should be able to use A4 for TN as well and this allows the UE to chose the next best cell without considering the source cell.  Apple thinks A4 alone is not sufficient as we need to handle the case where the UE can trigger the even before A4.   Interdigital also considers A4 a good contition but it should be linked with a time or window similar to NTN.

-	Oppo thinks we can have different threshold for an event whether NES is configured or not.  Xiaomi thinks that we should talk about the time point of triggering NES.

-	Vodafone explains that we have two cases, we switch off the cell and you don’t care about thresholds/configurations.  The other case when you want to keep some of the devices.

-	Lenovo thinks that we should give the option to the network to configure A3,A4,A5.  We can have two conditions.  Oppo thinks that Vodafone comments on cell off is very important.

-	CATT asks how the UE know the cell is in NES.  Lenovo assumes that the UE can use CELL_DTX/DRX configuration so the UE know when it will start sleeping.

-	ZTE is not sure why we would need two CHO execution condition.

-	LG asks how two different CHO execution condition would be achieved, one event and two thresholds or two events.

-	Mediatek asks if the assumption is that NES mode is for time domain only then the UE knows.

R2-2305511	Handover enhancement for NES	Sony	discussion	Rel-18	FS_Netw_Energy_NR

Proposal 1: Network should notify the UE to start performing NES CHO execution when the NES mode of source or candidate cells is going to change or has changed.

Proposal 2: The NES CHO execution trigger is based on L1 signaling.

-	Lenovo asks why the network wouldn’t send the message when it has decided to switch off.

-	Samsung clarifies that RAN1 agreed on L1 signaling and they would also like to have L2 signaling.  Samsung asks whether the intention is to use the RAN1 signalling for activation/deactivation that RAN1 agreed.  Sony explains that it would be a new signaling for CHO.

-	Nokia asks if this should be evaluation instead of being execution.  Sony explains that it is execution.  Qualcomm doesn’t like L1 stuff as L1 is not involved and what happens if the network would like to enable DTX but not handover the UEs.  Lenovo explains that the LS from RAN1 is related to deactivation/activation.

=>	Noted

R2-2304693	Discussion on UE mobility due to NES cell	Xiaomi	discussion	Rel-18

Proposal 4: The condEventT1 introduced in R17 for NTN is reused for time-based CHO in NES, the duration IE can be ignored or set to 1.

=>	Noted

R2-2305890	CHO Enhancements Supporting NES	Google Inc.	discussion

Proposal 1	CondEventA4 can be configured as a CHO execution condition in the NES scenario.

-	Lenovo thinks that the eventT1 is a condition on when the gNB is entering NES mode which is part of the second agreement FFS.    Apple thinks this should be for further study as it add a new requirement and NES UE shouldn’t use the UTC time.  Oppo thikns that the time based information can be added to the CHO configuration.   The current event T1 can be modified and we should just consider the start time.

-	CATT supports google proposal to be able use event A4 as it considers only the quality of neighboring cell.   CMCC also supports A4 event plus timer based.  Qualcomm indicates that the timer is useful to avoid RACH storming but there is not reason to really delay the evaluation.

-	Vodafone doesn’t thinks A4 is needed but timer based would be good.   CATT thinks that one strategy is to set the threshold very low but that increases the risk of failure.  Verizon thinks that the operators would find the timer based more useful but in any case the operator should be given the option to configure the events it finds most useful.

=>	Noted

Agreements

1.	We will have a CHO solution that considers NES mode of at least source cell.

2.	We can have a specific NES CHO execution condition based on source cell NES mode.   FFS how the UE determines is in NES mode.   FFS on how this is achieved in RRC

3.	We will not introduce new L1 signalling for the purpose of CHO

4.	Event A3, A4, A5 can be configured as a CHO execution condition in the NES scenario.   We will study the time based mechanism

Not treated

R2-2305252	Discussion on Connected mode mobility for NES	Samsung	discussion	Rel-18

Proposal 1. RAN2 needs to discuss and determine the conditions in a CHO configuration, which could be one or more than one of conditions of : A3, A4, A5, and a new condition of “NES activation signal reception”

R2-2305461	Triggering conditions and other aspects of the Handover to/from DTX/DRX cells	Vodafone GmbH	discussion	Rel-18

Proposal 1: For the case the “Source Cell is going to switch off” no new CHO evaluation conditions associated with the cell going to be switched off are needed.

Proposal 2: To trigger the CHO in case of Source Cell is going to be switched off, broadcast signalling is used

Proposal 3: To trigger the CHO in case of Source Cell is going to be switched off, RRC signalling is used

Proposal 4: It is proposed to discuss if additional timer is needed to facilitate the UEs in case  Source Cell is going to switch off.

Target cell NES state

R2-2306052	Discussion on CHO enhancements for NES	Sharp	discussion

Proposal 4: If the network knows the mode of a target cell, the network can indicate the mode of the target cell to the UE by explicit indication

Proposal 5: If the network can anticipate when a target cell will switch to NES mode or return to normal mode, a time based conditional handover event can be introduced to implicitly indicate the mode switching of target cell.

R2-2305206	Discussion on Connected mode mobility for network energy savings	Fujitsu	discussion	Rel-18	Netw_Energy_NR-Core

Proposal 5: 	RAN2 to confirm the UE does not have to obtain the NES mode of the cell from the target cell.

Proposal 6: 	The priority information is additionally provided by the source cell to select a suitable target cell.

Proposal 7: 	The UE selects the CHO candidate cell indicated as a high priority

R2-2305338	Conditional handover enhancement for network energy saving	vivo	discussion	Rel-18

Proposal 6: The NES mode/priority/NES capability of candidate cells are configured by the network, and it is up to the UE which one to select based on the NES mode and the signal quality of the candidate cells.

R2-2306362	CHO on NES	Nokia, Nokia Shanghai Bell	discussion	Rel-18	FS_Netw_Energy_NR

Proposal 4: It can be left up to UE implementation to select target cell out of multiple candidate CHO cells

R2-2305872	CHO enhancement for NES	CATT, Turkcell	discussion	Rel-18	FS_Netw_Energy_NR

Proposal 3  It is up to NW implementation whether to set a NES cell as a candidate cell. No specification change is needed.

R2-2305864	CHO procedure enhancement to support NES mode	NEC Telecom MODUS Ltd.	Discussion

Proposal-1: RAN2 send LS to RAN3 to enhance the signalling between the two gNBs to support NES mode notification between neighbour gNBs.

Proposal-2: Network implementation should be allowed for source cell to not select a target cell in NES mode as CHO candidate cell for the UE.

Proposal-3: The target cell’s NES mode should be indicated as part of the target cell’s CHO configuration to the UE during CHO preparation.

Proposal-4: The detailed configurations of target cell’s NES operation should be informed to the source cell and then to the UE during CHO preparation.

Proposal-5: Legacy CHO configuration update procedure is used to notify the UE the change of NES mode of the CHO candidate cell.

Failure handling

R2-2305860	CHO for NES	Ericsson	discussion	Rel-18	Netw_Energy_NR-Core

Proposal 2  Network needs to know if there are no good enough candidate target cells for CHO at the time cell is going to deactivate or enter cell DTX/DRX.

Proposal 	3 Enhance CHO procedure to enable priorization of candidate target cells by the UE based on NES mode.

R2-2305872	CHO enhancement for NES	CATT, Turkcell	discussion	Rel-18	FS_Netw_Energy_NR

Proposal 3  It is up to NW implementation whether to set a NES cell as a candidate cell. No specification change is needed.

Proposal 4: Considering the rare cases of HO failures, legacy connection re-establishment is an appropriate way to handle the case when HO fails and the source cell is about to enter NES mode

R2-2305084	Discussion on CHO enhancement in NES	Apple	discussion	Rel-18	Netw_Energy_NR-Core

if multiple CHO candidate cells fulfill the condition and the priority information is provided.

Proposal 3: If no triggered cell is available by the time source cell entering "NES mode", the UE re-evaluates candidate target cells with a delayed CondEvent Ax, which may be a CondEvent A3/A5 with a looser threshold or a CondEvent A4 which only evaluates radio condition of neighbor cell.

R2-2305322	Further discussion on connected mode mobility	ZTE corporation, Sanechips	discussion	Rel-18	Netw_Energy_NR-Core

R2-2305531	Discussion on connected mode mobility	OPPO	discussion	Rel-18	Netw_Energy_NR

R2-2305629	Discussion on Connected mode mobility	CMCC	discussion	Rel-18	Netw_Energy_NR-Core

R2-2305927	NES mobility aspects	InterDigital	discussion	Rel-18	Netw_Energy_NR-Core

R2-2306069	Discussion on CHO enhancement for NES	Huawei, HiSilicon	discussion	Rel-18	Netw_Energy_NR

R2-2306240	Mobility enhancement: mobility triggering by light handover command	LG Electronics Inc.	discussion	Rel-18	Netw_Energy_NR-Core

### 7.3.6	Others

This will be downprioritized

R2-2305123	Discussion of RAN3 LS on Restricting Paging	Qualcomm Incorporated	discussion	Rel-18

R2-2305512	Skip monitoring of CSI-RS during non-active periods	Sony	discussion	Rel-18	FS_Netw_Energy_NR

## 7.8	NR support for UAV

(NR_UAV -Core; leading WG: RAN2; REL-18; WID: RP-223545)

Time budget: 1 TU

Tdoc Limitation: 4

### 7.8.1	Organizational

Stage 2 running CR expected as input to this meeting

Expected output into next meeting, Running CRs for 38.331 (Qualcomm), 38.300 (Nokia)

Expected output later after capability discussions: 38.306 (Huawei)

R2-2305885	Uncrewed Aerial Vehicles in Rel-18 - Updated Workplan	Nokia, Nokia Shanghai Bell	Work Plan	Rel-18	NR_UAV-Core

=>	Noted

R2-2305886	Stage-2 Text Proposal for Rel-18 UAVs	Nokia, Nokia Shanghai Bell	discussion	Rel-18	NR_UAV-Core

=>	The CR is endorsed and will be updated after this meeting

### 7.8.2	Measurement reporting for mobility and interference control

Contributions should focus on further details related enhancement to measurement reports taking into account agreements made in RAN2#121bis-e

Height dependent configurations:

SSB-ToMeasure:

R2-2305056	Measurement and reporting enhancements	Qualcomm Incorporated	discussion	Rel-18	NR_UAV-Core

Proposal 1. Add height-based list of SSB-ToMeasure with corresponding height ranges and hysteresis in MeasObjectNR (TP shown above can be taken as baseline).

-	Xiaomi is asking how many height ranges should be configured.   Qualcomm thinks that more than 2 should be possible.

-	Nokia asks if we can also configure other measurements, like number of triggered cells.

=>	Noted

R2-2306491	Height-dependent measurement configuration	ZTE Corporation, Sanechips	discussion	Rel-18	NR_UAV-Core

Proposal 2: The only impact on L3 measurement is to capture in the SSB-ToMeasure field description that the UE applies the SSB-ToMeasure when it is in associated height-region.

-	LG thinks that we need to specify some timers.  Xiaomi thinks that using a similar procedure as RRC reconfiguration maybe better.  Vivo agrees  with Xiaomi and it is more aligned with the motivation of introducing such feature.

Proposal 3: As a basic principle, if no height-specific SSB-ToMeasure is configured for a specific height region, the legacy SSB-ToMeasure is applied.

-	LG and Qualcomm are good with this proposal.   Samsung thinks that we should have the option for the network to not measure any SSB so it should be controlled by the network.  Nokia thinks that if it is not configured the UE can either measure SSB-to measure without height spefic configuration or measure on all SSBs.

=>	Noted

R2-2305868	UAV measurement reports 	Ericsson 	discussion	Rel-18	NR_UAV-Core

Proposal 5: For UE behavior on L1 and L3 measurement, RAN2 to discuss, for example, whether to keep/discard the old samples while UE moves to a new height region with a different SSB-ToMeasure value

-	Ericsson thinks that this should be left up to UE implementation.  Qualcomm agrees.

-	Samsung thinks that the UE should discard old samples.  Qualcomm explains that there may be cases where you have the same SSBs across boundaries so the UE shouldn’t discard in this case.

-	CMCC and ZTE agrees that it can be left to UE implementation

=>	Noted

Numberoftriggeringcells:

R2-2305600	Discussion on Measurement Reporting for NR UAV	CMCC	discussion	Rel-18	NR_UAV-Core

Proposal 4: It is proposed to configure different numberoftriggeringcells value correspond to different height ranges, for example, height range 1 for NumberOfTriggeringCells value 1, height range 2 for NumberOfTriggeringCells value 2, and so on, and a step for height range could also be discussed.

-	Nokia asks if this is in the report config and how does this enable the event A4.  Qualcomm clarifies that this is for the measurement report config unlike SSB-tomeasure

=>	Noted

R2-2306215	Measurement report enhancement for UAV	Huawei, HiSilicon	discussion	Rel-18	NR_UAV-Core

Proposal 6: The UE should maintain the measured cells in cellTriggeredList when a new numberOfTriggeringCells is applied due to the UE reaching a new height range.

=>	Noted

A4-threshold:

R2-2306135	Discussion on measurement reporting for NR UAV	Xiaomi	discussion	Rel-18	NR_UAV-Core

Proposal 10: Height-dependent configuration of MR configuration parameters is supported using combination of events H1/H2 with other events (i.e event Ax).

=>	Noted

R2-2305691	Discussion on height dependent measurement for NR UAV	Lenovo	discussion	Rel-18

Proposal 2: Specify different A4 threshold values for different height region in measurement configuration, instead of combination of events

=>	Noted

Discussion on numberoftriggering cells and A4

Configure different numberoftriggeringcells value corresponding to different height ranges.  This configuration can be in the report config

-	Nokia thinks that event combination would require the UE to send extra data and H1/H2 are triggered when the thresholds are met and that’s it.  If it is a trigger it will happen only one for the A4 event so H1/H2 is not appropriate to implement this.

-	LG supports combination of events.

-	Samsung doesn’t support combination as with H1 and H2 we cannot specify a height region with upper and lower bound.

-	ZTE is generally fine with combination and there is a way to implement it to have a height range dependent configuration.

-	Vivo supports to have a unified solution for MR and MO and for combination of events there are more issues to address.

-	Ericsson thinks the combination can work and doesn’t see the advantage of having multiple ranges.

-	Huawei thinks both can work but the modeling of Lenovo is better, as it implements a height dependent threshold.

-	Ericsson asks if there is a technical reason why we want a unified solution. Vivo explains that this better for the specification.

=>	continue over offline in R2-2306800

R2-2306800	Offline of 304 (ZTE)

Proposal 1: A unified solution (e.g. for ASN.1 signalling structure, procedural text) is considered to implement both height-dependent MR configuration and combination of events.

-	Ericsson thinks this is is going against the previous agreement.  Nokia explains that combining the existing events was problematic and it was easier to have a separate event.  Qualcomm explains that it is true that we didn’t explict agree last meeting but we didn’t say we will never introduce new event.

-	Nokia thinks that proposal 1 is not needed. NEC doesn’t think the proposal is agreable.

=>	Noted

General Heigh-dependent aspects:

R2-2305429	Discussion on measurement reporting enhancement for NR UAV	vivo

Proposal 4: ToAddModList / ToRemoveList structure is used for configuring the height-dependent parameter, in which the height specific parameter is linked with the related height region by one entry.

=>	Noted

R2-2306046	Remaining issues on measurement reporting enhancements in NR UAV	Samsung Electronics Austria	discussion	Rel-18	NR_UAV-Core

Proposal 8: RAN2 to discuss the number of height regions we should consider for height-dependent configurations, e.g., one, two, or more than two.

Proposal 10: RAN2 to discuss how to avoid applying a height-specific value back and forth constantly caused by ping-poing effect.

-	Samsung proposes some timer.  Qualcomm explains that we have the hysteresis.  Interdigital agrees that hysteris works but we only agreed to the SSBstomeasure.    Samsung doesn’t thinks this is enough. Vodafone thought that they don’t change their heights very randomly it is predictable and hysteresis is sufficient.

-	Spirant explains that there is some anomaly within a height and the pilot is controlling the flight that can vary with the wind.

=>	Noted

Height/location reporting

R2-2306046	Remaining issues on measurement reporting enhancements in NR UAV	Samsung Electronics Austria	discussion	Rel-18	NR_UAV-Core

Proposal 1: When event H1 or H2 triggers, UE includes its height in the measurement report.

-	Qualcomm thinks that it can be optional and it is not always needed.  Nokia agrees that there are cases the gNB doesn’t need to know as the network knows approximately what the height.

-	Xiaomi thinks we should follow the LTE mechanism and have it mandatory.   Interdigital thinks that it should be up to the network.

Proposal 2: As in LTE, define the separate field (e.g. heightUE) to indicate UAV UE's height in the IE  MeasResults for event H1 and H2.

Proposal 3: When event H1 or event H2 triggers, location information (e.g. the IE CommonLocationInfo) is included in the measurement report as in legacy. No specification change is needed.

=>	Noted

R2-2305056	Measurement and reporting enhancements	Qualcomm Incorporated	discussion	Rel-18	NR_UAV-Core

Proposal 3: UE height, location and/or velocity reporting without being accompanied by other RRM report is supported.

-	LG would like to follow LTE mechanism.   Nokia explains that the network is generally interested to get RRM report but there may cases.   Samsung thinks that the PCell measurement are important but other things may not be needed.

-	Qualcomm explains that the expectation is the network will indicate what to report.

Proposal 4: Whether UE height is included when event H1 or H2 triggers is also configurable by the network.

-	Nokia supports this.   LG doesn’t understand why we make it complicated and what’s the harm in reporting it.  QC explains it is signaling overhead.   Vivo thinks that keeping it like LTE is better and the network can benefit from the information.  Interdigital explains that there are cases where the network wouldn’t benefit from the information.   Sony thinks that after the event is triggered and after that the information is not valid anymore and it is obsolete information.

-	Samsung thinks that the there no benefit to optionally reporting so they would like it mandatory.

-	ZTE thinks that LTE mechanism can be re-used.

-	ZTE asks if this is also applicable to the new events.

Proposal 5: H1/H2 triggered UE height reporting using uncompensated barometric pressure measurement is supported. (Height reporting using an RRC field as in LTE is not introduced.)

-	Samsung and Huawei doesn’t see a need and we can re-use LTE field.   Oppo clarifies that this is introduced for SON.

-	Nokia asks what is the justification.  Qualcomm explains it is to not introduce redundancy.

=>	no support

=>	Noted

Agreements

- Add height-based list of SSB-ToMeasure with corresponding height ranges and hysteresis in MeasObjectNR.  FFS on the number of height ranges

- As a basic principle, if no height-specific SSB-ToMeasure is configured for a specific height region, the legacy behaviour applies.

- For UE behavior on L1 and L3 measurement, it is left to UE implementation whether to keep/discard the old samples while UE moves to a new height region with a different SSB-ToMeasure value

- New event types will be introduced on the combination of event Ax and event Hx, at least for event A4 + event H1/H2. FFS for other event Ax + event H1/H2. FFS on details, e.g. whether to include one height threshold (H1 or H2 threshold) or a height range (both H1 and H2 threshold) in the new event, how to configure height-dependent numberOfTriggeringCells, etc.    This will be applied to all height dependent MR parameters.

- Whether UE height is included when UAV specific MR is triggered is configurable by the network.

- We will use LTE UEheight.

Combination of Hx and Ax events

Combination of events

R2-2305302	Discussion on Measurement Reports Enhancements	NEC Europe Ltd	discussion	Rel-18	LTE_UAV_enh-Core

Proposal 1: To minimize the impact on the current measurement report configuration and triggering structure, consider the following options to combine height-dependent conditions with RSRP/RSRQ/SINR-based conditions:

-	Option 1: Link ONE report configuration to TWO measurement events.

-	Option 2: Link ONE measurement ID to TWO report configurations.

When event is considered fulfilled:

R2-2306053	Discussion on measurement reporting for NR UAV	Sharp	discussion

Proposal 2: UE initiates measurement reporting procedure when entering condition of Ax event is fulfilled during Ax-timeToTrigger and entering condition of Hx event is fulfilled during Hx-timeToTrigger.

R2-2305143	On Height-dependent Measurement Report Configuration for UAVs	Nokia, Nokia Shanghai Bell	discussion	Rel-18	NR_UAV-Core

R2-2305144	On Interference Reporting for UAVs	Nokia, Nokia Shanghai Bell	discussion	Rel-18	NR_UAV-Core

R2-2305429	Discussion on measurement reporting enhancement for NR UAV	vivo	discussion	Rel-18	NR_UAV-Core

R2-2306171	Measurement reporting enhancement in UAV	Apple	discussion	Rel-18	NR_UAV

R2-2306288	Measurement Report Enhancement	LG Electronics	discussion	Rel-18

R2-2306337	Measurement reporting enhancements for NR UAV	China Telecom Corporation Ltd.	discussion	Revised

R2-2306458	Further discussion on NR support for UAV	NTT DOCOMO, INC.	discussion

R2-2306490	Measurement reporting enhancement in NR UAV	ZTE Corporation, Sanechips	discussion	Rel-18	NR_UAV-Core

R2-2306529	Measurement reporting for mobility and interference control	China Telecom	discussion	R2-2306337

### 7.8.3	Flight path reporting

Contributions on enhancements to flight path reporting

Flightpath update notification

R2-2305887	Further Details on Flight Path Plan (FPP)	Nokia, Nokia Shanghai Bell	discussion	Rel-18	NR_UAV-Core

Proposal 5: Study triggering of flightPathInfoAvailable in the UEAssistanceInformation based on time-based event trigger and distance-based event trigger. Consider the following options:

a)	inform the network of a new FPP periodically (configurable periodicity).

b)	inform the network of a new FPP if the timestamps for the included waypoints have changed more than a configurable threshold

c)	inform the network of a new FPP if it deviates more than a configurable distance threshold from the currently available FPP

d) 	number of way points

=>	Noted

R2-2305544	UAV Flight Path Reporting	Ericsson España S.A.	discussion	Rel-18

Proposal 1	Network configures a waypoint threshold with the understanding that a flight path update is triggered if more than the configured threshold of (e.g., number of) waypoints changes.

=>	Noted

R2-2306492	On flight path reporting	ZTE Corporation, Sanechips	discussion	Rel-18	NR_UAV-Core

Proposal 1: To introduce prohibit timer instead of threshold in distance/time/number of waypoints to control triggering the flightpath update indication in UAI.

=>	Noted

R2-2306289	Flight Path Information Report	LG Electronics	discussion	Rel-18

Proposal 1. Do not specify triggering conditions for updated flightpath available indication, i.e., it is up to UE implementation to trigger.

=>	Noted

Discussions

-	Interdigital is ok with b) and c) but doesn’t see a point for periodic the network can just configure. Qualcomm thinks it can be left to UE implementation but if we specify something a) is not need.

-	Samsung supports network configurable triggers but not sure about a) and b).  b) may not be useful if for example the waypoints are not located within the coverage of the current gNB.

-	LG doesn’t agree with triggering conditions, and it is up to UE implementation and a prohibit timer can work.

-	Huawei doesn’t think prohibit timer as the network is preparing the handover along the path, but the network doesn’t care if it is in the seconds range, and more than 10minutes for example, so it has to be a meaningful change.   The number of way points can be configurable even though 1 is sufficient.

-	ZTE also thinks it can be up to UE implementation.   CCMC thinks that initial flight report can be up to implementation but the followup update should be network controlled.

-	Nokia reminds everyone we agreed it has to be network controlled.  We can eliminate a) and we can eliminated prohibit timer for similar reasons.

-	Qualcomm thinks prohibit timer is meaningless if the network has control over the UE reporting the update.

Delta flightpath reporting

R2-2305109	Delta reporting of flight path plan	Qualcomm Incorporated	discussion	Rel-18	NR_UAV-Core

Proposal 1: Signalling of the changes in flight path information by the UE (‘delta’ signalling) is supported.

Proposal 2: Add a Way Point ID to identify a particular waypoint in the UE-reported flight path.

Proposal 3: Use xxToAddModList and xxToRemoveList to enable signalling of the changes in flight path.

-	Huawei indicates that this is not used currently in UL so there is a change when compared to legacy

Case 1.	Some outdated waypoints are removed as the UAV UE has already travelled through them.

Case 2.	Some waypoints’ timestamp is updated but location remains the same.

Case 3.	Some waypoints’ location (and possibly timestamp if it was indicated earlier) is updated due to change in planned path.

Case 4.	Some new waypoint(s) is/are inserted due to change in planned path.

=>	Noted

R2-2306216	Discussion on flight path reporting	Huawei, HiSilicon	discussion	Rel-18	NR_UAV-Core

Proposal 4: The delta report should be supported for flight path updates, especially partial flight path updates.

Proposal 5: Bitmap should be considered for the delta flight path report.

Proposal 6: For the delta flight path report, the flight path available indication does not need to be different if the bit map mechanism is supported.

=>	Noted

R2-2305939	Flightpath reporting for UAV	InterDigital	discussion	Rel-18	NR_UAV-Core

Proposal 1: Delta flightpath reporting is supported. As a baseline, UAV can report timestamp information only (i.e., a sequence of timestamps with size 1..maxWayPoint).

Proposal 2: FFS if delta reporting also supports updating individual waypoints/timestamps.

Proposal 3: NW decides whether UE reports full or delta flight path. FFS if delta flight path reporting is enabled by configuration or explicit request.

=>	Noted

R2-2305887	Further Details on Flight Path Plan (FPP)	Nokia, Nokia Shanghai Bell	discussion	Rel-18	NR_UAV-Core

Proposal 6: RAN2 does not pursue delta signaling for flight path reporting.

Proposal 7:  Confirm that a single indication is used for both initial and updated flightpath plan.

=>	Noted

Discussion on whether we support delta signaling

-	Nokia, LG, ZTE, and Samsung still is not convinced this is useful.   Delta signaling is not supported currently so it is introducing a new feature.

-	Interdigital thinks that the delta configuration is not as scary as it sounds and there is a way to do it in a simple way (i.e. just time information) and you do have up 63% savings.

-	Qualcomm thinks that it is not really delta it is just telling the network the changed timestamps.

-	Huawei brings up an example, what if the UE has already done half of the path.  So when it sends me somethings the network needs to understand where the UE is going next if it receives the whole path again.

Agreements

- The network can configure the UE to trigger a flightpath update notification based on a configured delta time (when timestamp is configured to be reported) or distance configuration.

Not treated

R2-2305304	Discussion on Flight Path Reporting	NEC Europe Ltd	discussion	Rel-18	LTE_UAV_enh-Core	R2-2303105

R2-2305430	discussion on Flight path reporting	vivo	discussion	Rel-18	NR_UAV-Core

R2-2305601	Discussion on Flight path Reporting	CMCC	discussion	Rel-18	NR_UAV-Core

R2-2305692	Remaining issues of flight path reporting for NR UAV	Lenovo	discussion	Rel-18

R2-2305938	Flightpath update notification for UAV	InterDigital	discussion	Rel-18	NR_UAV-Core

R2-2306054	Discussion on flight path reporting for NR UAV	Sharp	discussion

R2-2306124	Discussion on triggering of flight path report	ASUSTeK	discussion	Rel-18	NR_UAV-Core

R2-2306136	Discussion on flight path reporting for NR UAV	Xiaomi	discussion	Rel-18	NR_UAV-Core

R2-2306170	Flight path reporting in UAV	Apple	discussion	Rel-18	NR_UAV

R2-2306236	Leftover Issue on Flight Path Reporting	CATT	discussion	Rel-18	NR_UAV-Core

R2-2306241	Consideration on flight path reporting for NR UAV	DENSO CORPORATION	discussion	NR_UAV-Core

R2-2306338	Flight path reporting enhancements for NR UAV	China Telecom Corporation Ltd.	discussion

R2-2306449	Discussion on flight path reporting	Samsung	discussion	Rel-18	NR_UAV-Core

### 7.8.4	Subscription-based aerial-UE identification

Contributions should focus on signaling required to support subscription-based aerial-UE identification

Note: Work done in LTE is a starting point for this objective. NR-specific enhancements can be considered, if needed, while overall the LTE and NR solutions should be harmonized as much as possible.

R2-2305545	Subscription-Based Aerial UEs Identification	Ericsson España S.A	discussion	Rel-18	NR_UAV-Core	R2-2302906

Proposal 3: RAN2 clarify the use of AUE subscription information and capture it in TS 38.300. Clause 23.17.2 in TS 36.300 V17.2.0 is used as a starting point.

Proposal 4: RAN2 discuss which are the key features to be specified as conditionally mandatory to the Aerial UE subscription towards end of Release 18.

-	Qualcomm thinks that we need more information than UAV or not like in LTE.

-	Xiaomi thinks that we should first discuss and agree with the baseline.

-	Samsung thinks that the LTE baseline enough.

-	Nokia explains that RAN3 has agreed and this has been captured in the stage 2 CR.   No need to define different UAV types.

=>	Noted

Agreement:

- The subscription-based aerial-UE identification adopted in LTE can be taken as the baseline for NR UAV (i.e. the RAN3 endorsed CR will be captured in stage 2) .  No further NR specific enhancements will be pursued.

R2-2305431	discussion on Subscription-based aerial-UE identification	vivo	discussion	Rel-18	NR_UAV-Core

Proposal 1: The subscription-based aerial-UE identification adopted in LTE can be taken as the baseline for NR UAV.

R2-2305602	Subscription-based aerial-UE identification for NR UAV	CMCC	discussion	Rel-18	NR_UAV-Core

R2-2306030	Subscription-based Aerial-UE Identification in NR	Qualcomm Incorporated	discussion	Rel-18	NR_UAV-Core

Proposal 1.	From RAN2 point of view, it is beneficial for RAN to have the information about different UAV types, UAV subscription types, and/or UAV mission types.

Proposal 2.	Inform RAN3 and SA2 of the RAN2 agreement.

R2-2306048	Discussion on subscription-based aerial-UE identification for NR UAV	Samsung Electronics Austria	discussion	Rel-18	NR_UAV-Core

R2-2306217	Consideration on subscription-based UAV identification	Huawei, HiSilicon	discussion	Rel-18	NR_UAV-Core

R2-2306424	UAV Subscription and Identification	Beijing Xiaomi Mobile Software	discussion	Rel-18	NR_UAV-Core

### 7.8.5	UAV identification broadcast

UAV identification broadcast using PC5-U will be treated with higher priority.  Contributions analysing the gap for supporting DAA using the same framework as BRID can be submitted.

Separate SL resource pool for UAV

R2-2305110	Remaining aspects of PC5-based BRID and DAA support	Qualcomm Incorporated	discussion	Rel-18	NR_UAV-Core, LTE_UAV_enh-Core

Proposal 2: Separate SL resource pool for BRID and DAA broadcast is supported.

=>	Noted

R2-2306218	Discussion on UAV remote identification broadcast	Huawei, HiSilicon	discussion	Rel-18	NR_UAV-Core

Proposal 1: No separate resource pool is needed for A2X communication.

=>	Noted

R2-2305693	Discussion on broadcasting remote id for UAV	Lenovo	discussion	Rel-18

Proposal 1: Wait for SA2 reply to further discuss whether UAV specific resource pool is needed to support regional regulation and to fulfil U2X QoS requirements.

=>	Noted

Discussion

-	Vivo explains that the network can configure multiple resource pools and this can be considered furthered by SA2.

-	Qualcomm explains that the reason is to enable the receiver to know whether the message is DAA or something else.

-	Nokia has a similar understanding as Vivo and the network can make the decision that one is dedicated.   Qualcomm explains that there is no service to resource pool mapping.

-	Ericsson agrees that we need a separate resource pool and with mode 2 we need to ensure that these regulatory messages are not pre-empted.

-	Xiaomi thinks that different frequencies will resolve this issue.  Qualcomm explains that it doesn’t mean that there will be a dedicated frequency, each country will have their own regulation.

-	Ericsson clarifies that it is not QoS but it is also about ensuring that the UE receives the messages without being pre-empted.

=>	CB on Thursday pending SA2 discussion, but limited support for now

R2-2305306	Considerations on Enhancements for UAV identification broadcast	NEC Europe Ltd	discussion	Rel-18	LTE_UAV_enh-Core

R2-2305432	discussion on UAV identification broadcast	vivo	discussion	Rel-18	NR_UAV-Core	Withdrawn

R2-2305546	UAV Broadcast Identification	Ericsson España S.A.	discussion	Rel-18

R2-2305603	UAV identification broadcast	CMCC	discussion	Rel-18	NR_UAV-Core

R2-2305742	Resource configuration for UAV ID broadcast	Samsung	discussion	Rel-18	NR_UAV-Core

R2-2305888	On How To Ensure QoS for PC5-based BRID and DAA	Nokia, Nokia Shanghai Bell	discussion	Rel-18	NR_UAV-Core

R2-2306425	NR UAV BRID broadcast over PC5	Beijing Xiaomi Mobile Software	discussion	Rel-18	NR_UAV-Core	Late

R2-2306493	On UAV identification broadcast	ZTE Corporation, Sanechips	discussion	Rel-18	NR_UAV-Core

## 7.18	Mobile Terminated Small Data Transmission

(NR_NR_MT_SDT-Core; leading WG: RAN2; REL-18; WID: RP-222993)

Time budget: 0.5 TU

Tdoc Limitation: 2 tdoc

### 7.18.1	Organizational

Running CRs expected as input in this meeting: 38.300 (Nokia), 38.331 (ZTE), 38.321 (Huawei).

UE capabilities and running CR to 38.306 (Intel) will not be expected or discussed in this meeting

R2-2304795	Draft running CR for MAC spec for MT-SDT	Huawei, HiSilicon	draftCR	Rel-18	38.321	17.4.0	NR_MT_SDT-Core

=>	update with agreements and continue discussing over email discussion

R2-2305022	Introduction of MT-SDT (RRC Running CR)	ZTE Corporation (rapporteur)	draftCR	Rel-18	38.331	17.4.0	B	NR_MT_SDT-Core

=>	update with agreements and continue discussing over email discussion

R2-2305750	Introduction of MT-SDT in Stage-2	Nokia, Nokia Shanghai Bell	draftCR	Rel-18	38.300	17.4.0	NR_MT_SDT-Core

=>	update with agreements and continue discussing over email discussion

### 7.18.2	Control plane aspects

Paging message indication

R2-2304725	Control plane aspects of MT SDT Procedure in RRC_INACTIVE state	Samsung Electronics Co., Ltd	discussion	Rel-18	NR_MT_SDT-Core

Proposal 1: RAN2 to discuss and agree on one of the following signaling options for MT-SDT indication in paging message:

Option1:	1 bit MT-SDT indication is optionally included per paging record. This bit is added by extending legacy paging record;

Option 2	new list of paging records for MT-SDT indication is optionally included in paging message using non critical extension. Each record in this list optionally includes 1 bit MT-SDT indication. UE identity and access type are not included in paging record of this list.

Option 3	new list of paging records is optionally included in paging message using non critical extension. UE identity, access type and paging cause is included in paging record of this list. Paging record for UE’s with MT-SDT are included in this new list. Paging record for UE’s without MT-SDT are included in the legacy list.

Proposal 2: gNB may include MT-SDT indication in paging message only if UE’s I-RNTI is included in the paging message (i.e. MT-SDT is only used by RAN initiated paging).

=>	Noted

Discussion

-	Samsung thinks that the second option is the best from a complexity point of view.  Intel explains that when we use the … list it will add extra 24 bits.  Intel would prefer the option 2 in the minutes here.  ZTE implemented option 1 as it is the simplest and maybe we need a length indicator.   Huawei also prefers option 2 to avoid adding extra 24bits.  Vivo thinks we should adopt same solution as MU-SIM, option 2.

SIB configuration

R2-2305806	Control plane aspects of MT-SDT	Huawei, HiSilicon	discussion	NR_MT_SDT-Core

Proposal 3: The conditions checked by the RRC layer to determine whether to initiate MT-SDT procedure include:

•	Paging with mt-SDT indication is received;

•	SIB1 includes mt-SDT-ConfigCommon, mt-SDT-ConfigCommon may include at least sdt-RSRP-Threshold and t319a;

•	sdt-Config is configured;

•	Lower layers indicate that conditions for initiating MT-SDT are fulfilled.

=>	Noted

Discussion on whether we need separate SIB config for MT-SDT

-	Intel thinks that we should have a separate one and we should confirm that from NW point of view to allow only access for MT-SDT.   What remains to be discussed is what happens if both are configured

-	ZTE agrees with the intention but was wondering if the option would be to not configure the RA-SDT resources and if you don’t have it the cell then you don’t MO-SDT.  Intel thinks that works but not for CG-SDT.   Huawei clarifies that this is also linked to another discussion with threshold.

-	Nokia thinks that we need to have separate SIB MT-SDT configuration

Access identifies

R2-2304935	Discussion on subsequent transmission within MT-SDT	SHARP Corporation	discussion	NR_MT_SDT-Core

Proposal 1: UE selects '0' as the Access Category when the resumption of the RRC connection is triggered by response to the MT-SDT triggering in a PAGING message.

=>	Noted

R2-2305021	MT-SDT Control plane open isssues	ZTE Corporation, Sanechips	discussion

Proposal 4: MT-SDT is only applicable to the legacy MT-Access use case (i.e. it is not applicable to access identities 1, 2 and 11-15)

Proposal 3: Inform CT1 that from Rel-18 MT-SDT allows DL NAS messages to be received in INACTIVE state after paging for MT-SDT

=>	Noted

Discussion

-	Intel thinks that the intention is to use same AC as legacy.

-	ZTE clarifies that they are talking about access identifies in proposal 4.

-	Vodafone is not sure about access identity 10 and would like to double.

-	Intel and Huawei that think CT1 can do it by themselves and not sure if the note would have to be updated anyways.

R2-2304706	Discussion on Supporting MT-SDT from CP Perspective	vivo Mobile Com. (Chongqing)	discussion	Rel-18	NR_MT_SDT-Core

Proposal 1: A separate paging record list is introduced to carry per UE MT-SDT indication.

Proposal 2: The first entry of separate paging record list is mapped to the first entry of PagingRecordList, and so on.

Proposal 3: No new paging search space nor new P-RNTI are defined for MT-SDT procedure.

Proposal 4: No additional enhancement is needed specifically for RedCap UE to monitor paging for MT-SDT.

=>	Noted

Signaling details

R2-2305352	Further MT-SDT discussion	Ericsson	discussion	Rel-18	NR_MT_SDT-Core

Proposal 1	Common CG-SDT resources can be configured in SI.

=>	no support

Proposal 	2	The MT-SDT paging message indicates if a UE can use the common CG-SDT resources

=>	 no support

Proposal 4	In case the condition for paging triggered MT-SDT is not fulfilled, the UE initiates RRC Resume procedure with Resume cause “mt-Access”.

-	Intel, CATT and ZTE agrees

Proposal 5	The RSRP threshold for MT-SDT is configured separately from the sdt-RSRP-Threshold in Rel-17.

-	Intel doesn’t agree as from the UE side there is no reason to have two different values and we need to guarantee that we can do both UL and DL.  LG has same concerns

-	Nokia and Huawei support the proposal

-	Huawei thinks that we need to have separate IE to ensure that legacy UEs don’t think that MO SDT don’t think it is configure.  ZTE explains that this depends on how we configure

Agreements:

- Allow support of only MT-SDT in a cell.  A separate SIB configuration will be introduced.  FFS what is put in there.

- For paging indication signalling, a new list of paging records for MT-SDT indication is optionally included in paging message using non critical extension. Each record in this list optionally includes 1 bit MT-SDT indication. UE identity and access type are not included in paging record of this list.

- gNB may include MT-SDT indication in paging message only if UE’s I-RNTI is included in the paging message (i.e. MT-SDT is only used by RAN initiated paging).

- UE selects '0' as the Access Category when the resumption of the RRC connection is triggered by response to the MT-SDT triggering in a PAGING message

- MT-SDT is only applicable to the legacy MT-Access use case (i.e. it is not applicable to access identities 1, 2 and 11-15).

- SRB2 can be used for MT-SDT (i.e. similar to MO-SDT)

- No additional enhancement is needed specifically for RedCap UE to monitor paging for MT-SDT

- When RRC resume is triggered due to MT-SDT and in the case the condition for paging triggered MT-SDT is not fulfilled, the UE initiates RRC Resume procedure with Resume cause “mt-Access”.

- A separate sdt-RSRP threshold for MT-SDT can be configured, at least in the case where MO-SDT is not configured in the cell.

R2-2305527	Remaining procedures for MT-SDT	Sony	discussion	Rel-18	NR_MT_SDT-Core

Proposal 3: For beam failure recovery of DL/UL SDT in Rel-18, a UE should trigger RACH procedure if the RSRP of the current beam is below a certain pre-configured threshold.

-	Intel thinks that the topic is not part of the SDT WI and if there is something broken it should be Rel-17 CR or TEI18 if there is a new enhancement.

-	Nokia thinks that we already support this CG SDT in Rel-17 and it would be good for RA SDT.    Samsung has the same view as Nokia and we missed it in Rel-17 and are ok to support in R18.

-	ZTE asks if it will delay cell reselection if the beam is bad should UE select to a better cell or continue looking for a better beam in the same cell.    Nokia explains that this is not really related to cell reselection and it will not delay it.

-	LG and Qualcomm asks if there would be impact to another group in RAN1 as we haven’t discuss whether we have the beam failure procedure for SDT.   Nokia explains that this is already for CG SDT and there would be no RAN1 impacts.

=>	Noted

R2-2305299	Discussion on control plane issues for MT-SDT	OPPO	discussion	Rel-18	NR_MT_SDT-Core

R2-2305491	MT SDT mechanism (including configuration, paging, resume and capabilities)	Intel Corporation	discussion	Rel-18	NR_MT_SDT-Core

R2-2305583	Discussion on the configuration of MT-SDT	Xiaomi	discussion	Rel-18	NR_MT_SDT-Core

R2-2305735	Discussion on remaining CP issues for MT-SDT	Lenovo	discussion	Rel-18	NR_MT_SDT-Core

R2-2305791	Control plane aspects of MT-SDT	Qualcomm Incorporated	discussion	NR_MT_SDT-Core

R2-2305806	Control plane aspects of MT-SDT	Huawei, HiSilicon	discussion	NR_MT_SDT-Core

R2-2305906	CP aspects for MT-SDT procedure	Nokia, Nokia Shanghai Bell	discussion	Rel-18	NR_MT_SDT-Core

R2-2306128	Discussion on DL SPS for MT-SDT	ASUSTeK	discussion	Rel-18	NR_MT_SDT-Core

R2-2306141	Support of SPS in MT-SDT	LG Electronics Inc.	discussion	Rel-18	NR_MT_SDT-Core

R2-2306160	Discussion on MT-SDT	Apple	discussion	Rel-18	DUMMY

R2-2306341	Consideration on CP common aspects of MT-SDT	China Telecom Corporation Ltd.	discussion

R2-2306399	Consideration on CP aspects for MT-SDT	CATT	discussion	Rel-18	NR_MT_SDT-Core

### 7.18.3	User plane aspects

Initial RACH and CCCH message over CG-SDT

R2-2305751	MT-SDT UP impacts	Nokia, Nokia Shanghai Bell	discussion	Rel-18	NR_MT_SDT-Core

Proposal 1: RRC indicates to MAC whether MT-SDT or MO-SDT conditions need to be evaluated.

-	Intel thinks that we should discuss this based on running CRs.   LG thinks we can discuss this after we agree to thinks in MAC.   LG is not sure whether there is a difference from the MAC perspective.   Sony thinks that if the UE has UL data we need to check data volume.   Intel explains that we agreed that we have two independent procedures.

Observation 1: CCCH SDU can always be multiplexed into CG-SDT resource.

Proposal 2: For MT-SDT, LCH restrictions are not checked as CG-SDT condition.

-	ZTE thinks that we need to allow the CCCH to go through.  Intel agrees.  Huawei thinks that this is related to the case where there is data.   Intel doesn’t see any concerns to allowing the UE to used the grant.   LG has a different understanding from Intel.  If there is UL data it should be restricted like legacy procedure.   LG clarifies that LCH restriction should only be applied for DRB.   Intel thinks that we don’t check for UL data.  Sony disagrees, we should check for UL data.  LG explains that in case there is UL data we need to check.   Huawei thinks that we agreed to not check data volume in RRC.

Proposal 3: RA-SDT resources are not used for MT-SDT procedure.

-	Huawei thinks that the case where UL data becomes available can happen and we should ensure we can transmit UL data.   Ericsson agrees to proposal 3.

-	LG wonders why we are restricting use of RA-SDT if it is configured.    Huawei thinks that they can be used when there is UL data.  LG thinks that this makes the specification more complicated.   ZTE explains that we need to keep the MO-SDT and MT-SDT separate.  Intel agrees and also RAN3 is working on separate.

-	Qualcomm thinks that we didn’t discuss the RA-SDT resource for MT-SDT.

-	Sony is concerned about what happens when UL data is triggered.  Samsung explains that if the UE has UL data the UE can still check and trigger MO-SDT anyways and still check data threshold etc.

Proposal 4: In case CG-SDT resources cannot be used or are not available for MT-SDT, UE uses common RACH for RA-based MT-SDT.

=>	Noted

R2-2305805	User plane aspects of MT-SDT	Huawei, HiSilicon	discussion	NR_MT_SDT-Core

Proposal 4: A UE responds to paging with MT-SDT using R17 RA-SDT resource with the resume cause value set to mt-SDT when the UE has UL SDT data to send.

=>	Noted

SPS

R2-2305807	SPS support for MT-SDT	Huawei, HiSilicon, Xiaomi, vivo, LGE, CMCC	discussion	NR_MT_SDT-Core

Proposal 1: Allow DL SPS to be configured for SDT session.

Proposal 2: DL SPS for the next MT-SDT procedure can be configured in RRCRelease.

Proposal 3: Legacy SPS configuration can be reused for MT-SDT SPS. FFS whether to use PUCCH resource from SPS-Config or the one provided in SIB1 for HARQ feedback of SPS transmissions.

Proposal 4: Pre-configured SPS resources in RRCRelease are activated after RRCResumeRequest is sent by the UE in response to MT-SDT paging.

=>	Noted

R2-2306527	On support of DL SPS	MediaTek Inc.	discussion	Rel-18

Proposal 1	DL SPS is not pursued in Rel-18 MT-SDT. Postpone its support to Rel-19.

=>	Noted

Discussion

-	Mediatek, Ericsson, Sony, China Telecom,   and Intel think that this is not a very

=>	SPS will not be supported for Rel-18 SDT

CG SDT

R2-2305953	MT SDT mechanism (including CG and ROHC)	Intel Corporation	discussion	Rel-18	NR_MT_SDT-Core

Proposal 1.	There is no need to define new Rel-18 CG configurations specific to MT-SDT.

Proposal 3.	When resuming for MT-SDT, gNB informs whether UE whether the PDCP entity of the radio bearers configured for MT-SDT continues or resets the ROHC header compression protocol during PDCP re-establishment during SDT procedure.

-	ZTE thinks because we agreed to resume all bearers we would have a common configuration.

=>	Noted

Agreements:

- RRC explicitly indicates to MAC whether resume is trigged due to MT-SDT

- LCH restrictions are checked for DRBs as in MO-SDT (if UL data is available during SDT procedure).  Ensure CCCH can be transmitted in CG-SDT when MT-SDT is triggered in stage 3 discussion.

- Assumption is that if the UE has UL data the UE can still check and trigger MO-SDT (it is up to UE implementation)

- RA-SDT resources are not used for MT-SDT initiation RACH

- In case CG-SDT resources cannot be used or are not available for MT-SDT, UE uses non-SDT RACH for RA-based MT-SDT.  FFS whether new triggers are defined

- There is no need to define new Rel-18 CG configurations specific to MT-SDT.

- When the UE is configured with both MO and MT SDT the radio bearer configuration is common for both.

- For both MO and MT-SDT, if the next CG-SDT resource is too far, then RACH resource can be selected first.   This is checked at the point of initial resource selection (e.g. CG SDT selection).   FFS what is too far and how this is configured.   Assumption is that we will continue this discussion in SDT session.  CONFIRM with main session [CB]

R2-2305557	Discussion on MT-SDT procedure	Spreadtrum Communications	discussion	Rel-18

Proposal 1: If the CG periodicity of valid CG-SDT resource is too long, the RACH resource can be selected first.

R2-2305595	Discussion on MT-SDT procedure	Continental Automotive	discussion	Rel-18

Proposal 1: It would be beneficial for the UE to know which UL resources it has to use to transmit the paging response when the CG periodicity is too long.

Proposal 2: The network provides an indication in the paging message whether the UE should use RA-SDT or CG-SDT to transmit the paging response.

R2-2304706	Discussion on Supporting MT-SDT from CP Perspective	vivo Mobile Com. (Chongqing)

CG resources

Proposal 6: Only the initial BWP (including the separate initial BWP for RedCap) is used for MT-SDT procedure.

Proposal 7: Separate CG resources specific to MT-SDT is supported.

Proposal 8: Separate CG resources specific to MT-SDT can be configured on SUL carrier and NUL carrier.

R2-2306379	Handling BWP restrictions in MT-SDT	Ericsson	discussion	Rel-18	NR_MT_SDT-Core

Proposal 1	Choose from among the following options to handle BWP restriction in MT-SDT

Option 1: Remove CORESET 0 restriction for DL BWP for UEs from/after first DL MT-SDT data

Option 2: Introduce Channel Quality Indication (CQI) in Message 3 or Msg A

Option 3: Optimisations to reduce subsequent transmission overhead

Proposal 2	Introduce a DCQR MAC CE which can be multiplexed into Msg3, MsgA or the initial CG-SDT transmission in the MT-SDT procedure

Proposal 3	Send an LS to RAN1 to ask about the feasibility to introduce CQI reporting in Msg3 for MT-SDT.

Proposal 4	Discuss whether to introduce multi-TB support to reduce the number DCIs for scheduling multiple PDSCH transmissions as part of subsequent DL transmissions.

Proposal 5	For subsequent DL transmissions to transmit the new incoming data during an ongoing MT-SDT procedure, the gNB can schedule the PDSCH transmissions as in MO-SDT.

Proposal 6	For subsequent UL transmissions to transmit the new incoming data during an ongoing MT-SDT procedure, follow the dynamic scheduling based on the contents of BSR as in MO-SDT.

R2-2305805	User plane aspects of MT-SDT	Huawei, HiSilicon	discussion	NR_MT_SDT-Core

Proposal 1: MAC layer should know whether MO-SDT or MT-SDT conditions should be checked when the resume is triggered by RRC layer.

Proposal 3: When responding to MT-SDT Paging, the UE initiates the MT-SDT procedure using legacy RACH resource with the resume cause value set to mt-SDT when the UE has no UL data to send or there are no SDT RACH resources available.

Proposal 4: A UE responds to paging with MT-SDT using R17 RA-SDT resource with the resume cause value set to mt-SDT when the UE has UL SDT data to send.

R2-2306142	Discussion on MT-SDT procedure	LG Electronics Inc.	discussion	Rel-18	NR_MT_SDT-Core

Proposal 2. When the paging message for MT-SDT is received but MT-SDT criteria is not satisfied, RRCResume cause is set to mt-Access.

Proposal 3. UE transmits a positive HARQ feedback to the network upon receiving the paging message for MT-SDT.

Proposal 4. CG-SDT-TAT is restarted upon receiving the paging message for MT-SDT.

R2-2304707	Discussion on Supporting MT-SDT from UP Perspective	vivo Mobile Com. (Chongqing)	discussion	Rel-18	NR_MT_SDT-Core

R2-2304726	User plane aspects of MT SDT Procedure in RRC_INACTIVE state	Samsung Electronics Co., Ltd	discussion	Rel-18	NR_MT_SDT-Core

R2-2305023	MT-SDT user plane open isssues	ZTE Corporation, Sanechips	discussion

R2-2305300	Discussion on user plane issues for MT-SDT	OPPO	discussion	Rel-18	NR_MT_SDT-Core

R2-2305353	Handling BWP restrictions in MT-SDT	Ericsson	discussion	Rel-18	NR_MT_SDT-Core	Withdrawn

R2-2305736	Discussion on remaining UP issues for MT-SDT	Lenovo	discussion	Rel-18	NR_MT_SDT-Core

R2-2305751	MT-SDT UP impacts	Nokia, Nokia Shanghai Bell	discussion	Rel-18	NR_MT_SDT-Core

R2-2305793	User plane aspects of MT-SDT	Qualcomm Incorporated	discussion	NR_MT_SDT-Core

R2-2306342	Consideration on UP common aspects of MT-SDT	China Telecom Corporation Ltd.	discussion

R2-2306400	Consideration on UP aspects for MT-SDT	CATT	discussion	Rel-18	NR_MT_SDT-Core

## 7.23	Timing Resiliency and URLLC Enh

(NR_TRS_URLLC; leading WG: RAN3; REL-18; WID: RP-230754)

Time budget: 0.5 TU

Tdoc Limitation: 2 tdoc

### 7.23.1	Organizational

Incoming LSs, Rapporteur input etc.

Expected inputs to next meeting, running CRs for the following: 38.300 [Nokia], 38.331 [Ericsson],

R2-2304605	Response to Reply LS on Proposed method for Time Synchronization status reporting to UE(s) (C1-232942; contact: Nokia)	CT1	LS in	Rel-18	TRS_URLLC	To:RAN2, SA1	Cc:SA2, RAN3

=>	Noted

R2-2304621	Reply LS on proposed method for time synchronization status reporting to UE(s) (R3-230811; contact: Nokia)	RAN3	LS in	Rel-18	FS_5TRS_URLLC	To:SA2, RAN2

=>	Noted

R2-2305655	Stage 2 running CR on timing resiliency and URLLC	Nokia, Nokia Shanghai Bell	discussion	Rel-18	TRS_URLLC-NR-Core

=>	The CR will be updated after the meeting and can be reviewed over email discussion

### 7.23.2	General

R2-2305656	5GS network timing synchronization status and reporting	Nokia, Nokia Shanghai Bell	discussion	Rel-18	TRS_URLLC-NR-Core

Proposal 1: update of event ID is informed to UE by normal SI modification procedure.

=>	Noted

R2-2305129	Clock Quality Report Delivery	Qualcomm Incorporated	discussion	Rel-18

Proposal 1: Clock quality updates in SIB9 can be indicated to IDLE/INACTIVE UEs subscribed to TaaS using Short Messages.

=>	Noted

R2-2304842	Discussion on the update of event ID	Huawei, HiSilicon	discussion	Rel-18	TRS_URLLC-NR-Core

Proposal 1: The change of Event ID field in SIB9 should be excluded when determining changes in system information.

=>	Noted

Discussion

-	Nokia asks if the assumption that the UE always has to read SIB9.   Huawei thinks that the UE has to read the time anyways before.  Nokia explains that the UE only reads it once.  Apple thinks that it is not preferable that the UE continuously does it.

-	Samsung thinks that the change of event ID happens rarely so we can just follow existing SI modification procedure.  Ericsson asks if we know the frequency of the expected change.   Samsung explains that the major scenario doesn’t require the gNB to change the clock very frequently.

-	Apple thinks that we should use short message such that we don’t wake up the UE.   ZTE also doesn’t prefer to woke up the all the UEs.

-	Vivo thinks that the short message would be useful.

-	CATT agrees that we don’t need to do any specific optimization and anyways we have many other SIBs that require paging all UEs but they are not relevant to all UEs.    Ericsson would like to avoid using the extra short message bits.   Huawei also agrees.

=>	Noted

R2-2305656	5GS network timing synchronization status and reporting	Nokia, Nokia Shanghai Bell	discussion	Rel-18	TRS_URLLC-NR-Core

Proposal 2: Confirm the AS layer of the UE determines if there a change of event ID and gNB ID. If there is a change, the AS layer notifies the change in the RAN timing synchronization status to NAS layer. For both IDLE and INACTIVE mode, NAS layer requests the RRC layer to move to RRC_CONNECTED.

-	Apple thinks that perhaps the UE doesn’t always have to notify the NAS as we have tools to synchronize.  Vivo thinks that we can discuss further how the AS determines the change.

-	ZTE thinks that we need to discuss the case for same gNB so there may need to consider additional information besides the event ID.   Samsung explains that the RAN3 understanding is that the event can be uniquely set and there would be no problem.  Nokia also explains that even the LS provides this information that only event ID can be support.

Proposal 3: For RRC_CONNECTED mode UEs, the NW has the necessary information to determine whether to send detailed clock quality information to the UE, (i.e. it may choose to always send update or only when needed) and the details can be left to NW implementation.

=>	Noted

R2-2304704	Further Discussion on 5G Clock Quality Information Reporting	vivo Mobile Com. (Chongqing)	discussion	Rel-18	TRS_URLLC-NR-Core

Proposal 2: RAN2 to further discuss timing synchronization change based on the understanding that the field of event ID can be present or not.

Proposal 3: Under the same gNB, UE considers the change of timing synchronization status by comparison of stored and obtained event ID field from SIB9 in the following ways:

-	empty vs. present

-	present vs. empty

-	different values

-	Huawei asks when would the network set event ID to empty.   Vivo thinks this is NW implementation.  Qualcomm thinks that the only one to consider is different values.   Nokia thinks that different values makes sense and is not sure about the empty case.   Vivo explains that there is the case where the network doesn’t support the feature.  Samsung agrees that the only scenario that exists is that the NW support, otherwise it should always be there.

-	CATT clarifies that according to SA2 the field is optional.  Nokia thinks that we need to agree that in RAN2 we need to always broadcast the event ID otherwise it doesn’t work.    Vivo would be fine if this is captured in the specification.   CATT would like to check a bit offline.  Ericsson thinks we should have the assumption that if we support the feature we should broadcast the information.  Qualcomm thinks that if the field is absence there is nothing to do.

=>	Noted

R2-2305967	Discussion on the issue of RACH congestion	ZTE Corporation, Sanechips	discussion	Rel-18	TRS_URLLC-NR-Core

Proposal 3: RAN2 discusses that in RRC_ INACTIVE state UE acquire clock quality information through SDT procedure.

-	ZTE thinks that the time quality information can be provided in INACTIVE if SDT is supported/configured.

-	Nokia asks if we would need a change to SDT procedure because we allow SRB2 in SDT.   ZTE thinks that we may need another trigger.  Samsung thinks that this doesn’t happen so often so no need to optimize RACH.   Apple thinks that it makes sense to always move the UE in Connected.  The newtwork can also trigger a DL information transfer to the UE in active.

-	Huawei asks whether we are discussing MO-SDT or MT-SDT.   Nokia understood that the NAS will trigger the MO-SDT like in legacy.   Huawei is concerned that we don’t have any UL data.   ZTE explains that NAS will put a message and this can trigger MO-SDT.  For MT-SDT then the network can just page, the UE will put the MT-SDT cause and it is up to the network to put a message in a DL.

=>	Noted

Agreements

- Update of event ID is informed to UE by normal SI modification procedure.

- Confirm the AS layer of the UE determines if there a change of event ID and/or gNB ID. If there is a change, the AS layer notifies the change in the RAN timing synchronization status to NAS layer. For both IDLE and INACTIVE mode, NAS layer may requests the RRC layer to move to RRC_CONNECTED

- For RRC_CONNECTED mode UEs, the NW has the necessary information to determine whether to send detailed clock quality information to the UE, (i.e. it may choose to always send update or only when needed) and the details can be left to NW implementation

- Event ID is optional.  Under the same gNB, UE considers the change of timing synchronization if event ID field from SIB9 is different.   FFS if gNB always broadcasts event ID

- If the UE is in RRC_INACTIVE, the UE can acquire clock quality information using the SDT procedure, if it supports and/is configured with SDT procedure.

BAT discussion

Agreements Copied from XR session minutes

1. 	UE reports to RAN the range of jitter in its UL traffic, defined in the similar way as the one for N6 jitter.

2: Reference time is defined in similar way as BAT (Burst Arrival Time) at UE side.

3	RRC UAI framework is updated for Rel-18 to support signalling UL assistance information agreed so far for XR (Jitter, burst arrival time, FFS on periodicity).

R2-2305657	Reactive RAN feedback for upstream scheduling	Nokia, Nokia Shanghai Bell	discussion	Rel-18	TRS_URLLC-NR-Core

Proposal 1: confirm it is feasible for gNB implementation to derive BAT offset based on available information.

Proposal 2: confirm no UE report is needed for objective 3 on BAT offset derivation and we can close the objective in RAN2.

=>	Noted

Agreements:

-	No new UE report is needed for objective 3 on BAT offset derivation.  The XR mechanism can be used if the network configures it, otherwise we can use legacy BSR.   We can close the objective in RAN2

Not treated

R2-2304705	Discussion on RAN feedback for Upstream Scheduling	vivo Mobile Com. (Chongqing)	discussion	Rel-18	TRS_URLLC-NR-Core

R2-2304841	Discussion on TSS change notification procedure	Huawei, HiSilicon	discussion	Rel-18	TRS_URLLC-NR-Core

R2-2304972	RAN2 Impact of 5GS network timing synchronization status and reporting	CATT	discussion	Rel-18	TRS_URLLC-NR-Core

R2-2304973	Discussion on RAN feedback	CATT	discussion	Rel-18	TRS_URLLC-NR-Core

R2-2305079	RAN feedback for burst sending time adjustment	Apple	discussion	Rel-18	DUMMY

R2-2305080	5GS Network Timing Synchronization in RRC_INACTIVE	Apple	discussion	Rel-18	DUMMY

R2-2305130	UL BAT Derivation at RAN	Qualcomm Incorporated	discussion	Rel-18

R2-2305627	Discussion on the network timing synchronization status monitoring	CMCC	discussion	Rel-18

R2-2305738	Signaling of 5G Clock Quality Information	Samsung	discussion	Rel-18

R2-2305739	Time Synchronization Status Update via EventID	Samsung	discussion	Rel-18

R2-2305966	Further discussion on time synchronization status and reporting	ZTE Corporation, Sanechips	discussion	Rel-18	TRS_URLLC-NR-Core

R2-2306343	Discussion on 5G network timing synchronization status and reporting	China Telecom Corporation Ltd.	discussion

R2-2306464	Burst Arrival Time (BAT) offset derivation	Ericsson	discussion	Rel-18	TRS_URLLC-NR-Core

R2-2306473	Discussion on NR timing resiliency	Ericsson	discussion	Rel-18	TRS_URLLC-NR-Core