3GPP TSG-RAN WG2 Meeting #122	R2-2306563

# Incheon, Republic of Korea, 22 – 26 May , 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.331 | CR | 4935 | rev | 2 | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on QoE configuration release |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Google, Qualcomm, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_QMC_Streaming-Core |  |  |  |  |  | Date: |  |  | 2023-05-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | When the UE enters RRC_IDLE there may be a stored application layer measurement configuration in the UE. However, acc. to current specification the UE behaviour on handling the stored application layer measurement configuration is unspecified. After discussion in RAN2#121bis-e and RAN2#122 the following UE behaviour has been agreed: After entering RRC_IDLE, the UE should inform upper layers about the application layer measurement configuration release and discard received application layer measurement reports from upper layers, if any. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specify that the UE performs following actions after it enters RRC_IDLE: 1> release the stored serviceType, if any; 1> inform upper layers to clear the stored application layer measurement configuration; 1> discard received application layer measurement report information from upper layers, if any; 1> consider itself not to be configured to send application layer measurement report; Impact analysis Impacted functionality Application layer measurement functionality Inter-operability: If the network is implemented according to this CR while the UE is not, there is no inter-operability issue. If the UE is implemented according to this CR while the network is not, there is no inter-operability issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE may not inform upper layers about the application layer measurement configuration release and may not discard the received application layer measurement reports. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.12 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

### 5.3.12	UE actions upon leaving RRC_CONNECTED or RRC_INACTIVE

Upon leaving RRC_CONNECTED or RRC_INACTIVE, the UE shall:

1>	reset MAC;

1>	if leaving RRC_INACTIVE was not triggered by the reception of RRCConnectionRelease including idleModeMobilityControlInfo or altFreqPriorities:

2>	stop the timer T320 and T323, if running;

2>	if stored, discard the cell reselection priority information provided by the idleModeMobilityControlInfo;

2>	if stored, discard the altFreqPriorities provided by the RRCConnectionRelease;

1>	if entering RRC_IDLE was triggered by reception of the RRCConnectionRelease message including a waitTime:

2>	start timer T302, with the timer value set according to the waitTime;

2>	inform the upper layer that access barring is applicable for all access categories except categories '0' and '2';

1>	else if T302 is running:

2>	stop timer T302;

2>	if the UE is connected to 5GC:

3>	perform the actions as specified in 5.3.16.4;

1>	if T309 is running:

2>	stop timer T309 for all access categories;

2>	perform the actions as specified in 5.3.16.4.

1>	stop all timers that are running except T302, T320, T322, T323, T325, T330, T331;

1>	release crs-ChEstMPDCCH-ConfigDedicated, if configured;

1>	if leaving RRC_CONNECTED was triggered by suspension of the RRC:

2>	re-establish RLC entities for all SRBs and DRBs, including RBs configured with NR PDCP;

2>	remove all entries within VarConditionalReconfiguration, if any;

2>	for each measId, that is part of the current UE configuration in VarMeasConfig, if the associated reportConfig has condReconfigurationTriggerEUTRA/condReconfigurationTriggerNR configured:

3>	remove the entry with the matching reportConfigId from the reportConfigList within the VarMeasConfig;

3>	if the associated measObjectId is only associated with condReconfigurationTriggerEUTRA/condReconfigurationTriggerNR:

4>	remove the entry with the matching measObjectId from the measObjectList within the VarMeasConfig;

3>	remove the entry with the matching measId from the measIdList within the VarMeasConfig;

2>	store the UE AS Context including the current RRC configuration, the current security context, the PDCP state including ROHC state, C-RNTI used in the source PCell, the cellIdentity and the physical cell identity of the source PCell, and the spCellConfigCommon within ReconfigurationWithSync of the PSCell (if configured);

2>	store the following information provided by E-UTRAN:

3> if the UE connected to 5GC is a BL UE or UE in CE:

4>	the fullI-RNTI, if present;

4>	the shortI-RNTI, if present;

3>	else:

4>	the resumeIdentity;

3>	the nextHopChainingCount, if present. Otherwise discard any stored nextHopChainingCount that does not correspond to stored key KRRCint;

3>	the drb-ContinueROHC, if present. Otherwise discard any stored drb-ContinueROHC;

2>	suspend all SRB(s) and DRB(s), including RBs configured with NR PDCP, except SRB0;

2>	if the UE connected to 5GC is a BL UE or UE in CE, indicate PDCP suspend to lower layers of all DRBs;

2>	if the UE is connected to 5GC:

3>	indicate the idle suspension of the RRC connection to upper layers;

2>	else:

3>	indicate the suspension of the RRC connection to upper layers;

2>	configure lower layers to suspend integrity protection and ciphering;

NOTE 1:	Except when resuming an RRC connection after early security reactivation in accordance with conditions in 5.3.3.18, ciphering is not applied for the subsequent RRCConnectionResume message used to resume the connection and an integrity check is performed by lower layers, but merely upon request from RRC.

1>	else:

2>	upon leaving RRC_INACTIVE:

3>	discard the UE Inactive AS context;

3>	discard the KeNB, the KRRCenc key, the KRRCint and the KUPenc key;

2>	release rrc-InactiveConfig, if configured;

2>	remove all entries within VarConditionalReconfiguration, if any;

2>	for each measId, that is part of the current UE configuration in VarMeasConfig, if the associated reportConfig has condReconfigurationTriggerEUTRA/condReconfigurationTriggerNR configured:

3>	remove the entry with the matching reportConfigId from the reportConfigList within the VarMeasConfig;

3>	if the associated measObjectId is only associated with condReconfigurationTriggerEUTRA/condReconfigurationTriggerNR:

4>	remove the entry with the matching measObjectId from the measObjectList within the VarMeasConfig;

3>	remove the entry with the matching measId from the measIdList within the VarMeasConfig;

2>	release all radio resources, including release of the MAC configuration, the RLC entity and the associated PDCP entity and SDAP (if any) for all established RBs, except for the following:

-	pur-Config, if stored;

2>	indicate the release of the RRC connection to upper layers together with the release cause;

1>	release the stored serviceType, if any;

1>	inform upper layers to clear the stored application layer measurement configuration;

1>	discard received application layer measurement report information from upper layers, if any;

1>	consider itself not to be configured to send application layer measurement report;

1>	if leaving RRC_CONNECTED was triggered neither by reception of the MobilityFromEUTRACommand message nor by selecting an inter-RAT cell while T311 was running; or

1>	if leaving RRC_INACTIVE was not triggered by the inter-RAT cell reselection:

2>	if timer T350 is configured:

3>	start timer T350;

3>	apply rclwi-Configuration if configured, otherwise apply the wlan-Id-List corresponding to the RPLMN included in SystemInformationBlockType17;

2>	else:

3>	release the wlan-OffloadConfigDedicated, if received;

3>	if the wlan-OffloadConfigCommon corresponding to the RPLMN is broadcast by the cell:

4>	apply the wlan-OffloadConfigCommon corresponding to the RPLMN included in SystemInformationBlockType17;

4>	apply steerToWLAN if configured, otherwise apply the wlan-Id-List corresponding to the RPLMN included in SystemInformationBlockType17;

2>	enter RRC_IDLE and perform procedures as specified in TS 36.304 [4], clause 5.2.7;

1>	else:

2>	release the wlan-OffloadConfigDedicated, if received;

NOTE 2:	BL UEs or UEs in CE verifies validity of SI when released to RRC_IDLE.

1>	discard any segments of segmented RRC messages received;

1>	release the LWA configuration, if configured, as described in 5.6.14.3;

1>	release the LWIP configuration, if configured, as described in 5.6.17.3;

END OF CHANGE