3GPP TSG-RAN WG2 Meeting #122	R2-2306658

Incheon, Korea, May 22-26, 2023

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | 0676 | rev | 1 | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NTN stage-2 correction |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO, LG Electronics, Qualcomm, CATT, Huawei, Lenovo, Thales, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions-Core |  |  |  |  |  | Date: |  |  | 2023-05-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - HARQ enhancements are a big feature for NR NTN, which however have not been captured in the stage-2 spec. - For CG and SPS, it should be up to NW to configure the same HARQ modes. This should be reflected in the stage-2 spec. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add HARQ enhancements related text in the spec. - Add the NOTE that for CG and SPS, it should be up to NW to configure the proper HARQ modes. Impact analysis If the UE implements the CR, and the network does not, there is no interoperability issues. If the network implements the CR, and the UE does not, there is no interoperability issues. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The feature of HARQ enhancements is missing in the stage-2 spec for NR NTN. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.14.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The CR is based on the RAN2#121bis-e’s in-principle agreed CR R2-2304268 (which is resubmitted as R2-2306655 in RAN2#122). |  |  |  |  |  |  |  |  |

1st Change

16.14.2.1	Scheduling and Timing

DL and UL are frame aligned at the uplink time synchronization reference point (RP) with an offset given by NTA,offset (see clause 4.2 of TS 38.213 [38]).

To accommodate the propagation delay in NTNs, several timing relationships are enhanced by a Common Timing Advance (Common TA) and two offsets Koffset and kmac:

-	Common TA is a configured timing offset that is equal to the RTT between the RP and the NTN payload.

-	Koffset is a configured scheduling offset that needs to be larger or equal to the sum of the service link RTT and the Common TA.

-	kmac is a configured offset that is approximately equal to the RTT between the RP and the gNB.The scheduling offset Koffset is used to allow the UE sufficient processing time between a downlink reception and an uplink transmission, see TS 38.213 [38].

The offset kmac is used to delay the application of a downlink configuration indicated by a MAC CE command on PDSCH, see TS 38.213 [38], and in estimation of UE-gNB RTT, see TS 38.321 [6]. It may be provided by the network when downlink and uplink frame timing are not aligned at gNB. The kmac is also used in the random access procedure, to determine the start time of RAR window/MsgB window after a Msg1/MsgA transmission (see TS 38.213 [38]).

The Service link RTT, Feeder link RTT, RP, Common TA, kmac and TTA (see clause 16.14.2.2) are illustrated in Figure 16.14.2.1-1.

Figure 16.14.2.1-1: Illustration of timing relationship (for collocated gNB and NTN Gateway)

The network may configure the HARQ operation as follows:

-	For downlink, HARQ feedback can be enabled or disabled per HARQ process (as specified in clause 5.3.2.2 and clause 5.7 of TS 38.321 [6]). Disabling HARQ feedback allows scheduling a HARQ process before one HARQ RTT has elapsed since last scheduled.

-	For uplink, HARQ mode (i.e. HARQ mode A or HARQ mode B) can be configured per HARQ process (as specified in clause 5.4.3.1 and clause 5.7 of TS 38.321 [6]). HARQ mode B allows scheduling a HARQ process before one HARQ RTT has elapsed since last scheduled.

NOTE:	For the HARQ processes configured with HARQ feedback enabled/disabled, it is up to network implementation to ensure a proper configuration of HARQ feedback (e.g., either all enabled or all disabled) for HARQ processes used by an SPS configuration. For the HARQ processes configured with HARQ mode, it is up to network implementation to ensure a proper configuration of HARQ mode (e.g., either all HARQ mode A or all HARQ mode B) for HARQ processes used by a CG configuration.