3GPP TSG-RAN WG2 Meeting #122	                                      R2-2306696

Incheon, Korea, 22 – 26 May, 2023

Title:	Draft LS to SA2 on sidelink positioning agreements

Response to:

Release:	Rel-18

Work Items:	NR_pos_enh2

Source:	Intel (To be RAN2)

To:	SA2

CC:	RAN1

Contact Person:

#### Name:	Yi Guo

E-mail Address:	yi.guo@Intel.com

Attachment:

1. Overall Description:

RAN2 discussed the support of sidelink positioning, and made following agreements:

SLPP protocol stack:

Agreements:

SLPP over PC5-U/Uu will support reliable transport for at least unicast. FFS groupcast.

SLPP carried over NAS is used between UE and LMF. FFS on how to manage the session/transaction.

Anchor UE selection (addresses partly the question 1 in S2-2305735):

Agreements:

Anchor UE selection is supported by information about the candidate anchor UEs.  At least the following list can be discussed for use in anchor UE selection:

1.	UE roles

2.	Supported positioning method

3.	In coverage or not

4.	RSRP

5.	LOS/NLOS

6.	Location

7.	PLMN

A normative requirement on which anchor UEs to select (e.g., ranking) will not be specified.

RAN2 impact of this information to be determined.

FFS which information would be determined statically/dynamically.

Support of SLPP session (Addressed the question in S2-2303837):

Agreements:

SLPP can support multiple target UEs in the same session when LCS requests.

RAN2 will not specify group management for multiple target UEs.  RAN2 assumption is that a group ID will be provided from upper layers.

FFS how session IDs are managed between multiple UEs.

2. Actions:

To SA2

ACTION:

- RAN2 respectfully asks SA2 to take RAN2’s agreements into account in their future work.

3. Date of Next RAN WG2 Meetings:

TSG-RAN WG2#123                      2023-08-21 to 2023-08-25		Toulouse, FR