3GPP TSG RAN WG2#122    		 R2-2306764

Incheon, Korea, May 22-26, 2023

Agenda Item:	7.13.7

Source:	CATT

Title:   	Summary of 7.13.7 SONMDT enhancements for NPN(CATT)

Document for:	Discussion and Decision

Introduction

In this document, the summary of all the contributions submitted to 7.13.7 agenda item (SON/MDT enhancements for Non-Public Networks) of RAN2#122 meeting as in [1-9] will be presented. Taking the company proposals into account, the 3rd section provides sets of proposals for easy agreement, as well as for further discussions.

Discussion

RLF/HOF report enhancement for NPN

| TDoc | Company name | Proposals |
| --- | --- | --- |
| R2-2304931 | Xiaomi | Proposal 1: RAN2 agrees to introduce the CAG ID into the RLF/HOF report. Proposal 2: Introduce one SNPN ID (e.g. NID) into the RLF report which is applied for multiple IEs, i.e. previousPCellId-r16, failedPCellId-r16, reconnectCellId-r16, reestablishmentCellId-r16. |
| R2-2305325 | Vivo | Proposal 3: UE includes the registered SNPN (i.e., PLMN ID and NID) in rlf-Report in VarRLF-Report. Proposal 4: UE includes the allowed CAG list and CAG-only indication (if configured) in rlf-Report in VarRLF-Report. |
| R2-2305426 | Nokia, Nokia Shanghai Bell | Proposal 1.1: No PNI-NPN (CAG ID) should be added to the RLF/HOF reports. Proposal 1.3: An indication of the UE CAG subscription statues (if the UE has subscription with any of the CAG IDs broadcast by the cell, and whether the UE only allowed to access CAG cells) should be added to the RLF/HOF reports. |
| R2-2305487 | CATT | Proposal 3: Besides SNPN ID, also introduce PNI-NPN ID into the RLF/HOF Report to indicate UE NPN access limit. The PNI-NPN ID is the ID of PNI-NPN which the UE accesses to. Proposal 4: Includes the NID/CAG-ID as new fields in nr-RLF-Report, since there is no extension position in the CGI information related IE. Proposal 5: Confirm for SNPN ID (PLMN ID + NID) checking, only when the SNPN ID of the UE current registered network is included in the SNPN list stored in the UE, the UE could indicate the RLF/HOF result available to the network. |
| R2-2305647 | Samsung | Proposal 2: Introduce SNPN-ID in var-RLF report. Proposal 3: NID is not reported to the network in RLF report or logged measurement report. Proposal 4: CAG is not included in RLF report or logged measurement report. |
| R2-2305990 | Ericsson | Proposal 1 UE does not include PNI-NPN (CAG) ID in the RLF/HOF report. Proposal 2 UE includes the SNPN ID (NID) only in the variable VarRLF-Report. |
| R2-2306249 | ZTE Corporation, Sanechips | Proposal 1: For logged MDT in SNPN, UE stores SNPN IDs (i.e. NIDs) in logged MDT and RLF report results. Proposal 2: CGI-Info-Logging is enhanced to allow logging NPN identity info included in npn-IdentityInfoList of SIB1. |
| R2-2306293 | Huawei, HiSilicon | Proposal 1: No need to introduce PNI-NPN ID (e.g., CAG ID) in to RLF report. Proposal 2: Introduce one SNPN ID (e.g., NID) for both IE failedPCellId and previousPCellId. |

Rapporteur Summary:

For PNI-NPN ID in RLF report

2 companies of Xiaomi and CATT support to introduce the PNI-NPN ID (e.g. CAG ID) into the RLF/HOF report; 4 companies of Nokia, Samsung, Ericsson and Huawei do not support to include it.

2 companies of Vivo and Nokia support to include the information about UE CAG subscription information (e.g. allowed CAG list and CAG-only indication, or subscription indicator) in RLF/HOF reports.

Proposal 1: RAN2 to discuss whether to include PNI-NPN ID (e.g. CAG ID) in the RLF/HOF report.

For SNPN ID in RLF report

It has been agreed in RAN2#120 meeting that “Introduce SPNP ID (e.g., NID) to RLF/HOF report. Details of how to introduce it are FFS”. 5 companies of Xiaomi, Vivo, CATT, Huawei, ZTE support it with different detailed formats. Samsung do not support to report NID in RLF report. The supported formats include:

- NID only;

- PLMN ID + NID.

3 companies of Xiaomi, CATT and Huawei support to introduce one SNPN ID (e.g. NID) into the RLF report for multiple IEs (e.g. previousPCellId-r16, failedPCellId-r16, reconnectCellId-r16, reestablishmentCellId-r16). ZTE proposes to enhance CGI-Info-Logging IE to include NPN identity.

3 companies of Samsung, Ericsson and ZTE support to introduce SNPN ID/NID into variable VarRLF-Report for SNPN ID checking.

Proposal 2: RAN2 to discuss which format (NID only or PLMN+NID) and how to include SNPN related ID in the RLF/HOF report.

Logged MDT enhancement for NPN

| TDoc | Company name | Proposals |
| --- | --- | --- |
| R2-2304931 | Xiaomi | Proposal 3: No need to introduce the SNPN ID (e.g. NID) into logged MDT area configuration. Proposal 4: No need to introduce the SNPN ID (e.g. NID) into logged MDT report. Proposal 5: RAN2 agrees to introduce the CAG ID into the logged MDT area configuration. Proposal 6: RAN2 agrees to introduce the CAG ID into the logged MDT report. |
| R2-2305325 | Vivo | Proposal 6: RAN2 to agree introduce a CAG list for MDT area scope in the logged MDT configuration. Proposal 7: SNPN ID is not needed to be introduced for MDT area scope in the logged MDT configuration. UE in SNPN access mode stores registered SNPN in VarLogMeasReport upon receiving LoggedMeasurementConfiguration, and a new IE similar to plmn-IdentityList should be introduced for storing registered SNPN in VarLogMeasReport. |
| R2-2305426 | Nokia, Nokia Shanghai Bell | Proposal 1.2: No PNI-NPN (CAG ID) should be added to the logged MDT reports. Proposal 1.4: An indication of the UE CAG subscription statues (if the UE has subscription with any of the CAG IDs broadcast by the cell, and whether the UE only allowed to access CAG cells) should be added to the logged MDT reports. Proposal 1.6: Add CAG ID in the area configuration for logged MDT. Proposal 2.1: Include the SPNP ID (PLMN ID and NID) into the logged MDT report. Proposal 2.2: Include the SPNP ID (PLMN ID and NID) into logged MDT configuration. |
| R2-2305487 | CATT | Proposals 2: The current UE variable structure can be enhanced with some SNPN/PNI-NPN specific parameters. Proposal 7: Add PNI-NPN related information i.e. CAG-ID (list) in the area configuration for logged MDT. Proposal 9: Only the UE registered in SNPN can be configured with this SNPN related logged MDT, and the UE can continue logging across the MDT SNPN list. Proposal 10: The solution of adding the SNPN ID in the area scope configuration can wait and align with further RAN3 conclusion. Proposal 11: Confirm only the SNPN ID (PLMN ID + NID) checking is needed before sending the logged MDT availability indication related to NPN network. Proposal 12: It is not necessary to add NPN ID related information e.g. NID/CAG-ID into the logged MDT result. |
| R2-2305647 | Samsung | Proposal 1: LoggedMeasurementConfiguration can configure npn-IdentityList for SNPN. Proposal 3: NID is not reported to the network in RLF report or logged measurement report. Proposal 4: CAG is not included in RLF report or logged measurement report. |
| R2-2305990 | Ericsson | Proposal 8 RAN2 enhance the logged MDT report with cell type indication (e.g., NPN cell) as part of the measurement results. Proposal 9 RAN2 enhance the MDT configuration (interFreqTargetInfo) to enable logging only NPN or PN cells per frequency. |
| R2-2306249 | ZTE Corporation, Sanechips | Proposal 1: For logged MDT in SNPN, UE stores SNPN IDs (i.e. NIDs) in logged MDT results. Proposal 2: CGI-Info-Logging is enhanced to allow logging NPN identity info included in npn-IdentityInfoList of SIB1. Proposal 3: RAN2 wait for RAN3’s outcome on AreaConfiguration for logged MDT in SNPN . Proposal 4: areaConfiguration of logged MDT configuration is enhanced to allow include CAG list. Proposal 5: If configured with CAG list in areaConfiguration, CAG IDs in included in logged MDT results. |
| R2-2306293 | Huawei, HiSilicon | Proposal 3: No need to introduce PNI-NPN ID (e.g., CAG ID) or SPNP ID (e.g., NID) in to logged MDT report. Proposal 4: Introduce CAG list to logged MDT area configuration. Proposal 5: For logged MDT area configuration, RAN2 supports the enhancements for the following configurations: - CAG-only area scope configuration - both PN and CAG related area scope configuration Proposal 6: No need to add SPNP ID (e.g., NID) in logged MDT area configuration. |

Rapporteur Summary:

For SNPN ID in Logged MDT Configuration

3 companies of Xiaomi, Vivo and Huawei do not support to introduce the SNPN ID into the logged MDT area configuration;

2 companies of Nokia and Samsung support to introduce the SNPN ID;

2 companies of CATT and ZTE think the solution of adding the SNPN ID in the area scope configuration can wait and align with RAN3 conclusion.

Proposal 3: RAN2 to discuss whether to include SNPN ID in the logged MDT area configuration.

For PNI-NPN ID in Logged MDT Configuration

All 6 companies of Xiaomi, Vivo, Nokia, CATT, ZTE and Huawei support to introduce the CAG ID (list) into the logged MDT area configuration; Huawei proposes to enhance the MDT area scope to include CAG-only and both PN and CAG configuration.

Proposal 4: Include CAG ID(s) in the logged MDT area configuration.

For SNPN ID in Logged MDT Report

4 companies of Xiaomi, CATT, Samsung and Huawei do not support to introduce the SNPN ID into the logged MDT report; 2 companies of Nokia and ZTE support to introduce the SNPN ID into the logged MDT report.

For PNI-NPN ID in Logged MDT Report

4 companies of Nokia, CATT, Samsung and Huawei do not support to introduce the CAG ID(s) into the logged MDT report; 2 companies of Xiaomi and ZTE support to introduce the CAG ID(s) into the logged MDT report.

Nokia proposes to include an indication of the UE CAG subscription statues in Logged MDT Report.

Rapporteur thinks the logged cells can be SNPN only, CAG only, normal PLMN only or shared between/among different network types. The measurement result of logged MDT is the result of the cell which is used to draw a picture of the channel power/quality which is not affected by the cell types. The operator which receives the logged MDT result can obtain the network type information of the cell without UE reporting. So based on majority’s view, it is proposed that:

Proposal 5: Do not include the SNPN ID/CAG ID(s) in the logged MDT report.

If the Proposal 5 can be agreed, even SNPN ID does not need to be reported, 2 companies of CATT and ZTE suggest storing the SNPN ID in the UE variable to perform SNPN ID checking. But this issue can be considered after deciding whether to report the SNPN ID/CAG ID(s) in the logged MDT report.

Cell type indication

In addition, Ericsson proposes to enhance the logged MDT configuration and the report with cell type indication (e.g., NPN cell). Rapporteur thinks this can be discussed together with the above issues.

Co-existence of NPN and PNs

| TDoc | Company name | Proposals |
| --- | --- | --- |
| R2-2304931 | Xiaomi | Proposal 7: No need to introduce the new NPN specific variables for PNI-NPNs and SNPNs. |
| R2-2305325 | Vivo | Proposal 5: Common RLF report can be used for the public network and private network. |
| R2-2305426 | Nokia, Nokia Shanghai Bell | Proposal 1.5: Not to introduce new NPN specific variables for PNI-NPNs. Proposal 2.3: Introduce a new SNPN specific variable for logged MDT. FFS if the introduction of any storage limitation is needed. Proposal 2.4: Send a reply LS to RAN3 LS in R3-232118 that RAN2 agreed to introduce a new SNPN specific variable for logged MDT, which enables keeping the logged MDT report when the UE moves from one network type to another network type. (See draft reply LS in the Annex.) |
| R2-2305487 | CATT | Proposal 1: Not to introduce separate SON/MDT related UE variable(s) for SNPN or for PNI-NPN. Proposals 2: The current UE variable structure can be enhanced with some SNPN/PNI-NPN specific parameters. Proposal16: RAN2 does not enhance the continuous logged MDT recording between PN and SNPN networks, and send RAN2 decision to RAN3. |
| R2-2305647 | Samsung | Proposal 5: Common report is used for PN and NPN. Proposal 6: Keep the existing principle that UE releases any logged MDT report after deregistration for NPN. |
| R2-2305990 | Ericsson | Proposal 4 UE logs MDT report for SNPNs in a separate variable. Proposal 5 RAN2 discuss whether to consider additional memory (e.g., additional 64KB) for the SNPN MDT report. |
| R2-2306249 | ZTE Corporation, Sanechips | Proposal 8: RAN2 further discuss solutions to the loss of stored logged MDT reports upon moving from a network of one type to another type with considerations on logged MDT types, memory size and specs impact. |
| R2-2306293 | Huawei, HiSilicon | Proposal 7: No new NPN specific variables is introduces for both PNI-NPNs and SNPNs. |
| R2-2306358 | Xiaomi | Proposal 1: The motivation to have the enhancement for logged MDT report for the mobility between the SNPN and PLMN needs to be further clarified by RAN3. Proposal 2: RAN2 considers the draft LS reply to RAN3 in the annex. |

Rapporteur Summary:

5 companies of Xiaomi, Vivo, CATT, Samsung and Huawei do not support to introduce separate UE variables for PNI-NPNs and SNPNs for e.g. RLF/HOF report and logged MDT;

2 companies of Nokia and Ericsson support to introduce a new SNPN specific variable for logged MDT, based on the request by RAN3 LS;

1 company of ZTE suggests further discussion on this issue.

In legacy specification, more than one UE variable by any reason (e.g. for different PLMNs, for signaling based and management based logged MDT) for a specific SON/MDT use case is not allowed. So rapporteur thinks since the SNPN (which can be considered have the same level as PLMN) cannot be equivalent with other PLMNs, to follow majority’s view, no new NPN specific variables should be introduced.

Proposal 6: No new NPN specific variables are introduced for both PNI-NPNs and SNPNs.

For logged MDT, the UE can only perform the logged MDT if the RPLMN is included in the MDT PLMN list. The definition of the MDT PLMN list is a subset of the EPLMN list and RPLMN at the time when MDT is initiated. Since the SNPN cannot be equivalent with any normal PLMNs, and for the sake of UE memory saving, RAN2 should not enhance the continuous logged MDT recording between PN and SNPN networks, and send RAN2 decision to RAN3.

Proposal 7: RAN2 does not enhance the continuous logged MDT recording between PN and SNPN networks, and send RAN2 decision to RAN3.

Other aspects

OOC indication

| TDoc | Company name | Proposals |
| --- | --- | --- |
| R2-2304931 | Xiaomi | Proposal 8: UE access mode (e.g. SNPN access mode or with CAG-only indication) can be reported to network for the OOC analysis. |
| R2-2305990 | Ericsson | Proposal 6 RAN2 discuss to log an SNPN out-of-coverage (OoC) indication either in a. an RA report, or b. a CEF report, or c. a new report. |
| R2-2306249 | ZTE Corporation, Sanechips | Proposal 6: Support logging of OOC instance in logged MDT for NPN in R18. Proposal 7: Supports further differentiation OOC cause (e.g., whether due to weak coverage or due to cell being barred) when logging any cell selection state in NPN MDT results. |

Other use cases enhancement for NPN

| TDoc | Company name | Proposals |
| --- | --- | --- |
| R2-2305487 | CATT | Proposal 13: Report the NPN related information to the TCE together with the L2 measurement, e.g. throughput or data volume measurement. Proposal 14: No need to perform NPN related enhancement for immediate MDT in the air interface. Proposal 15: NPN enhancements for other SON related use cases can be further studied after the enhancement to RLF/HOF report and logged MDT are identified. |
| R2-2305990 | Ericsson | Proposal 3 Inclusion of TAC information in the SON reports (RA-Report, SHR, SPR) is mandatory. Proposal 7 UE logs time spent in the SNPN network in an entry in the existing PN MHI report. |

Equivalent SNPN list

| TDoc | Company name | Proposals |
| --- | --- | --- |
| R2-2305487 | CATT | Proposal 6: Add a NOTE to clarify that “SNPN list can only include one SNPN ID in SONMDT optimization of NPN in this Release”. Proposal 8: Introduce the definition of MDT SNPN list, and add a NOTE to clarify that “MDT SNPN list can only include one SNPN ID in this Release”. Proposal 9: Only the UE registered in SNPN can be configured with this SNPN related logged MDT, and the UE can continue logging across the MDT SNPN list. |

SNPN ID double checking

| TDoc | Company name | Proposals |
| --- | --- | --- |
| R2-2305325 | Vivo | Proposal 1: SNPN ID checking is needed before transmitting the information for the corresponding SON and MDT reports. Proposal 2a: The following steps can be adopted for SNPN ID checking for RLF report: - UE stores registered SNPN (i.e, PLMN ID and NID) when determining the content in the VarRLF-Report; a new IE similar to plmn-IdentityList should be introduced for storing registered SNPN in VarRLF-Report. - If the registered SNPN is the same as the stored SNPN in VarRLF-Report and if the UE has radio link failure or handover failure information available in VarRLF-Report, UE shall include the available indication of RLF report in the RRC Complete message; - If the registered SNPN is the same as the stored SNPN in VarRLF-Report, UE reports RLF-report upon the network requests for it. Proposal 2b: The following steps can be adopted for SNPN ID checking for logged MDT report: - UE stores registered SNPN (i.e, PLMN ID and NID) when receiving LoggedMeasurementConfiguration; a new IE similar to plmn-IdentityList should be introduced for storing registered SNPN in VarLogMeasReport. - If the registered SNPN is the same as the stored SNPN in VarLogMeasReport and if the UE has logged measurements available for NR, UE shall include the available indication of logged measurements in the RRC Complete message; - If the registered SNPN is the same as the stored SNPN in VarLogMeasReport, UE reports logged measurement report upon the network requests for it. |

Rapporteur Summary:

Furthermore,

- Xiaomi, Ericsson and ZTE propose to consider some information reporting for out-of-coverage analysis, e.g. UE access mode, SNPN OOC indication, OOC instance;

- CATT and Ericsson propose to consider other SON/MDT enhancements for NPN, e.g. L2 measurement, RA-Report, SHR, SPR, MHI;

- CATT proposes to consider equivalent SNPN list agreed in R18 eNPN item, and limit to one SNPN ID in this Release;

- Vivo thinks the SNPN ID checking is also needed before transmitting the corresponding report, besides the checking before sending the availability indication for SON and MDT report.

Proposal 8: RAN2 to discuss:

- Whether and how to introduce information reporting for OOC analysis involving NPN network;

- Whether and which to introduce other SON/MDT enhancements for NPN in this Release;

- Whether equivalent SNPN list (limit to one SNPN ID in this Release) needs to be considered to align with the future NPN evolution;

- Whether SNPN ID checking is needed before transmitting the information for the corresponding SON and MDT reports.

Conclusion

Based on summary of [1-9], following proposals are made for further discussion, and some proposals are only discussed under certain conditions.

For easy agreement

Proposal 4: Include CAG ID(s) in the logged MDT area configuration.

Proposal 5: Do not include the SNPN ID/CAG ID(s) in the logged MDT report.

Proposal 6: No new NPN specific variables are introduced for both PNI-NPNs and SNPNs.

Proposal 7: RAN2 does not enhance the continuous logged MDT recording between PN and SNPN networks, and send RAN2 decision to RAN3.

For online discussion

Proposal 1: RAN2 to discuss whether to include PNI-NPN ID (e.g. CAG ID) in the RLF/HOF report.

Proposal 2: RAN2 to discuss which format (NID only or PLMN+NID) and how to include SNPN related ID in the RLF/HOF report.

Proposal 3: RAN2 to discuss whether to include SNPN ID in the logged MDT area configuration.

Discussed if time allows

Proposal 8: RAN2 to discuss:

- Whether and how to introduce information reporting for OOC analysis involving NPN network;

- Whether and which to introduce other SON/MDT enhancements for NPN in this Release;

- Whether equivalent SNPN list (limit to one SNPN ID in this Release) needs to be considered to align with the future NPN evolution;

- Whether SNPN ID checking is needed before transmitting the information for the corresponding SON and MDT reports.

Reference

- R2-2304931, Discussion on the SONMDT enhancement for NPN, Beijing Xiaomi Software Tech

- R2-2305325, Discussion on SON enhancements for NPN, vivo

- R2-2305426, Discussion on NP related issues in SON/MDT, Nokia, Nokia Shanghai Bell

- R2-2305487, SON and MDT Enhancement for NPN, CATT

- R2-2305647, SON/MDT enhancements for NPN, Samsung R&D Institute India

- R2-2305990, SON support for NPN, Ericsson

- R2-2306249, Consideration on SON-MDT support for NPN, ZTE Corporation, Sanechips

- R2-2306293, Discussion on SONMDT enhancements for NPN, Huawei, HiSilicon

- R2-2306358, Discussion on the “LS on potential override of logged MDT reports upon moving from SNPN to PLMN” from RAN3 (R3-232118), Beijing Xiaomi Software Tech