3GPP TSG-RAN WG2 Meeting #122	R2-2306812

# Incheon, Korea, May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 4159 | rev | 1 | Current version: | 16.12.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on reference cell for TCI state |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_eMIMO-Core |  |  |  |  |  | Date: |  |  | 2023-05-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In Release-17, a term “applied” is used for a specific meaning for a TCI state when UE applies a TCI state in a serving cell but the TCI state itself is configured in another serving cell. The field description of cell in IE TCI-State use the term applied twice in Release-17 and only once with the specific meaning. Clarifying the wording in Release-17 creates a reason to align the sentence in Release-16 as well as the current form may cause uncertainty of what is the serving cell in which the referenceSignal is configured when the field cell is absent. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Section 6.2.2 - Clarification added in the field description cell in IE TCI-State Impact Analysis Impacted 5G architecture options: NR Standalone, (NG)EN-DC, NE-DC, NR-DC Impacted functionality: TCI state Inter-operability: 1. If the network is implemented according to the CR and the UE is not, there may be uncertainty of serving cell in which the referenceSignal is configured. 2. If the UE is implemented according to the CR and the network is not, there may be uncertainty of serving cell in which the referenceSignal is configured. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If the CR is not approved, there may be uncertainty of what is the serving cell in which the referenceSignal is configured. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | After Release-17 had defined meaning for “applied” and reason to update the wording it creates a reason to update Release-16 wording as well to keep the text clear and consistent. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

### 6.3.2	Radio resource control information elements

#### –	TCI-State

The IE TCI-State associates one or two DL reference signals with a corresponding quasi-colocation (QCL) type.

TCI-State information element

-- ASN1START

-- TAG-TCI-STATE-START

TCI-State ::=                       SEQUENCE {

tci-StateId                         TCI-StateId,

qcl-Type1                           QCL-Info,

qcl-Type2                           QCL-Info                                                    OPTIONAL,   -- Need R

...

}

QCL-Info ::=                        SEQUENCE {

cell                                ServCellIndex                                               OPTIONAL,   -- Need R

bwp-Id                              BWP-Id                                                      OPTIONAL, -- Cond CSI-RS-Indicated

referenceSignal                     CHOICE {

csi-rs                              NZP-CSI-RS-ResourceId,

ssb                                 SSB-Index

},

qcl-Type                            ENUMERATED {typeA, typeB, typeC, typeD},

...

}

-- TAG-TCI-STATE-STOP

-- ASN1STOP

QCL-Info field descriptions

bwp-Id The DL BWP which the RS is located in.

cell The UE's serving cell in which the referenceSignal is configured. If the field is absent, the referenceSignal is configured in the serving cell in which the TCI-State is configured. The RS can be located on a serving cell other than the serving cell in which the TCI-State is configured only if the qcl-Type is configured as typeC or typeD. See TS 38.214 [19] clause 5.1.5.

referenceSignal Reference signal with which quasi-collocation information is provided as specified in TS 38.214 [19] clause 5.1.5.

qcl-Type QCL type as specified in TS 38.214 [19] clause 5.1.5.

| Conditional Presence | Explanation |
| --- | --- |
| CSI-RS-Indicated | This field is mandatory present if csi-rs is included, absent otherwise |

END OF CHANGES