3GPP TSG-RAN2 Meeting #122	R2-2306827

# Incheon, Korea, 22-26 May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 4066 | rev | 2 | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Downlink positioning support and posSIB request for L2 UE-to-network remote UE [PosL2RemoteUE] |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc., CATT, Huawei, HiSilicon, Qualcomm Incorporated, Xiaomi, Intel Corporation, vivo, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18 |  |  |  |  |  | Date: |  |  | 2023-05-25 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - Downlink positioning methods require knowledge of the system time by the target UE. However, a L2 U2N remote UE may be unable to obtain the system time because of lacking a direct Uu link and potentially being out of Uu coverage. - A L2 U2N remote UE cannot request posSIBs from the relay UE, which reduces the benefits of posSIBs by forcing remote UEs to use unicast LPP to request assistance data. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - An SFN-DFN offset is added to the RRCReconfigurationSidelink message sent from the L2 U2N relay UE, allowing the remote UE to infer the SFN timeline from the DFN timeline. - A requested posSIB list is added to the RemoteUEInformationSidelink message. - An indication of support for the SFN-DFN offset by the remote UE is added to the UECapabilityInformationSidelink message. - An indication of support for forwarding the posSIB by the remote UE is added to the UECapabilityInformationSidelink message. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A L2 U2N remote UE may be unable to perform downlink positioning and will be unable to request posSIBs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.8.9.1.1, 5.8.9.1.2, 5.8.9.8.1, 5.8.9.8.2, 5.8.9.8.3, 5.8.9.9.2, 6.6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS 38.305 CR XXXX TS 37.355 CR XXXX TS 38.306 CR XXXX |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

##### 5.8.9.1.1	General

Figure 5.8.9.1.1-1: Sidelink RRC reconfiguration, successful

Figure 5.8.9.1.1-2: Sidelink RRC reconfiguration, failure

The purpose of this procedure is to modify a PC5-RRC connection, e.g. to establish/modify/release sidelink DRBs or PC5 Relay RLC channels, to (re-)configure NR sidelink measurement and reporting, to (re-)configure sidelink CSI reference signal resources, to (re)configure CSI reporting latency bound, to (re)configure sidelink DRX, to (re-)configure the latency bound of SL Inter-UE coordination report, and to indicate the SFN-DFN offset.

The UE may initiate the sidelink RRC reconfiguration procedure and perform the operation in clause 5.8.9.1.2 on the corresponding PC5-RRC connection in following cases:

-	the release of sidelink DRBs associated with the peer UE, as specified in clause 5.8.9.1a.1;

-	the establishment of sidelink DRBs associated with the peer UE, as specified in clause 5.8.9.1a.2;

-	the modification for the parameters included in SLRB-Config of sidelink DRBs associated with the peer UE, as specified in clause 5.8.9.1a.2;

-	the release of PC5 Relay RLC channels for L2 U2N Relay UE and Remote UE, as specified in clause 5.8.9.7.1;

-	the establishment of PC5 Relay RLC channels for L2 U2N Relay UE and Remote UE, as specified in clause 5.8.9.7.2;

-	the modification for the parameters included in SL-RLC-ChannelConfigPC5 of PC5 Relay RLC channels for L2 U2N Relay UE and Remote UE, as specified in clause 5.8.9.7.2;

-	the (re-)configuration of the peer UE to perform NR sidelink measurement and report.

-	the (re-)configuration of the sidelink CSI reference signal resources and CSI reporting latency bound;

-	the (re-)configuration of the peer UE to perform sidelink DRX;

-	the (re-)configuration of the latency bound of SL Inter-UE coordination report;

-	the request in a RemoteUEInformationSidelink message for the SFN-DFN offset from the L2 U2N Relay UE;

-	the change in the value of the SFN-DFN offset at the L2 U2N Relay UE.

NOTE: It is up to L2 U2N Relay UE implementation to determine when the SFN-DFN offset has changed in value to a degree requiring an update to be sent to the L2 U2N Remote UE.

In RRC_CONNECTED, the UE applies the NR sidelink communications parameters provided in RRCReconfiguration (if any). In RRC_IDLE or RRC_INACTIVE, the UE applies the NR sidelink communications parameters provided in system information (if any). For other cases, UEs apply the NR sidelink communications parameters provided in SidelinkPreconfigNR (if any). When UE performs state transition between above three cases, the UE applies the NR sidelink communications parameters provided in the new state, after acquisition of the new configurations. Before acquisition of the new configurations, UE continues applying the NR sidelink communications parameters provided in the old state.

##### 5.8.9.1.2	Actions related to transmission of RRCReconfigurationSidelink message

The UE shall set the contents of RRCReconfigurationSidelink message as follows:

1>	for each sidelink DRB that is to be released, according to clause 5.8.9.1a.1.1, due to configuration by sl-ConfigDedicatedNR, SIB12, SidelinkPreconfigNR or by upper layers:

2>	set the entry included in the slrb-ConfigToReleaseList corresponding to the sidelink DRB;

1>	for each sidelink DRB that is to be established or modified, according to clause 5.8.9.1a.2.1, due to receiving sl-ConfigDedicatedNR, SIB12 or SidelinkPreconfigNR:

2>	if a sidelink DRB is to be established:

3>	assign a new logical channel identity for the logical channel to be associated with the sidelink DRB and set sl-MAC-LogicalChannelConfigPC5 in the SLRB-Config to include the new logical channel identity;

2>	set the SLRB-Config included in the slrb-ConfigToAddModList, according to the received sl-RadioBearerConfig and sl-RLC-BearerConfig corresponding to the sidelink DRB;

1>	set the sl-MeasConfig as follows:

2>	If the frequency used for NR sidelink communication is included in sl-FreqInfoToAddModList in sl-ConfigDedicatedNR within RRCReconfiguration message or included in sl-ConfigCommonNR within SIB12:

3>	if UE is in RRC_CONNECTED:

4>	set the sl-MeasConfig according to stored NR sidelink measurement configuration information for this destination;

3>	if UE is in RRC_IDLE or RRC_INACTIVE:

4>	set the sl-MeasConfig according to stored NR sidelink measurement configuration received from SIB12;

2>	else:

3>	set the sl-MeasConfig according to the sl-MeasPreconfig in SidelinkPreconfigNR;

1>	set the sl-LatencyBoundIUC-Report;

1>	start timer T400 for the destination;

1>	set the sl-CSI-RS-Config;

1>	set the sl-LatencyBoundCSI-Report;

1>	set the sl-ResetConfig;

NOTE 1:	Whether/how to set the parameters included in sl-LatencyBoundIUC-Report, sl-CSI-RS-Config, sl-LatencyBoundCSI-Report and sl-ResetConfig is up to UE implementation.

1>	set the sl-DRX-ConfigUC-PC5 as follows:

2>	If the frequency used for NR sidelink communication is included in sl-FreqInfoToAddModList in sl-ConfigDedicatedNR within RRCReconfiguration message or included in sl-ConfigCommonNR within SIB12:

3>	if UE is in RRC_CONNECTED and if sl-ScheduledConfig is included in sl-ConfigDedicatedNR within RRCReconfiguration:

4>	set the sl-DRX-ConfigUC-PC5 according to stored NR sidelink DRX configuration information for this destination.

NOTE 2:	If UE is in RRC_IDLE or in RRC_INACTIVE or out of coverage, or in RRC_CONNECTED and sl-UE-SelectedConfig is included in sl-ConfigDedicatedNR within RRCReconfiguration, it is up to UE implementation to set the sl-DRX-ConfigUC-PC5.

1>	for each PC5 Relay RLC channel that is to be released due to configuration by sl-ConfigDedicatedNR:

2>	set the SL-RLC-ChannelID corresponding to the PC5 Relay RLC channel in the sl-RLC-ChannelToReleaseListPC5;

1>	for each PC5 Relay RLC channel that is to be established or modified due to receiving sl-ConfigDedicatedNR:

2>	if a PC5 Relay RLC channel is to be established:

3>	assign a new logical channel identity for the logical channel to be associated with the PC5 Relay RLC channel and set sl-MAC-LogicalChannelConfigPC5 in the SL-RLC-ChannelConfigPC5 to include the new logical channel identity;

2>	set the SL-RLC-ChannelConfigPC5 included in the sl-RLC-ChannelToAddModListPC5 according to the received SL-RLC-ChannelConfig corresponding to the PC5 Relay RLC channel, including setting sl-RLC-ChannelID-PC5 to the same value of sl-RLC-ChannelID received in SL-RLC-ChannelConfig;

1>	if the UE is operating as a L2 U2N Relay UE:

2>	if the destination UE requested the SFN-DFN offset in a previous RemoteUEInformationSidelink message:

3>	if the SFN-DFN offset has changed since a previous transmission of the RRCReconfigurationSidelink message, or no previous transmission of the RRCReconfigurationSidelink message has occurred since the reception of the RemoteUEInformationSidelink message:

4>	set the sl-SFN-DFN-Offset according to the relation between the SFN timeline of the PCell and the DFN timeline;

The UE shall submit the RRCReconfigurationSidelink message to lower layers for transmission.

##### […]

NEXT CHANGE

##### 5.8.9.8.1	General

Figure 5.8.9.8.1-1: Remote UE information

This procedure is used by the L2 U2N Remote UE in RRC_IDLE/RRC_INACTIVE to inform about the required SIB(s)/posSIB(s), provide Paging related information to the connected L2 U2N Relay UE, and request the SFN-DFN offset from the connected L2 U2N Relay UE.

NOTE:	MIB is not required by a L2 U2N Remote UE.

##### 5.8.9.8.2	Actions related to transmission of RemoteUEInformationSidelink message

When entering RRC_IDLE or RRC_INACTIVE, or upon change in any of the information in the RemoteUEInformationSidelink while in RRC_IDLE or RRC_INACTIVE, the L2 U2N Remote UE shall:

1>	if the UE has not stored a valid version of a SIB, in accordance with clause 5.2.2.2.1, of one or several required SIB(s) in accordance with clause 5.2.2.1 and the requested SIB has not been indicated in RemoteUEInformationSidelink message to the L2 U2N Relay UE before:

2>	include sl-RequestedSIB-List in the RemoteUEInformationSidelink to indicate the requested SIB(s);

1>	if the UE has not stored a valid version, in accordance with clause 5.2.2.2.1, of one or several posSIB(s) that the UE requires for a positioning operation, and the requested posSIB has not been indicated in RemoteUEInformationSidelink message to the L2 U2N Relay UE before, and the connected L2 U2N relay UE set posSIB-ForwardingSupported to true:

2>	include sl-RequestedPosSIB-List in the RemoteUEInformationSidelink to indicate the requested posSIB(s);

1>	if the UE has not sent sl-PagingInfo-RemoteUE in the RemoteUEInformationSidelink message to the L2 U2N Relay UE before, set sl-PagingInfo-RemoteUE as follows:

2>	if the L2 U2N Remote UE is in RRC_IDLE:

3>	include ng-5G-S-TMSI in the sl-PagingIdentityRemoteUE;

3>	if the UE specific DRX cycle is configured by upper layer, set sl-PagingCycleRemoteUE to the value of UE specific Uu DRX cycle configured by upper layer;

2>	else if the L2 U2N Remote UE is in RRC_INACTIVE:

3>	include ng-5G-S-TMSI and fullI-RNTI in the sl-PagingIdentityRemoteUE;

3>	if the UE specific DRX cycle is configured by upper layer,

4>	set sl-PagingCycleRemoteUE to the minimum value of UE specific Uu DRX cycles (configured by upper layer and configured by RRC);

3>	else:

4>	set sl-PagingCycleRemoteUE to the value of UE specific DRX cycle configured by RRC;

1>	submit the RemoteUEInformationSidelink message to lower layers for transmission;

When entering RRC_CONNECTED, if L2 U2N remote UE had sent sl-RequestedSIB-List, sl-RequestedPosSIB-List, and/or sl-PagingInfo-RemoteUE, the L2 U2N Remote UE shall:

1>	set the sl-RequestedSIB-List to the value release if requested before;

1>	set the sl-RequestedPosSIB-List to the value release if requested before;

1>	set the sl-PagingInfo-RemoteUE to the value release if sent before;

1>	submit the RemoteUEInformationSidelink message to lower layers for transmission;

##### 5.8.9.8.3	Reception of RemoteUEInformationSidelink message by the L2 U2N Relay UE

The L2 U2N Relay UE shall:

1>	if the RemoteUEInformationSidelink includes the sl-PagingInfo-RemoteUE:

2>	if the UE is in RRC_CONNECTED on an active BWP with common search space configured including pagingSearchSpace; or

2>	if the UE is in RRC_IDLE or RRC_INACTIVE:

3>	if the sl-PagingInfo-RemoteUE is set to setup:

4>	monitor the Paging message at the L2 U2N Remote UE's paging occasion calculated according to sl-PagingIdentityRemoteUE and sl-PagingCycleRemoteUE included in sl-PagingInfo-RemoteUE;

3>	else (the sl-PagingInfo-RemoteUE is set to release):

4>	stop monitoring the Paging message at the L2 U2N Remote UE's paging occasion;

4>	release the received paging information in sl-PagingInfo-RemoteUE;

2>	else (the UE is in RRC_CONNECTED on an active BWP without pagingSearchSpace configured):

3>	if the sl-PagingInfo-RemoteUE is set to setup:

4>	include the received sl-PagingIdentityRemoteUE in SidelinkUEInformationNR message and perform Sidelink UE information transmission in accordance with 5.8.3;

3>	else (the sl-PagingInfo-RemoteUE is set to release):

4>	initiate transmission of the SidelinkUEInformationNR message to release the sl-PagingIdentityRemoteUE in SidelinkUEInformationNR message in accordance with 5.8.3;

4>	release the received paging information in sl-PagingInfo-RemoteUE;

1>	if the RemoteUEInformationSidelink includes the sl-RequestedSIB-List:

2>	if the sl-RequestedSIB-List is set to setup:

3> if the L2 U2N Relay UE has not stored a valid version of SIB(s) indicated in sl-RequestedSIB-List:

4>	perform acquisition of the system information indicated in sl-RequestedSIB-List in accordance with 5.2.2;

3>	perform the Uu message transfer procedure in accordance with 5.8.9.9;

2>	if the sl-RequestedSIB-List is set to release:

3>	release received SIB request in sl-RequestedSIB-List;

1>	if the RemoteUEInformationSidelink includes the sl-RequestedPosSIB-List:

2>	if the sl-RequestedPosSIB-List is set to setup:

3> if the L2 U2N Relay UE has not stored a valid version of posSIB(s) indicated in sl-RequestedPosSIB-List:

4>	perform acquisition of the positioning system information indicated in sl-RequestedPosSIB-List in accordance with 5.2.2;

3>	else:

4>	perform the Uu message transfer procedure in accordance with 5.8.9.9;

2>	if the sl-RequestedPosSIB-List is set to release:

3>	release received posSIB request in sl-RequestedPosSIB-List.

##### […]

NEXT CHANGE

##### 5.8.9.9.2	Actions related to transmission of UuMessageTransferSidelink message

The L2 U2N Relay UE initiates the Uu message transfer procedure when at least one of the following conditions is met:

1>	upon receiving Paging message related to the connected L2 U2N Remote UE from network (including Paging message within RRCReconfiguration message);

1>	upon acquisition of the SIB(s) requested by the connected L2 U2N Remote UE (as indicated in sl-RequestedSIB-List in the RemoteUEInformationSidelink) or upon receiving the updated SIB(s) from network which has been requested by the connected L2 U2N Remote UE;

1>	upon acquisition of the posSIB(s) requested by the connected L2 U2N Remote UE (as indicated in sl-RequestedPosSIB-List in the RemoteUEInformationSidelink) or upon receiving the updated posSIB(s) from network which have been requested by the connected L2 U2N Remote UE;

1>	upon unsolicited SIB1 forwarding to the connected L2 U2N Remote UE or upon receiving the updated SIB1 from network;

For each associated L2 U2N Remote UE, the L2 U2N Relay UE shall set the contents of UuMessageTransferSidelink message as follows:

1>	include sl-PagingDelivery if the Paging message received from network containing the ue-Identity of the L2 U2N Remote UE;

1>	include sl-SIB1-Delivery if any of the conditions for initiating Uu message transfer procedure related to SIB1 are met;

1>	include sl-SystemInformationDelivery if any of the conditions for initiating Uu message transfer procedure related to System Information are met;

1>	submit the UuMessageTransferSidelink message to lower layers for transmission.

NOTE:	The L2 U2N Relay UE may perform unsolicited forwarding of SIB1 to the L2 U2N Remote UE based on UE implementation.

#### […]

NEXT CHANGE

### 6.6.2	Message definitions

#### […]

#### –	RemoteUEInformationSidelink

The RemoteUEInformationSidelink message is used to request SIB(s) or provide paging related information as specified in clause 5.8.9.8.1.

Signalling radio bearer: SL-SRB3

RLC-SAP: AM

Logical channel: SCCH

Direction: L2 U2N Remote UE to L2 U2N Relay UE

RemoteUEInformationSidelink message

-- ASN1START

-- TAG-REMOTEUEINFORMATIONSIDELINK-START

RemoteUEInformationSidelink-r17 ::=           SEQUENCE {

criticalExtensions                            CHOICE {

remoteUEInformationSidelink-r17               RemoteUEInformationSidelink-r17-IEs,

criticalExtensionsFuture                      SEQUENCE {}

}

}

RemoteUEInformationSidelink-r17-IEs ::=       SEQUENCE {

sl-RequestedSIB-List-r17                     SetupRelease { SL-RequestedSIB-List-r17}          OPTIONAL, -- Need M

sl-PagingInfo-RemoteUE-r17                    SetupRelease { SL-PagingInfo-RemoteUE-r17}         OPTIONAL, -- Need M

lateNonCriticalExtension                      OCTET STRING                                       OPTIONAL,

nonCriticalExtension                          RemoteUEInformationSidelink-v18xy-IEs             OPTIONAL

}

RemoteUEInformationSidelink-v18xy-IEs ::=    SEQUENCE {

sl-RequestedPosSIB-List-r18					SetupRelease { SL-RequestedPosSIB-List-r18 }		OPTIONAL,	-- Need M

sl-SFN-DFN-OffsetRequested-r18                  ENUMERATED { true }                              OPTIONAL,   -- Need R

nonCriticalExtension						SEQUENCE {}											OPTIONAL

}

SL-RequestedSIB-List-r17 ::=                 SEQUENCE (SIZE (maxSIB-MessagePlus1-r17)) OF SL-SIB-ReqInfo-r17

SL-PagingInfo-RemoteUE-r17 ::=                SEQUENCE {

sl-PagingIdentityRemoteUE-r17                 SL-PagingIdentityRemoteUE-r17,

sl-PagingCycleRemoteUE-r17                    PagingCycle                                        OPTIONAL  -- Need M

}

SL-SIB-ReqInfo-r17 ::=                   ENUMERATED { sib1, sib2, sib3, sib4, sib5, sib6, sib7, sib8, sib9, sib10, sib11, sib12, sib13,

sib14, sib15, sib16, sib17, sib18, sib19, sib20, sib21, spare11, spare10, spare9,

spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1, ... }

SL-RequestedPosSIB-List-r18 ::=                 SEQUENCE (SIZE (1..maxSIB)) OF SL-PosSIB-ReqInfo-r18

SL-PosSIB-ReqInfo-r18 ::=	SEQUENCE {

gnss-id-r18                  	GNSS-ID-r16                  OPTIONAL,

sbas-id-r18                  	SBAS-ID-r16                  OPTIONAL,

posSibType-r18				ENUMERATED { posSibType1-1, posSibType1-2, posSibType1-3, posSibType1-4, posSibType1-5, posSibType1-6,

posSibType1-7, posSibType1-8, posSibType2-1, posSibType2-2, posSibType2-3, posSibType2-4,

posSibType2-5, posSibType2-6, posSibType2-7, posSibType2-8, posSibType2-9, posSibType2-10,

posSibType2-11, posSibType2-12, posSibType2-13, posSibType2-14, posSibType2-15,

posSibType2-16, posSibType2-17, posSibType2-18, posSibType2-19, posSibType2-20,

posSibType2-21, posSibType2-22, posSibType2-23, posSibType3-1, posSibType4-1,

posSibType5-1,posSibType6-1, posSibType6-2, posSibType6-3,... }

-- TAG-REMOTEUEINFORMATIONSIDELINK-STOP

-- ASN1STOP

RemoteUEInformationSidelink-IEs field descriptions

sl-RequestedSIB-List Contains a list of requested SIBs.

sl-PagingInfo-RemoteUE Indicates the paging information used by L2 U2N Relay UE to perform the connected L2 U2N Remote UE's paging monitoring.

sl-PagingIdentityRemoteUE Indicates the L2 U2N Remote UE's paging UE ID.

sl-PagingCycleRemoteUE Indicates the L2 U2N Remote UE's UE specific DRX cycle as the minimum value of the one provided by upper layers (if configured) and the one provided by RRC layer (if configured). Value rf32 corresponds to 32 radio frames, value rf64 corresponds to 64 radio frames and so on.

sl-SFN-DFN-OffsetRequested If present, this field indicates that the L2 U2N Remote UE requests the L2 U2N Relay UE to provide the SFN-DFN offset in a subsequent RRCReconfigurationSidelink message.

#### –	RRCReconfigurationSidelink

The RRCReconfigurationSidelink message is the command to AS configuration of the PC5 RRC connection. It is only applied to unicast of NR sidelink communication.

Signalling radio bearer: SL-SRB3

RLC-SAP: AM

Logical channel: SCCH

Direction: UE to UE

RRCReconfigurationSidelink message

-- ASN1START

-- TAG-RRCRECONFIGURATIONSIDELINK-START

RRCReconfigurationSidelink ::=          SEQUENCE {

rrc-TransactionIdentifier-r16           RRC-TransactionIdentifier,

criticalExtensions                      CHOICE {

rrcReconfigurationSidelink-r16          RRCReconfigurationSidelink-r16-IEs,

criticalExtensionsFuture                SEQUENCE {}

}

}

RRCReconfigurationSidelink-r16-IEs ::=  SEQUENCE {

slrb-ConfigToAddModList-r16             SEQUENCE (SIZE (1..maxNrofSLRB-r16)) OF SLRB-Config-r16             OPTIONAL, -- Need N

slrb-ConfigToReleaseList-r16            SEQUENCE (SIZE (1..maxNrofSLRB-r16)) OF SLRB-PC5-ConfigIndex-r16    OPTIONAL, -- Need N

sl-MeasConfig-r16                       SetupRelease {SL-MeasConfig-r16}                                    OPTIONAL, -- Need M

sl-CSI-RS-Config-r16                    SetupRelease {SL-CSI-RS-Config-r16}                                 OPTIONAL, -- Need M

sl-ResetConfig-r16                      ENUMERATED {true}                                                   OPTIONAL, -- Need N

sl-LatencyBoundCSI-Report-r16           INTEGER (3..160)                                                    OPTIONAL, -- Need M

lateNonCriticalExtension                OCTET STRING                                                        OPTIONAL,

nonCriticalExtension                    RRCReconfigurationSidelink-v1700-IEs                                OPTIONAL

}

RRCReconfigurationSidelink-v1700-IEs ::= SEQUENCE {

sl-DRX-ConfigUC-PC5-r17                 SetupRelease { SL-DRX-ConfigUC-r17 }                                OPTIONAL, -- Need M

sl-LatencyBoundIUC-Report-r17           SetupRelease { SL-LatencyBoundIUC-Report-r17 }                      OPTIONAL, -- Need M

sl-RLC-ChannelToReleaseListPC5-r17      SEQUENCE (SIZE (1..maxSL-LCID-r16)) OF SL-RLC-ChannelID-r17         OPTIONAL, -- Need N

sl-RLC-ChannelToAddModListPC5-r17       SEQUENCE (SIZE (1..maxSL-LCID-r16)) OF SL-RLC-ChannelConfigPC5-r17  OPTIONAL, -- Need N

nonCriticalExtension                    RRCReconfigurationSidelink-v18xy-IEs                                OPTIONAL

}

RRCReconfigurationSidelink-v18xy-IEs ::= SEQUENCE {

sl-SFN-DFN-Offset-r18                   SL-SFN-DFN-Offset-r18                                               OPTIONAL, -- Need N

nonCriticalExtension                    SEQUENCE {}                                                         OPTIONAL

}

SL-LatencyBoundIUC-Report-r17::=            INTEGER (3..160)

SLRB-Config-r16::=                      SEQUENCE {

slrb-PC5-ConfigIndex-r16                SLRB-PC5-ConfigIndex-r16,

sl-SDAP-ConfigPC5-r16                   SL-SDAP-ConfigPC5-r16                                               OPTIONAL, -- Need M

sl-PDCP-ConfigPC5-r16                   SL-PDCP-ConfigPC5-r16                                               OPTIONAL, -- Need M

sl-RLC-ConfigPC5-r16                    SL-RLC-ConfigPC5-r16                                                OPTIONAL, -- Need M

sl-MAC-LogicalChannelConfigPC5-r16      SL-LogicalChannelConfigPC5-r16                                      OPTIONAL, -- Need M

...

}

SLRB-PC5-ConfigIndex-r16 ::=            INTEGER (1..maxNrofSLRB-r16)

SL-SDAP-ConfigPC5-r16 ::=               SEQUENCE {

sl-MappedQoS-FlowsToAddList-r16         SEQUENCE (SIZE (1.. maxNrofSL-QFIsPerDest-r16)) OF SL-PQFI-r16      OPTIONAL, -- Need N

sl-MappedQoS-FlowsToReleaseList-r16     SEQUENCE (SIZE (1.. maxNrofSL-QFIsPerDest-r16)) OF SL-PQFI-r16      OPTIONAL, -- Need N

sl-SDAP-Header-r16                      ENUMERATED {present, absent},

...

}

SL-PDCP-ConfigPC5-r16 ::=               SEQUENCE {

sl-PDCP-SN-Size-r16                     ENUMERATED {len12bits, len18bits}                                   OPTIONAL, -- Need M

sl-OutOfOrderDelivery-r16               ENUMERATED { true }                                                 OPTIONAL,  -- Need R

...

}

SL-RLC-ConfigPC5-r16 ::=                CHOICE {

sl-AM-RLC-r16                           SEQUENCE {

sl-SN-FieldLengthAM-r16                 SN-FieldLengthAM                                                OPTIONAL, -- Need M

...

},

sl-UM-Bi-Directional-RLC-r16            SEQUENCE {

sl-SN-FieldLengthUM-r16                 SN-FieldLengthUM                                                OPTIONAL, -- Need M

...

},

sl-UM-Uni-Directional-RLC-r16           SEQUENCE {

sl-SN-FieldLengthUM-r16                 SN-FieldLengthUM                                                OPTIONAL, -- Need M

...

}

}

SL-LogicalChannelConfigPC5-r16 ::=      SEQUENCE {

sl-LogicalChannelIdentity-r16           LogicalChannelIdentity,

...

}

SL-PQFI-r16 ::=                         INTEGER (1..64)

SL-CSI-RS-Config-r16 ::=                SEQUENCE {

sl-CSI-RS-FreqAllocation-r16            CHOICE {

sl-OneAntennaPort-r16                   BIT STRING (SIZE (12)),

sl-TwoAntennaPort-r16                   BIT STRING (SIZE (6))

}                                                                                                           OPTIONAL, -- Need M

sl-CSI-RS-FirstSymbol-r16               INTEGER (3..12)                                                     OPTIONAL, -- Need M

...

}

SL-RLC-ChannelConfigPC5-r17::=          SEQUENCE {

sl-RLC-ChannelID-PC5-r17                SL-RLC-ChannelID-r17,

sl-RLC-ConfigPC5-r17                    SL-RLC-ConfigPC5-r16                                                OPTIONAL, -- Need M

sl-MAC-LogicalChannelConfigPC5-r17      SL-LogicalChannelConfigPC5-r16                                      OPTIONAL, -- Need M

...

}

SL-SFN-DFN-Offset-r18 ::=					SEQUENCE {

sl-FrameOffset-r18                          INTEGER (1..1023)                                               OPTIONAL, -- Need S

sl-SubframeOffset-r18                       INTEGER (1..9)                                                  OPTIONAL, -- Need S

sl-SlotOffset-r18                           INTEGER (1..31)                                                 OPTIONAL  -- Need S

}

-- TAG-RRCRECONFIGURATIONSIDELINK-STOP

-- ASN1STOP

#### […]

#### –	UECapabilityInformationSidelink

The UECapabilityInformationSidelink message is used to transfer UE radio access capabilities. It is only applied to unicast of NR sidelink communication.

Signalling radio bearer: SL-SRB3

RLC-SAP: AM

Logical channel: SCCH

Direction: UE to UE

UECapabilityInformationSidelink message

-- ASN1START

-- TAG-UECAPABILITYINFORMATIONSIDELINK-START

UECapabilityInformationSidelink ::=         SEQUENCE {

rrc-TransactionIdentifier-r16               RRC-TransactionIdentifier,

criticalExtensions                          CHOICE {

ueCapabilityInformationSidelink-r16         UECapabilityInformationSidelink-r16-IEs,

criticalExtensionsFuture                    SEQUENCE {}

}

}

UECapabilityInformationSidelink-r16-IEs ::= SEQUENCE {

accessStratumReleaseSidelink-r16            AccessStratumReleaseSidelink-r16,

pdcp-ParametersSidelink-r16                 PDCP-ParametersSidelink-r16                                             OPTIONAL,

rlc-ParametersSidelink-r16                  RLC-ParametersSidelink-r16                                              OPTIONAL,

supportedBandCombinationListSidelinkNR-r16  BandCombinationListSidelinkNR-r16                                       OPTIONAL,

supportedBandListSidelink-r16               SEQUENCE (SIZE (1..maxBands)) OF BandSidelinkPC5-r16                    OPTIONAL,

appliedFreqBandListFilter-r16               FreqBandList                                                            OPTIONAL,

lateNonCriticalExtension                    OCTET STRING                                                            OPTIONAL,

nonCriticalExtension                        UECapabilityInformationSidelink-v1700-IEs                               OPTIONAL

}

UECapabilityInformationSidelink-v1700-IEs ::= SEQUENCE {

mac-ParametersSidelink-r17                    MAC-ParametersSidelink-r17                                            OPTIONAL,

supportedBandCombinationListSidelinkNR-v1710  BandCombinationListSidelinkNR-v1710                                   OPTIONAL,

nonCriticalExtension                          UECapabilityInformationSidelink-v18xy-IEs                             OPTIONAL

}

UECapabilityInformationSidelink-v18xy-IEs ::= SEQUENCE {

sfn-DFN-OffsetSupported-r18                   ENUMERATED { supported }                                                   OPTIONAL,

posSIB-ForwardingSupported-r18                ENUMERATED { supported }                                                   OPTIONAL,

nonCriticalExtension                          SEQUENCE {}                                                           OPTIONAL

}

MAC-ParametersSidelink-r17 ::= SEQUENCE {

drx-OnSidelink-r17                          ENUMERATED {supported}                                                  OPTIONAL,

...

}

AccessStratumReleaseSidelink-r16 ::= ENUMERATED { rel16, rel17, spare6, spare5, spare4, spare3, spare2, spare1, ... }

PDCP-ParametersSidelink-r16 ::= SEQUENCE {

outOfOrderDeliverySidelink-r16              ENUMERATED {supported}      OPTIONAL,

...

}

BandCombinationListSidelinkNR-r16 ::= SEQUENCE (SIZE (1..maxBandComb)) OF BandCombinationParametersSidelinkNR-r16

BandCombinationListSidelinkNR-v1710 ::= SEQUENCE (SIZE (1..maxBandComb)) OF BandCombinationParametersSidelinkNR-v1710

BandCombinationParametersSidelinkNR-r16 ::= SEQUENCE (SIZE (1..maxSimultaneousBands)) OF BandParametersSidelink-r16

BandCombinationParametersSidelinkNR-v1710 ::= SEQUENCE (SIZE (1..maxSimultaneousBands)) OF BandParametersSidelink-v1710

BandParametersSidelink-v1710 ::=    SEQUENCE {

--32-5a-1

tx-IUC-Scheme1-Mode2Sidelink-r17    ENUMERATED {supported}                      OPTIONAL,

--32-5b-1

tx-IUC-Scheme2-Mode2Sidelink-r17    ENUMERATED {n4, n8, n16}                    OPTIONAL

}

BandSidelinkPC5-r16 ::=           SEQUENCE {

freqBandSidelink-r16              FreqBandIndicatorNR,

--15-1

sl-Reception-r16                  SEQUENCE {

harq-RxProcessSidelink-r16        ENUMERATED {n16, n24, n32, n64},

pscch-RxSidelink-r16              ENUMERATED {value1, value2},

scs-CP-PatternRxSidelink-r16      CHOICE {

fr1-r16                           SEQUENCE {

scs-15kHz-r16                     BIT STRING (SIZE (16))                            OPTIONAL,

scs-30kHz-r16                     BIT STRING (SIZE (16))                            OPTIONAL,

scs-60kHz-r16                     BIT STRING (SIZE (16))                            OPTIONAL

},

fr2-r16                           SEQUENCE {

scs-60kHz-r16                     BIT STRING (SIZE (16))                            OPTIONAL,

scs-120kHz-r16                    BIT STRING (SIZE (16))                            OPTIONAL

}

}                                                                                           OPTIONAL,

extendedCP-RxSidelink-r16         ENUMERATED {supported}                                    OPTIONAL

}                                                                                               OPTIONAL,

--15-10

sl-Tx-256QAM-r16                  ENUMERATED {supported}                                        OPTIONAL,

--15-12

lowSE-64QAM-MCS-TableSidelink-r16 ENUMERATED {supported}                                        OPTIONAL,

...,

[[

--15-14

csi-ReportSidelink-r16                SEQUENCE {

csi-RS-PortsSidelink-r16              ENUMERATED {p1, p2}

}                                                                                               OPTIONAL,

--15-19

rankTwoReception-r16                  ENUMERATED {supported}                                    OPTIONAL,

--15-23

sl-openLoopPC-RSRP-ReportSidelink-r16 ENUMERATED {supported}                                    OPTIONAL,

--13-1

sl-Rx-256QAM-r16                      ENUMERATED {supported}                                    OPTIONAL

]],

[[

--32-5a-2

rx-IUC-Scheme1-PreferredMode2Sidelink-r17     ENUMERATED {supported}                           OPTIONAL,

--32-5a-3

rx-IUC-Scheme1-NonPreferredMode2Sidelink-r17  ENUMERATED {supported}                           OPTIONAL,

--32-5b-2

rx-IUC-Scheme2-Mode2Sidelink-r17               ENUMERATED {n5, n15, n25, n32, n35, n45, n50, n64} OPTIONAL,

--32-6-1

rx-IUC-Scheme1-SCI-r17                         ENUMERATED {supported}                           OPTIONAL,

--32-6-2

rx-IUC-Scheme1-SCI-ExplicitReq-r17             ENUMERATED {supported}                           OPTIONAL,

--32-7

scheme2-ConflictDeterminationRSRP-r17          ENUMERATED {supported}                           OPTIONAL

]]

}

-- TAG-UECAPABILITYINFORMATIONSIDELINK-STOP

-- ASN1STOP

END OF CHANGE