3GPP TSG RAN WG2 Meeting#122	R2-2306873

Incheon, South Korea, 22nd - 26th May, 2023

# Title:	LS on L1 measurements for LTM

# Response to:

# Release:	Rel-18

# Work Item:	NR_Mob_enh2-Core

Source:	Ericsson (To be RAN2)

To:	RAN1, RAN3

Cc:	RAN4

Contact person:

Name:	Antonino Orsino

Tel. Number:

E-mail Address:	antonino.orsino@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	none.

1. Overall Description:

RAN2 has discussed the L1 measurements for LTM, and the outcome is captured in the following agreements:

- For L1 measurements for LTM The RS configuration is provided to the UE per LTM candidate cell. RAN2 assumes that Each candidate DU needs to know the RS configuration of each candidate DUs in order to provide the LTM candidate configuration. RAN2 assumes that The CU transmits to each C-DU the RS configuration of S-DU (if this is an LTM candidate cell) and/or other C-DUs, to generate the corresponding L1 configuration for LTM. RAN2 assumes C-DU generates the RS configuration and send to the CU. The CU transmits to the Source DU the RS configuration per LTM candidate cell and the associated LTM candidate (when the CU receives LTM candidate configuration(s) from the C-DU). It is up to RAN3 whether the RS configuration is sent before (or at the same time of) the C-DU creates the LTM candidate configuration (and whether is semi-statis or UE associated). The RS configuration and/or CSI resource configuration for measuring LTM candidate cells is included in the LTM-Config IE and is a separate configuration, e.g. outside of the LTM candidate configuration. CSI reports for LTM candidates (neighbour cell reports for the purpose of LTM cell switch) are configured by the serving cell in an IE that is like CSI-ReportConfig for LTM within the ServingCellConfig since this is the cell in which the report is to be transmitted. RAN2 assumes the following about CSI measurement reporting for LTM (final decision up to RAN1): - a. UE reports all measured LTM candidate cells in a single report; or - b. UE reports one or a subset of measured LTM candidate cell(s) in a report.

In order to finalize the work on this, RAN2 would like to ask RAN1 to provide the RRC parameters that are part of the L1 RS configuration and the supported association(s) of reporting instances and LTM candidate cells (i.e., see last RAN2 agreement).

Also, RAN2 would like to ask RAN1 and RAN3 to take these agreements into account and provide feedbacks if RAN1 and/or RAN3 identify any major technical issues.

2. Actions:

To RAN1.

ACTION: 	RAN2 asks RAN1 to provide the RRC parameters that are part of the L1 RS configuration and the supported association(s) of reporting instances and LTM candidate cells (i.e., see last RAN2 agreement).

To RAN1 and RAN3.

ACTION:	RAN2 asks RAN1 and RAN3 to take the above information into account and provide feedbacks if RAN1 and/or RAN3 identify any major technical issues.

3. Date of Next RAN2 Meetings:

TSG RAN WG2 Meeting #123	August 21 – August 25, 2023		Toulouse, France

TSG RAN WG2 Meeting #123bis	October 9 – October 13, 2023		Xiamen, China