3GPP TSG-RAN WG2 Meeting #122	R2-2306947

# Incheon, Korea: 22nd May – 26th May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 4172 | rev | 1 | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on the release of logged measurement configuration as well as logged measurement information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT-Core |  |  |  |  |  | Date: |  |  | 2023-05-25 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | From the state 2 descriptions of logged MDT in TS 37.320, we have the following, “There is only one RAT-specific logged measurement configuration for Logged MDT in the UE. When the network provides a configuration, any previously configured logged measurement configuration will be entirely replaced by the new one. Moreover, logged measurements corresponding to the previous configuration will be cleared at the same time. It is left up to the network to retrieve any relevant data before providing a new configuration.” However, it is not clear from stage 3 whether UE releases previously received logged measurement configuration and logged measurement information if a new configuration is received from the same RAT. In our understanding, the logged measurement configuration and logged measurement information should be released even upon receiving a new logged measurement configuration from the same RAT. For example, the UE action upon reception of new loggedMeasurementConfiguration by the UE (section 5.5a.1.3) refers to section 5.5a.2 for discarding the logged measurement configuration as well as logged measurement information. #### 5.5a.1.3 Reception of the LoggedMeasurementConfiguration by the UE Upon receiving the LoggedMeasurementConfiguration message the UE shall: 1> discard the logged measurement configuration as well as the logged measurement information as specified in 5.5a.2; However, section 5.5a.2 talks about action on the releasing “logged measurement configuration in another RAT” #### 5.5a.2.2 Initiation The UE shall initiate the procedure upon receiving a logged measurement configuration in another RAT. The UE shall also initiate the procedure upon power off or upon deregistration.. |
| Summary of change: |  | The proposed CR clarifies that logged measurement configuration as well as logged measurement information is released when a new loggedMeasurmentConfiguration is received by the UE, irrespective of RAT. Impact analysis: Impacted functionality: Logged measurement report Inter-operability: No. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE behavior is not clear upon receiving from same RAT. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5a.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

### 5.5a.2	Release of Logged Measurement Configuration

#### 5.5a.2.1	General

The purpose of this procedure is to release the logged measurement configuration as well as the logged measurement information.

#### 5.5a.2.2	Initiation

The UE shall initiate the procedure upon receiving a logged measurement configuration in same or another RAT. The UE shall also initiate the procedure upon power off or upon deregistration.

The UE shall:

1>	stop timer T330, if running;

1>	if stored, discard the logged measurement configuration as well as the logged measurement information, i.e. release the UE variables VarLogMeasConfig and VarLogMeasReport.

END OF CHANGE