3GPP TSG-RAN WG2 Meeting #122          	          R2-2306923

Incheon, Korea, 22 – 26 May, 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | 0680 | rev | 2 | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of In-Device Co-existence (IDC) enhancements for NR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IDC_enh-Core |  |  |  |  |  | Date: |  |  | 2023-05-29 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This CR introduces the support of Rel-18 In-Device Co-existence (IDC) enhancements for NR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of general description of Rel-18 In-Device Co-existence (IDC) enhancements for NR. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Rel-18 In-Device Co-existence (IDC) enhancements are not supported in NR. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.9,10.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 37.340 CR 0367 TS 38.331 CR 4164 TS 38.306 CR 0915 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 0 - Initial version in R2-2305995 Rev 1 - Minor editorial updates in R2-2306592. Rev 2 – Updates to CR cover page to add the other specs running CR numbers. |  |  |  |  |  |  |  |  |

Start of 1st change

## 7.9	UE Assistance Information

When configured to do so, the UE can signal the network through UEAssistanceInformation:

-	If it prefers an adjustment in the connected mode DRX cycle length, for the purpose of delay budget reporting;

-	If it is experiencing internal overheating;

-	If it prefers certain DRX parameter values, and/or a reduced maximum number of secondary component carriers, and/or a reduced maximum aggregated bandwidth and/or a reduced maximum number of MIMO layers and/or minimum scheduling offsets K0 and K2 for power saving purpose;

-	If it expects not to send or receive any more data in the near future, and in this case, it can provide its preference to transition out of RRC_CONNECTED where this indication may express its preferred RRC state, or alternately, it may cancel an earlier indicated preference to transition out of RRC_CONNECTED;

-	If it prefers (not) to be provisioned with reference time information;

-	If it prefers to transition out of RRC_CONNECTED state for MUSIM operation and its preferred RRC state after transition;

-	If it wants to include assistance information for setup or release of gaps for MUSIM operation;

-	When affected by IDC problems that it cannot solve by itself:

-	The list of frequencies affected by IDC problems (see clause 23.4 of TS 36.300 [2]); or

-	The list of frequency ranges/frequency range combinations affected by the IDC problems; and

-	TDM assistance information;

-	Its RRM measurement relaxation status indicating whether RRM measurement relaxation criteria are met or not;

-	Its RLM measurement relaxation status indicating whether the UE is applying RLM measurements relaxation;

-	Its BFD measurement relaxation status indicating whether the UE is applying BFD measurements relaxation.

NOTE:	The requirements on RRM/RLM/CSI measurements in different phases of IDC interference defined in TS 36.300 [2] are applicable except that for NR serving cell, the requirements in TS 38.133 [13] and TS 38.101-1 [18], TS 38.101-2 [35], TS 38.101-3 [36] apply.

In the second case, the UE can express a preference for temporarily reducing the number of maximum secondary component carriers, the maximum aggregated bandwidth and the number of maximum MIMO layers. In all cases, it is up to the gNB whether to accommodate the request.

For sidelink, the UE can report SL traffic pattern(s) to NG-RAN, for periodic traffic.

End of 1st change

Start of 2nd change

## 10	Scheduling

## 10. x    Autonomous Denial for IDC

The network may configure a long-term denial rate by dedicated RRC signalling to limit the amount of NR UL autonomous denials. Once configured by the network, the UE can autonomously deny NR UL transmission. Otherwise, the UE shall not perform any NR UL autonomous denials for IDC.

End of 2nd change