3GPP TSG RAN WG2#122	R2-2304610

Incheon, Korea, 22nd - 26th May, 2023

3GPP TSG RAN WG1 #112bis-e	R1-2304099

e-Meeting, April 17th – April 26th, 2023

Title:	LS to RAN2 on scheduling and HARQ issues for FR2-2

Release:	Release 17

Work Item:	NR_ext_to_71GHz-Core

Source:	TSG RAN WG1

To:	TSG RAN WG2

Cc:

Contact person:	Seonwook Kim

seonwook.kim@lge.com

# 1	Overall description

RAN1 identified three issues related to scheduling and HARQ for FR2-2.

First of all, RAN1 observed that according to current TS 38.331 specification, enhanced type 3 HARQ-ACK codebook supports up to 16 HARQ process numbers per serving cell since bitmap size allocated for a serving cell equals to 16 as highlighted below.

PDSCH-HARQ-ACK-EnhType3-r17 ::=         SEQUENCE {

pdsch-HARQ-ACK-EnhType3Index-r17    PDSCH-HARQ-ACK-EnhType3Index-r17,

applicable-r17   CHOICE {

perCC                            SEQUENCE (SIZE (1..maxNrofServingCells)) OF INTEGER (0..1),

perHARQ                          SEQUENCE (SIZE (1..maxNrofServingCells)) OF BIT STRING (SIZE (16))

},

pdsch-HARQ-ACK-EnhType3NDI-r17         ENUMERATED {true}                                            OPTIONAL,   -- Need R

pdsch-HARQ-ACK-EnhType3CBG-r17         ENUMERATED {true}                                            OPTIONAL,   -- Need S

...

}

However, if a UE is provided with nrofHARQ-ProcessesForPDSCH-v1700 for a serving cell, a maximum of 32 HARQ processes for the serving cell can be used for the downlink. Therefore, RAN1 respectfully request RAN2 to update 331 specification for enhanced type 3 HARQ-ACK codebook, taking into account that up to 32 HARQ processes can be configured for a serving cell.

Secondly, RAN1 reached the following agreement regarding CBG configuration.

Agreement - It is RAN1’s understanding that if at least one DL (or UL) BWP configured in a cell has 480 or 960 kHz, the network does not configure the higher layer parameter codeBlockGroupTransmission for DL (or UL).

Therefore, RAN1 respectfully request to update 331 specification based on RAN1’s understanding, if needed.

Thirdly, RAN1 made the following agreement in RAN1#104-e. As highlighted below, a UE does not expect to be configured with both numberOfSlotsTBoMS-r17 and pusch-TimeDomainAllocationListForMultiPUSCH-r16.

Agreement: (RAN1#104-e) - For a UE and for a serving cell, scheduling multiple PDSCHs by single DL DCI and scheduling multiple PUSCHs by single UL DCI are supported. - Each PDSCH or PUSCH has individual/separate TB(s) and each PDSCH/PUSCH is confined within a slot. - FFS: The maximum number of PDSCHs or PUSCHs that can be scheduled with a single DCI - FFS: Whether multiple PDSCH scheduling applies to 120 kHz in addition to 480 and 960 kHz - At least for 120 kHz SCS, single-slot scheduling with slot-based monitoring will still be supported as specified in Rel-15/Rel-16 - The followings will not be considered in this WI. - Single DCI to schedule both PDSCH(s) and PUSCH(s) - Single DCI to schedule one or multiple TBs where any single TB can be mapped over multiple slots, where mapping is not by repetition - Single DCI to schedule N TBs (N>1) where a TB can be repeated over multiple slots (or mini-slots) - Note: This does not imply that existing slot aggregation and/or repetition for PDSCH and PUSCH by single DCI is precluded for the serving cell.

Therefore, RAN1 respectfully request RAN2 to update 331 specification in line with the above RAN1 agreement.

# 2	Actions

To TSG RAN2

ACTION: 	RAN1 respectfully ask RAN2 to update TS 38.331 specification by taking the above information into account.

# 3	Dates of next TSG RAN WG1 meetings

RAN1#113	22 - 26 May 2023	 	Incheon, Korea

RAN1#114	21 - 25 Aug 2023	 	Toulouse, France