3GPP TSG RAN WG2#122	R2-2304618

Incheon, Korea, 22nd - 26th May, 2023

3GPP TSG RAN WG1 Meeting #112bis-e	R1-2304257

e-Meeting, April 17th – 26th, 2023

Title:	LS on MCSt resource (re-)selection

Response to:	-

Release:	Rel-18

Work Item:	NR_SL_enh2-Core

Source:	RAN1

To:	RAN2

Cc:	-

Contact person:	Kevin Lin

Kevin.Lin@oppo.com

Send any reply LS to: 3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN1 has discussed the following approaches to implement/achieve MCSt for SL-U communication. RAN1 would like to seek RAN2’s opinion on the following questions.

Approach 1: “best effort for multiple TBs”

- Step 1: Higher layer triggers L1 resource selection for one TB with one set of parameters (prioTX, remaining PDB, LsubCH and Prsvp_TX) - R16/17 behavior.

- Step 2: L1 report a set of candidate single-slot resource (SA) according to existing L1 resource allocation procedure - R16/17 behavior.

- Step 3: Higher layer selects a set of resources either randomly (R16/17 behavior) or according to a consecutive-slots criterion (new behavior) to achieve MCSt.

- Step 4: Repeat Step 1-3 for different TB if required.

Approach 2: “guarantee MCSt for single TB and best effort for multiple TBs”

- Step 1: Higher layer triggers L1 resource selection for one TB with one set of parameters (prioTX, remaining PDB, LsubCH and Prsvp_TX) + “number of slots for MCSt” which could be derived based on CAPC of the logical channel/TB or other means.

- Step 2: L1 report a set of candidate multi-slot resource (SA) according to most of the existing L1 resource allocation procedure (FFS: RSRP calculation / threshold may need to change)

- Step 3: Higher layer selects a candidate multi-slot resource either randomly (R16/17 behavior) or according to a consecutive-slots criterion (new behavior).

- Step 4: Repeat Step 1-3 for different TB if required.

Approach 3: “guarantee MCSt for multiple TBs”

- Step 1: Higher layer triggers L1 resource (re-)selection one time for one or multiple TBs with one set of parameters (prioTX, remaining PDB, LsubCH and Prsvp_TX) + “number of slots for MCSt” which could be derived based on CAPC of the multiple TBs.

- Step 2: L1 report a set of candidate multi-slot resource (SA) according to most of the existing L1 resource allocation procedure (FFS: RSRP calculation / threshold may need to change)

- Step 3: Higher layer selects transmission resource for the one or multiple TB(s) from the reported set of candidate multi-slot resource (SA).

Question 1 (for Approach 1/ Approach 2): feasibility of selecting the resource for a single TB in MAC layer (single-slot under Approach 1, multi-slot under Approach 2) with the principle of “concatenating” across separate resource selection triggers (across TBs)

Question 2 (for Approach 3): feasibility of triggering the resource selection procedures for multiple SL processes at the same time

Question 3 (Approach 2/ Approach 3): feasibility of providing a new parameter “number of slots for MCSt” to L1 when triggering resource (re-)selection for MCSt

2. Actions:

To RAN2: RAN1 respectfully asks RAN2 to provide an answer to the questions above.

3. Date of Next RAN1 Meetings:

RAN1#113	May 22-26, 2023	Incheon, KR

RAN1#114	August 21-25, 2023	Toulouse, FR