3GPP TSG-RAN WG2 Meeting #123	R2-2307077

Toulouse, France, 21~25 August 2023

Agenda item:	7.5.3

Source:	Qualcomm Incorporated

Title:	Power saving enhancements for XR

WID/SID:	NR_XR_enh-Core

Document for:	Discussion and Decision

# Introduction

In this paper, we discuss the remaining issues in enhancements for the DRX mismatch problem and SFN wrap-around problem.

# Discussion

## DRX mismatch

In the last RAN2 meeting (#122), company agreed to introduce new DRX cycles in rational numbers, as the way to address the DRX cycle mismatch problem for XR traffic. It is left to be discussed whether the new DRX cycles in rational numbers are applicable to both short and long DRX cycles or only one of them, how DRX formula should be updated and how DRX cycles in rational numbers should be specified in ASN.1.

Considering the fact that XR traffic is composed of continuous, periodic data bursts, we think either short DRX cycle or long DRX cycle would work fine. There is no good reason to exclude one of them from using the new DRX cycles, and it will be really up to operators to choose which type of DRX cycle(s) to configure.

Proposal 1. 	New DRX cycles in rational numbers are supported for both short and long DRX cycles.

In legacy, if short DRX cycle is configured, the length of long DRX cycle shall be an integer multiple of the short DRX cycle. We think this requirement should be applicable to the new DRX cycles as well, for the same reasons behind the requirement in legacy.

Proposal 2. 	If short DRX cycle in rational number is configured, the length of the long DRX cycle shall be an integer multiple of the short DRX cycle, as in legacy.

As to how to capture the changes to the DRX formula, we think it is desirable to reuse the legacy DRX formula as much as possible, because it has been in use for a long time and would be easier for developers to identify and understand how the new enhancements differs from the legacy one.

One approach to accomplish that is to add only a floor operation to the legacy DRX formula. To ensure the modulo operation in the formula does not produce rounding error, a note instead of normative text is added to specify this requirement. We think a note is better because there are different ways to implement modulo operation on rational numbers without rounding errors. Hence 3GPP should not mandate one particular implementation method over another. For example,

- One may use the formula that A modulo B = A – B x floor(A/B). For example, 17 mod (50/3) = 17 – (50/3) x floor (17/(50/3)) = 17 – (50/3) x 1 = 1/3.

- Or one may use the method of least common multiples (LCM), i.e. compute the LCM of the denominators, which then transforms the modulo operation to integers that can be calculated exactly. Mathematically, for integers A, B and C, A modulo (B/C) = (A x C/C) modulo (B/C) = [(AxC) modulo B] / C. So the original modulo operation becomes the ratio of two integers. For example, 17 mod (50/3) = (51/3) mod (50/3) = (51 mod 50) /3 = 1/3.

It is also worth noting that several modern programming languages support modulo operation on rational numbers without rounding errors. For example, the BigDecimal class in Java can provide arbitrary precision for decimal data types. It allows storing rational numbers with near infinite precision to avoid rounding errors. Or one may also use fraction data type, which represents rational numbers as fractions instead of decimals. That also can help avoid rounding errors in modulo operation.

Observation 1. There are different mathematical ways to implement modulo operations on rational numbers without rounding errors. And modern programming languages also can support such operations without rounding errors.

Based on the reasons above, an example TP can be the following:

1> if the drx-NonIntegerShortCycle is used for a DRX group, and floor ([(SFN × 10) + subframe number] modulo (drx-NonIntegerShortCycle)) = floor ((drx-StartOffset) modulo (drx-NonIntegerShortCycle)): 2> start drx-onDurationTimer for this DRX group after drx-SlotOffset from the beginning of the subframe. 1> if the drx-NonIntegerLongCycle is used for a DRX group, and floor ([(SFN × 10) + subframe number] modulo (drx-NonIntegerLongCycle)) = drx-StartOffset: NOTE x: If drx-NonIntegerShortCycle or drx-NonIntegerLongCycle is used, the modulo operation should be implemented by a method that does not produce rounding errors. The exact method can be up to UE implementation.

Proposal 3. 	In TS 38.321, capture the following changes for the case where the DRX cycle is in rational numbers:

- add floor operation to the legacy DRX formula;

- in a note, capture the requirement that the modulo operation should be implemented by a method that does not produce rounding errors. The exact method is up to UE implementation.

In legacy, long DRX cycle and start offset are specified in pairs of integers. However, since new DRX cycles are in rational numbers, it is not possible to directly specify a new DRX cycle in ASN.1. One possible approach is to specify the same corresponding frame rate in the IE and define how to derive the corresponding DRX cycle in rational number in the field description. An example TP is the following:

drx-NonIntegerLongCycleStartOffset CHOICE { 24 INTEGER(0..40), 30 INTEGER(0..32), 45 INTEGER(0..21), 48 INTEGER(0..19), 60 INTEGER(0..15), 80 INTEGER(0..11), 90 INTEGER(0..10), 120 INTEGER(0..7), …

DRX-Config field descriptions

drx-HARQ-RTT-TimerDL Value in number of symbols of the BWP where the transport block was received. drx-HARQ-RTT-TimerDL-r17 is only applicable for SCS 480 kHz and 960 kHz. If configured, the UE shall ignore drx-HARQ-RTT-TimerDL (without suffix) for SCS 480 kHz and 960 kHz.

drx-HARQ-RTT-TimerUL Value in number of symbols of the BWP where the transport block was transmitted. drx-HARQ-RTT-TimerUL-r17 is only applicable for SCS 480 kHz and 960 kHz. If configured, the UE shall ignore drx-HARQ-RTT-TimerUL (without suffix) for SCS 480 kHz and 960 kHz.

drx-InactivityTimer Value in multiple integers of 1 ms. ms0 corresponds to 0, ms1 corresponds to 1 ms, ms2 corresponds to 2 ms, and so on.

drx-NonIntegerLongCycleStartOffset Frame rate in fps corresponding to a non-integer long DRX cycle and drx-StartOffset in multiples of 1 ms. The actual value of long DRX cycle is the canonical representation of the reciprocal of the frame rate in unit of ms. For example, for the frame rate of 60 fps, the long DRX cycle is 50/3 ms. If drx-NonIntegerShortCycle is configured, the value of drx-NonIntegerLongCycle shall be a multiple of the drx-NonIntegerShortCycle value. If drx-NonIntegerLongCycleStartOffset is configured, the UE shall ignore drx-LongCycleStartOffset (without suffix).

drx-LongCycleStartOffset drx-LongCycle in ms and drx-StartOffset in multiples of 1 ms. If drx-ShortCycle is configured, the value of drx-LongCycle shall be a multiple of the drx-ShortCycle value. If drx-LongCycleStartOffset-r18 is configured, the UE shall ignore drx-LongCycleStartOffset (without suffix).

… (omitted fields)

Note that for each frame rate, the range of the corresponding start offset is between 0 and floor(1000 / frame rate) – 1.

Proposal 4. 	In TS 38.331, specify a long DRX cycle in rational number as follows:

- Represent it by the corresponding frame rate in pair with its associated start offset, which has the range of 0~ [floor(1000 / frame rate) – 1];

- In its field description, specify the actual value of a long DRX cycle as the canonical representation of the reciprocal of its correspond frame rate, in unit of msec.

For consistency, short DRX cycle can be captured in ASN.1 in the same way as how long DRX cycle is captured.

Proposal 5.	Short DRX cycle in rational numbers can be captured in ASN.1 in the same way as specified in Proposal 4 for long DRX cycles.

As to the value range of new DRX cycles in Rel-18, we think RAN2 should consult SA4 to obtain the set of frame rates that need to be supported.

Proposal 6. 	Ask SA4 to provide a set of frame rates that need to be supported in Rel-18.

## SFN wrap-around

In the last two RAN2 meeting (#121bis and #122), the following agreements on SFN wrap-around were agreed:

- To address SFN wrap around, it is proposed to adopt option with a counter in DRX formula that increments at every SFN wrap around and an DRX reference SFN signalled by network. - Not use broadcast signalling for counter and reference SFN in XR.

In the following, we discuss a few open issues on SFN wrap-around.

The first issue is whether the counter (denoted by NSFN) needs a maximum value or not. There have been views at the maximum of the counter does not need to be specified, i.e. it can keep incrementing without wrapping around, because a RRC connection will not last forever. If that is agreed, then it is up to UE/NW implementation to make sure that the storage for the counter is large enough so that it never overflows in any cases.

Such arguments may seem fine on paper. However, if overflow does happen or NW and UE allocate different field lengths for their counters, there can be inter-operability issue. Therefore, we think a maximum for the counter shall be specified.

Observation 2.	If maximum value of the counter is not specified, there can be inter-operability issue between different spec implementations.

Since defining or configuring a maximum for the counter does not create much complexity, we think it is better to have it defined/configured. To ensure that DRX cycles always align with the boundaries of SFN, this maximum should be chosen such that 10240 x M is an integer multiple of any DRX cycles that UE supports (which directly relates to frame rates supported by UE). Considering the uncertainty around new DRX cycle values at the moment (e.g. integer versus rational numbers) and beyond, we think it is safer to have network configure it during UE’s DRX re-/configuration than to predefine it in the spec. That will provide better flexibility and forward compatibility.

Proposal 7. 	The maximum of the counter (NSFN) is RRC configured by network during DRX re-/configuration.

The initial value of the counter may depend on how the DRX reference SFN is used. We see two possible options:

- Option A: the DRX reference SFN is directly added to the DRX formula, in the same fashion as how time reference SFN is used in the Rel-16 configured grant formula;

- Option B: the DRX reference is not directly used in the DRX formula. Instead, UE uses the DRX reference SFN signalled by network to set the initial value of the counter.

In Option A, there is no coupling or interaction between the counter (denoted by NSFN) and DRX reference SFN (denoted by drx-ReferenceSFN):

- The counter NSFN is applied to ensure boundaries of DRX cycles and SFN always are aligned, for all possible DRX cycles. In the MAC spec, NSFN can be added to the left hand side of the DRX formula;

- The DRX reference SFN drx-ReferenceSFN is used to avoid the RRC configuration ambiguity problem. In the MAC spec, drx-ReferenceSFN can be added to the right hand side of the DRX formula.

The enhanced formula for long DRX cycle can be the following:

[(NSFN × 10240) + (SFN × 10) + subframe number] modulo (drx-LongCycle) =

(drx-ReferenceSFN × 10 + drx-StartOffset) modulo (drx-LongCycle)

And similarly for short DRX cycle:

[(NSFN × 10240) + (SFN × 10) + subframe number] modulo (drx-ShortCycle) =

(drx-ReferenceSFN × 10 + drx-StartOffset) modulo (drx-ShortCycle)

In this option, the counter NSFN can be initialized to 0 when DRX is configured.

Option B is motivated by the question whether there is an alternative that has less impact on the legacy DRX formula.

We observe that the main purpose of DRX reference SFN is to add a “shift” to the start offset in the DRX formula if the RRC configuration is sent and received on different sides of a SFN boundary. This “shift” allows both sides of the DRX formula to have a common “start”.

Note that the same effect can be achieved if the initial value of NSFN is set based on DRX reference SFN. More specifically, assume network assigns the values of DRX reference SFN in the same way as in Rel-16, i.e. drx-ReferenceSFN = 0 if RRC configuration is first sent between SFN 0 and 512. Otherwise, network sets drx-ReferenceSFN = 512. Then based on the received value of DRX reference SFN and the SFN in which UE successfully receives RRC configuration message, UE can determine if ambiguity may exist, i.e. if drx-ReferenceSFN = 512 and UE receives RRC configuration in a SFN between 0 and 512. In that case, UE can set the initial value of NSFN to 1. In other cases, ambiguity does not exist. Hence UE can set the initial value of NSFN to 0.

By setting the initial value of NSFN based on the value of DRX reference SFN, a “shift” is applied to the left hand side of the DRX formula when RRC configuration is sent and received across SFN boundary. As a result, DRX reference SFN does not need to be added to the DRX formula anymore. The resulting DRX formulas thus are:

[(NSFN × 10240) + (SFN × 10) + subframe number] modulo (drx-LongCycle) = drx-StartOffset;

[(NSFN × 10240) + (SFN × 10) + subframe number] modulo (drx-ShortCycle) =

(drx-StartOffset) modulo (drx-ShortCycle).

These changes obviously have less impact on the current DRX formula than those in Option A.

Based on the above discussions, we’d propose that

Proposal 8. 	Network sets DRX reference SFN (drx-ReferenceSFN) to either 0 or 512, in the same way as in Rel-16 IIoT.

Proposal 9.  	RAN2 discuss and select one of the following options:

- Option A: both the counter NSFN and the DRX reference SFN drx-ReferenceSFN are added to the DRX formula. NSFN is initialized to 0;

- Option B: only NSFN is added to the DRX formula. However, NSFN is initialized according to drx-ReferenceSFN as follows:

  - If UE successfully receives RRC configuration in SFNUE, UE initializes NSFN to 1 if 0 ≤ SFNUE < 512 and drx-ReferenceSFN = 512;

  - Otherwise, UE initializes NSFN to 0.

# Conclusion

Based on the above analysis, we’d recommend RAN2 to discuss and adopt the following proposals:

Non-integer DRX cycles

Proposal 1. 	New DRX cycles in rational numbers are supported for both short and long DRX cycles.

Proposal 2. 	If short DRX cycle in rational number is configured, the length of the long DRX cycle shall be an integer multiple of the short DRX cycle, as in legacy.

Observation 1. There are different mathematical ways to implement modulo operations on rational numbers without rounding errors. And modern programming languages also can support such operations without rounding errors.

Proposal 3. 	In TS 38.321, capture the following changes for the case where the DRX cycle is in rational numbers:

- add floor operation to the legacy DRX formula;

- in a note, capture the requirement that the modulo operation should be implemented by a method that does not produce rounding errors. The exact method is up to UE implementation.

Proposal 4. 	In TS 38.331, specify a long DRX cycle in rational number as follows:

- Represent it by the corresponding frame rate in pair with its associated start offset, which has the range of 0~ [floor(1000 / frame rate) – 1];

- In its field description, specify the actual value of a long DRX cycle as the canonical representation of the reciprocal of its correspond frame rate, in unit of msec.

Proposal 5.	Short DRX cycle in rational numbers can be captured in ASN.1 in the same way as specified in Proposal 4 for long DRX cycles.

Proposal 6. 	Ask SA4 to provide a set of frame rates that need to be supported in Rel-18.

SFN wrap-around

Observation 2.	If maximum value of the counter is not specified, there can be inter-operability issue between different spec implementations.

Proposal 7. 	The maximum of the counter (NSFN) is RRC configured by network during DRX re-/configuration.

Proposal 8. 	Network sets DRX reference SFN (drx-ReferenceSFN) to either 0 or 512, in the same way as in Rel-16 IIoT.

Proposal 9.  	RAN2 discuss and select one of the following options:

- Option A: both the counter NSFN and the DRX reference SFN drx-ReferenceSFN are added to the DRX formula. NSFN is initialized to 0;

- Option B: only NSFN is added to the DRX formula. However, NSFN is initialized according to drx-ReferenceSFN as follows:

  - If UE successfully receives RRC configuration in SFNUE, UE initializes NSFN to 1 if 0 ≤ SFNUE < 512 and drx-ReferenceSFN = 512;

  - Otherwise, UE initializes NSFN to 0.