3GPP TSG-RAN WG2 Meeting #123	R2-2307078

Toulouse, France, 21~25 August 2023

Agenda item:	7.5.3

Source:	Qualcomm Incorporated

Title:	BSR and delay status reporting for XR

WID/SID:	NR_XR_enh-Core

Document for:	Discussion and Decision

# Introduction

In this paper, we discuss open issues for the new BSR table and enhancements for delay status reporting.

# Discussion

## New BSR table

In the last two RAN2 meeting (#121bis and #122), the following agreements on the new BSR table were made:

- At least linear distribution is used for generating code points in new BSR table(s). FFS whether exponential distribution can be considered too. FFS if piecewise linear distribution is supported. - As working assumption (depending on how we create the new BSR table(s) and the MAC CE format), If more than one new BSR table are introduced, all of them have the same size BS field. FFS on the exact size. - Support one static BSR table with 8 bits BS field for Rel-18 XR (for all cases). - We do not support additional piecewise linear BSR table in Rel-18. Can consider piecewise linearity when discussing how the BSR table values are defined.

We first discuss how to determine the range of the new BSR table.

Since it has been agreed that the number of code points in the table is a constant (256), the parameters required to define the table is its maximum, minimum and distribution function.

Since the objective for the new BSR table is to help network schedule UL grants that better match UE’s buffered data, the range of the new BSR table can be selected so that it matches the expected size range of UE’s UL data bursts.

For XR traffic, the size range of UE’s data burst is determined by the size range of video frames, which can be known beforehand based on the bit rate (denoted by R) and frame rate (denoted by F) used by the application. For example, suppose an XR application applies AVC codec with 4K resolution, which produces a bit rate in the range of 20~150 Mbps [2]. Then at 60 fps, the size range of a data burst = R/8/F = 20/8/60 ~ 150/8/60 = 41~312 KB.

Since quantization error matters more at high end of buffer sizes, the maximum data burst size should be covered by the new BSR table(s). We thus can set it as the maximum buffer size in the new BSR table (denoted by Bmax). It can be derived by the following steps:

- Since size of a data burst = R/8/F, it is easy to see that maximum burst size is associated with maximum bit rate and lowest frame rate. From Table 4.5-1 in [2], which is copied below for reference, we can see that AVC 8K high-quality produces the highest bit rate of 150 Mbps.

Table 1. Expected Video coding standards performance and bitrate target

| # Codec | # Coding performance # (Random-Access) |  | # Targeted bitrate # (Random Access) |
| --- | --- | --- | --- |
|  | Objective | Subjective |  |
| AVC |  |  | 4k: - Statmux: 20-25 Mbps - CBR: 35 - 50 Mbps 8k: - CBR: 80 - 100 Mbps - High quality: 100 - 150 Mbps [33][34][35] |
| HEVC | -40% vs AVC [33][34][35] | -60% vs AVC [33][34][35] | 4k: - Statmux: 10-13 Mbps - CBR: 18-25 Mbps 8k: - CBR: 40-56 Mbps - High quality: 80-90 Mbps [33][34][35] |

- Since the set of common frame rates are 24, 25, 30, 50, 60, 90, 120, 144 and 240, we can derive Bmax to be 150Mbps/8/24 = 781,250 B.

There are two possible approaches for determining the minimum of the new BSR table (denoted by Bmin).

In the first approach, Bmin can be the minimum size of all possible data bursts (denoted as Dmin), if the resulting quantization error at Bmin is no worse than the legacy one. By the same approach used to determine Bmax, one may find Bmin = minimum bit rate / maximum frame rate = 10 Mbps/8/ 240 fps = 5,208 B.

  - If linear distribution for the code points is used, then the step size between the code points is (781,250 – 5,208) / 256 = 3,031 B. The resulting quantization error at Bmin = 3,031/5,208 = 58%, which is way higher than the one with legacy table (6.5%). So linear distribution does not work in this approach.

  - If exponential distribution is used, since Bmax = Bmin x (1+p)^(N-1), where p is the quantization error and N is the number of code points (256), p = (Bmax/Bmin) ^ (1/(N-1)) = (781,250/5,208) ^ (1/255) = 2.0%.

In the second approach, Bmin is derived based on the objective of minimize the average quantization error (denoted by AQE). Without assuming a particular probability distribution on video frame size, this average quantization error can be defined as arithmetic average. Such a minimum exists because of the following reasons:

If Bmin is selected between Dmin and Bmax, then the quantization error = 6.5% for buffer size in the range [Dmin, Bmin] (i.e. legacy BSR table is used) and a lower value (denoted by QEnew) for buffer size in the range [Bmin, Bmax] (i.e. new BSR table is used). The AQE can be calculated as follows:

AQE=6.5% × (Bmin-Dmin)Bmax-Dmin+QEnew x (Bmax-Bmin)Bmax-Dmin

When Bmin moves towards Dmin, the weight for legacy part of quantization error in the average quantization error reduces, but the weight from QEnew increases, due to both higher quantization error and wider range of [Bmin, Bmax]. The opposite happens when Bmin moves towards Bmax. In summary, the two terms in AQE move in opposite directions and offset each other. Therefore, an optimal Bmin exists between Dmin and Bmax, which can be determined through a numerical study.

Figure 1 shows how AQE varies with different values of Bmin (shown in logarithmic scale for better illustration) over the range [Dmin, Bmin], for both linear and exponential distributions. One may see that exponential distribution consistently outperforms linear distribution over the entire range of Bmin. The minimum AQE of 1.4% is achieved at Bmin = 53,054 B.

Comparing the results produced by the above two approaches, we can see that the 2nd approach with exponential distribution is better, in term of lower average quantization error (1.4% vs 2.0%).

A point worth noting about the above analysis is that the bit rates and frame rates we used are independent from each other. For example, we used maximum bit rate (AVC at 8K resolution) and minimum frame rate (24 fps) to derive Bmax. In reality, this assumption may not hold. For example, it is not common for an application to run at a very high resolution at a very flow frame rate. Hence these two parameters should be considered in a pair, when we use them to derive the new BSR table. In addition, some of the rates in the table may not be used in UL-centric application. For example, it is unlikely in the time frame of Rel-18 that 8K resolution will be used for UL-centric XR applications.

Since RAN2 do not have expertise in codec, RAN2 should consult with SA4 to obtain that information.

Proposal 1. 	Use exponential distribution to generate code points in the new BSR table.

Proposal 2. 	Maximum buffer size can be determined based on the ratio between maximum bit rate and minimum frame rate of UL-centric XR applications.

Proposal 3. 	Minimum buffer size is the one which minimizes average quantization error over the size range of data bursts of UL-centric XR applications.

Proposal 4. 	Ask SA4 to provide a list of bit rate and frame rate (in pair) used by UL-centric XR applications. Finalize the entries in the new BSR table after receiving reply LS back from SA4.

The motivation of using a new BSR table is to reduce quantization error in BSR. Therefore, UE should always choose the BSR table which has the lower quantization error. This implies that if an LCG’s buffer size is within the range of the new BSR table, then it should use the new BSR table. Otherwise, it should use the legacy one.

Proposal 5. 	UE uses the new BSR table to report the buffer size of an LCG if it is within the range of the new BSR table. Otherwise, the legacy BSR table is used.

Since UE may use either of the two BSR tables, network needs to know which BSR table UE has used when reporting. Hence UE has to provide an indication for that in the new BSR MAC CE.

Proposal 6. 	The enhanced BSR MAC CE can be extended from the legacy one by introducing a new bitmap that indicates which BSR table each LCG has used to encode its BS field.

## Delay status reporting

We may first discuss which entity should report DSR, e.g. whether it should be per LCG or per LCH. For the same reason why LCG is used for BSR reporting, we think network likely would only group LCHs with compatible priorities (delay requirements) into the same LCG. And per-LCG reporting can allow the possibility that delay status is reported together with buffer status. Hence LCG can be a good choice for the report entity for DSR.

An XR application may generate different types of flows. Some flows have tight delay requirements, while others do not. For those flows that do not have tight delay requirements, then there probably is no need for them to report their delay status, even when such a report is triggered by another flow. Therefore, network can configure which LCG(s) should report its delay status. Not every LCG needs to report their delay status in a DSR.

Proposal 7. 	Network can configure which LCG(s) should report its delay status.

For the level of urgence required by XR traffic (e.g. 15~30 msec), we do not think it is necessary for UE to include a DSR in every of its PUSCH transmission. Instead, network can configure a time threshold for an LCG to trigger DSR, i.e. UE triggers a DSR if the maximum remaining time among data buffered in the layer-2 buffers corresponding to the LCG drops below the configured time threshold.

Proposal 8. 	UE triggers a DSR when an LCG configured for reporting and its associated L2 buffer has data whose remaining time drops below a configured triggering threshold.

Depends on the preferred granularity of reporting, network may configure one or more reporting thresholds. UE then reports the amount of data whose remaining time is below a reporting threshold. For example, suppose network configures two reporting thresholds, T1 and T2, where T1 < T2. Then UE reports the amount of data whose remaining time is shorter than T1 and T2, respectively.

Proposal 9. 	Network can configure one or more reporting thresholds for an LCG. For each reporting threshold, UE reports the amount of data whose remaining time is below that threshold.

DSR may be sent more frequently than BSR. To avoid unnecessary overhead on UL, when UE sends a DSR to network, we think it is more efficient to exclude buffer sizes of LCGs which are not configured to report delay status. In addition, even for an LCG is configured for reporting delay status, if it does not have any data with remaining time is below its configured reporting threshold(s), those LCGs should not be included in a DSR. Buffer sizes of those LCGs can be reported via BSR.

Proposal 10. 	DSR does not include buffer size of LCGs that are not configured for DSR or LCGs that do not have data whose remaining time is below their reporting threshold(s).

It has been agreed that DSR will be sent in a MAC CE. We think this MAC CE should at least include the following elements:

- As we have argued above, not every LCG configured for DSR need to be included in a DSR. Therefore, this MAC CE should include indications on which LCGs are included in this report.

- If an LCG is included in the MAC CE, its buffer size is also included. We think both legacy and the new BSR table may be used to report its buffer size, as there is no good reason to limit DSR to use only the legacy BSR table. Therefore, the MAC CE should include indications on which BSR table is used in the reporting of the buffer size of an LCG.

Proposal 11. 	The MAC CE for sending DSR should include at least the following elements:

- A bitmap that indicates which LCGs are reporting their delay status;

- A bitmap that indicates which BSR table is used in the reporting of an LCG;

- For each LCG included in the report, amount of data whose remaining time is below each of its configured reporting threshold(s).

At the last RAN2 meeting, there as a brief discussion on how intra-UE prioritization may impact how reference time for remaining time may be determined. Indeed, if UE performs autonomous retransmission over CG, network is not able to determine the exact reference time for DSR (i.e. at the first PUSCH transmission). We see two options to resolve this issue:

- Alt 1. When autonomous retransmission is configured and a DSR is sent over a configured grant, this  DSR includes the reference time which is the 5G system time of the first PUSCH transmission which includes this DSR.

- Alt 2. UE recalculates remaining times each time it performs autonomous retransmission, with the reference time set to the time of the latest PUSCH transmission. This option requires UE to rebuild the TB, which is not desirable from UE’s perspective.

Comparing the pros and cons of the two options, one can see that Alt 1 is relatively simpler and thus more preferred.

Proposal 12. 	When autonomous retransmission is configured and a DSR is sent over a configured grant, this DSR includes the reference time which is the 5G system time of the first PUSCH transmission that includes this DSR.

# Conclusion

Based on the above analysis, we’d recommend RAN2 to discuss and adopt the following proposals:

Proposal 1. 	Use exponential distribution to generate code points in the new BSR table.

Proposal 2. 	Maximum buffer size can be determined based on the ratio between maximum bit rate and minimum frame rate of UL-centric XR applications.

Proposal 3. 	Minimum buffer size is the one which minimizes average quantization error over the size range of data bursts of UL-centric XR applications.

Proposal 4. 	Ask SA4 to provide a list of bit rate and frame rate (in pair) used by UL-centric XR applications. Finalize the entries in the new BSR table after receiving reply LS back from SA4.

Proposal 5. 	UE uses the new BSR table to report the buffer size of an LCG if it is within the range of the new BSR table. Otherwise, the legacy BSR table is used.

Proposal 6. 	The enhanced BSR MAC CE can be extended from the legacy one by introducing a new bitmap that indicates which BSR table each LCG has used to encode its BS field.

Delay status report

Proposal 7. 	Network can configure which LCG(s) should report its delay status.

Proposal 8. 	UE triggers a DSR when an LCG configured for reporting and its associated L2 buffer has data whose remaining time drops below a configured triggering threshold.

Proposal 9. 	Network can configure one or more reporting thresholds for an LCG. For each reporting threshold, UE reports the amount of data whose remaining time is below that threshold.

Proposal 10. 	DSR does not include buffer size of LCGs that are not configured for DSR or LCGs that do not have data whose remaining time is below their reporting threshold(s).

Proposal 11. 	The MAC CE for sending DSR should include at least the following elements:

- A bitmap that indicates which LCGs are reporting their delay status;

- A bitmap that indicates which BSR table is used in the reporting of an LCG;

- For each LCG included in the report, amount of data whose remaining time is below each of its configured reporting threshold(s).

Proposal 12. 	When autonomous retransmission is configured and a DSR is sent over a configured grant, this DSR includes the reference time which is the 5G system time of the first PUSCH transmission that includes this DSR.