3GPP TSG-RAN WG2 Meeting #123	R2-2307079

Toulouse, France, 21~25 August 2023

Agenda item:	7.5.4.2

Source:	Qualcomm Incorporated

Title:	PDU discard operation for XR

WID/SID:	NR_XR_enh-Core

Document for:	Discussion and Decision

# Introduction

In this paper, we discuss open issues related to the following aspects of PDU discard:

- Modelling of PDCP discard timer

- PSI-based PDU discard during congestion

- PDU discard when PSIHI is set

- Impact of PDU discard on L2 protocols

# Discussion

## PDCP discard timer

RAN2 have the following agreements regarding PDCP discard timer:

- For UE transmitter, the PDCP discard should be performed per PDU set basis - PDU set discard is modelled using the existing PDCP discard timer for the uplink

And from SA2, PSIHI is defined as: “PSIHI indicates whether all PDUs of the PDU set are needed for the usage of PDU set by application layer.”

Therefore, if PSIHI is set, once there is a loss of PDU (e.g. due to discard), other PDUs in the same PDU set are no longer needed by the application layer. This requirement aligns well the current agreement that PDCP discard is performed per PDU set basis. Its implication on UE behavior is that:

- All PDUs in a PDU set share a common PDCP discard timer (Note: There can be alternative ways to model this, as long as all PDUs in the same PDU set share a common delay/discard deadline)

- The PDCP discard timer for the PDU set is started when the first PDU in the PDU set arrives. When the PDCP discard timer expires, all PDUs in the PDU set are discarded.

If PSIHI is not set, it means that delivery of PDUs in the same PDU set are not coupled. The loss of one PDU should not affect the delivery of other PDUs in the same PDU set, i.e. UE and NW should still make the best effort to deliver other PDUs in the PDU set. Therefore, PDUs in a PDU set do not need to share a common PDCP discard timer. Instead, each PDU should have its own PDCP discard timer, and the timers should be independently managed.

Therefore, we’d propose to update the RAN2 agreement as follows:

Proposal 1. 	PDCP discard is performed per PDU set if PSIHI is set. Otherwise, PDCP discard is performed per PDU as in legacy.

It is worth noting that with this new understanding, PSDB should used as the delay requirement when PSIHI is set. Otherwise, PDB should be used instead.

Next, we discuss how PDCP discard timer should be modelled when PSIHI is set. There can be two possible options:

- Option 1. One PDU set is associated with only one instance of PDCP discard timer. This timer is started when the first PDU in the PDU set arrives.

- Option 2. Each PDU in a PDU set has its own instance of PDCP discard timer. However, the duration of the discard timer can be different for different PDUs, e.g. if PDUs do not arrive at the same time due to jitter. When a PDU arrives at PDCP, its discard timer is set to a duration equal to the configured PDCP discard timer minus the time interval between this PDU and the first PDU in the same PDU Set. In this way, PDCP discard timer of all PDUs in the same PDU set expire at the same time, thus achieving the same effect as having a common instance of PDCP discard timer.

We think both options can work. Option 1 may be simpler to model in the spec, but Option 2 allows a single, consistent model for both cases (whether PSIHI is set or not). And it is also more friendly for UE to implement delay status reporting. Because delay information (e.g. amount of data below a reporting threshold) is per PDU, Option 2 makes it easier for UE to track the remaining time of each PDU. We hence prefer Option 2 more.

Proposal 2.  Each PDU maintains its own instance of PDCP discard timer as follows:

- If PSIHI is set, the duration of discard timer is set to the RRC configured PDCP discard timer for its associated PDU set minus the interval between the arrival times of this PDU and the first PDU in the same PDU Set.

- Otherwise, the duration of discard timer is set to the RRC configured PDCP discard timer for its associated PDU set.

## PSI-based discard

At the last RAN2 meeting (#122), companies discussed PSI based discard and made the following agreements:

- Network indicates UE to apply PSI-based XR discard mechanism via dedicated signalling. - FFS how/whether to minimize additional UL signalling after this indication. - FFS if the NW indication is a one-shot or also subsequent packets

In this section, we discuss the open issues remained from the last meeting on PSI-based discard.  For example, after UE receives network’s indication for PSI-based discard,

- Which set of PSIs this indication is applicable for?

- For an applicable PSI, which PDU sets are subject to discard, e.g. all PDUs with that PSI or only some of them? If it is the latter, what is a suitable criterion?

- How long does the indication stays in effect? One shot or good until further indication from network?

First, we think it is desirable to have some flexibility for network to specify which set of PSIs are subject to PSI-based discard during congestion, e.g. based on severity of the congestion. If congestion is not severe, PDU sets with high importance can be exempted. The selection of PSIs can be signaled via the activation indication.

Proposal 3. 	Network can indicate which set of PSIs are subject to PSI-based discard, e.g. via its activation indication.

If a particular PSI is subject to PSI-based discard, we do not think it is necessary to discard all PDU sets with that PSI. For example, it may not be necessary to discard a PDU which has just arrived at PDCP, because congestion may be cleared before its PDCP discard timer expires. Therefore, we think it is more sensible to discard PDUs based on remaining time when PSI-based discard is activated. Here the term “remaining time” is defined as PDCP discard timer minus the duration that a PDU has been buffered (i.e. in the same way as how it is defined for delay status report). More specifically, network can either configure or dynamically signal a discard threshold for remaining time. When PSI-based discard is activated, UE drops PDUs which meet the two conditions: its PSI is subject to discard and its remaining time is shorter than the discard threshold.

The discard threshold can be PSI specific, e.g. the more important a PSI is, the smaller its threshold is.

The discard threshold can be either configured or signalled by network in the indication that activates PSI-based discard. The latter offers the flexibility that network may choose the threshold based on the severity of congestion.

Proposal 4. 	After network activates PSI-based discard, UE discards PDUs whose PSI is subject to discard and remaining time is shorter than a threshold.

- This discard threshold is PSI specific, e.g. the more important a PSI is, the smaller the threshold is.

- FFS this threshold is configured or dynamically signalled in the indication that activates PSI-based discard.

Once PSI-based discard (congestion) is activated, we think it makes more sense to keep applying it until congestion is cleared than applying it for only one shot, because congestion may not be cleared after only one-shot of discard. As a baseline, we think network can send an indication to deactivate PSI-based discard. Since a de-/activating indication may fail to go through the air link during congestion, it is also desirable to have an event based deactivation trigger. For example, PSI-discard can be deactivated after the associated DRB has no more buffered data.

Proposal 5. 	UE deactivates PSI-based discard when it receives a deactivation indication from network or after the associated DRB has no more buffered data.

Lastly, among different options for signaling the de-/activation indication, e.g. RRC, MAC CE, or PDCP control PDU, we prefer PDCP control PDU, as it is directly communicated to the entity which performs PDU discards.

Proposal 6. 	The de-/activation indication for PSI-based discard is sent in a PDCP control PDU.

  - PSIHI-based discard

Applications may react differently to the loss/discard of PDU sets depend on whether PSIHI is set or not. For example,

- For PDU sets based on “all-or-nothing”, after a PDU is lost/discarded, whether the rest of the PDUs in the same PDU set is delivered to the application does not matter to the application.

- On the other hand, for PDU sets based on AL-FEC, even after application receives enough number of PDUs in a PDU set to complete its decoding, it is still important to deliver the remaining PDUs to the application. That is because codecs using AL-FEC typically adapts its FEC code rate based on measured loss rate. Unnecessary discards can trick the adaptation algorithm into thinking there is an increase in end to end loss rate and thus increase its code rate. That in turn leads to over protection and wasted capacity in delivering those redundant PDUs. Therefore, network/UE should avoid discarding PDUs in application traffic with AL-FEC unless it is absolutely necessary.

Based on the above observation, one can see that it is necessary to have different discard rules for PDU sets depending on whether PSIHI is set or not, especially those which have already been submitted to the lower layer:

- If PSIHI set is set for a PDU set, it is fine to discard all PDUs in that PDU set if one of the PDUs is discarded. That can include PDUs that have already been submitted to the lower layers. To avoid unintended impact of PDU discard on applications (i.e. try not to discard PDUs only when it is necessary, as recommended by SA4), we think this type of discard should be controlled (enabled/disabled) based on network configuration. For example, discard in lower layer should be performed only when there is congestion (PSI-based discard is activated).

- If PSIHI is not set for a PDU set, after PDCP discard timer expires, only PDUs which have not been submitted to the lower layers may be discarded.

Proposal 7. 	A PDU discard results in discard of other PDUs in the same PDU set that have already been submitted to RLC, only if PSIHI is set and congestion has been activated.

## Impact on PDU discard on RLC/MAC

In the previous section, we discussed the scenarios in which PDUs have already submitted to the lower layers may be discarded after DPCP discard timer for the PDU set expires. In this section, we discuss how discard of those PDUs may impact RLC and MAC protocols.

If a RLC transmitter, which can be either UE or RAN, discards a RLC PDU, that will create a gap in the RLC sequence number. This gap can cause stall in the ARQ window operation and unnecessary expiry of RLC timers. Therefore, it is desirable for the transmitter to send an indication to the receiver to inform it of the discarded PDUs. This indication does not need to be sent for every discarded PDU. For instance, it may be subject to some prohibit timer; or it is sent only after a configured number of PDUs have been discarded.

Proposal 8.	When a RLC transmitter discards a RLC PDU, it sends an indication to the receiver. FFS mechanisms to control the frequency of such an indication.

When a PDCP PDU is to be discarded, it is possible that it is multiplexed with other PDUs in the same MAC PDU which do not need to be discarded. In that case, it is unnecessary and wasteful to discard a whole MAC PDU only because one of its MAC SDUs is no longer needed.

Or even if PDCP PDUs associated with all MAC SDUs in a MAC PDU are subject to discard, it is also not desirable to discard the MAC PDU, as long as HARQ transmissions of the MAC PDU have already taken place. Because otherwise, discard of the MAC PDU would cause unnecessary packet loss and waste the radio resources that already have been used for the transmissions.

Proposal 9.	UL MAC PDUs are not subject to discard.

# Conclusion

Based on the above analysis, we’d recommend RAN2 to discuss and adopt the following proposals:

Proposal 1. 	PDCP discard is performed per PDU set if PSIHI is set. Otherwise, PDCP discard is performed per PDU as in legacy.

Proposal 2.  	Each PDU maintains its own instance of PDCP discard timer as follows:

- If PSIHI is set, the duration of discard timer is set to the RRC configured PDCP discard timer for its associated PDU set minus the interval between the arrival times of this PDU and the first PDU in the same PDU Set.

- Otherwise, the duration of discard timer is set to the RRC configured PDCP discard timer for its associated PDU set.

Proposal 3. 	Network can indicate which set of PSIs are subject to PSI-based discard, e.g. via its activation indication for PSI-based discard.

Proposal 4. 	After network activates PSI-based discard, UE discards PDUs whose PSI is subject to discard and remaining time is shorter than a threshold.

- This discard threshold is PSI specific, e.g. the more important a PSI is, the smaller the threshold is.

- FFS this threshold is configured or dynamically signalled in the indication that activates PSI-based discard.

Proposal 5. 	UE deactivates PSI-based discard when it receives a deactivation indication from network or after the associated DRB has no more buffered data.

Proposal 6. 	The de-/activation indication for PSI-based discard is sent in a PDCP control PDU.

Proposal 7. 	A PDU discard results in discard of other PDUs in the same PDU set that have already been submitted to RLC, only if PSIHI is set and congestion has been activated.

Proposal 8.	When a RLC transmitter discards a RLC PDU, it sends an indication to the receiver. FFS mechanisms to control the frequency of such an indication.

Proposal 9.	UL MAC PDUs are not subject to discard.