3GPP TSG-RAN WG2 Meeting #123	R2-2307080

Toulouse, France, 21~25 August 2023

Agenda item:	7.5.4.3

Source:	Qualcomm Incorporated

Title:	Configured grant enhancements for XR

WID/SID:	NR_XR_enh-Core

Document for:	Discussion and Decision

# Introduction

In this contribution we discuss configuration aspects of multi-PUSCH CG and upper-layer enhancements for supporting UTO-UCI for unused CG occasions.

# Discussion

## Multi-PUSCH CG

Up to RAN1#113, RAN1 have finished a good part of the PHY-layer designs for multi-PUSCH CG. Their agreements which have RAN2 impacts include the following:

From RAN1#112: - Multi-PUSCHs CG is supported for Type-1 configured grant. - Multi-PUSCHs CG is supported for Type-2 configured grant. From RAN1#113: From RAN1 perspective, for determination of HARQ process IDs associated to PUSCHs in multi-PUSCHs CG assuming one TB per PUSCH: - The HARQ process ID for the first configured PUSCH in a period is determined based on the legacy CG procedure when cg-RetransmissionTimer is not configured, and applying the following formula, whichever is applicable - HARQ Process ID = [X*floor( (CURRENT_symbol ) / periodicity)] modulo nrofHARQ-Processes - HARQ Process ID = [X*floor((CURRENT_symbol ) / periodicity)] modulo nrofHARQ-Processes + harq-ProcID-Offset2 - X= the number of configured PUSCHs in the CG period - The HARQ process ID of the remaining configured and valid CG PUSCHs in the period is determined by incrementing the HARQ process ID of the preceding PUSCH in the period by one with module operation with nrofHARQ-Processes or module operation with (nrofHARQ-Processes + harq-ProcID-Offset2), whichever applicable. Note: A configured CG PUSCH is invalid if the CG PUSCH is dropped due to collision with DL symbol(s) indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated or SSB. • To determine corresponding slots for CG PUSCHs in a period of a multi-PUSCH CG configuration: o For the first PUSCH in the period, follow the legacy procedures. o For remaining PUSCHs in the period - ForType-1 and Type-2, reuse the corresponding procedures for NR-U by applying the RRC parameters N and M, instead of cg-nrofSlots-r16 and cg-nrofPUSCH-InSlot-r16, respectively.

Since the above agreements are supposed to be captured in RAN2 specs, in principle RAN2 have the responsibility in their designs. However, if companies can agree that they are reasonable designs and do not have obvious faults, then for the sake of saving valuable meeting time, RAN2 can endorse the above agreements as a baseline. If any company has any concerns about them, they can still raise the issue and request a discussion.

Proposal 1. 	Confirm the RAN1 agreement that multi-PUSCH CG is supported for both type-1 and type-2.

Proposal 2. 	Endorse the RAN1 agreement on time-domain configuration of PUSCH transmission occasions as a baseline for RAN2 discussion.

Proposal 3. 	Endorse the RAN1 agreement on HARQ process ID determination for multi-PUSCH CG as a baseline for RAN2 discussion.

However, we believe there is an error in the RAN1 agreement on how to determine HARQ processing ID for the Nth (N>1) CG occasion in a period. More specifically, the RAN1 agreement states that “incrementing the HARQ process ID of the preceding PUSCH in the period by one with module operation with nrofHARQ-Processes or module operation with (nrofHARQ-Processes + harq-ProcID-Offset2), whichever applicable.”

The reason is that when the purpose of HARQ process ID offset 2 is to shift the entire sequence of HARQ process IDs by an offset. It does not change the number of HARQ process IDs in a CG period. Therefore, the module operation shall be based only on nrofHARQ-Processes, instead of the sum nrofHARQ-Processes + harq-ProcID-Offset2.

Proposal 4. 	Correct the error in the RAN1 LS as follows: The modulo operation in the determination of HARQ process ID of the Nth (N>1) CG occasion is with nrofHARQ-Processes, not (nrofHARQ-Processes + harq-ProcID-Offset2).

One issue that has not been discussed in neither RAN1 nor RAN2 so far is possible values of periodicity of multi-PUSCH CG. One of the key motivations for introducing multi-PUSCH CG is that only a single CG configuration is needed to support XR traffic with periodic data bursts. However, it is well known by now that XR traffic has non-integer periodicity. That does not match with the current values of CG periodicity (all of them are integers). As a result, there can be misalignment between data arrivals and PUSCH occasions of a multi-PUSCH CG. Such misalignments can lead to increased UL latency and/or wasted radio resources.

Proposal 5.  	Support non-integer periodicity for multi-PUSCH CG.

Technically, this issue is very similar to the problem of DRX cycle mismatch that was discussed in the last few RAN2 meetings. Since RAN2 have already agreed on a solution for the DRX problem, we think it is desirable to apply the same approach (i.e. introduce new non-integer DRX cycles defined as ratios of rational numbers) to the periodicity of multi-PUSCH CGs. That will help avoid redundant discussions and save valuable meeting times.

Proposal 6.  	Periodicity of multi-PUSCH CGs can include non-integer values defined as ratio of rational numbers, in the same way as how new non-integer DRX cycles are defined.

## Indication of unused CG occasions

Generally speaking, it is MAC and higher layer that determine whether UE has UL data to send or not or any data can be sent over a configured grant or not. We thus think that functionalities to support indication of unused CG occasions should be split between MAC and PHY layers.

More specifically, MAC determines whether there will be UL data to send over an upcoming CG occasion. If none, MAC sends an indication to PHY layer to inform this determination. It is then the responsibility of PHY layer to generate UTO-UCI and multiplex it in a PUSCH transmission.

Proposal 7.	MAC determines whether there will be UL data to send over an upcoming CG occasion and inform PHY of this determination via cross-layer indication.

How UE makes that decision obviously depends on smart UE implementation, as it involves prediction on future traffic arrivals.

Proposal 8.	How MAC determines whether an CG occasion will be used or not is up to UE implementation.

For the same reasons, given the uncertainty in future traffic arrivals, it is difficult for UE to make the determination following some mandatory timeline, e.g. N slots before the corresponding CG occasion.

Proposal 9. 	There is no timeline requirement on when UE needs to determine if a CG occasion is not going to be used.

A scenario related to intra-UE prioritization is that a UTO-UCI may be deprioritized by another UCI with higher priority. In such a case, to avoid misunderstanding between network and UE, absence of UTO-UCI in a CG PUSCH transmission should be interpreted as that UE may use the corresponding CG occasion(s).

Proposal 10.	If UTO-UCI is absent in a CG PUSCH transmission (e.g. due to intra-UE prioritization), network should consider the corresponding CG occasion(s) as “not unused”, unless that occasion(s) has already been indicated as “unused”.

When Rel-16 enhanced UL skipping is configured, if a UCI overlaps with a PUSCH occasion, then UE is required to multiplex the UCI on that PUSCH, even if UE has no data eligible to use that PUSCH (i.e. a dummy TB has to be sent). This behavior is inefficient for UE but saves gNB from blind decode both PUCCH and PUSCH in the same slot.

However, if skipping indication for CG is configured for a UE, then this inefficient can be avoided. More specifically, if UE has a UCI whose PUCCH occasion overlaps with a CG occasion and does not have any UL data eligible for the CG occasion, UE sends UL skipping indication before the CG occasion. Then network knows UE will not transmit anything over PUSCH, and it needs to decode only PUCCH. For UE, it can still transmit the UCI over PUCCH and no PUSCH transmission is required. This behavior eliminates the decoding ambiguity at gNB side and is more power efficient for UE than the Rel-16 behavior.

Proposal 11.  	If UE has a UCI (e.g. HARQ feedback or CSI) overlapping with a CG occasion and does not have any UL data eligible for that occasion, UE can send UTO-UCI before that occasion and transmit the UCI over PUCCH, i.e. UE is not required to transmit over PUSCH in this case.

# Conclusion

Based on the above analysis, we’d recommend RAN2 to discuss and adopt the following proposals:

Proposal 1. 	Confirm the RAN1 agreement that multi-PUSCH CG is supported for both type-1 and type-2.

Proposal 2. 	Endorse the RAN1 agreement on time-domain configuration of PUSCH transmission occasions as a baseline for RAN2 discussion.

Proposal 3. 	Endorse the RAN1 agreement on HARQ process ID determination for multi-PUSCH CG as a baseline for RAN2 discussion.

Proposal 4. 	Correct the error in the RAN1 LS as follows: The modulo operation in the determination of HARQ process ID of the Nth (N>1) CG occasion is with nrofHARQ-Processes, not (nrofHARQ-Processes + harq-ProcID-Offset2).

Proposal 5.  	Support non-integer periodicity for multi-PUSCH CG.

Proposal 6.  	Periodicity of multi-PUSCH CGs can include non-integer values defined as ratio of rational numbers, in the same way as how new non-integer DRX cycles are defined.

Proposal 7.	MAC determines whether there will be UL data to send over an upcoming CG occasion and inform PHY of this determination via cross-layer indication.

Proposal 8.	How MAC determines whether an CG occasion will be used or not is up to UE implementation.

Proposal 9. 	There is no timeline requirement on when UE needs to determine if a CG occasion is not going to be used.

Proposal 10.	If UTO-UCI is absent in a CG PUSCH transmission (e.g. due to intra-UE prioritization), network should consider the corresponding CG occasion(s) as “not unused”, unless that occasion(s) has already been indicated as “unused”.

Proposal 11.  	If UE has a UCI (e.g. HARQ feedback or CSI) overlapping with a CG occasion and does not have any UL data eligible for that occasion, UE can send UTO-UCI before that occasion and transmit the UCI over PUCCH, i.e. UE is not required to transmit over PUSCH in this case.