3GPP TSG-RAN WG2 Meeting #123	                                                   R2-2307084

Toulouse, France, August 21-25, 2023

Agenda item:	7.11.2.1

Source:	TD Tech, Chengdu TD Tech

Title:	Control plane for multicast reception in RRC_INACTIVE state

Document for:	Discussion and Decision

- Introduction

During the RAN2#122 meeting, the following agreements on control plane of multicast reception in RRC_INACTIVE state were made for the 3GPP R18 WI “Enhancements of NR Multicast and Broadcast services”.

The multicast MCCH configuration takes the broadcast MCCH configuration structure (i.e., mcch-Config-r17) as baseline.

To notify the multicast MCCH change, change notification mechanism for Rel-17 broadcast MCCH is the baseline.

Working assumption (to be confirmed by RAN1 via pending reply LS): One bit in the MCCH DCI is used to notify the change of the multicast MCCH. We reuse the bit used for MCCH change indication from Rel-17 MBS broadcast. This does not cover session deactivation which is FFS.

It is not supported to provide the PTM configuration of intra-gNB neighbour cells in the dedicated signalling.

For PTM configuration structure on the multicast MCCH, Rel-17 broadcast PTM configuration structure is taken as baseline.

As a baseline, The PTM configuration in the RRCRelease message with suspendconfig has the same structure as the PTM configuration in multicast MCCH.

FFS how existing MRBs are handled.

Introduce a new indication per tmgi in the group paging which informs Rel-18 UEs having a valid PTM configuration to receive the multicast in RRC_INACTIVE.

MCCH is used for notifying MC session deactivation for multicast reception in RRC_INACTIVE to enable Rel-18 UE to stay in RRC_INACTIVE and stop monitoring corresponding G-RNTI.

This is assumed to have no/minor impact on RAN1/PHY

In the contribution, the following content is discussed for multicast reception in RRC_INACTIVE state with the related proposals suggested.

- PTM configuration structure

- Multicast MCCH configuration and multicast MCCH handling by the UE

- Service continuity and multicast NCL on multicast MCCH

- Session deactivation notification on multicast MCCH

- Frequency prioritization

- PTM configuration structure

Both dynastic scheduling and SPS are supported for multicast sessions in RRC_CONNECTED state. Therefore, we suggest to support dynastic scheduling and SPS for multicast sessions in RRC_INACTIVE state.

When dynastic scheduling is used for a multicast session, the PTM configuration of the multicast session on multicast MCCH needs to include the following information:

- TMGI

- G-RNTI

- DRX mode: support transmission/retransmission of TB over PTM mode

- PDCCH occasion to SSB mapping window

- PDSCH parameters:

  - scrambling ID for PDSCH

  - scrambling ID for PDSCH DMRS

  - PDSCH aggregation factor

- MRB list

- Bit mapping for indicating whether or not a neighbour cell provides the same multicast session in RRC_INACTIVE state: N bits are needed with each bit for a neighbouring cell

- Bit mapping for indicating whether or not the same PTM configuration is applied in a neighbour cell providing the same multicast session in RRC_INACTIVE state: N bits are needed with each bit for a neighbouring cell

When SPS is used for a multicast session, the PTM configuration of the multicast session on multicast MCCH needs to include the following information:

- TMGI

- G-CS-RNTI

- Active SPS resource

  - Basic information of active SPS resource

    - period, radio frame offset and timeslot index: used to define the timeslot position of the active SPS resource in each period

    - MCS table indicator: used to select an MCS table among the MCS tables in TS38.214

    - PDSCH aggregation factor

  - Scheduling information: with DCI format for activating an SPS resource as baseline

- DRX mode: support transmission/retransmission of TB over PTM mode

- PDCCH occasion to SSB mapping window

- PDSCH parameters

  - scrambling ID for PDSCH

  - scrambling ID for PDSCH DMRS

  - PDSCH aggregation factor

- MRB list

- Bit mapping for indicating whether or not a neighbour cell provides the same multicast session in RRC_INACTIVE state

- Bit mapping for indicating whether or not the same SPS resource is activated in a neighbour cell providing the same multicast session

For a multicast session with SPS in RRC_INACTIVE state, if the SPS resources for the multicast session are configured to UE via dedicated signalling where each SPS resource only consists of the basic information of an SPS resource (period, radio frame offset, timeslot index, MCS table indicator and aggregation factor), the PDCCH with CRC scrambled by G-CS-RNTI can be used to activate an SPS resource where the DCI format on the PDCCH provides the index of the activated SPS resource if more than one SPS resources are configured to UE and the scheduling information of the activated SPS resource. At the same time multicast MCCH can be used to send the configuration information of the activated SPS resource periodically where the configuration information of the activated SPS resource includes both the basic information and the scheduling information of the activated SPS resource. Each time the activated SPS resource is updated, multicast MCCH can be used to send the configuration information of the latest activated SPS resource.

For the PDSCH DMRS transmission of a multicast session in RRC_INACTIVE state, there are the following parameters:

- DMRS type

- DMRS additional position

- Max length

For the PDSCH DMRS transmission of a broadcast session, the above parameters use the following fixed values as indicated in the L1 specification:

- DMRS type=type 1

- DMRS additional position=2

- Max length=1

Therefore an LS to RAN1 is needed to ask for the answer to the following question:

Question 1: For the PDSCH DMRS transmission of a multicast session in RRC_INACTIVE state, do the following parameters need to be configured in L3? If needed, RAN2 kindly asks RAN1 to provide the default values of the following parameters.

- DMRS type

- DMRS additional position

- Max length

Based on the above discussion, the following proposals are suggested:

Proposal 1: When dynastic scheduling is used for a multicast session, the PTM configuration of the multicast session on multicast MCCH includes the following information:

- TMGI

- G-RNTI

- DRX mode

- PDCCH occasion to SSB mapping window

- PDSCH parameters

  - scrambling ID for PDSCH

  - scrambling ID for PDSCH DMRS

  - PDSCH aggregation factor

- MRB list

- Bit mapping for indicating whether or not a neighbour cell provides the same multicast session in RRC_INACTIVE state

- Bit mapping for indicating whether or not the same PTM configuration is applied in a neighbour cell

Proposal 2: When SPS is used for a multicast session, the PTM configuration of the multicast session on multicast MCCH includes the following information:

- TMGI

- G-CS-RNTI

- Active SPS resource

  - Basic information of active SPS resource

    - period, radio frame offset and timeslot index: used to define the timeslot position of active SPS resource in each period

    - MCS table indicator: used to select an MCS table among the MCS tables in TS38.214

    - PDSCH aggregation factor

  - Scheduling information: with DCI format for activating an SPS resource as baseline

- DRX mode:

- PDCCH occasion to SSB mapping window

- PDSCH parameters

  - scrambling ID for PDSCH

  - scrambling ID for PDSCH DMRS

  - PDSCH aggregation factor

- MRB list

- Bit mapping for indicating whether or not a neighbour cell provides the same multicast session in RRC_INACTIVE state

- Bit mapping for indicating whether or not the same SPS resource is activated in a neighbour cell providing the same multicast session

Proposal 3: If the SPS resources for a multicast session are configured to UE via dedicated signalling, the PDCCH with CRC scrambled by G-CS-RNTI can be used to activate an SPS resource where the DCI format on the PDCCH provides the index of the activated SPS resource if needed and the scheduling information of the activated SPS resource.

Proposal 4：Multicast MCCH is used to send the configuration information of the activated SPS resource periodically where the configuration information of the activated SPS resource includes both the basic information and the scheduling information of the activated SPS resource.

Proposal 5: Send an LS to RAN1 to ask for the answer to the following question:

Question 1: For the PDSCH DMRS transmission of a multicast session in RRC_INACTIVE state, do the following parameters need to be configured in L3? If needed, RAN2 kindly asks RAN1 to provide the default values of the following parameters.

- DMRS type

- DMRS additional position

- Max length

- Multicast MCCH configuration

Multicast MCCH configuration can be provided by a new SIB denoted by SIB x. SIB x provides almost the same information as SIB 20 for broadcast MCCH. In detail, SIB x provides the following IEs:

- MulticastMCCH-Config-R18: this IE has the same content as IE “MCCH-Config-r17”

- cfr-ConfigMulticastMCCH-MulticastMTCH-R18: this IE has almost the same content as IE “CFR-ConfigMCCH-MTCH-r17”

In detail, IE” cfr-ConfigMulticastMCCH-MulticastMTCH-R18” includes the following information:

- locationAndBandwidthMulticast-r18

  - If the CFR for multicast MCCH and multicast sessions in RRC_INACTIVE state contains the CFR for broadcast sessions, this IE is configured to provide the starting PRB and the number of the PRBs of the CFR

  - Otherwise, this IE is not configured, which means the CFR for broadcast sessions is also used for multicast MCCH and multicast sessions in RRC_INACTIVE state

- pdsch-ConfigMulticastMCCH-r18: this IE has the same content as IE” PDSCH-ConfigBroadcast-r17”

- commonControlResourceSetExt-r18: this IE is used to configure an additional CORESET different from the CORESETs in SIB1 and has the same content as IE” ControlResourceSet”

- searchSpaceMulticastMCCH-r18: this IE is used to configure an additional CSS for multicast MCCH different from the CSSs in SIB1

- searchSpaceMulticastMTCH-r18: this IE is used to configure an additional CSS for multicast sessions in RRC_INACTIVE state different from the CSSs in SIB1

In addition, the CORESET and CSS for multicast MCCH can be selected among the CORESETs and CSSs in SIB1 respectively. The CORESET and CSS for multicast sessions in RRC_INACTIVE state can be selected among the CORESETs and CSSs in SIB1 respectively. Therefore, SIB1 can provide the following IEs:

- searchSpaceMulticastMCCH-r18

- searchSpaceMulticastMTCH-r18

In order for UE to monitor multicast MCCH, a new RNTI with a fixed value can be allocated to multicast MCCH. This new RNTI can be denoted by MMCCH-RNTI. Beam sweeping is used for transmitting multicast MCCH in the multicast MCCH transmission window in each repetition period within each modification period. In detail, the PDCCH monitoring occasions for multicast MCCH which are not overlapping with UL symbols are sequentially numbered from one in the multicast MCCH transmission window. The [x×N+K]th PDCCH monitoring occasion in the multicast MCCH transmission window corresponds to the Kth transmitted SSB, where x = 0, 1, ...X-1, K = 1, 2, …N, N is the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1 and X is equal to CEIL(number of PDCCH monitoring occasions in the multicast MCCH transmission window/N). The actual transmitted SSBs are sequentially numbered from one in ascending order of their SSB indexes. The UE assumes that, in the multicast MCCH transmission window, the PDCCH for multicast MCCH is transmitted in at least one PDCCH monitoring occasion corresponding to each transmitted SSB and thus the selection of SSB for the reception of multicast MCCH is up to UE implementation.

Based on the above discussion, the following proposals are suggested.

Proposal 6: A new SIB denoted by SIB x is used to configure multicast MCCH. The new SIB carries the following IEs:

- MulticastMCCH-Config-R18: this IE has the same content as IE “MCCH-Config-r17”

- cfr-ConfigMulticastMCCH-MulticastMTCH-R18”

  - locationAndBandwidthMulticast-r18

    - If the CFR for multicast MCCH and multicast sessions in RRC_INACTIVE state contains the CFR for broadcast sessions, this IE is configured to provide the starting PRB and the number of the PRBs of the CFR

    - Otherwise, this IE is not configured, which means the CFR for broadcast sessions is also used for multicast MCCH and multicast sessions in RRC_INACTIVE state

  - pdsch-ConfigMulticastMCCH-r18: this IE has the same content as IE” PDSCH-ConfigBroadcast-r17”

  - commonControlResourceSetExt-r18: this IE is used to configure an additional CORESET different from the CORESETs in SIB1 and has the same content as IE” ControlResourceSet”

  - searchSpaceMulticastMCCH-r18: this IE is used to configure an additional CSS for multicast MCCH different from the CSSs in SIB1

  - searchSpaceMulticastMTCH-r18: this IE is used to configure an additional CSS for multicast sessions in RRC_INACTIVE state different from the CSSs in SIB1

Proposal 7: SIB1 can be used to configure the search spaces for multicast MCCH and multicast sessions in RRC_INACTIVE state respectively.

Proposal 8: Multicast MCCH is identified over Uu by a fixed RNTI denoted by MMCCH-RNTI.

Proposal 9: The multicast MCCH transmission window in each repetition period within each modification period of multicast MCCH is used to transmit multicast MCCH. The PDCCH monitoring occasions for multicast MCCH which are not overlapping with UL symbols are sequentially numbered from one in the multicast MCCH transmission window. The [x×N+K]th PDCCH monitoring occasion in the multicast MCCH transmission window corresponds to the Kth transmitted SSB, where x = 0, 1, ...X-1, K = 1, 2, …N, N is the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1 and X is equal to CEIL(number of PDCCH monitoring occasions in the multicast MCCH transmission window/N). The actual transmitted SSBs are sequentially numbered from one in ascending order of their SSB indexes.

Proposal 10: The UE assumes that, in the multicast MCCH transmission window, the PDCCH for multicast MCCH is transmitted in at least one PDCCH monitoring occasion corresponding to each transmitted SSB and thus the selection of SSB for the reception of multicast MCCH is up to UE implementation.

- Service continuity and multicast NCL on multicast MCCH

Based on the agreement, UE can trigger RRC connection resumption if the reception quality of the multicast session received by UE is below a configured threshold. But how to specify the reception quality is FFS.

The BLER of a multicast session can be used to evaluate the reception quality of the multicast session. Each kind of multicast sessions has a fixed QCI and a fixed BLER requirement. If the measured BLER of a multicast session during a configured time interval is higher than the BLER requirement of the multicast session, UE can trigger RRC connection resumption.

Correspondingly the reception quality information for UE to evaluate the reception quality can be sent to UE via dedicated signalling before UE is released to RRC_INACTIVE state. Each time the reception quality information is updated, the updated information can be sent to UE on multicast MCCH as a part of the PTM configuration of the corresponding multicast session.

In order to lower the PRACH collision probability for RRC connection resumption for bad reception quality of a multicast session in RRC_INACTIVE state, the specific PRACH resource can be configured for the multicast session. The specific PRACH resource as a part of PTM configuration of a multicast session can be sent to UE via dedicated signalling and/or multicast MCCH. If the specific PRACH resource is sent to UE, UE can trigger RRC connection resumption with the specific PRACH resource. Otherwise, UE can trigger RRC connection resumption with the normal PRACH resource.

Based on the above discussion, the following proposals are suggested.

Proposal 11: The following reception quality information is configured as a part of the PTM configuration of a multicast session.

      - Expected BLER of corresponding multicast session (optional): if the BLER requirement of a QCI is applied, this item has no need to configure.

      - Duration for reception quality evaluation (optional): indicate to UE a time interval during which the reception quality evaluation can be made for the corresponding multicast session.

Proposal 12: The reception quality information can be sent to UE via dedicated signalling before UE is released to RRC_INACTIVE state.

Proposal13: Each time the reception quality information is updated, the updated reception quality information as a part of the PTM configuration of the corresponding multicast session can be sent to UE on multicast MCCH.

Proposal 14: The specific PRACH resource for a multicast session can be sent to UE via dedicated signalling before UE is released to RRC_INACTIVE state.

Proposal15: Each time the specific PRACH resource is updated, the updated PRACH resource as a part of the PTM configuration of the corresponding multicast session can be sent to UE on multicast MCCH.

In order to reduce service interruption time during UE’s mobility from the source cell to the target cell, the new SIB for configuring multicast MCCH can be area specific just as an existing SIB can be area specific. If the source cell and target cell belong to the same area, UE has no need to read the new SIB in the target cell.

In order to support service continuity for RRC_INACTIVE UEs, a bit mapping of N bits long can be provided as the multicast NCL of the corresponding multicast session, where the n-th bit of the bit mapping is used to indicate whether or not the n-th neighbouring cell provides the corresponding multicast session, n=1,2,…..,N and N is the number of the neighbouring cells.

In order to reduce service interruption time during UE’s mobility from the source cell to the target cell, the PTM configuration of a multicast session can be applied in an application area. Considering the complexity of supporting the area specific PTM configuration, the application area can consist of all or some cells of a same gNB.

If the same PTM configuration is applied for the intra-gNB case, an additional bit mapping of N bits long can be introduced as a part of the multicast NCL to indicate whether or not a neighbour cell providing the corresponding multicast session has the same PTM configuration as the source cell.

In order to reduce service interruption time, for the intra-gNB case, if the PTM configurations of a neighbour cell is different from that of the source cell but available in the source cell, the PTM configuration of the neighbouring cell can be provided as a part of the multicast NCL.

Based on the above discussion, the following proposals are suggested.

Proposal 16: The SIB for configuring multicast MCCH can be area specific.

Proposal 17: For a multicast session in RRC_INACITVE state, a bit mapping of N bits long is introduced on multicast MCCH as the multicast NCL of the corresponding multicast session to indicate whether or not a neighbour cell provides the multicast session as the source cell, where N is the number of the neighbour cells.

Proposal 18: The same PTM configuration of a multicast session in RRC_INACTIVE state can be applied for the intra-gNB case.

Proposal 19: For a multicast session in RRC_INACITVE state, an additional bit mapping of N bits long is introduced as a part of the multicast NCL to indicate whether or not a neighbour cell providing the multicast session has the same PTM configuration as the source cell, where N is the number of the neighbour cells.

Proposal 20: For the intra-gNB case, if the PTM configuration of a neighbour cell is different from that of the source cell but available in the source cell, the PTM configuration of the neighbouring cell can be provided as a part of the multicast NCL.

- Session deactivation notification

Session deactivation notification is sent on multicast MCCH. But the detail of session deactivation notification needs to be decided. In detail, session deactivation notification can be sent as an optional part of the PTM configuration of the corresponding multicast session on multicast MCCH with the following information included:

- 1-bit indicator: used to indicate whether or not the corresponding multicast session is deactivated.

- PTM configuration: used to provide to UE the PTM configuration which can be used by UE when the corresponding multicast session is re-activated.

According to the above discussion, the following proposal is suggested.

Proposal 21: Session deactivation notification can be sent as an optional part of the PTM configuration of the corresponding multicast session on multicast MCCH with the following information included:

- 1-bit indicator: used to indicate the corresponding multicast session is deactivated.

- PTM configuration: used to provide to UE the PTM configuration which can be used by UE when the corresponding multicast session is re-activated.

- Frequency prioritization

For UE interested to receive or receiving a multicast session in RRC_INACTIVE state, frequency prioritization during cell selection/reselection shall be considered with frequency prioritization during cell selection/reselection for a UE interested to receive or receiving a broadcast session as baseline.

In detail, the following proposals are suggested.

Proposal 22: A new SIB denoted by SIB y is introduced to provide the mapping between FSAI and a multicast frequency for multicast sessions in RRC_INACTIVE state.

Proposal 23: The following frequency prioritization rules are supported:

Rule 1:

For the UE capable of receiving multicast sessions in RRC_INACTIVE state, if the UE is receiving or interested to receive a multicast session in RRC_INACTIVE state and can only receive this multicast session by camping on a frequency on which it is provided, the UE may consider that frequency to be of the highest priority during the multicast session as specified in TS 38.300 [2] as long as the two following conditions are fulfilled:

1)	SIB 1 scheduling information of the cell reselected by the UE due to frequency prioritization for multicast session in RRC_INACTIVE state contains SIB x which carries the multicast MCCH configuration;

2)	Either:

-	MBS FSAIs of that frequency are indicated in SIBy of the serving cell and the same MBS FSAIs are also indicated for this multicast session in MBS User Service Description (USD) as specified in TS 26.346 [20], or

-	SIB y is not provided in the serving cell and that frequency is included in the USD of this multicast session, or

-	SIB y is provided in the serving cell but does not provide the frequency mapping for the concerned multicast session, and that frequency is included in the USD of this multicast session.

NOTE 1: It is up to UE implementation which frequency to select, when the USD provides multiple frequencies for the multicast session the UE is receiving or interested in receiving.

Rule 2:

For the UE capable of receiving multicast sessions in RRC_INACTIVE state, if the UE is receiving or interested to receive an multicast session, the UE may consider cell reselection candidate frequencies at which it cannot receive the multicast session to be of the lowest priority during the multicast session as specified in TS 38.300 [2], as long as SIB 1 scheduling information of the cell contains SIB x on the multicast frequency which the UE monitors and as long as the condition 2) above is fulfilled for the serving cell.

NOTE 2:	Example scenarios in which such down-prioritisation may be needed include the cases where camping is not possible for the UE on the multicast frequency (e.g. the multicast frequency belongs to a PLMN different from UE's registered PLMN) while the UE can receive the multicast session when camped on another frequency than the multicast frequency or current frequency.

NOTE 3:	The frequency prioritization for multicast session in RRC_INACTIVE state may override the re-selection priorities for slice-based cell reselection.

- Conclusion

Based on the discussion in the former sections, the following proposals are suggested.

Proposal 1: When dynastic scheduling is used for a multicast session, the PTM configuration of the multicast session on multicast MCCH includes the following information:

- TMGI

- G-RNTI

- DRX mode

- PDCCH occasion to SSB mapping window

- PDSCH parameters

  - scrambling ID for PDSCH

  - scrambling ID for PDSCH DMRS

  - PDSCH aggregation factor

- MRB list

- Bit mapping for indicating whether or not a neighbour cell provides the same multicast session in RRC_INACTIVE state

- Bit mapping for indicating whether or not the same PTM configuration is applied in a neighbour cell

Proposal 2: When SPS is used for a multicast session, the PTM configuration of the multicast session on multicast MCCH includes the following information:

- TMGI

- G-CS-RNTI

- Active SPS resource

  - Basic information of active SPS resource

    - period, radio frame offset and timeslot index: used to define the timeslot position of active SPS resource in each period

    - MCS table indicator: used to select an MCS table among the MCS tables in TS38.214

    - PDSCH aggregation factor

  - Scheduling information: with DCI format for activating an SPS resource as baseline

- DRX mode:

- PDCCH occasion to SSB mapping window

- PDSCH parameters

  - scrambling ID for PDSCH

  - scrambling ID for PDSCH DMRS

  - PDSCH aggregation factor

- MRB list

- Bit mapping for indicating whether or not a neighbour cell provides the same multicast session in RRC_INACTIVE state

- Bit mapping for indicating whether or not the same SPS resource is activated in a neighbour cell providing the same multicast session

Proposal 3: If the SPS resources for a multicast session are configured to UE via dedicated signalling, the PDCCH with CRC scrambled by G-CS-RNTI can be used to activate an SPS resource where the DCI format on the PDCCH provides the index of the activated SPS resource if needed and the scheduling information of the activated SPS resource.

Proposal 4：Multicast MCCH is used to send the configuration information of the activated SPS resource periodically where the configuration information of the activated SPS resource includes both the basic information and the scheduling information of the activated SPS resource.

Proposal 5: Send an LS to RAN1 to ask for the answer to the following question:

Question 1: For the PDSCH DMRS transmission of a multicast session in RRC_INACTIVE state, do the following parameters need to be configured in L3? If needed, RAN2 kindly asks RAN1 to provide the default values of the following parameters.

- DMRS type

- DMRS additional position

- Max length

Proposal 6: A new SIB denoted by SIB x is used to configure multicast MCCH. The new SIB carries the following IEs:

- MulticastMCCH-Config-R18: this IE has the same content as IE “MCCH-Config-r17”

- cfr-ConfigMulticastMCCH-MulticastMTCH-R18”

  - locationAndBandwidthMulticast-r18

    - If the CFR for multicast MCCH and multicast sessions in RRC_INACTIVE state contains the CFR for broadcast sessions, this IE is configured to provide the starting PRB and the number of the PRBs of the CFR

    - Otherwise, this IE is not configured, which means the CFR for broadcast sessions is also used for multicast MCCH and multicast sessions in RRC_INACTIVE state

  - pdsch-ConfigMulticastMCCH-r18: this IE has the same content as IE” PDSCH-ConfigBroadcast-r17”

  - commonControlResourceSetExt-r18: this IE is used to configure an additional CORESET different from the CORESETs in SIB1 and has the same content as IE” ControlResourceSet”

  - searchSpaceMulticastMCCH-r18: this IE is used to configure an additional CSS for multicast MCCH different from the CSSs in SIB1

  - searchSpaceMulticastMTCH-r18: this IE is used to configure an additional CSS for multicast sessions in RRC_INACTIVE state different from the CSSs in SIB1

Proposal 7: SIB1 can be used to configure the search spaces for multicast MCCH and multicast sessions in RRC_INACTIVE state respectively.

Proposal 8: Multicast MCCH is identified over Uu by a fixed RNTI denoted by MMCCH-RNTI.

Proposal 9: The multicast MCCH transmission window in each repetition period within each modification period of multicast MCCH is used to transmit multicast MCCH. The PDCCH monitoring occasions for multicast MCCH which are not overlapping with UL symbols are sequentially numbered from one in the multicast MCCH transmission window. The [x×N+K]th PDCCH monitoring occasion in the multicast MCCH transmission window corresponds to the Kth transmitted SSB, where x = 0, 1, ...X-1, K = 1, 2, …N, N is the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1 and X is equal to CEIL(number of PDCCH monitoring occasions in the multicast MCCH transmission window/N). The actual transmitted SSBs are sequentially numbered from one in ascending order of their SSB indexes.

Proposal 10: The UE assumes that, in the multicast MCCH transmission window, the PDCCH for multicast MCCH is transmitted in at least one PDCCH monitoring occasion corresponding to each transmitted SSB and thus the selection of SSB for the reception of multicast MCCH is up to UE implementation.

Proposal 11: The following reception quality information is configured as a part of the PTM configuration of a multicast session.

- Expected BLER of corresponding multicast session (optional): if the BLER requirement of a QCI is applied, this item has no need to configure.

- Duration for reception quality evaluation (optional): indicate to UE a time interval during which the reception quality evaluation can be made for the corresponding multicast session.

Proposal 12: The reception quality information can be sent to UE via dedicated signalling before UE is released to RRC_INACTIVE state.

Proposal13: Each time the reception quality information is updated, the updated reception quality information as a part of the PTM configuration of the corresponding multicast session can be sent to UE on multicast MCCH.

Proposal 14: The specific PRACH resource for a multicast session can be sent to UE via dedicated signalling before UE is released to RRC_INACTIVE state.

Proposal15: Each time the specific PRACH resource is updated, the updated PRACH resource as a part of the PTM configuration of the corresponding multicast session can be sent to UE on multicast MCCH.

Proposal 16: The SIB for configuring multicast MCCH can be area specific.

Proposal 17: For a multicast session in RRC_INACITVE state, a bit mapping of N bits long is introduced on multicast MCCH as the multicast NCL of the corresponding multicast session to indicate whether or not a neighbour cell provides the multicast session as the source cell, where N is the number of the neighbour cells.

Proposal 18: The same PTM configuration of a multicast session in RRC_INACTIVE state can be applied for the intra-gNB case.

Proposal 19: For a multicast session in RRC_INACITVE state, an additional bit mapping of N bits long is introduced as a part of the multicast NCL to indicate whether or not a neighbour cell providing the multicast session has the same PTM configuration as the source cell, where N is the number of the neighbour cells.

Proposal 20: For the intra-gNB case, if the PTM configuration of a neighbour cell is different from that of the source cell but available in the source cell, the PTM configuration of the neighbouring cell can be provided as a part of the multicast NCL.

Proposal 21: Session deactivation notification can be sent as an optional part of the PTM configuration of the corresponding multicast session on multicast MCCH with the following information included:

- 1-bit indicator: used to indicate the corresponding multicast session is deactivated.

- PTM configuration: used to provide to UE the PTM configuration which can be used by UE when the corresponding multicast session is re-activated.

Proposal 22: A new SIB denoted by SIB y is introduced to provide the mapping between FSAI and a multicast frequency for multicast sessions in RRC_INACTIVE state.

Proposal 23: The following frequency prioritization rules are supported:

Rule 1:

For the UE capable of receiving multicast sessions in RRC_INACTIVE state, if the UE is receiving or interested to receive a multicast session in RRC_INACTIVE state and can only receive this multicast session by camping on a frequency on which it is provided, the UE may consider that frequency to be of the highest priority during the multicast session as specified in TS 38.300 [2] as long as the two following conditions are fulfilled:

1)	SIB 1 scheduling information of the cell reselected by the UE due to frequency prioritization for multicast session in RRC_INACTIVE state contains SIB x which carries the multicast MCCH configuration;

2)	Either:

-	MBS FSAIs of that frequency are indicated in SIBy of the serving cell and the same MBS FSAIs are also indicated for this multicast session in MBS User Service Description (USD) as specified in TS 26.346 [20], or

-	SIB y is not provided in the serving cell and that frequency is included in the USD of this multicast session, or

-	SIB y is provided in the serving cell but does not provide the frequency mapping for the concerned multicast session, and that frequency is included in the USD of this multicast session.

NOTE 1: It is up to UE implementation which frequency to select, when the USD provides multiple frequencies for the multicast session the UE is receiving or interested in receiving.

Rule 2:

For the UE capable of receiving multicast sessions in RRC_INACTIVE state, if the UE is receiving or interested to receive an multicast session, the UE may consider cell reselection candidate frequencies at which it cannot receive the multicast session to be of the lowest priority during the multicast session as specified in TS 38.300 [2], as long as SIB 1 scheduling information of the cell contains SIB x on the multicast frequency which the UE monitors and as long as the condition 2) above is fulfilled for the serving cell.

NOTE 2:	Example scenarios in which such down-prioritisation may be needed include the cases where camping is not possible for the UE on the multicast frequency (e.g. the multicast frequency belongs to a PLMN different from UE's registered PLMN) while the UE can receive the multicast session when camped on another frequency than the multicast frequency or current frequency.

NOTE 3:	The frequency prioritization for multicast session in RRC_INACTIVE state may override the re-selection priorities for slice-based cell reselection.

- Annex 1

–	SIB20

SIB20 contains the information required to acquire the MCCH/MTCH configuration for MBS broadcast.

SIB20 information element

-- ASN1START

-- TAG-SIB20-START

SIB20-r17 ::=	SEQUENCE {

mcch-Config-r17                MCCH-Config-r17,

cfr-ConfigMCCH-MTCH-r17        CFR-ConfigMCCH-MTCH-r17 OPTIONAL,  -- Need S

lateNonCriticalExtension       OCTET STRING            OPTIONAL,

...

}

MCCH-Config-r17 ::= SEQUENCE {

mcch-RepetitionPeriodAndOffset-r17   MCCH-RepetitionPeriodAndOffset-r17,

mcch-WindowStartSlot-r17             INTEGER (0..79),

mcch-WindowDuration-r17              ENUMERATED {sl2, sl4, sl8, sl10, sl20, sl40,sl80, sl160}     OPTIONAL, -- Need S

mcch-ModificationPeriod-r17          ENUMERATED {rf2, rf4, rf8, rf16, rf32, rf64, rf128, rf256,

rf512, rf1024, r2048, rf4096, rf8192, rf16384, rf32768, rf65536}

}

MCCH-RepetitionPeriodAndOffset-r17 ::= CHOICE {

rf1-r17                                INTEGER(0),

rf2-r17                                INTEGER(0..1),

rf4-r17                                INTEGER(0..3),

rf8-r17                                INTEGER(0..7),

rf16-r17                               INTEGER(0..15),

rf32-r17                               INTEGER(0..31),

rf64-r17                               INTEGER(0..63),

rf128-r17                              INTEGER(0..127),

rf256-r17                              INTEGER(0..255)

}

-- TAG-SIB20-STOP

-- ASN1STOP

–	CFR-ConfigMCCH-MTCH

The IE CFR-ConfigMCCH-MTCH is used to configure the common frequency resource used for MCCH and MTCH reception.

CFR-ConfigMCCH-MTCH information element

-- ASN1START

-- TAG-CFR-CONFIGMCCH-MTCH-START

CFR-ConfigMCCH-MTCH-r17 ::= SEQUENCE {

locationAndBandwidthBroadcast-r17          LocationAndBandwidthBroadcast-r17  OPTIONAL,  -- Need S

pdsch-ConfigMCCH-r17                       PDSCH-ConfigBroadcast-r17          OPTIONAL,  -- Need S

commonControlResourceSetExt-r17            ControlResourceSet                 OPTIONAL   -- Cond NotSIB1CommonControlResource

}

LocationAndBandwidthBroadcast-r17 ::= CHOICE {

sameAsSib1ConfiguredLocationAndBW          NULL,

locationAndBandwidth                       INTEGER (0..37949)

}

-- TAG-CFR-CONFIGMCCH-MTCH-STOP

-- ASN1STOP

–	PDSCH-ConfigBroadcast

The IE PDSCH-ConfigBroadcast is used to configure parameters for acquiring the PDSCH for MCCH and MTCH.

PDSCH-ConfigBroadcast information element

-- ASN1START

-- TAG-PDSCH-CONFIGBROADCAST-START

PDSCH-ConfigBroadcast-r17 ::= SEQUENCE {

pdschConfigList-r17                    SEQUENCE (SIZE (1..maxNrofPDSCH-ConfigPTM-r17) ) OF PDSCH-ConfigPTM-r17,

pdsch-TimeDomainAllocationList-r17     PDSCH-TimeDomainResourceAllocationList-r16                          OPTIONAL,   -- Need R

rateMatchPatternToAddModList-r17       SEQUENCE (SIZE (1..maxNrofRateMatchPatterns)) OF RateMatchPattern   OPTIONAL,   -- Need R

lte-CRS-ToMatchAround-r17              RateMatchPatternLTE-CRS                                             OPTIONAL,   -- Need R

mcs-Table-r17                          ENUMERATED {qam256, qam64LowSE}                                     OPTIONAL,   -- Need S

xOverhead-r17                          ENUMERATED {xOh6, xOh12, xOh18}                                     OPTIONAL    -- Need S

}

PDSCH-ConfigPTM-r17 ::= SEQUENCE {

dataScramblingIdentityPDSCH-r17        INTEGER (0..1023)         OPTIONAL,   -- Need S

dmrs-ScramblingID0-r17                 INTEGER (0..65535)        OPTIONAL,   -- Need S

pdsch-AggregationFactor-r17            ENUMERATED {n2, n4, n8}   OPTIONAL    -- Need S

}

-- TAG-PDSCH-CONFIGBROADCAST-STOP

-- ASN1STOP

- References

- RP-221458, Revised WID: Enhancements of NR Multicast and Broadcast Services, RAN Meeting#96-e

- R2-2211271, Analysis of options for sending PTM configuration, RAN2#120, TD Tech, Chengdu TD Tech,

- R2-2206990, A new MCCH transmission method, RAN2#119-e, Chengdu TD Tech, TD Tech