3GPP TSG-RAN WG2 Meeting #123	                                                   R2-2307085

Toulouse, France, August 21-25, 2023

Agenda item:	7.11.2.1

Source:	TD Tech, Chengdu TD Tech

Title:	MCCH change notification for multicast sessions in RRC_INACTIVE state

Document for:	Discussion and Decision

- Introduction

During the RAN2#122 meeting, the following agreements on control plane of multicast reception in RRC_INACTIVE state were made for the 3GPP R18 WI “Enhancements of NR Multicast and Broadcast services”.

The multicast MCCH configuration takes the broadcast MCCH configuration structure (i.e., mcch-Config-r17) as baseline.

To notify the multicast MCCH change, change notification mechanism for Rel-17 broadcast MCCH is the baseline.

Working assumption (to be confirmed by RAN1 via pending reply LS): One bit in the MCCH DCI is used to notify the change of the multicast MCCH. We reuse the bit used for MCCH change indication from Rel-17 MBS broadcast. This does not cover session deactivation which is FFS.

It is not supported to provide the PTM configuration of intra-gNB neighbour cells in the dedicated signalling.

For PTM configuration structure on the multicast MCCH, Rel-17 broadcast PTM configuration structure is taken as baseline.

As a baseline, The PTM configuration in the RRCRelease message with suspendconfig has the same structure as the PTM configuration in multicast MCCH.

FFS how existing MRBs are handled.

Introduce a new indication per tmgi in the group paging which informs Rel-18 UEs having a valid PTM configuration to receive the multicast in RRC_INACTIVE.

MCCH is used for notifying MC session deactivation for multicast reception in RRC_INACTIVE to enable Rel-18 UE to stay in RRC_INACTIVE and stop monitoring corresponding G-RNTI.

This is assumed to have no/minor impact on RAN1/PHY

In the contribution, MCCH change notification for multicast sessions in RRC_INACTIVE state is discussed with the related proposals suggested.

- MCCH change notification

In order to reduce the power consumption in UE for monitoring multicast MCCH, MCCH change notification can be sent under the following cases:

- When a new multicast session is carried on multicast MCCH

- When the PTM configuration of a multicast session in RRC_INACTIVE state is changed

- When the reception quality information of a multicast session in RRC_INACTIVE state is changed

- When the multicast NCL of a multicast session is changed

- When a multicast session in RRC_INACTIVE state is deactivated

MCCH change notification is sent with a MCCH change notification field of 2 bits long on the DCI format of multicast MCCH. The first bit is used for indicating a new multicast session on multicast MCCH while the second bit is used for the other cases for the MCCH change notification to be triggered.

Multicast MCCH can be sent according to the configured modification period Pm, repetition period Pr and multicast MCCH transmission window. In detail, within each repetition period of a modification period, multicast MCCH is sent in the multicast MCCH transmission window. Multicast MCCH carries the PTM configurations of all multicast sessions in RRC_INACTIVE state, which means the PTM configuration of each multicast session is sent with a same frequent degree of Pm/Pr times per modification period.

Different multicast sessions have different QoS requirements, which means different multicast sessions have different delay requirements and different BLER requirements. If all multicast sessions have their PTM configurations sent with a same frequent degree, the modification period and repetition period shall be configured respectively with a smaller value to meet the most sincere QoS requirement among the QoS requirements of all multicast sessions, which mean more radio resource is needed for sending multicast MCCH. For the multicast sessions not with the most sincere QoS requirement, the smaller modification period and smaller repetition period have the PTM configurations of these multicast sessions sent with a frequent degree greater than expected. For the UE joining a multicast session among these multicast sessions, more power consumption than expected is suffered by the UE for monitoring multicast MCCH with a greater frequent degree than expected.

In order to reduce radio resource for sending multicast MCCH and power consumption in UE for monitoring multicast MCCH, different multicast sessions can be sent with different frequent degrees. Therefore M>1 transmission periods T(m), m=1,…..,M can be introduced to transmit different multicast sessions with different frequent degrees, where T(m)=2k(m)Pm, Pm is the modification period, k(m)>=0, m=1,…..,M. Select a transmission period among M transmission periods T(m)， m=1,……,M for each multicast session according to its QoS requirement. For a multicast session with the transmission period of T(k) ms long, the PTM configuration information of the multicast session is only transmitted in the first modification period of each transmission period of T(k) ms long, which means the multicast session has its PTM configuration sent with a frequent degree of Pm/Pr times every T(k)/Pm modification periods. Correspondingly for a multicast session, the transmission period of the multicast session is configured to UE before UE is released to RRC_INACTIVE state. Then UE can monitor multicast MCCH only in the first modification period of each transmission period instead of monitoring multicast MCCH in each modification period. Usually UE only joins one multicast sessions. If UE joins several multicast sessions, gNB can allocate a same G-RNTI/G-CS-RNTI to these multicast sessions. Or gNB allocates different G-RNTI/G-CS-RNTI and a same transmission period to these multicast sessions according to the most sincere QoS requirement among the QoS requirements of these multicast sessions, which means UE only needs to monitor multicast MCCH according to the same transmission period.

Based on the above discussion, the introduction of transmission periods can save radio resource over Uu and reduce power consumption in UE.

An example with three transmission periods is illustrated in Figure1. In Figure 1, the following information is shown:

- The modification period is Pm=80ms and the repetition period is Pr=20ms.

- The services using T(1)=Pm=80ms have their configuration transmitted in each modification period.

- The services using T(2)=2*Pm=160ms have their configuration transmitted in the first modification period within each transmission period of T(2)=160ms long.

- The services using T(3)=4*Pm=320ms have their configuration transmitted in the first modification period within each transmission period of T(3)=320ms long.

- The PTM configuration of some service using T(3)=4*Pm=320ms is changed in the transmission period from 320ms to 640ms, MCCH change notification is sent in the first modification period within the transmission period.

- If the multicast session received by a UE uses T(3)=4*Pm=320ms, T(3) is configured to the UE via dedicated signalling before the UE is released to RRC_INACTIVE state. Then the UE can only monitor multicast MCCH in the first modification period of each transmission period of 320ms long, which means the UE only monitors multicast MCCH in the first 80ms of each time interval of 320ms long and thus 75% power consumption in UE for monitoring multicast MCCH can be saved.

Figure 1: multicast MCCH with three transmission periods

Based on the above discussion, the following proposals are suggested.

Proposal 1: For multicast MCCH, MCCH change notification can be sent under the following cases:

- When a new multicast session is carried on multicast MCCH

- When the PTM configuration of a multicast session in RRC_INACTIVE state is changed

- When the reception quality information of a multicast session in RRC_INACTIVE state is changed

- When the multicast NCL of a multicast session is changed

- When a multicast session in RRC_INACTIVE state is deactivated

Proposal 2: MCCH change notification is sent with a MCCH change notification field of 2 bits long on the DCI format of multicast MCCH. The first bit is used for indicating a new multicast session on multicast MCCH while the second bit is used for the other cases for the MCCH change notification to be triggered.

Proposal 3: Several transmission periods are introduced for multicast MCCH to transmit the PTM configurations of different multicast sessions with different frequent degrees.

Proposal 4: For multicast MCCH, a modification period Pm, a repetition period Pr, M transmission periods T(m), m=1=1,….,M can be introduced. For a multicast session using the k-th transmission period T(k), the PTM configuration information of the multicast session is sent only in the first modification period of each transmission period of T(k) ms long, where Pm=2nPr, T(m)=2k(m)Pm, n>=0  is an integer, k(m)>=0 is an integer, m=1,….,M, and M>=1 is an integer.

- Conclusion

Based on the discussion in the former sections, the following proposals are suggested.

Proposal 1: For multicast MCCH, MCCH change notification can be sent under the following cases:

- When a new multicast session is carried on multicast MCCH

- When the PTM configuration of a multicast session in RRC_INACTIVE state is changed

- When the reception quality information of a multicast session in RRC_INACTIVE state is changed

- When the multicast NCL of a multicast session is changed

- When a multicast session in RRC_INACTIVE state is deactivated

Proposal 2: MCCH change notification is sent with a MCCH change notification field of 2 bits long on the DCI format of multicast MCCH. The first bit is used for indicating a new multicast session on multicast MCCH while the second bit is used for the other cases for the MCCH change notification to be triggered.

Proposal 3: Several transmission periods are introduced for multicast MCCH to transmit the PTM configurations of different multicast sessions with different frequent degrees.

Proposal 4: For multicast MCCH, a modification period Pm, a repetition period Pr, M transmission periods T(m), m=1=1,….,M can be introduced. For a multicast session using the k-th transmission period T(k), the PTM configuration information of the multicast session is sent only in the first modification period of each transmission period of T(k) ms long, where Pm=2nPr, T(m)=2k(m)Pm, n>=0  is an integer, k(m)>=0 is an integer, m=1,….,M, and M>=1 is an integer.

- References

- RP-221458, Revised WID: Enhancements of NR Multicast and Broadcast Services, RAN Meeting#96-e

- R2-2211271, Analysis of options for sending PTM configuration, RAN2#120, TD Tech, Chengdu TD Tech,

- R2-2206990, A new MCCH transmission method, RAN2#119-e, Chengdu TD Tech, TD Tech