3GPP TSG-RAN WG2 #123	R2-2307090

Toulouse, France, August 2023

Agenda Item:	7.15.3

Source:	OPPO

Title:	Discussion on Resource (Re)selection and LCP Enhancement

Document for:	Discussion, Decision

Introduction

This paper is to further discuss resource (re)selection.

Discussion on Resource (Re)selection

Issue-1: Impact on Resource (Re)selection for MCSt

For the resource (re)selection for MCSt, R2 has replied R1 on the LS, where the down-selection of 3 approaches would be up to R1 in the Toulouse meeting.

R2 wait for R1 on the down-selection of the 3 approaches of MCSt-based resource (re)selection.

In 122, the following issue has been discussed

MCSt (FFS whether SL LBT failure triggers resource (re)selection or not)

- Yes (P3:4806/P4-5:5686)

  - Proposal 4: For MCST case, reselect consecutive resources for dropped PSSCH transmission due to an LBT failure indication from L1

  - Proposal 5: For MCST case, the resource (re)selection is triggered when LBT succeed and there has dropped PSSCH transmission due to an LBT failure indication from L1

- No (P3:4793)

  - Proposal 3. When it comes to the MCSt, RAN2 does not pursue the UE behavior that UE triggers resource (re-)selection when the PSSCH transmission was not performed due to the LBT failure.

In the case of MCSt, it is not reasonable to trigger reselection upon LBT failure of a single resource in the MCSt, but also maybe not reasonable to not trigger reselection at all regardless of LBT failure.

Considering the controversial view, it seems OK to leave it to UE implementation.

Rely on UE implementation to decide whether / when to trigger resource reselection upon LBT failure of transmission in case of MCSt.

Issue-2: Impact on Resource (Re)selection for COT

Based on the latest R1 WA

For Type 1 LBT block issue (inter-UE case), the following option 2 and option 1 are supported separately based on UE capability

- Option 2: If transmission in slot(s) before a reserved resource is able to share its initiated COT to the reservation [with high L1 SL priority], UE may prioritize/select resource(s) in the slot(s) for transmission.

  - FFS: details of applying this prioritization, which layer to perform above prioritization behaviour, and if the reserved resource belongs to a MCSt, the COT initiating UE should be able to share the COT to cover the whole MCSt

  - (pre)configuring enabling/disabling option 2 is supported

- Option 1:

  - UE may avoid selection of N consecutive resource(s) before a reserved resource with high L1 SL priority.

    - The value of N can be selected from {0, 1, 2}

    - The selection of the value of N is up to UE implementation

      - FFS: unless (pre-)configured or indicated by UE reserved resource in SCI

  - (pre)configuring enabling/disabling option 1 is supported

I.e., Option-2 is to require UE to prioritize the resource before another reserved resource, while option-1 is to require UE to avoid selecting the M resource before another reserved resource, and thus if both are configured for the same pool, there would be a collision in-between.

Considering it is still WA in R1, and R1 is likely to address this issue, R2 can rely on R1 progress before working on it.

For resource prioritization for COT-sharing during resource (re)selection, R2 relies on R1 progress.

Issue-3: Impact on Resource (Re)selection for LBT

R1 made the following WA

Working assumption

For Type 1 LBT block issue (inter-UE case), the following option 2 and option 1 are supported separately based on UE capability

- Option 2: If transmission in slot(s) before a reserved resource is able to share its initiated COT to the reservation [with high L1 SL priority], UE may prioritize/select resource(s) in the slot(s) for transmission.

  - FFS: details of applying this prioritization, which layer to perform above prioritization behaviour, and if the reserved resource belongs to a MCSt, the COT initiating UE should be able to share the COT to cover the whole MCSt

  - (pre)configuring enabling/disabling option 2 is supported

- Option 1:

  - UE may avoid selection of N consecutive resource(s) before a reserved resource with high L1 SL priority.

    - The value of N can be selected from {0, 1, 2}

    - The selection of the value of N is up to UE implementation

      - FFS: unless (pre-)configured or indicated by UE reserved resource in SCI

  - UE may avoid selection of M consecutive resource(s) after a reserved resource when the transmitting symbols of the reserved resource overlap with LBT of the selected resource.

    - M is determined based on UE implementation (at least including 0)

  - FFS: Which layer to perform above behaviour

  - FFS: any restriction of M

  - (pre)configuring enabling/disabling option 1 is supported

- FFS: Whether the above high priority is determined according to a (pre)configured threshold

- Note: both option1 and option2 are optional UE features

Note that R1 reached this WA but using ‘may’, and leaving the selection of N/M value to UE implementation.

The same spirit can be adopted for intra-UE cases, i.e., to allow sufficient time for the UE to perform type-1 between two reserved resources which belong to different MCSt.

On the other hand, considering the following R1 WA

Working assumption

For the case where a COT initiating UE uses Type 1 channel access procedure to initiate a SL transmission,

- it is supported that the COT initiating UE can transmit transmission(s) within the same channel occupancy that follows a COT responding UE’s SL transmission(s) according to the channel access procedures.

It is possible that the two resources which belong to different MCSt, do not require a type-1 LBT in-between. Yet it is quite hard to predict, during the resource (re)selection phase, whether type-1 LBT is needed, since the responding UE presence is hard to predict, and it is hard to predict which two transmissions would fall into the same COT, given that the CAPC value and LBT check result of each attempt is unknown in advance. E.g, Even if ideally the UE knows the value of CAPC, e.g., if maximum COT length = 6ms, and if the resources are at Xms, (X+5)ms, (X+10)ms, so maybe finally,

- it is <X, X+5> in a COT if the LBT of X succeeds, or

- it is the <X+5, X+10> in a COT, if the LBT of X fails, but the LBT of X+5 succeeds.

So it is hard to optimize the procedure for this reason during the resource (re)selection phase.

R2 makes the WA that UE may avoid selection of N consecutive resource(s) before a reserved resource of its own, if the two belong to different MCSt transmission. Where the selection of N is up to UE implementation from {0,1,2}. Further details are to be clarified after R1 confirmation.

R2 makes the WA that UE may avoid selection of M consecutive resource(s) after a reserved resource of its own, if the two belong to different MCSt transmission. Where the selection of M is up to UE implementation (at least including 0). Further details are to be clarified after R1 confirmation.

Discussion on LCP left issues

Issue-1: When to use enhanced LCP

In 121, it was concluded that

Agreement on SL LCP and COT

1: 	UE can select 1/ either to do a changed-LCP, in order to satisfy the COT requirement, and to do the type-2 LBT (How to do the LCP can be decided after RAN1 agreement) 2/ or to do a legacy-LCP, e.g. using type-1, type-2 LBT. FFS on the need of assistance INFO to initiating UE. FFS on spec impact, e.g., conditions for UE to choose either solution.

In 121bis, it was concluded that

Agreement:

If the resource to be used is within a shared COT, and if PDU not generated before COT arrival, and there is data in buffer satisfying COT requirement, at least enhanced LCP should be allowed. FFS on the condition for UE to use enhanced LCP. FFS on spec impact.

For whether an additional condition is needed on top of the ones agreed on last meeting, e.g., when the initiating UE is of the one with the highest prio LCH, different implementations may have different preferences, based on different evaluations of the fairness importance. Considering that the selection of legacy / enhanced LCP is up to UE implementation, and R2 has already defined some conditions for enhanced LCP selection, there is no need to add more conditions for enhanced LCP selection but can leave it to UE implementation.

R2 does not pursue additional conditions to allow UE to select enhanced LCP besides the agreed ones.

Issue-2: Which layer to decide on LBT / LCP type

Firstly, for LBT type, most part of this issue can be left to UE implementation, considering (even if we limit to PSCCH/PSSCH)

- There could be an implementation that type-1/2 LBT is down-selected upon COT availability, e.g., based on buffer status;

- Or there could be an implementation that both type-1/2 are performed in parallel (e.g., type-1 is initiated by default, and type-2 can be initiated when COT is available), so that data transmission can be done when either succeed.

So there seems no need to restrict UE implementation on the conditions/time point for UE to decide on LBT type.

There are various UE implementations of conditions / time point to decide on LBT type(s).

On the other hand, it is not clear whether there is a need to define which layer for UE to select type-2 LBT.

- For S-SSB and PSFCH: they can use type-2A LBT no matter whether there is COT sharing. This can be determined by PHY since when to perform such transmission is known by PHY.

- For PSCCH/PSSCH: UE may assume type-1 LBT can be used by default and when to start type-1 LBT is up to the implementation. If COT is received and the UE can share the COT, type-2 LBT can be performed.

Whether the UE can use the shared COT is based on some criteria (i.e., destination ID and CAPC value) considering this information is known by MAC, seems MAC layer can make the decision.

Since R1 agreed that

Agreement

The container for carrying the COT sharing information from a COT initiator UE includes at least the SCI.

•	FFS 1st and/or 2nd stage SCI

if it relies on MAC layer to decide, it requires cross-layer indication from PHY to MAC on the COT information.

MAC can decide on the LBT type for PSCCH/PSSCH, based on COT sharing information reported by PHY.

Secondly, for LCP type decisions, obviously, it should be MAC to decide on it, based on COT sharing information reported by PHY.

MAC can decide on LCP type for PDU generation, based on COT sharing information reported by PHY.

PHY informs MAC on the COT sharing information.

Conclusion

We have the following proposals:

Proposal 1	R2 wait for R1 on the down-selection of the 3 approaches of MCSt-based resource (re)selection.

Proposal 2	Rely on UE implementation to decide whether / when to trigger resource reselection upon LBT failure of transmission in case of MCSt.

Proposal 3	For resource prioritization for COT-sharing during resource (re)selection, R2 relies on R1 progress.

Proposal 4	R2 makes the WA that UE may avoid selection of N consecutive resource(s) before a reserved resource of its own, if the two belong to different MCSt transmission. Where the selection of N is up to UE implementation from {0,1,2}. Further details are to be clarified after R1 confirmation.

Proposal 5	R2 makes the WA that UE may avoid selection of M consecutive resource(s) after a reserved resource of its own, if the two belong to different MCSt transmission. Where the selection of M is up to UE implementation (at least including 0). Further details are to be clarified after R1 confirmation.

Proposal 6	R2 does not pursue additional conditions to allow UE to select enhanced LCP besides the agreed ones.

Proposal 7	PHY informs MAC on the COT sharing information.

Reference

[1] 3GPP RP-230077, WID revision: NR sidelink evolution, OPPO.