3GPP TSG-RAN2 Meeting # 123	R2-2309076

# Toulouse, France, August 21 - August 25

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 4175 | rev | - | Current version: | 16.13.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on PUCCH resource field description and SSB transmission initiation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Core |  |  |  |  |  | Date: |  |  | 2023-06-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - In 6.3.5, Currently the field description for sl-N1PUCCH-AN and sl-N1PUCCH-AN-Type2 describes the IE as “HARQ resource for PUCCH”, which is misleading since the two IEs indicates PUCCH resource carrying HARQ A/N feedback. - In 5.8.5.2, for out of coverage case, different from in-coverage case, the clause of 5.8.5.3 is not mentioned, yet clause 5.8.5.3 also includes the case for out-of-coverage, so it is not clear why the misalignement between the two cases is needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In 6.3.5, correct the field description from “HARQ resource for PUCCH” to “PUCCH resource for HARQ feedback” - In 5.8.5.3, for out of coverage case, add the reference to “5.8.5.3” to align with in-coverage case as well. Impact analysis Impacted 5G architecture options: NR SA Impacted functionality PUCCH configuration for NR SL type1 CG, Sync resource selection for out-of-coverage case. Inter-operability: If the network is implemented according to this CR while the UE is not, there is no inter-operability issue because this CR is to correct misleading FD and to add missing clause. If the UE is implemented according to this CR while the network is not, there is no inter-operability issue because this CR is to correct misleading FD and to add missing clause. If one UE is implemented according to this CR while the other UE is not, there is no inter-operability issue because this CR is to correct misleading FD and to add missing clause. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Misleading field description of “HARQ resource for PUCCH” - It is unclear how for UE to select sync resource for out-of-coverage case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.5, 5.8.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Change

#### –	SL-ConfiguredGrantConfig

The IE SL-ConfiguredGrantConfig specifies the configured grant configuration information for NR sidelink communication.

SL-ConfiguredGrantConfig information element

-- ASN1START

-- TAG-SL-CONFIGUREDGRANTCONFIG-START

SL-ConfiguredGrantConfig-r16 ::=           SEQUENCE {

sl-ConfigIndexCG-r16                       SL-ConfigIndexCG-r16,

sl-PeriodCG-r16                            SL-PeriodCG-r16                                                       OPTIONAL, -- Need M

sl-NrOfHARQ-Processes-r16                  INTEGER (1..16)                                                       OPTIONAL, -- Need M

sl-HARQ-ProcID-offset-r16                  INTEGER (0..15)                                                       OPTIONAL, -- Need M

sl-CG-MaxTransNumList-r16                  SL-CG-MaxTransNumList-r16                                             OPTIONAL, -- Need M

rrc-ConfiguredSidelinkGrant-r16            SEQUENCE {

sl-TimeResourceCG-Type1-r16                INTEGER (0..496)                                                  OPTIONAL, -- Need M

sl-StartSubchannelCG-Type1-r16             INTEGER (0..26)                                                   OPTIONAL, -- Need M

sl-FreqResourceCG-Type1-r16                INTEGER (0..6929)                                                 OPTIONAL, -- Need M

sl-TimeOffsetCG-Type1-r16                  INTEGER (0..7999)                                                 OPTIONAL, -- Need R

sl-N1PUCCH-AN-r16                          PUCCH-ResourceId                                                  OPTIONAL, -- Need M

sl-PSFCH-ToPUCCH-CG-Type1-r16              INTEGER (0..15)                                                   OPTIONAL, -- Need M

sl-ResourcePoolID-r16                      SL-ResourcePoolID-r16                                             OPTIONAL, -- Need M

sl-TimeReferenceSFN-Type1-r16              ENUMERATED {sfn512}                                               OPTIONAL  -- Need S

}                                                                                                                OPTIONAL, -- Need M

...,

[[

sl-N1PUCCH-AN-Type2-r16                    PUCCH-ResourceId                                                      OPTIONAL  -- Need M

]]

}

SL-ConfigIndexCG-r16 ::=          INTEGER (0..maxNrofCG-SL-1-r16)

SL-CG-MaxTransNumList-r16 ::=     SEQUENCE (SIZE (1..8)) OF SL-CG-MaxTransNum-r16

SL-CG-MaxTransNum-r16 ::=                  SEQUENCE {

sl-Priority-r16                            INTEGER (1..8),

sl-MaxTransNum-r16                         INTEGER (1..32)

}

SL-PeriodCG-r16 ::=            CHOICE{

sl-PeriodCG1-r16               ENUMERATED {ms100, ms200, ms300, ms400, ms500, ms600, ms700, ms800, ms900, ms1000, spare6,

spare5, spare4, spare3, spare2, spare1},

sl-PeriodCG2-r16               INTEGER (1..99)

}

-- TAG-SL-CONFIGUREDGRANTCONFIG-STOP

-- ASN1STOP

SL-ConfiguredGrantConfig field descriptions

sl-ConfigIndexCG This field indicates the ID to identify configured grant for sidelink.

sl-CG-MaxTransNumList This field indicates the maximum number of times that a TB can be transmitted using the resources provided by the configured grant. sl-Priority corresponds to the logical channel priority.

sl-FreqResourceCG-Type1 Indicates the frequency resource location of sidelink configured grant type 1. An index giving valid combinations of one or two starting sub-channel and length (jointly encoded) as resource indicator value (RIV), as defined in TS 38.214 [19].

sl-HARQ-ProcID-Offset Indicates the offset used in deriving the HARQ process ID for SL configured grant type 1 or SL configured type 2, see TS 38.321 [3], clause 5.8.3.

sl-N1PUCCH-AN This field indicates the PUCCH resource for HARQ feedback for sidelink configured grant type 1. The actual PUCCH-Resource is configured in sl-PUCCH-Config and referred to by its ID.

sl-N1PUCCH-AN-Type2 This field indicates the PUCCH resource for HARQ feedback for PSCCH/PSSCH transmissions without a corresponding PDCCH on sidelink configured grant type 2. The actual PUCCH-Resource is configured in sl-PUCCH-Config and referred to by its ID.

sl-NrOfHARQ-Processes This field indicates the number of HARQ processes configured for a specific configured grant. It applies for both Type 1 and Type 2.

sl-PeriodCG This field indicates the period of sidelink configured grant in the unit of ms.

sl-PSFCH-ToPUCCH-CG-Type1 This field, for configured grant type 1, indicates slot offset between the PSFCH associated with the last PSSCH resource of each period and the PUCCH occasion used for reporting sidelink HARQ.

sl-ResourcePoolID Indicates the resource pool in which the configured sidelink grant Type 1 is applied.

sl-StartSubchannelCG-Type1 This field indicates the starting sub-channel of sidelink configured grant Type 1. An index giving valid sub-channel index.

sl-TimeOffsetCG-Type1 This field indicates the slot offset with respect to logical slot defined by sl-TimeReferenceSFN-Type1, as specified in TS 38.321 [3].

sl-TimeReferenceSFN-Type1 Indicates SFN used for determination of the offset of a resource in time domain. If it is present, the UE uses the 1st logical slot of associated resource pool after the starting time of the closest SFN with the indicated number preceding the reception of the sidelink configured grant configuration Type 1 as reference logical slot, see TS 38.321 [3], clause 5.8.3. If it is not present, the reference SFN is 0.

sl-TimeResourceCG-Type1 This field indicates the time resource location of sidelink configured grant Type 1. An index giving valid combinations of up to two slot positions (jointly encoded) as time resource indicator value (TRIV), as defined in TS 38.212 [17].

Next Change

#### 5.8.5.2	Initiation

A UE capable of NR sidelink communication and SLSS/PSBCH transmission shall, when transmitting NR sidelink communication, and if the conditions for NR sidelink communication operation are met and when the following conditions are met:

1>	if in coverage on the frequency used for NR sidelink communication, as defined in TS 38.304 [20]; and has selected GNSS or the cell as synchronization reference as defined in 5.8.6.3; or

1>	if out of coverage on the frequency used for NR sidelink communication, and the frequency used to transmit NR sidelink communication is included in sl-FreqInfoToAddModList in sl-ConfigDedicatedNR within RRCReconfiguration message or included in sl-FreqInfoList within SIB12; and has selected GNSS or the cell as synchronization reference as defined in 5.8.6.3:

2>	if in RRC_CONNECTED; and if networkControlledSyncTx is configured and set to on; or

2>	if networkControlledSyncTx is not configured; and for the concerned frequency syncTxThreshIC is configured; and the RSRP measurement of the reference cell, selected as defined in 5.8.6.3, for NR sidelink communication transmission is below the value of syncTxThreshIC:

3>	transmit sidelink SSB on the frequency used for NR sidelink communication in accordance with 5.8.5.3 and TS 38.211 [16], including the transmission of SLSS as specified in 5.8.5.3 and transmission of MasterInformationBlockSidelink as specified in 5.8.9.4.3;

1>	else:

2>	for the frequency used for NR sidelink communication, if syncTxThreshOoC is included in SidelinkPreconfigNR; and the UE is not directly synchronized to GNSS, and the UE has no selected SyncRef UE or the PSBCH-RSRP measurement result of the selected SyncRef UE is below the value of syncTxThreshOoC; or

2>	for the frequency used for NR sidelink communication, if the UE selects GNSS as the synchronization reference source:

3>	transmit sidelink SSB on the frequency used for NR sidelink communication in accordance with 5.8.5.3 and TS 38.211 [16] , including the transmission of SLSS as specified in 5.8.5.3 and transmission of MasterInformationBlockSidelink as specified in 5.8.9.4.3;

End of Change