3GPP TSG-RAN WG2 Meeting #123	R2-2307105

Toulouse, France, August 21-25, 2023

Agenda item:	7.6.2.1

Source:	vivo

Title:	Discussion on HARQ Enhancement for IOT NTN

Document for:	Discussion and Decision

Introduction

In the past meetings, RAN2 has discussed the HARQ enhancement for R18 IOT NTN and achieved lots of agreements. In this contribution, we will continue to discuss the remaining issue for this topic.

Discussion

In the RAN2#121bis meeting, there was an FFS left for further discussion as below.

- RAN2 further discuss whether UL transmission using PUR can be configured with HARQ mode B.

In Rel-17 NR NTN, two UL HARQ Modes, i.e. HARQ Mode-A and HARQ Mode-B, were introduced to handle the “HARQ process stalling” issue in the NTN environment caused by the extremely large propagation delay (i.e., up to 540ms). This mechanism was introduced to support different HARQ strategies for UL scheduling and impacted UE behaviors on how to receive the UL scheduling w.r.t to DRX operations. Obviously, HARQ mode A and HARQ mode B apply to the connected state. However, transmission using PUR allows one uplink transmission from RRC_IDLE using a pre-configured uplink resource without performing the random access procedure, which applies to the idle state. Actually, there is not much relationship between HARQ mode A/B and the PUR mechanism.

Observation 1: There is not much relationship between HARQ mode A/B and PUR, considering that the HARQ mode A/B mechanism are connected feature while the PUR mechanism is an idle feature.

Proposal 1: HARQ mode A and mode B are not applicable for PUR transmission.

According to TS 36.321, to avoid unnecessary PDCCH monitoring, the PUR mechanism has been enhanced by delaying the start of the pur-ResponseWindowTimer by UE-eNB RTT. It can be found that UE is not expected to receive the retransmission schedule for the first transmission after the 4 subframes plus UE-eNB RTT. This seems to have the same transmission characteristics as HARQ mode A, i.e., retransmission scheduling is based on the decoding result. Then, we have the issue of whether UL transmission using PUR can be configured with HARQ mode B. In essence, this issue is actually equivalent to discuss if the blind retransmission is supported for transmission using PUR, i.e., whether NW can schedule retransmission for the transmission using PUR without decoding result.

TS 36.321 After transmission using PUR, the MAC entity shall monitor PDCCH identified by PUR-RNTI in the PUR response window using timer pur-ResponseWindowTimer: - if PUR was transmitted in a non-terrestrial network and UE supports delaying the start of the pur-ResponseWindowTimer: - the MAC entity shall start pur-ResponseWindowTimer at the subframe that contains the end of the corresponding PUSCH transmission plus 4 subframes plus UE-eNB RTT. - else: - the MAC entity shall start pur-ResponseWindowTimer at the subframe that contains the end of the corresponding PUSCH transmission plus 4 subframes.

Observation 2: About the issue of whether UL transmission using PUR can be configured with HARQ mode B, it is equivalent to discuss if the blind retransmission is supported for transmission using PUR, i.e., whether NW can schedule retransmission without decode result.

For the first transmission using PUR, as the PUR resource is selected by UE based on the PUR configuration provided via the RRCRelease message, NW has no idea which PUR resource will be used for the first transmission before receiving the transmission. Thus, it is difficult for NW to perform blind retransmission scheduling. This means that schedule retransmission without decoding result is not a resource-efficient way.

A similar issue has been discussed in NR NTN for blind retransmission for initial msg3 transmission. The main difference is that the UL grant for initial msg3 transmission is scheduled by NW, which makes the blind retransmission for initial msg3 transmission feasible at a certain degree. However, considering the additional standard work and the limited benefit, the blind retransmission for initial msg3 transmission has been excluded.

Return back to the issue of the blind transmission for the first transmission using PUR, compared with the blind retransmission for the initial msg3 transmission, it is not technically reasonable, and should not be pursued.

Proposal 2: Blind retransmission for the first transmission using PUR is not supported.

While the pur-ResponseWindowTimer is running, UE will monitor PDDCH for the potential UL grant for retransmission. On the UE side, UE may receive blind retransmission scheduling during the pur-ResponseWindowTimer. On the NW side, for the retransmission using PUR, different from the first transmission using PUR, NW has known that UE performed transmission on specific PUR resource. Then, it is feasible that NW can provide blind retransmission scheduling during the pur-ResponseWindowTimer. In short, based on the current spec, blind retransmission for the retransmission using PUR has been supported, which is up to NW implementation.

Observation 3: Blind retransmission for the retransmission using PUR has been supported in the current spec.

Conclusion

In this contribution, we discussed the HARQ enhancement for R18 IOT NTN. This discussion includes the following observations and proposals.

Observation 1: There is not much relationship between HARQ mode A/B and PUR, considering that the HARQ mode A/B mechanism are connected feature while the PUR mechanism is the idle feature.

Observation 2: About the issue of whether UL transmission using PUR can be configured with HARQ mode B, it is equivalent to discuss if the blind retransmission is supported for transmission using PUR, i.e., whether NW can schedule retransmission without decode result.

Observation 3: Blind retransmission for the retransmission using PUR has been supported in the current spec.

Proposal 1: HARQ mode A and mode B are not applicable for PUR transmission.

Proposal 2: Blind retransmission for the first transmission using PUR is not supported.

Reference

[1] RAN2#121bis chairman note.