3GPP TSG-RAN WG2 Meeting #123	   R2-2307115

Toulouse, France, August 21-25, 2023

Agenda item:		7.21.2

Source:	vivo

Title:	Further Discussion on PRACH Repetition from CP

Document for:		Discussion and Decision

1 Introduction

In the previous RAN2 meeting, the following agreements are achieved for multiple PRACH transmissions [1],

RAN2#122 Agreements: - MSG1 repetition can be applicable to the 4-step CBRA procedure initiated by Msg3-based SI request - FFS for MSG1 repetition can be applicable to the 4-step CBRA procedure initiated by Msg1-based SI request. - RAN2 intends to support CFRA for msg1 repetition for ReconfigurationWithSync case, FFS for other cases. - RAN2 to agree to configure multiple RSRP thresholds for different repetition numbers - The RSRP threshold(s) for triggering Msg1 repetition are configured per-BWP - RAN2 to further discusss fallback from lower number of MSG1 repetition to higher number which is also FFS for now. We need to understand how to signal this and how this impacts MAC procedure.

In this contribution, we would like to further discuss some remaining issues about PRACH repetition from CP perspective, including CFRA configuration and RACH partitioning configuration.

2 Discussion

2.1 CFRA configuration

In the previous meeting, CFRA with Msg1 repetition is supported at least for ReconfigurationWithSync case. Then during the [Post122#801] email discussion, some details on the CFRA configuration were discussed and there came a proposal that NW indicates ONE MSG1 repetition number applicable for CFRA MSG1 repetition by RRC for Reconfiguration with sync.

Basically, this proposal is suitable for most cases considering that UE anyway reports the latest quality of RSs of candidate cells to the network before receiving the ReconfigurationWithSync message. As a result, the target cell may have sufficient knowledge about the radio link quality between the UE and it, configuring only the CFRA with Msg1 repetition resource of a given Msg1 repetition number. However, this proposal may not be applicable to the CHO case considering that the UE measurement report is no longer available and conditional trigger event for execution may not be precisely enough to guarantee RA success with only CFRA with Msg1 repetition resource of a given number. To this end, we think the NW should have the flexibility to allocate one or more than one MSG1 repetition number applicable for CFRA MSG1 repetition by RRC ReconfigurationWithSync case. If NW cares about the resource overhead, it can just configure one available Msg1 repetition resource. It is totally up to NW implementation. At the UE side, if more than one Msg1 repetition resources of different repetition number are configured, the UE can select a resource based on RSRP, similar to the CBRA with Msg1 repetition case. In this sense, no additional complexity will be brought to UE. Based on the above consideration, we propose that

Proposal 1: RAN2 confirms that CFRA with Msg1 repetition is applicable to Conditional Handover case.

Proposal 2: It is up to NW implementation to configure one or more than one CFRA with Msg1 repetition resources of different repetition numbers for ReconfigurationWithSync case.

2.2 RACH partitioning configuration

As the Rel-17 RACH partitioning framework is reused for Msg1 repetition configuration, we think some configuration principles of Rel-17 RACH partitioning are supposed to be inherited for Msg1 repetition. For example, Preamble Group B is applicable to Rel-18 PRACH repetition (i.e. it is up to the network to decide whether and how to configure Rel-18 PRACH repetition with Preamble Group B). That is because there exists a valid use case, e.g. realizing robust transmission in case of RRC resumption with a CCCH message of 64-bits. Meanwhile, it is naturally supported when reusing the Rel-17 partitioning signalling (i.e. no additional spec impact incurred). For another example, a separate configuration for the RSRP threshold for SSB selection during the RA procedure should also be inherited.

Proposal 3: RAN2 confirms that Preamble Group B can be configured for Msg1 repetition.

Proposal 4: RAN2 confirms that a separate RSRP threshold for SSB selection can be configured for an Msg1 repetition type.

Last but not least, in the Rel-17 RACH partitioning framework, it is not supported to configure a separate preambleReceivedTargetPower, preambleTransMax nor powerRampingStep for the shared RO case. But this rule may degrade performance and efficiency for Rel-18 Msg1 repetition. Specifically, it is possible that a Msg1 repetition feature associated with 2 repeitions share the same RO with another Msg1 repetition feature associated with 4 repetitions. In this case, preambleReceivedTargetPower for Msg1 repetition with 2 generally can be different from that for Msg1 repetition with 4 (i.e. the value for 4 is lower than that for 2). This is because the UE currently will only use the same preamble and the UL Tx beam to perform multiple PRACH transmissions. On the network side, the NW may detect the preamble only after receiving all the PRACH transmissions using joint energy detection. So the network may use a lower value of preambleReceivedTargetPower for the Msg1 repetition of a higher number of transmissions (i.e. time diversity gain brought by multiple times of repetition allows lower transmit power, same as LTE IoT CE enhancment), as depicted in Figure 1.

Figure 1: Multiple PRACH transmissions in shared RO

For the powerRampingStep, the value of it is supposed to be higher for the Msg1 repetition feature with a higher number of transmissions. That is a RA attempt with a higher number of PRACH transmissions generally means more times of PRACH failure. It seems a spontaneous logic to use a powerRampingStep of higher value in this case so that higher transmission power can be used in the next RA attempt.

Then for preambleTransMax, considering that all Msg1 repetitions within one attempt are considered as one time of Msg1 transmission, thus, the transmission counter will be incremented by 1 only after N times of Msg1 repetitions, requiring a long time. To some degree, the UE needs a very long period to detect the random access problem. To allow early random access problem detection, it seems beneficial to set the preambleTransMax to a lower value for the case of Msg1 repetition with higher numbers. That is, separately configuring preambleTransMax for an Msg1 repetition type (per partition level) should be allowed.

Proposal 5: In the shared RO case, separate configuration of preambleReceivedTargetPower or powerRampingStep is allowed for an Msg1 repetition type.

Proposal 6: Separate configuration of preambleTransMax is allowed for an Msg1 repetition type

3 Conclusion

In this contribution, we have further discussed the PRACH repetition from CP perspective, including CFRA configuration and RACH partitioning configuration. All proposals are summarized below:

Proposal 1: RAN2 confirms that CFRA with Msg1 repetition is applicable to Conditional Handover case.

Proposal 2: It is up to NW implementation to configure one or more than one CFRA with Msg1 repetition resources of different repetition numbers for ReconfigurationWithSync case.

Proposal 3: RAN2 confirms that Preamble Group B can be configured for Msg1 repetition.

Proposal 4: RAN2 confirms that a separate RSRP threshold for SSB selection can be configured for an Msg1 repetition type.

Proposal 5: In the shared RO case, separate configuration of preambleReceivedTargetPower or powerRampingStep is allowed for an Msg1 repetition type.

Proposal 6: Separate configuration of preambleTransMax is allowed for an Msg1 repetition type

4 References

[1] Draft_R2_122_meeting_report_v2.