3GPP TSG-RAN WG2 Meeting #122	   R2-2307118

Toulouse, France, August 21-25, 2023

Agenda item:		7.18.3

Source:	vivo

Title:	Remaining Issues on MT-SDT from UP

Document for:		Discussion and Decision

1 Introduction

In the last RAN2 meeting [1], agreements related to MT-SDT UP aspect were achieved as follows,

- RAN2#122 Agreements: A separate sdt-RSRP threshold for MT-SDT can be configured, at least in the case where MO-SDT is not configured in the cell. RRC explicitly indicates to MAC whether resume is trigged due to MT-SDT LCH restrictions are checked for DRBs as in MO-SDT (if UL data is available during SDT procedure). Ensure CCCH can be transmitted in CG-SDT when MT-SDT is triggered in stage 3 discussion. Assumption is that if the UE has UL data the UE can still check and trigger MO-SDT (it is up to UE implementation) RA-SDT resources are not used for MT-SDT initiation RACH In case CG-SDT resources cannot be used or are not available for MT-SDT, UE uses non-SDT RACH for RA-based MT-SDT. FFS whether new triggers are defined There is no need to define new Rel-18 CG configurations specific to MT-SDT. When the UE is configured with both MO and MT SDT the radio bearer configuration is common for both. For both MO and MT-SDT, if the next CG-SDT resource is too far, then RACH resource can be selected first. This is checked at the point of initial resource selection (e.g. CG SDT selection). FFS what is too far and how this is configured. Assumption is that we will continue this discussion in SDT session. CONFIRM with main session [CB]

In this contribution, we would like to discuss some potential remaining issues on the MT-SDT in terms of early RO selection and CG-SDT in MT-SDT.

2 Discussion

  - 2.1	Early RO selection

In the previous meeting, it was agreed that if the next CG-SDT resource is too far, then RACH resource can be selected first for both MO and MT-SDT. In our understanding, it means if there will be an early RO that can be selected before the next CG-PUSCH resource (there is at least a valid RO that can be selected during the gap time), then the UE will initiate RA-SDT or legacy RA. And if no valid RO is available during the time gap, the UE should keep on waiting for the CG-PUSCH occasion. Based on this understanding, we have the following proposals for MO-SDT and MT-SDT, respectively,

Proposal 1: During ongoing MO-SDT:

If the time gap is larger than a threshold and at least one valid RO configured by RA-SDT resources can be selected during the time gap, the UE performs RA-SDT;

Elseif the time gap is larger than a threshold and at least one valid RO configured by legacy RA resources can be selected during the time gap, the UE performs RA using legacy RA resources;

Elseif CG-SDT MAC conditions are satisfied, wait for the next available CG-PUSCH occasion;

Else UE performs RA-SDT or RA using legacy RA resources, same as Rel-17.

Proposal 2: During ongoing MT-SDT:

If the time gap is larger than a threshold and at least one valid RO configured by legacy RA resources can be selected during the time gap, the UE performs RA using legacy RA resources；

Elseif CG-SDT MAC conditions are satisfied, wait for the next available CG-PUSCH occasion;

Else UE performs RA using legacy RA resources.

  - 2.2	CG-SDT in MT-SDT

5.27.1 General <the unrelated part is omitted> 2> if CG-SDT is configured on the selected UL carrier, and TA for CG-SDT is valid according to clause 5.27.2 in the first available CG occasion for initial CG-SDT transmission with CCCH message according to clause 5.8.2; and 2> if the time gap between the initiation of the SDT procedure and the next CG-SDT occasion is less than cg-SDT-MaxDurationToNext-CG-Occasion, if configured; and 2> if the SDT procedure is initiated for MO-SDT as in TS 38.331 [5], for each RB having data available for transmission, configuredGrantType1Allowed, if configured, is configured with value true for the corresponding logical channel; and 2> if at least one SSB configured for CG-SDT with SS-RSRP above cg-SDT-RSRP-ThresholdSSB is available: 3> indicate to the upper layers that the conditions for initiating SDT procedure are fulfilled; 3> perform CG-SDT procedure on the selected UL carrier according to clause 5.8.2. 2> else if the SDT procedure is initiated for MO-SDT as specified in TS 38.331 [5], and a set of Random Access resources for RA-SDT is configured and can be selected according to clause 5.1.1b on the selected UL carrier; or 2> if the SDT procedure is initiated for MT-SDT as specified in TS 38.331 [5]: 3> if cg-SDT-TimeAlignmentTimer is running, consider cg-SDT-TimeAlignmentTimer as expired and perform the corresponding actions in clause 5.2; 3> indicate to the upper layers that the conditions for initiating SDT procedure are fulfilled. <the unrelated part is omitted>

According to the running MAC CR for MT-SDT, it seems that the UE cannot use the configured CG-SDT resources if there is no pending UL data (i.e. only MT-SDT is initiated by RRC). This is because the CG-SDT resources can only be used when MO-SDT is initiated by RRC. In our understanding, for the pure MT-SDT case (i.e. there is no pending UL data / signalling), pre-allocated CG-SDT resources should be used for CCCH message transmission to improve access performance (reducing access latency and avoiding contention resolution), as shown in Figure 1. Otherwise, the CG-SDT resources anyway will be wasted. Thus, we have the proposal below,

Proposal 3: UE can use CG-SDT resources for the initial transmission including only CCCH message in MT-SDT procedure.

Figure 1: MT-SDT with CG transmission

3 Conclusion

In this contribution, we have discussed potential remaining issues on the MT-SDT in terms of early RO selection and CG-SDT in MT-SDT. All the proposals are given as follows,

Proposal 1: During ongoing MO-SDT:

If the time gap is larger than a threshold and at least one valid RO configured by RA-SDT resources can be selected during the time gap, the UE performs RA-SDT;

Elseif the time gap is larger than a threshold and at least one valid RO configured by legacy RA resources can be selected during the time gap, the UE performs RA using legacy RA resources;

Elseif CG-SDT MAC conditions are satisfied, wait for the next available CG-PUSCH occasion;

Else UE performs RA-SDT or RA using legacy RA resources, same as Rel-17.

Proposal 2: During ongoing MT-SDT:

If the time gap is larger than a threshold and at least one valid RO configured by legacy RA resources can be selected during the time gap, the UE performs RA using legacy RA resources；

Elseif CG-SDT MAC conditions are satisfied, wait for the next available CG-PUSCH occasion;

Else UE performs RA using legacy RA resources.

Proposal 3: UE can use CG-SDT resources for the initial transmission including only CCCH message in MT-SDT procedure.

4 References

[1] Draft_R2_122_meeting_report_v2.