3GPP TSG-RAN WG2 Meeting #123	R2-2307121

Toulouse, France, 21st – 25th Aug, 2023

Title:		Discussion on LPHAP

Source:	Huawei, HiSilicon

Agenda item:	7.2.4

Document for:	Discussion and Decision

- Introduction

Within the WID RP-223549, the following objectives have been specified for LPHAP:

- Specify enhancements for enabling LPHAP use-case 6 as defined in TS 22.104 including: - Extending eDRX cycle beyond 10.24s in RRC_INACTIVE state towards meeting the battery life requirement for LPHAP [RAN2, RAN3, RAN4] - Positioning-specific enhancement for eDRX cycle beyond 10.24s to be defined as part of Rel-18 WI on expanded and improved NR positioning. - NOTE: Work on this objective should be coordinated with that in Rel-18 WI on eRedCap. Towards this, the feature of extending eDRX cycle beyond 10.24s should be defined as part of Rel-18 WI on eRedCap. - NOTE: Inputs from RAN1 as necessary may be facilitated via LSs - For UL and DL+UL positioning for UEs in RRC_INACTIVE state, specify SRS configuration enhancements based on SRS positioning validity area to avoid frequent RRC connection for SRS (re)configuration [RAN2, RAN1, RAN3]. - SRS for positioning configurations in multiple cells [RAN2, RAN1]. - Note: Details including issues such as interference, timing advance, spatial relation information, pathloss reference and common SRS parameters across multiple cells can be further discussed during normative work. - Pre-configuration of one or multiple SRS for positioning configurations [RAN2, RAN3]. - SRS for positioning activation/request procedure(s) [RAN2, RAN1]. - Specify solutions for DL PRS measurements for a UE in RRC_IDLE state and reporting of the measurements in RRC_CONNECTED state [RAN2]. - Specify solutions for alignment between eDRX and PRS configurations [RAN2]. - Specify corresponding new core requirements, as well as identifying and specifying the impact on the existing RAN4 specification, including RRM measurements and procedures [RAN4].

This paper provides our thinking on the further enhancements on LPHAP.

- SRS Configuration with Validity Area

2.1	SRS Configuration Request

The following has been agreed on the SRS configuration request during RAN2 #121bis [1].

Agreement:

SRS configuration request can be indicated via Msg3/MsgA transmission.  FFS if the request is in the RRC message or an accompanying MAC CE.

A remaining problem is whether the SRS configuration request is in the RRC message or an accompanying MAC CE. For the SRS configuration request, it needs to be discussed what needs to be included in the RRC message or MAC CE. For the UE’s operation in RRC_INACTIVE, in order for the network to be able to identify the UE and authenticate the UE, clearly the I-RNTI and the resumeMAC-I need to be carried. Hence, we can reuse the legacy RRCResumeRequest message for the SRS configuration request, and a new resume cause value is needed. However, it is not clear what needs to be carried by the accompanying MAC CE. The network knows the cell where the UE performs SRS configuration update and the SRS configuration is configured by the network. Hence, there is no need to carry cell id/SRS configuration in the MAC CE. In addition, an accompany MAC CE will cause cell coverage issue for the msg3/msgA.

Proposal1: SRS configuration request is performed by the UE sending the legacy RRCResumeRequest message with a new cause value when the UE reselects out of a validity area in RRC_INACTIVE. An accompanying MAC CE is not needed.

2.2	Release of SRS Configurations

RAN1 has made the agreement to introduce area-specific TA timer for validity area [2]:

Agreement From RAN1 perspective, it is feasible to configure SRS positioning validity area-specific TA timer (e.g., with larger values) for a UE in RRC_INACTIVE state. Details can be up to RAN2. - For TA validation, use of area-specific RSRP change threshold is feasible - FFS: which RS is the reference RS for the RSRP change threshold

In terms of the mechanisms of the timer, the following agreements were made by RAN2 [3]:

Agreements:

Define an SRS for positioning validity-area specific TA timer (e.g., with larger values) for a UE in RRC_INACTIVE state.

- The UE starts/restarts the area-specific TA timer when it receives the TA command.

- The UE stops the SRS transmission when the area-specific TA timer expires.

- The UE stops the area-specific TA timer when it reselects to a cell out of the SRS validity area.

- Other stop/restart conditions can be discussed.

In Rel-17, If the TA timer expires, the MAC entity notifies the RRC to release SRS configurations for positioning in RRC_INACTIVE. However, it is still unclear when the UE should release the SRS configurations for LPHAP. In Rel-18, the study on preconfigured SRS is included in the WID. From our understanding, SRS configurations can be preconfigured to the UE in advance, e.g. before a positioning session. Then the UE can request for activation of SRS transmission by sending RRC message upon event report in the configured validity area. In this case, the SRS configuration with area validity should not be released when the TA timer expires. But when there is no pre-configured SRS configured, the UE behaviour should be the same as R17 that it releases the SRS configuration.

Proposal2: SRS configuration with area validity is released when the area-specific TAT expires and SRS pre-configuration is not configured.

In addition, in Rel-17, SRS configurations are released if the UE performs cell reselection:

1> else if cell reselection occurs when srs-PosRRC-Inactive is configured: 2> indicate to the lower layer to stop inactivePosSRS-TimeAlignmentTimer; 2> release the srs-PosRRC-Inactive.

Similarly, if the UE reselects to a cell out of the validity area, and SRS pre-configuration is not configured, the UE should release the SRS configuration same as R17 UE behaviour. But if pre-configured SRS is configured, the UE should not release it when cell reselection happens. There is possibility that the UE reselects back to the cell belonging to the validity area and then, the SRS configuration can be used.

Proposal3: SRS configuration with validity area is released when it reselects to a cell out of the validity area and SRS pre-configuration is not configured.

2.3	SRS Configuration Procedure

The following agreements have been made for SRS configuration procedures:

Agreements:

RRCRelease can be used to provide SRS configuration with validity area for use in RRC_INACTIVE.

Agreements:

RAN2 consider that the LMF should determine the area-specific SRS configuration. Details are up to RAN3.

To enable this mechanism of positioning SRS validity area, an issue to discuss is how to configure the UE with validity area. Since the configurations are configured for multiple cells, the coordination between gNBs and/or between the gNB and the LMF may be required. A potential solution is that the LMF firstly collects the candidate SRS configurations for LPHAP from the gNBs. Then the LMF can provide the validity area and available SRS configurations to the gNBs based on the collected information. To minimize spec. impacts, TRP Information Exchange procedure can be reused.

Figure 2, Generation of SRS configuration by TRP information exchange

Proposal4: Reuse the legacy TRP Information Exchange procedure for coordination of SRS configurations between gNBs and LMF.

- Preconfigured SRS

3.1	What is Pre-configuration?

According to the WID, the pre-configuration of one or multiple SRS for positioning configurations should be discussed. From our understanding, if a UE is preconfigured with SRS, the UE can send the activation request to the receiving gNB when an event is detected, and the receiving gNB can directly activate the SRS by only updating the TA information without LMF involvement, which reduces latency and saves signaling overhead.

Observation1: Pre-configured SRS with validity area means that the UE is configured with SRS before the positioning event is triggered.

During the email discussion, it is noticed that most companies think the pre-configured SRS should be UE-specific, i.e. one or multiple SRS configurations are delivered to the UE via dedicated signaling. An issue to address is the pervious possible mis-understanding on the “multiple SRS configuration”. We think that for a certain cell, it is not necessary to define multiple validity areas which contain the same cell. The correspondence between cell and validity area is important for determining the SRS configuration associated to the validity area. While if we configure multiple validity area corresponding to a single cell, the UE needs to determine which validity area to adopt among the multiple of them, which is not necessary. The correspondence between the cell and validity area can be determined by the LMG un-ambiguously.

Proposal5: If multiple SRS configurations can be pre-configured to UE, one cell cannot be configured in multiple SRS configurations with validity area.

For preconfigured SRS, UE may stay within an area for a long time, and the UE would never release the SRS configurations. In this case, NW-initiated release of SRS can be supported. For example, the LMF sends deactivation message to the gNB, and then the gNB releases the UE with RRCRelease message.

Proposal6: Support NW-initiated release for preconfigured-SRS.

3.2 SRS Activation Request for pre-configured SRS

During the last RAN2 meeting, the following agreement was made for preconfigured SRS [3]:

Agreement: RAN2 will introduce an activation indication and/or request for preconfigured SRS using at least Msg3/MsgA; FFS if Msg1 would be supported also. FFS RRC signalling or MAC CE for the Msg3/MsgA case, as for the configuration request. This agreement does not imply that the UE will be allowed to transmit autonomously.

To perform the activation, the gNB needs to know the UE identity for activation, therefore Msg 1 is not a reasonable solution that the UE has to send msg3. Only after contention resolution in msg4, the UE can begin to transmit SRS. Similar with SRS configuration request, SRS activation request can also be supported by the legacy RRCResumeRequest message with a new cause value. However, instead of introducing two separate new cause values, we think only a single resume caused is needed. We prefer a unified design for SRS configuration and activation. The issue is that the gNB should distinguish them when receiving the request, which needs the involvement of RAN3. Therefore, it is also proposed to send LS to RAN3.

Proposal7: Support a unified design for SRS Configuration Request and SRS Activation Request for pre-configured SRS when positioning event is triggered, i.e., same resume cause for the RRCResumeRequest message. Send LS to RAN3.

4.	Overall Stage-2 Procedure for LPHAP

In summary, the following gives a detailed procedure for the SRS configuration with positioning validity area. It is proposed to adopt the procedure as the baseline for the discussion.

Proposal8: Adopt the following procedure as the baseline for the discussion on SRS configuration with validity area and SRS pre-configuration.

Figure 3, SRS configuration request for UL positioning with validity area 1. LMF collects the candidate SRS configurations of the TRPs via TRP INFORMATION REQUEST message. 2. The gNBs sends the response with available SRS configurations via TRP INFORMATION RESPONSE message. 3-4. The LMF sends POSITIONING INFORMATION REQUEST message to request the serving gNB to configure the UE with SRS configurations and validity area. The LMF can suggest SRS configurations and the associated validity area to the serving gNB, then the serving gNB determines the configurations and sends to the LMF via POSITIONING INFORMATION RESPONSE message. 5. The gNB provides SRS configuration to the UE via RRCRelease message and it sends the UE to RRC_INACTIVE. 6. Event is triggered within the UE that the UE needs to initiate the positioning procedure with the network; or when cell reselection happens, the UE selects one cell out of the current positioning validity area. The UE needs to request the SRS configuration to the network; 7. The UE sends RRC Resume Request message to the newly selected gNB; 8. The network determines SRS configuration and the associated validity area for the UE, the detailed procedures pending to RAN3. 9. The serving gNB sends RRCRelease message to the UE, configuring the UE with new SRS and validity area configuration; Or the serving gNB activates the SRS transmission preconfigured in the UE. 10. LMF sends NRPPa message MEASUREMENT REQUEST to the gNBs where SRS receptions are expected; 11. UE performs SRS transmission according to the SRS configuration that the UE receives and the gNBs perform SRS reception; 12. The gNBs measure the SRS and sends measurement report to the LMF.

5.	UE capabilities for LPHAP

In the WID of the LPHAP, SRS with validity area and preconfigured SRS have been listed as two independent objectives. This means that one of them can work on its own or the two features can work together. The configuration of SRS with validity area for positioning in RRC_INACTIVE state is subject to UE capability, which should be exposed to both the serving gNB and the LMF. To support SRS with validity area, the UE capability should include the support of area-specific TA timer, area-specific RSRP change threshold and the capability of sending SRS configuration request from RAN2 perspective. In addition, the UE should also send the capability of supporting the pre-configuration to the serving gNB and the LMF if pre-configuration of SRS validity area for positioning is expected.

Proposal9. Introduce UE capability of whether to support SRS with validity area in RRC_INACTIVE to LPP and RRC.

Proposal10 Introduce UE capability of whether to support SRS pre-configuration in RRC_INACTIVE to LPP and RRC.

- Alignment between (e)DRX and PRS

6.1	PRS Alignment with fixed DRX

To summarize, there are three approaches for the DRX configuration in RRC_IDLE/INACTIVE to be delivered to the UE:

- PCCH-Config in SIB1 for both RRC_IDLE and RRC_INACTIVE

- PCCH-Config in RRCRelease message for RRC_INACTIVE when the UE enters RRC_INACTIVE

- NAS message for both RRC_IDLE and RRC_INACTIVE received when the UE is in RRC_CONNECTED

We can either choose to align PRS with fixed DRX or align DRX with fixed PRS. From our perspective, the latter one is not appropriate because there are cell-specific DRX parameters which should not be adjusted due to LPHAP UEs. Also, the alignment of DRX to PRS may impact other services of the UE.

Proposal11: Support PRS alignment with fixed DRX and deprioritize DRX alignment with fixed PRS.

When multiple DRX cycles are configured with the options above, the UE should choose the shortest cycle to monitor paging such that the UE does not miss any paging occasion, either from the gNB or from the AMF. Based on the discussion during last meeting, there are two solutions for the alignment of PRS with DRX configurations:

- Option 1: Reuse UE-initiated on-demand DL-PRS procedures

- Option 2: Support LMF-initiated mechanism for the alignment

In this section, we will discuss both solutions.

6.2	UE-initiated alignment between PRS and DRX

For the case of aligning PRS with fixed DRX, UE-based on-demand PRS procedure introduced in Rel-17 can be reused to change the PRS configuration, e.g., periodicity and offset. The PRS alignment with fixed DRX can be discussed in the following two cases for UE in RRC_INACTIVE and RRC_IDLE.

6.2.1	Alignment of PRS and DRX in RRC_IDLE

For DRX configuration in RRC_IDLE, since the UE’s configuration is by NAS and SIB1, the UE knows the DRX configuration when it is in RRC_CONNECTED. Since for positioning in RRC_IDLE, the UE should have already obtained the PRS configuration in RRC_CONNECTED, it can know whether the PRS needs any adjustment. While, to realize the adjustment, the legacy UE-based on-demand PRS request can be performed. For this, no stage3 change is required.

Obeservation2: For the PRS alignment with fixed DRX for UE in RRC_IDLE, the legacy UE-based on-demand PRS procedure can be sent in RRC_CONNECTED with no stage-3 impacts.

6.2.2	Alignment of PRS and DRX in RRC_INACTIVE

If the UE obtains the DRX configuration in RRC_CONNECTED, the PRS alignment with fixed DRX can be achieved by the UE sending LPP message for on-demand PRS request to the LMF. If the UE obtains the DRX configuration only when the UE is in RRC_INACTIVE, the UE can also send LPP message to the LMF for UE-based on-demand PRS request with SDT and SRB2 configured for SDT when the deferred MT-LR procedure is on-going. With the discussion above, it can be seen that if we perform on-demand PRS request to change the PRS configuration with DRX fixed, the legacy mechanism for on-demand PRS request can be reused with no stage3 impacts.

Observation3: For the PRS alignment with fixed DRX for UE in RRC_INACTIVE, the legacy UE-based on-demand PRS procedure can be sent in RRC_CONNECTED or RRC_INACTIVE by SDT with no stage-3 impacts.

Proposal12: For the DL-PRS alignment with fixed (e)DRX, UE-initiated on-demand DL-PRS procedures are used.

6.3	LMF-initiated alignment between PRS and DRX

To supported LMF-initiated mechanism, the LMF should firstly obtain the DRX information of the UE from the gNB and/or the AMF. The DRX information may include the UE ID, the paging cycles and the cell-specific DRX parameters of the serving cell, etc. It should be noticed that the LMF does not know when the UE performs cell-reselection, therefore it may not be able to properly align PRS with DRX considering UE mobility.

In addition, LMF also does not know the RRC state of the UE. For example, if UE transmit from the RRC Connected state to RRC Inactive state, it is more reasonable that the UE itself to request for the alignment.

Proposal13: LMF-initiated alignment between PRS and (e)DRX is not supported.

- Summary

SRS configuration with validity area

Proposal1: SRS configuration request is performed by the UE sending the legacy RRCResumeRequest message with a new cause value when the UE reselects out of a validity area in RRC_INACTIVE. An accompanying MAC CE is not needed.

Proposal2: SRS configuration with area validity is released when the area-specific TAT expires and SRS pre-configuration is not configured.

Proposal3: SRS configuration with validity area is released when it reselects to a cell out of the validity area and SRS pre-configuration is not configured.

Proposal4: Reuse the legacy TRP Information Exchange procedure for coordination of SRS configurations between gNBs and LMF.

Pre-configured SRS

Proposal5: If multiple SRS configurations can be pre-configured to UE, one cell cannot be configured in multiple SRS configurations with validity area.

Proposal6: Support NW-initiated release for preconfigured-SRS.

Proposal7: Support a unified design for SRS Configuration Request and SRS Activation Request for pre-configured SRS when positioning event is triggered, i.e., same resume cause for the RRCResumeRequest message. Send LS to RAN3.

Overall stage2 procedure for LPHAP

Proposal8: Adopt the following procedure as the baseline for the discussion on SRS configuration with validity area and SRS pre-configuration.

Figure 3, SRS configuration request for UL positioning with validity area 1. LMF collects the candidate SRS configurations of the TRPs via TRP INFORMATION REQUEST message. 2. The gNBs sends the response with available SRS configurations via TRP INFORMATION RESPONSE message. 3-4. The LMF sends POSITIONING INFORMATION REQUEST message to request the serving gNB to configure the UE with SRS configurations and validity area. The LMF can suggest SRS configurations and the associated validity area to the serving gNB, then the serving gNB determines the configurations and sends to the LMF via POSITIONING INFORMATION RESPONSE message. 5. The gNB provides SRS configuration to the UE via RRCRelease message and it sends the UE to RRC_INACTIVE. 6. Event is triggered within the UE that the UE needs to initiate the positioning procedure with the network; or when cell reselection happens, the UE selects one cell out of the current positioning validity area. The UE needs to request the SRS configuration to the network; 7. The UE sends RRC Resume Request message to the newly selected gNB; 8. The network determines SRS configuration and the associated validity area for the UE, the detailed procedures pending to RAN3. 9. The serving gNB sends RRCRelease message to the UE, configuring the UE with new SRS and validity area configuration; Or the serving gNB activates the SRS transmission preconfigured in the UE. 10. LMF sends NRPPa message MEASUREMENT REQUEST to the gNBs where SRS receptions are expected; 11. UE performs SRS transmission according to the SRS configuration that the UE receives and the gNBs perform SRS reception; 12. The gNBs measure the SRS and sends measurement report to the LMF.

LPHAP UE capability

Proposal9. Introduce UE capability of whether to support SRS with validity area in RRC_INACTIVE to LPP and RRC.

Proposal10 Introduce UE capability of whether to support SRS pre-configuration in RRC_INACTIVE to LPP and RRC.

Alignment between (e)DRX and PRS:

Proposal11: Support PRS alignment with fixed DRX and deprioritize DRX alignment with fixed PRS.

Proposal12: For the DL-PRS alignment with fixed (e)DRX, UE-initiated on-demand DL-PRS procedures are used.

Proposal13: LMF-initiated alignment between PRS and (e)DRX is not supported.

References

[1] RAN2 #121bis Chair’s Note

[2] RAN1 #112 Chair’s Notes

[3] RAN2 #122 Chair’s Notes