3GPP TSG-RAN WG2 #123	R2-2307122

Toulouse, France, 21st – 25th August, 2023

Agenda item:	7.2.2

Source:	Huawei, HiSilicon

Title:	Discussion on higher layer aspects for Sidelink Positioning

Document for:	Agreement

- Introduction

In this paper, we discuss the SL positioning issues with respect to priority issues, LS from SA2, reliable transport, and other RAN2 related issues based on RAN1 and SA2’s progress.

2.	Discussion

2.1	Priority issues

RAN2#121 meeting discussed the support of SLPP protocol and agreed the SLPP over PC5-U option.

RAN2#121 Agreement: PC5-U is used for transport of SLPP.

Adopting the SLPP over PC5-U option means that the SLPP message will be transferred via DRB. Then UE needs to know which DRB to use when transmitting the SLPP message, so that the SLPP transmission of different positioning services can be treated differently and the positioning services with high requirement can be assured. In SL communication, UE uses the corresponding configuration of sidelink including radio bearer config and RLC bearer config, etc., based on the Qos profile of the current traffic. For example, UE can obtain the mapping between Qos profile and configuration of sidelink from SIB12 when in RRC IDLE, or NW configures the configuration of sidelink for RRC Connected UE, or UE uses the pre-configuration including the mapping between Qos profile and configuration of sidelink when in OoC.

For SL positioning, SA2 has agreed the “RSPP transport QoS” to ensure the transmission quality of SLPP message. The “RSPP transport QoS” for SL positioning is similar as the Qos profile for SL communication, therefore, we can use the similar way to provide UE with the sidelink related configuration of SLPP message. In the sidelink configuration, logical channel priority is configured via the MAC logical channel config. With the SL positioning service specific configuration of sidelink that includes “priority” field, the different SL positioning service with different Qos requirement can be distinguished and achieving the assurance of the positioning services with high requirement. Therefore, the adopted PC5-U solution can reuse the legacy design and not to introduce new spec impacts.

## 5.7 QoS Handling ### 5.7.1 General QoS of Ranging/SL Positioning includes 2 aspects: - Ranging/SL Positioning QoS, which refers to the quality of the Ranging/SL Positioning result - RSPP transport QoS, which refers to the quality of the PC5-U communication for transmitting RSPP traffic. ### 5.7.2 Handling of Ranging/SL Positioning QoS Ranging/SL Positioning QoS requirement may be provided in the Ranging/SL Positioning service request generated at the application layer, and is provided from the application layer to the Ranging/SL Positioning layer. Ranging/SL Positioning QoS information contains attributes defined in clause 4.1b of TS 23.273 [8] with the following additions: - The accuracy attribute also includes the accuracy of horizontal/vertical distance and the accuracy of horizontal/vertical direction; - Range, which indicates the applicability of the QoS attributes in the Ranging/SL Positioning operation over PC5.

Proposal1: Based on PC5-U, the SLPP message related configuration can reuse the legacy procedure of sidelink communication, e.g., how priority is configured for each SLPP message.

2.2	SA2 LS related issue

In [3], SA2 sent an LS to RAN2 asking the following question:

Question 1: What are the criteria from RAN2 perspective for the selection of a Located UE and SL Positioning Server UE? And when does the selection take place?

With respect to Server UE selection, RAN2#121 agreed the following:

RAN2 confirm that for cases without LMF involvement, besides method determination, assistant data distribution and anchor UE selection (agreed in RAN2), the SL positioning server UE may perform SL-PRS configuration coordination and location calculation.

According to the above agreements, for Server UE selection, UE should consider whether the corresponding functionalities supported by a Server UE can meet the requirement of current positioning, e.g., UE role to be server UE, supported positioning method, capability for calculation, assistant data distribution, anchor UE selection and SL-PRS configuration coordination.

Proposal2: Reply to SA2 that for Server UE selection, RAN2 consider the following information of UE:

- UE role to be server UE;

- supported positioning method;

- capability for calculation;

- assistant data distribution;

- anchor UE selection;

- SL-PRS configuration coordination.

In RAN2#121bis-e meeting, RAN2 agreed the following, where UE role is agreed to be included in discovery message, but other information is FFS.

RAN2#121bis-e Agreements: RAN2 confirms that discovery messages will be used to carry information for targeted discovery and candidate selection of SL positioning UEs, including at least the indication of anchor UE, target UE. and server UE roles. FFS how much information is indicated about anchor UEs (e.g., knowledge of location).

However, considering that the discovery message payload is limited, it is not reasonable to place all the related information in discovery message. Therefore, the final anchor UE selection procedure can happen after unicast establishment, for example, the other information can be transferred using SLPP capability transfer and SLPP assistant data transfer procedure.

Observation: The UE capability fields might be too large for the DCR/discovery message that the coverage of the message will be impacted. There might also be security issues for carrying UE capability in the DCR/discovery message

With respect to the timing for selection, considering that discovery message can’t carry all the selected information, then it is reasonable that UE selection happens after discovery procedure. Based on the above discussion, we propose the following

Proposal3: No other information than UE role needs to be carried within the discovery message/DCR message. And reply to SA2 that for Located UE or Server UE selection can happen after discovery.

2.3	Reliable transport

In last RAN2#122 meeting, RAN2 discussed the reliable transport for SL positioning, and agreed to support reliable transport at least for unicast case.

RAN2#122 Agreements: SLPP over PC5-U/Uu will support reliable transport for at least unicast. FFS groupcast.

In Uu positioning, the reliable transport for LPP includes LPP duplicate detection, LPP acknowledgement, and LPP retransmission, which ensure the reliable and in-sequence delivery of LPP message between UE and LMF. However, for groupcast case of SL positioning, all the group members within the group receive the SLPP message sent by the transmitter UE, which is different from the one-to-one unicast case. For example, assume UE1, UE2 and UE3 form a group. If UE1 sends SLPP message1 with a sequence number, UE2 receives SLPP message1 correctly and response an acknowledgement with the corresponding sequence number, but UE3 doesn’t receive SLPP message1 correctly, then UE3 won’t response UE1 which leads to UE1 to retransmit the SLPP message1 again to both UE2 and UE3. In the second round of transmission, if UE3 receives correctly but UE2 fails, then UE1 will perform the retransmission again, which is not power efficient for UE1.

Proposal4: Reliable transport is not supported for groupcast case.

2.4	SLPP measurement report

With respect to measurement report and location information for SL positioning, RAN1 agreed the following, where RAN2 needs to discuss the signaling design.

RAN1#112b Agreement Support higher layer signaling for sidelink positioning measurement report and report triggering. - Up to RAN2 to discuss detailed signaling design. FFS on SCI based report triggering. RAN1#113 Agreement Location information of the target UE based on sidelink positioning measurements can be reported at least to LMF. - FFS: on whether quality information of location is included, e.g., uncertainty etc - Up to other WGs to determine whether location information of the target UE can be reported to another UE - Up to RAN2 for signaling details FFS: whether and how to report per ARP location information.

For measurement report and location information, RAN2 agreed that SLPP can support the “SL Location Information Transfer” function, where SLPP is terminated not only between Server/Anchor UE and Target UE, but also between Target UE and LMF.

For SLPP between UE and LMF: using SLPP request location information message, LMF can request Target UE to report location information results, then Target UE response LMF with calculated location information results. Or LMF can request Target UE and anchor UEs to report location information measurements, then Target UE and anchor UEs response LMF with location information measurements. A special case is the anchor UE may report location information results of target UE to the LMF, e.g., for the absolute positioning of Target UE using Located UE in [2].

For assistance information, RAN1 discussed and agreed the following:

RAN1#112b Agreement For provision of assistance information for absolute SL positioning, the anchor UE location information can be provided to LMF or UE. FFS: which UEs can receive the anchor UE location information (note: which may be decided by other WGs) FFS on quality information of anchor UE location information. RAN1#113 Agreement For provision of assistance information for sidelink positioning, the ARP location information can be provided to LMF or UE. - FFS: which UEs can receive the location information (note: which may be decided by other WGs) - FFS: details on the location information, e.g., relative location information - Note: different ARPs have their own location information

RAN2 agreed that Server UE is responsible for performing calculation, therefore, it is reasonable for Server UE to obtain the above anchor UE location information and ARP related location information.

Proposal5: Server UE/target UE can receive the Anchor UE location information and ARP related location information.

2.5	Issues related to SA2 procedure

SA2 discussed the general procedure for SL positioning, and has reached agreements for the whole procedure including “Ranging and Sidelink Positioning MO-LR procedure (SL-MO-LR)” [1], “Ranging and Sidelink Positioning MT-LR procedure (SL-MT-LR)” [1] and other sidelink positioning related procedures in [2]. However, some of the details need alignment with RAN2.

2.5.1	SL-MO-LR

For SL-MO-LR, SA2 agreed the following procedure:

For step 4 and step 5, there is an Editor’s note from SA2:

4. UE1 and UEs 2 to n may communicate over PC5 for authorization of Ranging/SL positioning and receiving QoS parameters if needed. Each of UEs verifies that Ranging/SL positioning is permitted, including whether Ranging/SL positioning results may be transferred to an LCS Client or AF if this is used, according to any service authorization and policy/parameter provisioning received at step 1. QoS requirements for the Ranging/SL positioning may be also provided based on QoS requirements in the service request. 5. UE1 may obtain the Sidelink positioning capabilities of UEs 2 to n using the groupcast and/or unicast links established in step 3. Step 4 and 5 may be performed to transfer the information of UEs which are not served by the LMF. NOTE 2: UE2/.../UEn is not assumed to be served by the same LMF serving UE1. Editor's note: It needs to be verified that RAN2 will provide support for steps 4 and 5.

From RAN2 perspective, RAN2 agreed that SLPP supports the functional of capability transfer. The specific details of SLPP capability can take LPP capability as baseline, e.g., SLPP capability includes SL-PRS resources capability, SL-PRS measurement capability, SL-PRS process capability, calculation capability etc.

Proposal6: RAN2 to discuss the details of SLPP related capability for SL positioning.

For step 8, there is an Editor’s note from SA2:

8. UE1 sends a supplementary services SL-MO-LR request to the serving AMF in an UL NAS TRANSPORT message. The SL-MO-LR request indicates the other UEs 2 to n (using application layer ID and/or GPSI), indicates any assistance data needed, indicates whether location calculation assistance is needed, and indicates whether location results should be transferred to an LCS client or AF. The message shall include the identity of the LCS client or the AF and may include the address of the GMLC through which the LCS client or AF (via NEF) should be accessed. In addition, a Service Type indicates which MO-LR service of the LCS Client is requested by the UE may be included. For location calculation assistance from the LMF, the preferred type of Sidelink positioning/ranging location results (e.g. absolute locations, relative locations or distances and directions between pairs of UEs) and the required QoS are included. If UE1 is Located UE and one of UE2/.../UEn is the target UE that does not have NAS connection, the supplementary services SL-MO-LR request includes an indication that one of UE2/.../UEn is the target UE instead of UE1. Editor's note: It is FFS whether indication about the need of assistance data from UE, which will be aligned with RAN WGs.

The editor’s note here means whether UE needs to send an indication to request assistance data. In Uu positioning, UE can also send LPP request assistance data message to LMF to request assistance data during positioning session. For SL positioning, UE requesting the assistance data here in step 8 can reduce another SLPP request message to LMF. Therefore, it is acceptable for UE to send the indication about the need of assistance data in step 8.

Proposal7: UE can send indication about the need of assistance data using SL-MO-LR request sent to LMF.

In step 10, the LMF requests the capability of UE1-UEn from UE1 if there is not enough capability information provided in step 8. In addition, UE1 requests the capability of other UEs in Step 4 and 5 if other UEs are not served by the same LMF.

10. The LMF sends a request to UE1 for the capabilities of UEs 1 to n. NOTE 3: UE2/.../UEn is not assumed to be served by the same LMF serving UE1. Editor's note: It is FFS the update when the LMF also serves UE2/.../UEn with alignment with RAN. NOTE 4: Steps 10 and 11 can be omitted if UE1 includes a message containing the capabilities of UEs 1 to n in the SL-MO-LR request at step 8. Step 12 can be omitted if UE1 includes a message containing the request for specific assistance data in the SL-MO-LR request at step 8.

With respect of the above Editor’s note, if LMF is only the serving LMF of UE1, then LMF can also requests UE1 to send the capability of UE2-UEn. If LMF is the serving LMF of not only UE1, but also UE2-UEn, then SA2 needs to align with RAN whether LMF requests the capability of UE2-UEn from UE1(option 1), or from UE2-UEn themselves (option 2).

From RAN2 perspective, for this LMF involved case where LMF is the serving LMF of UE1-UEn, the procedure of the two options include:

- Option 1: LMF requests UE1 to send the capabilities of all related UEs. UE1 requests the capabilities of UE2-UEn, then UE2-UEn response UE1 with their capabilities. Then UE1 sends capabilities of UE1-UEn to LMF.

- Option 2: LMF requests UE1-UEn to send their own capabilities. UE1-UEn response LMF with their capabilities.

Therefore, it is more power efficient and signaling overhead can be reduced if option 2 is used.

Another Editor’s note related to RAN2 is “Whether step 10-11 are needed”.

11. UE 1 returns its capabilities to the LMF. UE1 may additionally return the capabilities of the UEs obtained at step 5 if requested by the LMF at step 10. 12. UE1 may send a request for specific assistance data to the LMF. 13. LMF sends the requested assistance data to UE1, and UE1 forwards the assistance data received from LMF to UE2/.../UEn. The assistance data may assist UEs 1 to n to obtain Sidelink location measurements at step 15 and/or may assist UE1 to calculate Sidelink positioning/ranging location results at step 16. NOTE 4: Steps 10 and 11 can be omitted if UE1 includes a message containing the capabilities of UEs 1 to n in the SL-MO-LR request at step 8. Step 12 can be omitted if UE1 includes a message containing the request for specific assistance data in the SL-MO-LR request at step 8. Editor's note: Whether step 10-11 are needed will be aligned with RAN WGs.

In step 10 and step 11, LMF requests positioning related capabilities from UE. In legacy Uu positioning, LMF also requests UE capabilities from UE, e.g., the LMF sends LPP RequestCapablities to UE, and UE responses LPP ProvideCapabilities to LMF. From RAN2 perspective, step 10 and step 11 are similar procedures as in legacy, so it is reasonable to perform step 10 and step 11 for LMF and UE.

Proposal8: LMF requesting SLPP capabilities from UE and UE sending SLPP capabilities of itself and the peer UEs within the SLPP session to LMF are needed in SA2 SL-MO-LR procedure.

For candidate Located UEs, the Editors’ note in SA2 procedure is as shown follows:

14. If the SL-MO-LR request at step 8 indicated location calculation assistance is needed and/or indicated transfer of Sidelink positioning/ranging location results to an LCS Client or AF, the LMF sends a request for location information to UE1 and may also send a request for location information to UE2/.../UEn if it is served by the LMF. If LMF determines to apply UE based SL Positioning, LMF includes in the request the indication of UE based SL Positioning. LMF may also provide the list of candidate Located UE(s), if absolute location is requested at step 8. If scheduled location time is received at step 14. LMF may include a scheduled location time. Editor's note: It needs to be aligned with RAN WG2 if the list of candidate Located UE(s) is provided in step 13 or step 14.

From RAN2 perspective, the list of candidate Located UE should be taken as assistance data, same as the TRP information provided from LMF to UE in Uu positioning.

Proposal9: The list of candidate Located UE is provided in assistance data message.

2.5.2	SL-MT-LR

For SL-MT-LR, SA2 agreed the following procedure:

For step1, there is one Editor’s note about application layer ID.

1. The LCS Client or the AF (via NEF) sends an LCS service request to the (H)GMLC for Ranging/Sidelink Positioning location results for the n UEs which may each be identified by a GPSI or a SUPI. The request may include the required QoS, the required location results (e.g. absolute locations, relative locations or distances and/or directions related to the UEs), the SL reference UE(s) in case of relative locations, distance, or direction. The (H)GMLC or NEF authorizes the LCS Client or the AF for the usage of the LCS service. If the authorization fails, the remaining steps are skipped and the (H)GMLC or NEF responds to the LCS Client or the AF with the failure of the service authorization. In addition, an Application Layer ID shall be included for each of the n UEs to enable discovery of the UEs at step 12. Editor's note: Whether Application Layer ID will be included in LPP operation is FFS and needs the coordination with RAN WGs.

In SA2 procedure, after reception of location request from AMF, if LMF determines to use SL positioning, then LMF sends an SL-MT-LR request to serving AMF in step 10, based on which the AMF forwards the SL-MT-LR to UE in step 11.

10. The LMF sends an SL-MT-LR request to the serving AMF as a supplementary services message, using the Namf_Communication_N1N2MessageTransfer service operation, and the session ID parameter is set to the LCS Correlation identifier. The SL-MT-LR request may include the application layer IDs of the other UEs 2 to n, the types of required location results (e.g. relative locations or distances and/or directions) and SL reference UE(s) in case of relative locations. Editor's note: Whether supplementary services message or lpp will be sent from LMF to UE is FFS and needs the coordination with RAN WGs.

For the above Editor’s note, if AMF is responsible to transfer the SL-MT-LR request to UE, then there is no need for LMF to send the SL-MT-LR again to UE through LPP, it is also similar as in legacy that AMF is responsible to send the MT-LR to UE.

Proposal10: It is AMF to send the SL-MT-LR to UE instead of LMF.

2.5.3	Application Layer ID in SLPP between UE and LMF

The following is captured from clause 5.5.2 of TS 23.586 [2]. For network-based SL positioning for UE with NAS connection, the target UE can send the measurement data together with application layer ID. The LMF can interact with GMLC to resolve UE identities.

- The Target UE and Located UE(s) perform Ranging/SL positioning. The Target UE includes the UE identity i.e. Application Layer ID of the Located UE(s) to the LMF together with the Ranging measurement data or estimation result. The LMF may interact with GMLC to get the location of Located UE.

Similarly, as the SL-MO-LR shows, UE1 and the LMF may interact the capability information or assistance data of other UEs. The application layer ID should be used to identify different UEs.

Proposal11: Application layer ID is carried over SLPP messages between UE and LMF to identify other UEs.

2.6	Session Management

Explicit session management, e.g. request/response/reject for the start/end/modify of SLPP sessions, introduces extra complexity and latency to the positioning procedures without clear benefits. A potential use case might be group management in group positioning. However, RAN2 has agreed that group management for multiple target UE will not be specified. Therefore, it is proposed that explicit session management for SLPP is not supported for Rel-18.

Proposal12: Explicit session management of SLPP procedure is not needed for Rel-18.

3. Conclusion

Based on the discussion in this paper, we recommend RAN2 to discuss and adopt the following proposals:

Priority issues

Proposal1: Based on PC5-U, the SLPP message related configuration can reuse the legacy procedure of sidelink communication, e.g., how priority is configured for each SLPP message.

LS from SA2

Proposal2: Reply to SA2 that for Server UE selection, RAN2 consider the following information of UE:

- UE role to be server UE;

- supported positioning method;

- capability for calculation;

- assistant data distribution;

- anchor UE selection;

- SL-PRS configuration coordination.

Proposal3: No other information than UE role needs to be carried within the discovery message/DCR message. And reply to SA2 that for Located UE or Server UE selection can happen after discovery.

Reliable transport

Proposal4: Reliable transport is not supported for groupcast case.

SLPP measurement report

Proposal5: Server UE/target UE can receive the Anchor UE location information and ARP related location information.

Issues related to SA2 procedure

Proposal6: RAN2 to discuss the details of SLPP related capability for SL positioning.

Proposal7: UE can send indication about the need of assistance data using SL-MO-LR request sent to AMF.

Proposal8: LMF requesting SLPP capabilities from UE and UE sending SLPP capabilities of itself and the peer UEs within the SLPP session to LMF are needed in SA2 SL-MO-LR procedure.

Proposal9: The list of candidate Located UE is provided in assistance data message.

Proposal10: It is AMF to send the SL-MT-LR to UE instead of LMF.

Application Layer ID in SLPP between UE and LMF

Proposal11: Application layer ID is carried over SLPP messages between UE and LMF to identify other UEs.

Session management

Proposal12: Explicit session management of SLPP procedure is not needed for Rel-18.

4.	References

- 23.273-i20

- 23.586-i00

- S2-2305735_was_5287r11 draft reply LS on Sidelink positioning procedure, April 17 – 21, 2023