3GPP TSG-RAN WG2 Meeting #123 	R2-2307123

Toulouse, France, 21st – 25th August, 2023

Title:		Discussion on lower-layer aspects for SL positioning

Source:	Huawei, HiSilicon

Agenda item:	7.2.2

Document for:	Discussion and Decision

- Introduction

Within the WID RP-223549, the following objectives have been specified for sidelink positioning:

The specific objectives of this work item are: - Specify solutions for support of sidelink positioning (including ranging) in NR systems, including the following [RAN1, RAN2, RAN3, RAN4]: - Specify SL PRS for support of sidelink positioning such that the SL PRS uses a comb-based (full RE mapping pattern is not precluded) frequency domain structure and a pseudorandom-based sequence where the existing sequence of DL-PRS is used as a starting point [RAN1]. - Specify support for SL PRS bandwidths of up to 100 MHz in FR1 spectrum. - NOTE: SL PRS transmission in FR2 is not precluded but no FR2 specific aspects will be specified. - Specify measurements to support RTT-type solutions using SL, SL-AoA, and SL-TDOA [RAN1, RAN2]. - Specify support of resource allocation for SL PRS: - Including resource allocation Scheme 1 and Scheme 2, where Scheme 1 corresponds to a network-centric SL PRS resource allocation and Scheme 2 corresponds to UE autonomous SL PRS resource allocation [RAN1]. - For resource allocation mechanism for SL PRS in Scheme 2: - Study and specify support of sensing-based resource allocation, and/or a random resource selection [RAN1]. - Study and specify solutions for congestion control for SL PRS and/or inter-UE coordination for SL-PRS [RAN1]. - Support resource allocation for shared resource pool with Rel-16/17/18 sidelink communication and dedicated resource pool for SL PRS [RAN1]. - NOTE: For SL positioning resource (pre-)configuration in a shared resource pool with Rel-16/17/18 sidelink communication, backward compatibility with legacy Rel-16/17 UEs should be ensured. - Specify procedures for transmit power control for SL PRS transmissions at least based on open loop power control (OLPC) [RAN1]. - Specify signalling and associated UE behavior for support of unicast, groupcast (not including many to one) and broadcast of SL PRS transmissions [RAN1, RAN2]. - Specify reporting signalling and procedures to facilitate support of SL positioning in all coverage scenarios and for PC5-only and joint PC5-Uu scenarios [RAN2, RAN3]: - Specify the protocol and procedures for SL positioning between UEs (Protocol for Sidelink positioning procedures (SLPP)). - Specify the protocol and procedures for SL positioning between UEs and LMF. - Specify signalling to NG-RAN for sidelink positioning and ranging service authorizations as needed. [RAN3, RAN2] - Specify corresponding new core requirements, as well as identifying and specify the impact on the existing RAN4 specification, including RRM measurements and procedures [RAN4].

This paper provides our thinking on the lower-layer aspects for sidelink positioning, mainly including the discussion on RRC and MAC spec.

2	Discussions on the MAC aspects

2.1 SL-PRS priority

SL-PRS is agreed in RAN1 to be transmitted in dedicated resource pool or shared resource pool. According to the agreements related to the SL-PRS priority, SL-PRS has multiple priority in dedicated resource pool.

Agreement For the scheme 2 sensing-based resource allocation, - Rel-16/17 resource (re)-selection procedure is reused for SL-PRS in the shared resource pool. - Study if/what changes are needed - Rel-16[/17] resource (re)-selection procedure with periodic and without periodic reservations is the starting point for the design of SL-PRS in the dedicated resource pool. - Study what changes, if any, are needed at least with regards to the following: sensing window, resource selection window, reservation interval, Resource exclusion mechanism (e.g. definition of resource set for SL-PRS, how RSRP is measured, etc) - From RAN1 perspective, priority value for SL PRS should be provided by higher layers from Tx UE perspective Agreement For Scheme 2, in a dedicated resource pool, - Multiple L1 SL-PRS priority are allowed in a resource pool - A SL PRS resource within the resource selection window is used as a candidate resource - with regards the reservation interval of SL-PRS, it is provided by UE’s higher layers with values TBD. The set of values is (pre-)configured. - Use the periodicities available for legacy SL communication and the ones defined for DL-PRS as a starting point. - with regards to the resource (re)-selection procedure - support re-evaluation & pre-emption for SL-PRS using the Rel-16 re-evaluation and pre-emption respectively as a starting point.

The agreements above mean that L1 priority is used for one of SCI transmission parameters in the dedicated resource pool. For the transmission of SL-PRS using shared pool, as the legacy, priority is indicated in the first SCI. Considering we have the case that only the SL-PRS is to be transmitted via the SL grant in the shared resource pool, the priority value of the SL-PRS should also be defined. Hence, for both dedicated and shared RP, SL-PRS priority needs to be provided to the lower layer for resource selection and collision handling, etc. Then, it needs to be determined what priority levels are defined for this SL-PRS priority.

In order for the priority of SL-PRS to be comparable with data in the shared resource pool, which has 8 levels of PHY priorities, and also align the priority of the SL-PRS in shared and dedicated resource pool, the number of priorities for SL-PRS should also be 8.

Proposal1: Define 8 priority levels for SL-PRS priority. Send an LS to RAN1 and SA2

Then, a second issue needs to be discussed is who provides the priority level for the SL-PRS transmission. In the previous RAN1 meeting, the following has been agreed on the trigger of the SL-PRS transmission.

Agreement In Scheme 2, with regards to the triggering of SL-PRS, support one or both of the following options: - Option 1: Support SL-PRS triggering at the physical layer by the UE’s own higher layers. - Note: this also includes higher layer triggering from another UE - Option 2: Support UE-A to request UE-B to transmit SL-PRS via lower layer signaling sent by UE-A. - FFS: Whether lower-layer signaling is SCI or SL MAC-CE Agreement In Scheme 2, with regards to the triggering of SL-PRS, confirm the related WA for shared and dedicated resource pools. - With regards to the lower-layer signalling, support SCI associated with SL-PRS transmission - FFS: whether this is enabled by (pre)configuration FFS: to support also SL-PRS

Since, SL-PRS can be triggered by the UE’s own higher layer or the lower layer signaling from the peer UE, it is reasonable that the priority level is also provided by the higher layer and the lower layer signaling from the peer UE.

Proposal2: SL-PRS priority is provided by the UE’s own higher layer or the SCI from the peer UE that trigger the SL-PRS transmission.

2.2	Resource Allocation

In the following, we first give an overview of the resource allocation procedure for SL-SCH data transmission in the legacy. Then, we discuss the issues for resource allocation involving the SL-PRS transmission.

2.2.1	Resource allocation mechanism for SL-SCH in Rel-16/17

The current spec for resource allocation for SL-SCH can be summarized in the following two steps:

Step 1: Determine the SL grant

- Mode 1: when receiving a DCI for dynamic grant or configured with CG type1/2, determine the SL grant based on the DCI or CG configuration.

For mode 1, the gNB can know the TX UE’s transmission needs via BSR. With the BSR for LCGs, the gNB can know the priority of the SL-SCH data to be transmitted, which helps gNB indicate the TX UE an appropriate SL grant.

- Mode 2: when there is MAC PDU to be transmitted, including LCH data arrival and MAC CE to be transmitted, the resource selection procedure is triggered. The resource pool is selected first according to the data to be transmitted (e.g., resource pool selection for discovery message and data from logical channel). Then, the parameters (e.g., L1 priority) are indicated by MAC to lower layer to select resources, and finally MAC randomly selects from the reported resource to determine the SL grant.

Note that for mode 2, when there are multiple LCH data arrival, resource selection procedures can be separately triggered for each logical channel. The sidelink grant selection triggered by the lower priority data transmission may come first before that triggered by the higher priority data as shown in the following figure.

Step 2: For each SL grant, preform LCP, to guarantee that higher priority data can be transmitted first

  - Determine the destination associated with the SL grant (e.g. SL grant 1 in the following figure) based on the priority of the LCH data and/or MAC CE to be transmitted. In the following example, the priority of the LCH2 is higher than that of LCH1 and the priority of the LCH3 is lower than that of LCH2.

  - Construct the MAC PDU according to the LCP and TB size. Set the parameters (including the priority parameter) on the SCI associated to the SL grant.

Based on the current procedure, we can see that no matter the intention to trigger resource selection, high priority data is always transmitted first when the SL grant can satisfy its traffic.

Observation: For mode 1, resource can be requested based on BSR, and SL grant is indicated by DCI.

Observation: For mode 2, resource selection can be triggered by LCH data and/or MAC CE, and SL grant is selected based on the priority parameters of the LCH data and/or MAC CE.

Observation: For both mode 1 and mode 2, the final transmission of the SL grant depends on the LCP procedure with the priority of the data from each logical channels.

2.2.2	Resource Allocation for SL-PRS

Similar to the Mode1/2 for SL-SCH transmission, according to the WID, Scheme 1 and Scheme 2 have been adopted for SL-PRS transmission. Also, RAN1 agrees that the SL-PRS can be transmitted in dedicated resource pool and shared resource pool.

2.2.2.1	Scheme1 discussion

In the RAN1 discussion, the following has been agreed on the resource allocation in scheme1

Agreement For Scheme 1 SL-PRS resource allocation, a transmitting UE can receive a SL-PRS resource allocation signaling from gNB through a - Dynamic grant - FFS Reuse DCI format 3_0 for signalling SL-PRS resource allocation or Support a new DCI format (3_X) and consider DCI format 3_0 as a starting point - Configured grant type 1 - the SL-PRS transmission(s) follows the higher layer configuration - Configured grant type 2 - Support activating and releasing the configured grant using a new DCI format 3_X or 3_0 (to be down-selected between the two DCI formats) - The above mechanisms use NR Rel-16 mode-1 signaling as a starting point - FFS: whether same/different DCI format(s) are applied for shared pool and dedicated pool. - FFS: Further details

It can be seen that the gNB indicates dynamic grant/configured type of resource selection for scheme1 for either dedicated resource pool or shared resource pool. However, it needs to be discussed how the gNB determines the UE needs the SL grant for SL-PRS transmission in the first place. Some assistance information needs to be provided to the gNB from the UE to let the gNB make informed decisions.

2.2.2.1.1	Scheme 1 SL-PRS transmission by dynamic grant

For dynamic grant, the legacy spec use BSR MAC CE to inform the gNB about the current buffer status for data in the UE side such that the gNB can schedule data according to the buffer size as discussed above. According to the BSR MAC CE, the gNB can identify the priority of the data in LCHs. While for SL-PRS transmission, to make the gNB select an appropriate SL-grant for SL-PRS, the gNB needs to know the requirements of the SL-PRS transmission. Then we think a similar mechanism as data transmission can be adopted to enable a quick information transmission: the UE can send a MAC CE for SL-PRS resource request for one-shot PRS transmission.

Proposal3: When aperiodic SL-PRS transmission is triggered for UE configured with Scheme 1 SL-PRS resource allocation, the UE sends a MAC CE to the gNB for SL-PRS resource request. LS to RAN1 on the detailed content in the MAC CE.

Then, if we agree the proposal for the transmission of UL MAC CE, it is possible that there is no UL-SCH resource and the MAC CE cannot be transmitted. In this case, a SR can be triggered for the request of UL-SCH resource for the transmission of the MAC CE.

Proposal4: SR can be triggered for SL-PRS resource allocation request MAC CE when there is no UL-SCH resources to accommodate the MAC CE.

2.2.2.1.2	Scheme 1 SL-PRS transmission by configured grant

In the other case of configured grant type 1/2, legacy spec has enabled the UE to send the SL traffic pattern to the gNB for requesting configured grant configuration, as can be seen in the following:

SL-UE-AssistanceInformationNR-r16 ::= SEQUENCE (SIZE (1..maxNrofTrafficPattern-r16)) OF SL-TrafficPatternInfo-r16 SL-TrafficPatternInfo-r16::= SEQUENCE { trafficPeriodicity-r16 ENUMERATED {ms20, ms50, ms100, ms200, ms300, ms400, ms500, ms600, ms700, ms800, ms900, ms1000}, timingOffset-r16 INTEGER (0..10239), messageSize-r16 BIT STRING (SIZE (8)), sl-QoS-FlowIdentity-r16 SL-QoS-FlowIdentity-r16 }

For the periodic type of SL-PRS transmission, we think the same methodology can be applied that the UE requests the periodic SL-PRS transmission by configured grant with RRC message.

Proposal5: When periodic SL-PRS transmission is triggered for UE configured with Scheme1 SL-PRS resource allocation, the UE sends an RRC message to the gNB for sending the assistance information for CG configuration. The detailed content in the RRC message is FFS.

For configured grant type 2, CG type 2 in the legacy releases are reused as baseline according to the agreement. For configured grant type 2, if the resource is activated/deactivate by the new DCI 3_x, similar to legacy releases, a CG confirmation MAC CE needs to be triggered and sent to the gNB by the UE when the UE successfully decodes the DCI. For CG type 2 for SL-PRS transmission, and same mechanism should be used

Proposal6: The UE sends a CG confirmation MAC CE when the DCI for CG type 2 activation/deactivation is successfully received.

2.2.2.2	Scheme2 discussion

The following agreement has been made in RAN1 regarding the resource allocation for scheme2.

Agreement In Scheme 2, with regards to the triggering of SL-PRS, support one or both of the following options: - Option 1: Support SL-PRS triggering at the physical layer by the UE’s own higher layers. - Note: this also includes higher layer triggering from another UE - Option 2: Support UE-A to request UE-B to transmit SL-PRS via lower layer signaling sent by UE-A. - FFS: Whether lower-layer signaling is SCI or SL MAC-CE Agreement In Scheme 2, with regards to the triggering of SL-PRS, confirm the related WA for shared and dedicated resource pools. - With regards to the lower-layer signalling, support SCI associated with SL-PRS transmission - FFS: whether this is enabled by (pre)configuration FFS: to support also SL-PRS Agreement For SL-PRS transmission, at least support the following - SL-PRS transmissions with periodic reservation: SL-PRS transmissions which are being reserved with a similar mechanism as the SL periodic resource reservation for another TB in legacy SL communication - FFS: whether/what changes are needed - SL-PRS transmissions without periodic reservation: SL-PRS transmissions in which the SL-PRS is transmitted at least once without periodic reservation, with a similar mechanism as in legacy SL communication with SL resource without periodic reservation. - FFS: Maximum number of reservations and transmissions after triggering

For SL-PRS transmission, it has been agreed in RAN1 that the SL-PRS transmission can be triggered with the reception of SCI in the lower layer or the UE’s own higher layer. When the SL-PRS transmission is triggered in scheme 2, the MAC entity should first trigger the resource selection for the SL-PRS. And for each upcoming SL grant, the MAC entity needs to consider whether the SL grant can be used to transmit the SL-PRS.

As both dedicated resource pool and shared resource pool can be configured for the UE, when the resource selection for SL-PRS is triggered, the UE can perform resource pool selection among shared resource pool and dedicated resource pool.

At the time when SL-PRS transmission is triggered, there might and might not be on-going data transmission. The intention for the design for shared resource pool is that there can be simultaneous transmission of data and SL-PRS for the same destination UE. Then, UE can select the pool based on the pending transmission. For example, when both sidelink data and SL-PRS are pending for transmission, shared resource pool should be selected. And in the other case of only SL-PRS transmission, we think it is reasonable to prioritize on the dedicated resource pool for SL-PRS transmission.

Proposal4a: When there are both sidelink data and SL-PRS pending for transmission, select shared resource pool.

Proposal4b: When there are only SL-PRS pending for transmission while there is no data, prioritize dedicated resource pool.

Agreement In Scheme 2, with regards to the triggering of SL-PRS, support one or both of the following options: - Option 1: Support SL-PRS triggering at the physical layer by the UE’s own higher layers. - Note: this also includes higher layer triggering from another UE - Option 2: Support UE-A to request UE-B to transmit SL-PRS via lower layer signaling sent by UE-A. - FFS: Whether lower-layer signaling is SCI or SL MAC-CE

Agreement For the scheme 2 sensing-based resource allocation, - Rel-16/17 resource (re)-selection procedure is reused for SL-PRS in the shared resource pool. - Study if/what changes are needed - Rel-16[/17] resource (re)-selection procedure with periodic and without periodic reservations is the starting point for the design of SL-PRS in the dedicated resource pool. - Study what changes, if any, are needed at least with regards to the following: sensing window, resource selection window, reservation interval, Resource exclusion mechanism (e.g. definition of resource set for SL-PRS, how RSRP is measured, etc) - From RAN1 perspective, priority value for SL PRS should be provided by higher layers from Tx UE perspective Agreement For the scheme 2 sensing-based resource allocation: - Alt. 2: Rel-16 resource (re)-selection procedure with periodic and without periodic reservations is the starting point for the design of SL-PRS in the dedicated resource pool. - Note: This means that Rel-17 partial sensing is not considered a starting point for the design

As agreed by RAN1, SL-PRS transmission by periodic reservation is supported for dedicated resource pool. For periodic reservation, the legacy spec defines a counter for the triggering for resource reselection. The counter is randomly selected according to the following spec. With the resource reservation interval larger or smaller than 100ms, the value of the counter is randomly selected within an interval.

3> randomly select, with equal probability, an integer value in the interval [5, 15] for the resource reservation interval higher than or equal to 100ms or in the interval 5×100max20, Prsvp_TX,15×100max20, Prsvp_TX for the resource reservation interval lower than 100ms and set SL_RESOURCE_RESELECTION_COUNTER to the selected value;

Then, the counter is maintained per HARQ process and after the last transmission, the counter is decreased by 1. Then, if the counter reaches 0, the UE is required to perform resource reselection.

2> else if SL_RESOURCE_RESELECTION_COUNTER = 0 and when SL_RESOURCE_RESELECTION_COUNTER was equal to 1 the MAC entity randomly selected, with equal probability, a value in the interval [0, 1] which is less than or equal to the probability configured by RRC in sl-ProbResourceKeep: 3> clear the selected sidelink grant, if available; 3> randomly select, with equal probability, an integer value in the interval [5, 15] for the resource reservation interval higher than or equal to 100ms or in the interval 5×100max20, Prsvp_TX,15×100max20, Prsvp_TX for the resource reservation interval lower than 100ms and set SL_RESOURCE_RESELECTION_COUNTER to the selected value; 3> reuse the previously selected sidelink grant for the number of transmissions of the MAC PDUs determined in TS 38.214 [7] with the resource reservation interval to determine the set of PSCCH durations and the set of PSSCH durations according to TS 38.214 [7].

We think the same rationale also applies here that the resource can be reselected when a counter reduces to zero for reserved resource, the UE needs to reselect the resource for the next sensing period. The difference might be that for SL-PRS transmission on dedicated resource pool, there is no concept of HARQ process. It can be further discussed how the counter is maintained.

Proposal: Resource reselection by resource reselection counter is applied for SL-PRS transmission on dedicated resource pool.

In the legacy MAC spec, the following conditions are specified as triggered for resource selection/reselection and the checking for these conditions are performed persistently.

If the TX resource (re-)selection check procedure is triggered on the selected pool of resources for a Sidelink process according to clause 5.22.1.1, the MAC entity shall for the Sidelink process: 1> if PSCCH duration(s) and 2nd stage SCI on PSSCH for all transmissions of a MAC PDU of any selected sidelink grant(s) are not in SL DRX Active time as specified in clause 5.28.3 of the destination that has data to be sent; or 1> if SL_RESOURCE_RESELECTION_COUNTER = 0 and when SL_RESOURCE_RESELECTION_COUNTER was equal to 1 the MAC entity randomly selected, with equal probability, a value in the interval [0, 1] which is above the probability configured by RRC in sl-ProbResourceKeep; or 1> if the pool of resources is configured or reconfigured by RRC; or 1> if there is no selected sidelink grant on the selected pool of resources; or 1> if neither transmission nor retransmission has been performed by the MAC entity on any resource indicated in the selected sidelink grant during the last second; or 1> if sl-ReselectAfter is configured and the number of consecutive unused transmission opportunities on resources indicated in the selected sidelink grant, which is incremented by 1 when none of the resources of the selected sidelink grant within a resource reservation interval is used, is equal to sl-ReselectAfter; or 1> if the selected sidelink grant cannot accommodate a RLC SDU by using the maximum allowed MCS configured by RRC in sl-MaxMCS-PSSCH associated with the selected MCS table and the UE selects not to segment the RLC SDU; or NOTE 1: If the selected sidelink grant cannot accommodate the RLC SDU, it is left for UE implementation whether to perform segmentation or sidelink resource reselection. 1> if transmission(s) with the selected sidelink grant cannot fulfil the remaining PDB of the data in a logical channel, and the MAC entity selects not to perform transmission(s) corresponding to a single MAC PDU: NOTE 2: If the remaining PDB is not met, it is left for UE implementation whether to perform transmission(s) corresponding to single MAC PDU or sidelink resource reselection. NOTE 3: It is left for UE implementation whether to trigger the TX resource (re-)selection due to the latency requirement of the MAC CE triggered according to clause 5.22.1.7. 2> clear the selected sidelink grant associated to the Sidelink process, if available; 2> trigger the TX resource (re-)selection.

Then, with the dedicated and shared resource pool introduced for SL-PRS transmission, we need to examine the conditions one by one that whether they are still applicable.

First, for shared resource pool, since SL-SCH and SL-PRS will be transmitted, we think all of the legacy conditions for resource selection should be adopted.

Proposal8: For shared resource pool, all the legacy conditions for resource selection/reselection can be reused.

Then, for dedicated resource pool, we think the following conditions are not needed anymore

- if PSCCH duration(s) and 2nd stage SCI on PSSCH for all transmissions of a MAC PDU of any selected sidelink grant(s) are not in SL DRX Active time as specified in clause 5.28.3 of the destination that has data to be sent

  - for dedicated resource pool, there is only a single stage SCI and the think DRX does not need to be supported together with dedicated resource pool

- if the selected sidelink grant cannot accommodate a RLC SDU by using the maximum allowed MCS configured by RRC in sl-MaxMCS-PSSCH associated with the selected MCS table and the UE selects not to segment the RLC SDU

  - when SL-PRS is transmitted on dedicated resource pool, there is no MCS configured

- if transmission(s) with the selected sidelink grant cannot fulfil the remaining PDB of the data in a logical channel, and the MAC entity selects not to perform transmission(s) corresponding to a single MAC PDU

  - The notion of PDB does not hold for SL-PRS transmission in dedicated resource pool anymore.

Proposal9: For SL-PRS transmission on dedicated resource pool, the following legacy conditions are not needed for resource selection/reselection:

- if PSCCH duration(s) and 2nd stage SCI on PSSCH for all transmissions of a MAC PDU of any selected sidelink grant(s) are not in SL DRX Active time as specified in clause 5.28.3 of the destination that has data to be sent

- if the selected sidelink grant cannot accommodate a RLC SDU by using the maximum allowed MCS configured by RRC in sl-MaxMCS-PSSCH associated with the selected MCS table and the UE selects not to segment the RLC SDU

- if transmission(s) with the selected sidelink grant cannot fulfil the remaining PDB of the data in a logical channel, and the MAC entity selects not to perform transmission(s) corresponding to a single MAC PDU

Another issue for resource selection in RA scheme 2 is the PDB/latency requirement for PRS. RAN1 has agreed on the following for the resource selection window

Agreement For Scheme 2, in a dedicated resource pool, using Rel-16 resource (re)-selection procedure as the starting point, consider at least the following potential modifications: - Modification 1: For the RS used to derive L1 SL-RSRP for resource exclusion: - Option 1: SL-PRS - Option 2: PSCCH DMRS - Option 3: PSSCH DMRS (if PSSCH is included in the dedicated resource pool) - Modification 2: For the resource selection window: - Option 1: for the derivation of the window, using the legacy approach as a starting point, substitute the Packet Delay Budget (PDB) with a new delay budget - Option 2: the selection window is provided by higher layers

While PDB is related to the QoS for data transmission, for SL-PRS transmission, there is no definition of PDB per se. While, RAN1 has not determined the specific option for the resource selection window, we can come back to the issue later when there is an option in RAN1.

2.3	Logical channel prioritization

For shared resource pool in both scheme 1 and scheme 2, both SL-PRS and data can be transmitted in a single slot. For the dedicated resource pool in both scheme 1 and scheme 2, only SL-PRS can be transmitted. For the scheme 1, the SL grant is indicated for the UE, according to the agreement in RAN1, it is still FFS that how to indicate the SL PRS related information for the SL grant. For the scheme 2, the SL grant is selected by the UE.

No matter which way the UE obtained the SL grant, the final transmission on the SL grant needs to be based on the LCP to make sure the priority data and/or SL-PRS can be delivered first.

When considering the SL-PRS transmission,

- For SL grant of dedicated resource pool, since no data can be transmitted via the dedicated resource pool, only PRS can be transmitted. The TX UE may have several RX UEs to receive the SL-PRS with different priorities. Follow the similar procedure in LCP, MAC entity can select the L2 destination associated with the SL grant for the highest priority SL-PRS allowed. Then the MAC entity sets the related parameters in SCI (e.g. priority) and indicates the information to the lower layer.

- For SL grant of shared resource pool, in addition to the issue above for destination selection, SL-SCH and the SL-PRS may be transmitted together and the SL-PRS has the corresponding priority. According to the defined SL-PRS priority, the urgent data and/or SL-PRS should be transmitted via the SL grant in the shared resource pool. For example, in the following figure, the available data in LCH when a grant is determined, for destination 2, the LCH data is PC5- RRC message, which is quite important for the connection, and for destination 1, the SL-PRS is only the reference signal for the positioning service which will not affect the connection. The connection control message should be more important.

Then, it is reasonable that the MAC entity can follow a similar procedure in LCP to decide the transmission in the SL grant. For destination selection, we think that for both shared resource pool and dedicated resource pool, the destination should be selected according to highest priority level, as in legacy SL-SCH transmission.

Proposal10: For destination selection for SL-PRS transmission on both shared and dedicated resource pool, the UE should select the destination with the highest priority in the LCP procedure

According to the LCP procedure, the MAC entity can determine whether and which SL PRS is transmitted. The MAC entity can set the related parameters in SCI (e.g. priority) and indicate the information to the lower layer.

For the case when SL PRS and SL-SCH are both transmitted in the SL grant of shared resource pool, the PHY layer can provide the TBS for MAC entity considering the SL PRS related information. Then the MAC entity can construct the MAC PDU according to the TBS considering the SL PRS. Considering in this case, there exists both SL-PRS priority and SL-SCH priority for the SL grant, highest priority should be adopted for SCI parameter setting.

Proposal11: For a SL-SCH and SL-PRS transmission on shared resource pool, SL-PRS priority and SL-SCH priority are used for multiplexing the SL-PRS and SL-SCH transmission within the allocated time and frequency resources. LS to RAN1 on the agreement.

2.4	HARQ operations

For the current MAC spec, when a MAC PDU is ACKed (either by the maximum number of blind retransmissions, positive ACK or negative ACK), the HARQ buffer is flushed and the UE stops the SL transmission:

1> if sl-MaxTransNum corresponding to the highest priority of the logical channel(s) in the MAC PDU has been configured in sl-CG-MaxTransNumList for the sidelink grant by RRC and the number of transmissions of the MAC PDU has been reached to sl-MaxTransNum; or 1> if a positive acknowledgement to this transmission of the MAC PDU was received according to clause 5.22.1.3.2; or 1> if negative-only acknowledgement was enabled in the SCI and no negative acknowledgement was received for this transmission of the MAC PDU according to clause 5.22.1.3.2: 2> flush the HARQ buffer of the associated Sidelink process.

For SL-PRS and SL-SCH transmission on shared RP, it is also possible that the transmission is enabled with PFSCH. It needs to be discussed when ACK is assumed for the SL-PRS and SL-SCH transmission, whether the UE should continue to use the SL grant for retransmission of SL-PRS.

Proposal12: The UE stops the SL-PRS transmission on shared resource pool when (a) the maximum number of transmissions by CG is reached (b) positive ACK is received (c) negative ACK is not received.

2.6	Collision handling

For the transmission of PUSCH transmission when there is a simultaneous NR sidelink/V2X sidelink transmission, collision handling is needed in the following cases/combinations in the current MAC spec in Section 5.4.2.2. in these cases, PUSCH can be transmitted.

- Both sidelink grant for NR sidelink and configured for V2X, neither V2X nor NR sidelink transmission is prioritized

- Both sidelink grant for NR sidelink and configured grant for V2X, PUSCH and V2X/NR sidelink can be performed simultaneously

- Only configured grant for V2X sidelink, PUSCH is prioritized or they can be transmitted simultaneously

- Only a sidelink grant for NR sidelink, PUSCH is prioritized or they can be transmitted simultaneously

- Both sidelink grant for NR sidelink and configured grant or V2X

Prioritization between PUCCH transmission and SL-SCH transmission has been discussed in Section 5.4.4.

Section 5.22.1.3.1a of the current MAC spec specifies the condition when the SL transmission can be prioritized over Uu transmission, including both PUSCH and PUCCH. In the following, the condition that SL-SCH transmission MAC PDU is prioritized over uplink transmission is listed as follows.

The transmission of the MAC PDU is prioritized over uplink transmission(s) of the MAC entity or the other MAC entity if the following conditions are met: 1> if the MAC entity is not able to perform this sidelink transmission simultaneously with all uplink transmission(s) at the time of the transmission, and 1> if none of the uplink transmission(s) is prioritized by upper layer according to TS 23.287 [19], and 1> if none of the NR uplink MAC PDU(s) includes any MAC CE prioritized as described in clause 5.4.3.1.3, and 1> if ul-PrioritizationThres is configured and if the value of the highest priority of logical channel(s) of all the NR uplink transmission(s) is not lower than ul-PrioritizationThres, and 1> if sl-PrioritizationThres is configured and if the value of the highest priority of logical channel(s) or MAC CE(s) in the MAC PDU is lower than sl-PrioritizationThres. NOTE 2: If the MAC entity is not able to perform this sidelink transmission simultaneously with all uplink transmissions as specified in clause 5.4.2.2 of TS 36.321 [22] at the time of the transmission, and prioritization-related information is not available prior to the time of this sidelink transmission due to processing time restriction, it is up to UE implementation whether this sidelink transmission is performed.

Then we have following observation from the sidelink transmission perspective and only the comparison between SL-SCH MAC PDU and uplink transmission used for the handling the collision.

Observation: SL-SCH transmission MAC PDU is prioritized over uplink transmission(s), only when all the following conditions are satisfied.

- Resource to transmission SL-SCH overlaps with PUCCH.

- None of uplink transmission is prioritized.

- The value of the highest priority of logical channel(s) of all the uplink transmission(s) is not lower than a configured threshold, i.e., the priority of the uplink transmission is low enough.

- The value of the highest priority of logical channel(s) or MAC CE(s) is lower than a configured threshold, i.e., the priority of the SL-SCH transmission is high enough.

The SL-PRS may be transmitted in both the dedicated resource pool and the shared resource pool. For the SL-PRS transmission in the dedicated resource pool, it is already agreed that there is no PSSCH is transmitted with the SL-PRS. In this case, there is no MAC PDU transmitted for SL grant. For the SL-PRS transmission in the shared resource pool, there exists the case that the SL-PRS is transmitted without any SL-SCH. In this case, the MAC PDU may not have any logical channel or MAC CE priority to used. Therefore, the original collision handling cannot work. We have the following proposal.

Proposal13: Support collision handling between SL/UU for SL-PRS based on the SL-PRS priority. SL-PRS is prioritized over PUSCH/PUCCH when

- The value of the priority of PUSCH/PUCCH is higher than a threshold, as in legacy

- The value of the priority of SL-PRS is lower than a threshold

The transmission of the MAC PDU or the SL-PRS is prioritized over uplink transmission(s) of the MAC entity or the other MAC entity if the following conditions are met: 1> if the MAC entity is not able to perform this sidelink transmission simultaneously with all uplink transmission(s) at the time of the transmission, and 1> if none of the uplink transmission(s) is prioritized by upper layer according to TS 23.287 [19], and 1> if none of the NR uplink MAC PDU(s) includes any MAC CE prioritized as described in clause 5.4.3.1.3, and 1> if ul-PrioritizationThres is configured and if the value of the highest priority of logical channel(s) of all the NR uplink transmission(s) is not lower than ul-PrioritizationThres, and 1> if sl-PrioritizationThres is configured and if the value of the highest priority of logical channel(s) or MAC CE(s) in the MAC PDU or SL-PRS(es) is lower than sl-PrioritizationThres. NOTE 2: If the MAC entity is not able to perform this sidelink transmission simultaneously with all uplink transmissions as specified in clause 5.4.2.2 of TS 36.321 [22] at the time of the transmission, and prioritization-related information is not available prior to the time of this sidelink transmission due to processing time restriction, it is up to UE implementation whether this sidelink transmission is performed.

2.7	DRX

In R17, DRX was introduced for sidelink communication with the intention of power saving, also supported under all the cast modes. Similarly, power consumption is a very important evaluation metric for the SL positioning. UE is benefit if we have the DRX to save the power.

Observation: Power saving mechanism should be pursued for Rel-18 SL positioning.

For the SL-PRS transmitted via the shared resource pool, since the DRX has been supported for the legacy NR sidelink transmission, it is reasonable that it should be supported for shared resource pool for SL-PRS transmission. There also does not seem to be spec change required for supporting this.

While for SL-PRS transmitted via the dedicated resource pool, it has been agreed that there will not be PSSCH and PSFCH along with the SL-PRS transmission on the dedicated resource pool. Consider the timer operation for DRX associated HARQ procedure, it is better to disable the DRX when the dedicated pool is configured for the UE. Therefore, we propose the following

Proposal14: DRX and dedicated resource pool for PRS transmission should not be configured together.

2.8	MAC PDU with SL-PRS transmission on shared RP

For shared resource pool, SL-PRS needs to be transmitted together with a SL MAC PDU subheader carrying a source/destination ID.

The SL MAC PDU and the SL-SCH MAC subheader are shown as follows:

It can be observed that the SL-MAC PDU consists of SL-SCH subheader and subsequent MAC subPDUs containing subPDU subheaders and MAC SDUs/MAC CEs.

The following conditions have been specified for the MAC PDU generation

The MAC entity shall not generate a MAC PDU for the HARQ entity if the following conditions are satisfied: - there is no Sidelink CSI Reporting MAC CE generated for this PSSCH transmission as specified in clause 5.22.1.7; and - there is no Sidelink DRX Command MAC CE generated for this PSSCH transmission as specified in clause 5.22.1.8; and - there is no Sidelink Inter-UE Coordination Request MAC CE generated for this PSSCH transmission as specified in clause 5.22.1.9; and - there is no Sidelink Inter-UE Coordination Information MAC CE generated for this PSSCH transmission as specified in clause 5.22.1.10; and - the MAC PDU includes zero MAC SDUs.

Based on the current specified conditions, the MAC entity will not generate MAC PDU if the MAC PDU includes zero MAC SDUs and zero MAC CEs. While, for the transmission of SL-PRS without SL-SCH data, since there is no MAC SDUs, according to the current spec, MAC PDU will not be generated and cannot transmit the MAC subheader, which contains the source and destination layer-2 ID. To support the case that the SL-PRS transmitted without data in the SL grant in the shared resource pool, the constraint on the MAC PDU construction needs to be broken, e.g., the MAC PDU is not allowed to be constructed when there is no MAC SDUs, no MAC CEs and no SL-PRS.

Proposal15: When SL-PRS is transmitted without SL-SCH in the shared resource pool, the Multiplexing and Assembly entity generates MAC PDU containing only padding MAC subPDU for the transmission along with SL-PRS.

- Discussion on the RRC aspects

  - Connection control

In the legacy releases for SL communications, the UE will perform connection setup/resume request when the received system information does not have valid version of resource pool for sidelink transmission in mode2 when the carrier for sidelink transmission is included in the system information. The intention is that the UE should not use the pre-configured resource for SL transmission when it is in coverage and while the transmission in mode1 would offer better performance.

For the above connection control procedure, we think the same rationale also applies for sidelink PRS transmission. The UE should perform connection setup/resume request when the carrier for SL-PRS transmission is included in the frequency list in the system information, while the system information does not have RP configuration for Scheme2 selection.

Proposal16: UE should perform connection setup/resume request with the following conditions:

- SL-PRS transmission is triggered; and

- the carrier for SL-PRS transmission is included in the frequency list in the system information; and

- the system information does not have resource pool configuration for Scheme2 selection.

  - Sidelink UE capability

In the legacy, when the UE is in coverage, the UE sends the peer UE’s sidelink UE capability to the gNB via the RRC message sidelinkUEInformationNR. For sidelink positioning, we think that the peer UE’s SLPP capability should also be sent to the gNB, e.g., for proper scheduling in resource allocation scheme 1. Hence, we propose the following for the sidelink UE capability transfer

Proposal17: Support the transport of SLPP capability in RRC message

- SLPP capability for the UE itself can be transferred from the UE to the gNB via a container in the RRC message UECapabilityInformation

- SLPP capability from the peer UE can be transferred from the UE to the gNB via a container in RRC message SidelinkUEInformationNR

  - Sidelink measurement

In RAN1 discussion, the following has been agreed regarding the CBR measurements

Agreement For Scheme 2 SL-PRS resource allocation, specify congestion control mechanisms using the existing congestion control mechanisms as a starting point. - Study at least the following aspects on potential changes over the existing congestion control mechanisms: - CBR and CR definition for SL-PRS - Which parameters of a SL-PRS configuration could be impacted by the congestion control mechanism, the mapping between congestion measurements, SL-PRS priority and SL-PRS parameters - CR and CBR measurement time window - Congestion control processing time - Number of CBR ranges - Whether any proposed changes could be applicable to shared resource pools in addition to the dedicated resource pool Agreement In Scheme 2, congestion control can restrict the range of parameters for SL PRS configuration per resource pool by CBR and priority. Consider further the following parameter(s): - Option 1: SL PRS transmission power - Option 2: Periodicity of SL PRS - Option 3: Number of occupied subchannels of SL-PRS (for shared resource pool) - Option 4: Number of SL PRS resources in a slot - Option 5: comb-size of a SL PRS resource in a slot - Option 7: Number of OFDM symbols of a SL PRS resource in a slot - Option 8: Number of SL PRS (re-)transmissions - FFS: Other options are not precluded

Observation: CBR measurement needs to be supported in dedicated resource pool.

Since the CBR measurement is already supported in the pool to transmit data, as shown below, the UE should support the CBR measurement in the shared resource pool.

2> perform CBR measurement on the transmission resource pool(s) indicated by sl-TxPoolSelectedNormal, sl-TxPoolScheduling or sl-TxPoolExceptional for NR sidelink communication transmission, as specified in 5.5.3;

Observation: CBR measurement is already supported in the shared resource pool.

With the RAN1 agreement on the congestion control, we think that CBR measurement are needed for both shared and dedicated resource pool.

Proposal18: Support CBR measurement on both shared and dedicated resource pool for SL-PRS transmission.

3.4	RLF

In sidelink communication, RLF is detected upon the following conditions as defined in 38.331:

#### 5.8.9.3 Sidelink radio link failure related actions The UE shall: 1> upon indication from sidelink RLC entity that the maximum number of retransmissions for a specific destination has been reached; or 1> upon T400 expiry for a specific destination; or 1> upon indication from MAC entity that the maximum number of consecutive HARQ DTX for a specific destination has been reached; or 1> upon integrity check failure indication from sidelink PDCP entity concerning SL-SRB2 or SL-SRB3 for a specific destination: 2> consider sidelink radio link failure to be detected for this destination; 2> release the DRBs of this destination, according to clause 5.8.9.1a.1; 2> release the SRBs of this destination, according to clause 5.8.9.1a.3; 2> release the PC5 Relay RLC channels of this destination if configured, in according to clause 5.8.9.7.1; 2> discard the NR sidelink communication related configuration of this destination; 2> reset the sidelink specific MAC of this destination; 2> consider the PC5-RRC connection is released for the destination; 2> indicate the release of the PC5-RRC connection to the upper layers for this destination (i.e. PC5 is unavailable);

RAN1#113 agreed that PSFCH is not included in dedicated resource pool:

Agreement PSFCH is not included in dedicated resource pool for SL positioning.

The above HARQ DTX condition is on the premise of a configured PSFCH, where HARQ-based sidelink RLF is detected based on a number of consecutive DTX on PSFCH reception occasion. Therefore, for dedicated resource pool or shared resource pool without a configured PSFCH, it can be seen that none of the above conditions can be applied for UE to detect SL-PRS related RLF.

In sidelink positioning, there are cases where UEs only transmit SL-PRS without data transmission, the following case can be taken as an example, where Target UE has a positioning requirement:

- Phase 1: Target UE and Anchor UE perform SLPP capability and SLPP assistant data exchange.

- Phase 2: Target UE measures SL-PRS sent from an anchor UE and performs calculation, e.g., for RSTD based SL-TDOA, Target UE doesn’t need to send SLPP providelocationreuqest information to Anchor UE.

The radio link between Target UE and Anchor UE is good during Phase 1, but becomes worse after Phase 1, which leads to a case that Target UE can not receive Anchor UE’s SL-PRS, and also Anchor UE can not trigger RLF because there is only SL-PRS transmission between UEs during Phase 2. To avoid such case, RAN2 should discuss how to can UE detect RLF for SL-PRS.

Proposal19: RAN2 to discuss how can UE detect RLF for SL-PRS.

4.	Summary

Based on the discussion above, we make the following proposals:

SL PRS priority

Proposal1: Define 8 priority levels for SL-PRS priority. Send an LS to RAN1 and SA2

Proposal2: SL-PRS priority is provided by the UE’s own higher layer or the SCI from the peer UE that trigger the SL-PRS transmission. Send an LS to RAN1 and SA2

Resource allocation scheme 1 by DG

Proposal3: When aperiodic SL-PRS transmission is triggered for UE configured with Scheme 1 SL-PRS resource allocation, the UE sends a MAC CE to the gNB for SL-PRS resource request. LS to RAN1 on the detailed content in the MAC CE.

Proposal4: SR can be triggered for SL-PRS resource allocation request MAC CE when there is no UL-SCH resources to accommodate the MAC CE.

Resource allocation scheme 1 by CG

Proposal5: When periodic SL-PRS transmission is triggered for UE configured with Scheme1 SL-PRS resource allocation, the UE sends an RRC message to the gNB for sending the assistance information for CG configuration. LS to RAN1 on the detailed content of the RRC message

Proposal6: The UE sends a CG confirmation MAC CE when the DCI for CG type 2 activation/deactivation is successfully received.

Resource allocation scheme 2

Proposal7a: When there are both sidelink data and SL-PRS pending for transmission, select shared resource pool.

Proposal7b: When there are only SL-PRS pending for transmission while there is no data, prioritize dedicated resource pool

Proposal8: For shared resource pool, all the legacy conditions for resource selection/reselection can be reused.

Proposal9: For SL-PRS transmission on dedicated resource pool, the following legacy conditions are not needed for resource selection/reselection:

- if PSCCH duration(s) and 2nd stage SCI on PSSCH for all transmissions of a MAC PDU of any selected sidelink grant(s) are not in SL DRX Active time as specified in clause 5.28.3 of the destination that has data to be sent

- if the selected sidelink grant cannot accommodate a RLC SDU by using the maximum allowed MCS configured by RRC in sl-MaxMCS-PSSCH associated with the selected MCS table and the UE selects not to segment the RLC SDU

- if transmission(s) with the selected sidelink grant cannot fulfil the remaining PDB of the data in a logical channel, and the MAC entity selects not to perform transmission(s) corresponding to a single MAC PDU

Logical Channel Prioritization

Proposal10: For destination selection for SL-PRS transmission on both shared and dedicated resource pool, the UE should select the destination with the highest priority in the LCP procedure

Proposal11: For a SL-SCH and SL-PRS transmission on shared resource pool, SL-PRS priority and SL-SCH priority are used for multiplexing the SL-PRS and SL-SCH transmission within the allocated time and frequency resources. LS to RAN1 on the agreement.

HARQ operations

Proposal12: The UE stops the SL-PRS transmission on shared resource pool when (a) the maximum number of transmissions by CG is reached; (b) positive ACK is received; (c) negative ACK is not received.

Collision Handling

Proposal13: Support collision handling between SL/UU for SL-PRS based on the SL-PRS priority. SL-PRS is prioritized over PUSCH/PUCCH when

- The value of the priority of PUSCH/PUCCH is higher than a threshold, as in legacy

- The value of the priority of SL-PRS is lower than a threshold

DRX

Proposal14: DRX and dedicated resource pool for PRS transmission should not be configured together.

MAC PDU generation

Proposal15: When SL-PRS is transmitted without SL-SCH in the shared resource pool, the Multiplexing and Assembly entity generates MAC PDU containing only padding MAC subPDU for the transmission along with SL-PRS.

Connection control

Proposal16: UE should perform connection setup/resume request with the following conditions:

- SL-PRS transmission is triggered; and

- the carrier for SL-PRS transmission is included in the frequency list in the system information; and

- the system information does not have resource pool configuration for Scheme2 selection.

Sidelink UE capability

Proposal17: Support the transport of SLPP capability in RRC message

- SLPP capability for the UE itself can be transferred from the UE to the gNB via a container in the RRC message UECapabilityInformation

- SLPP capability from the peer UE can be transferred from the UE to the gNB via a container in RRC message SidelinkUEInformationNR

Sidelink measurement

Proposal18: Support CBR measurement on both shared and dedicated resource pool for SL-PRS transmission.

RLF

Proposal19: RAN2 to discuss how can UE detect RLF for SL-PRS.

5.	References

- RAN1-112 Chair’s notes