3GPP TSG-RAN WG2 #123 	R2-2307147

Toulouse, France, August 21-25 2023

Agenda Item:	7.3.2

Source:	NEC

Title:	Joint Operation between UE C-DRX and Cell DTX

Document for:	Discussion, Decision

# Introduction

In the last RAN2 meeting, the following agreements were reached for the cell DTX/DRX operation for NES:

1	UE monitors PDCCH for RAR during Cell DTX non-active time. The ra-ResponseWindow could be started as legacy.

2	UE monitors PDCCH for msg4 during Cell DTX non-active time. The ra-ContentionResolutionTimer could be started as legacy.

3	Working assumption:  When the retransmission timer is running (if C-DRX is configured), the UE is expected to monitor PDCCH, like in legacy.  It is up to the network whether it schedules retransmissions out of the Cell DTX active period, i.e., when the DRX retransmission timer is running, the UE should monitor PDCCH regardless of the Cell DTX.

4	Once gNB recognizes there is an emergency call or public safety related service (e.g. MPS/MCS), the NW should ensure there is no impact to the emergency call (e.g. may deactivate Cell DTX/DRX).  The behavior is captured in stage 2 spec

5	When an DG grant is received, by the gNB during cell DRX/DTX, the UE follows the grant assignment (i.e. like in legacy).  This includes DL HARQ feedback.

This paper intends to firstly discuss the overlapping case between UE reception of DL retransmission and Cell DTX, and then discuss the alignment between cell DTX and UE-C-DRX.

# Discussion

## Joint Operation between DL Retransmission and cell DTX

In the last meeting, there is a working assumption made for retransmission that when the retransmission timer is running (if C-DRX is configured), the UE is expected to monitor PDCCH, like in legacy. It is up to the network whether it schedules retransmissions out of the Cell DTX active period, i.e., when the DRX retransmission timer is running, the UE should monitor PDCCH regardless of the Cell DTX.

However, we think that network implementation based approach may be not the best approach, since it may be negative for the UE to keep monitoring the PDCCH but the network actually want to delay the transmission of the retransmission for the purpose of energy saving. Otherwise, if the network changes its scheduling, the network needs to notify the UE for the new scheduled PDCCH occasion, which may introduce additional signalling.

As shown at Figure 1 (a), when the UE’s expected DL retransmission overlaps with the Cell DTX Inactive period, the UE can assume that the retransmission would be delayed by the gNB until the end of the Cell DTX Inactive period as shown at Figure 1 (b). In this manner, there is no further signalling introduced but the aligned UE-gNB behaviour is ensured, since both UE and gNB are aware of the configuration of the Cell DTX period.

Fig.1(a): Overlapped DL Retransmission and Cell DTX

Fig.1(b): Delayed DL Retransmission and Cell DTX

Proposal-1: RAN2 to confirm that UE is not monitoring PDCCH during cell DTX non-active time.

Proposal-2: When UE expected DL retransmission(if C-DRX is configured) overlaps with the Cell DTX Inactive period, the UE assumes that the retransmission would be delayed by the gNB until the end of the Cell DTX Inactive period.

## Alignment between UE C-DRX and cell DTX

The WID [1] captured the following with regards to alignment between cell DTX/DRX and C-DRX:

- Specify enhancement on cell DTX/DRX mechanism including the alignment of cell DTX/DRX and UE DRX in RRC_CONNECTED mode, and inter-node information exchange on cell DTX/DRX [RAN2, RAN1, RAN3]

- Note: No change for SSB transmission due to cell DTX/DRX.

- Note: The impact to IDLE/INACTIVE UEs due to the above enhancement should be avoided.

Alignment between cell DTX/DRX and UE-C-DRX was not treated on line at RAN2#122 meeting due to shortage of time. It was included as a part of a post meeting e-mail discussion [Post122][307][NES].

This contribution summarizes our views on this topic.

In typical cell DTX deploying scenario, we think cell DTX OnDuration would be longer than UE C-DRX OnDuration. In such a scenario, we see three cases shown in Fig.2.

- UE A: only a part of UE C-DRX On-duration occasion fully falls within Cell DTX on-duration period (In Fig.1, the first UE C-DRX OnDuariton occasion fully falls within Cell DTX on-duration period whereas the second overlaps with Cell DTX off period)

- UE B: a part of UE C-DRX On-duration partially falls within Cell DTX on-duration period (In Fig.1, the first UE C-DRX OnDuariton occasion partially falls within Cell DTX on-duration period whereas the second overlaps with Cell DTX off period)

- UE C: no UE C-DRX On-duration falls within Cell DTX on-duration period (In Fig.1, the first and second UE C-DRX OnDuariton occasions overlap with Cell DTX off period)

Fig.2: Three cases of cell-DTX and UE C-DRX

- Among the above 3 cases, it is obviously that UE C’s C-DRX needs to be re-configured to align with cell DTX at least for reception of new transmission. For UE B, since the occasions for reception of new transmission are limited, it would be good to re-configure its C-DRX to align with cell DTX. Finally, for UE A, re-configuring its C-DRX may have no big benefits, therefore can be left to network.

Observation-1: When C-DRX and cell DTX OnDuration have no overlap period, network shall reconfigure UE C-DRX to align with cell DTX pattern.

Observation-2: When C-DRX and cell DTX Onduration overlaps partially, re-configuring UE C-DRX to align with cell DTX pattern may improve occasions for reception of new transmission.

Proposal-3: When C-DRX and cell DTX OnDuration overlaps partially, whether re-configure UE C-DRX to align with cell DTX pattern is left to network.

When reconfiguring UE C-DRX happens, we think at least some principles on how to align cell DTX and C-DRX OnDuration would be necessary. Seems anyway reconfiguration will happen associated with configuring DTX pattern by UE specific RRC signaling, an aligned UE C-DRX can also be provided via the reconfiguration. Then we propose the basic principle as following

Proposal-4: An aligned UE C-DRX configuration with cell DTX means that all OnDuration of C-DRX fully falls within Cell DTX OnDuration period.

Obviously, at least for UE C in Fig.1, an aligned C-DRX that differs from the previous C-DRX configuration shall be applied/re-configured by the network. It means that two different C-DRX configurations are applied depends on cell DTX status, i.e., one C-DRX configuration is applied when cell DTX is activated whereas another C-DRX configuration is applied when cell DTX is deactivated.

On how to activate/deactivate cell DTX/DRX pattern, RAN2#122 meeting agreed that “As a baseline Cell DTX/DRX is activated/deactivated implicitly by RRC signaling, i.e., activated immediately once configured by RRC and deactivated once the RRC configuration is released. Furthermore, RAN1#113 meeting agreed to

support the group common L1 signaling using PDCCH for cell DTX/DRX activation and deactivation without HARQ feedback.

Agreement

RAN1 supports the group common L1 signalling using PDCCH for cell DTX/DRX activation and deactivation without HARQ feedback

•	Send an LS to RAN2 to consider the additional support of a MAC CE based indication

•	Subject to UE capability

To establish a baseline to efficiently change the C-DRX of UEs to align the C-DRX when cell DTX is activated/deactivated, we think a reasonable solution is to configure two UE C-DRX configurations for cell DTX activation and deactivation status respectively. When cell DTX/DRX activation and deactivation status are changed via the RRC signalling or the group common L1 signalling using PDCCH, the associated UE C-DRX configuration is applied by default depending on cell DTX activation and deactivation status.

Proposal-5: Configure two UE C-DRX configurations for cell DTX activation and deactivation status, respectively. When cell DTX/DRX activation and deactivation status are changed via the RRC signalling or the group common L1 signalling using PDCCH, the associated UE C-DRX configuration is applied by default depending on cell DTX activation and deactivation status.

## Alignment between Cell DTX and Cell DRX

The alignment between Cell DTX and Cell DRX was discussed by RAN1. In general, the decouple configuration between Cell DTX and Cell DRX will cause some trouble for both DL and UL and in some times it may not be efficient for UE power saving.

For example, the UE may transmit UL during cell  DRX active and may not receive any HARQ feedback for such UL transmission due to cell DTX inactive time. The consequence is that the UE may need to receive the HARQ in a delayed time slot when the cell goes to cell DTX active time. We can image the similar operation when the UE receives the DL retransmission but cannot transmits the UL HARQ feedback, since the cell may be in cell DRX inactive period in the following time slot applying for the UL HARQ feedback.

Hence, it is assumed that the network will always configure aligned cycles for Cell DRX and Cell DTX. Even though this may be network implementation, however if this assumption is confirmed, the design of the detailed mechanism for cell DTX/DRX operation will be having a baseline.

During the Post-RAN2#122 Email discussion [307] for this issue, the majority companies acknowledged the need to make a fully aligned configuration for both Cell DRX and Cell DTX in Q5:

Question 5: Do you agree that when Cell DRX is configured together with Cell DTX it must be fully aligned with Cell DTX (i.e. exactly the same periodicity, offset and on-duration) for one serving cell?

•	Yes, must be fully aligned together as in Fig.1 case b).

•	No, Cell DTX and Cell DRX can be configured with different parameter values (e.g. different offset, on-duration, periodicity) as in Fig.1 case a)

Based on the discussion above we have the following proposal:

Proposal-6: When Cell DRX is configured together with Cell DTX, it must be fully aligned with Cell DTX (i.e. exactly the same periodicity, offset and on-duration) for one serving cell.

# Conclusion

The following observations and proposals are summarized for this paper:

Observation-1: When C-DRX and cell DTX OnDuration have no overlap period, network shall reconfigure UE C-DRX to align with cell DTX pattern.

Observation-2: When C-DRX and cell DTX Onduration overlaps partially, re-configuring UE C-DRX to align with cell DTX pattern may improve occasions for reception of new transmission.

Proposal-1: RAN2 to confirm that UE is not monitoring PDCCH during cell DTX non-active time.

Proposal-2: When UE expected DL retransmission(if C-DRX is configured) overlaps with the Cell DTX Inactive period, the UE assumes that the retransmission would be delayed by the gNB until the end of the Cell DTX Inactive period.

Proposal-3: When C-DRX and cell DTX OnDuration overlaps partially, whether re-configure UE C-DRX to align with cell DTX pattern is left to network.

Proposal-4: An aligned UE C-DRX configuration with cell DTX means that all OnDuration of C-DRX fully falls within Cell DTX OnDuration period.

Proposal-5: Configure two UE C-DRX configurations for cell DTX activation and deactivation status, respectively. When cell DTX/DRX activation and deactivation status are changed via the RRC signalling or the group common L1 signalling using PDCCH, the associated UE C-DRX configuration is applied by default depending on cell DTX activation and deactivation status.

Proposal-6: When Cell DRX is configured together with Cell DTX, it must be fully aligned with Cell DTX (i.e. exactly the same periodicity, offset and on-duration) for one serving cell.

# References

[1] RP-223540, “New WID: Network energy savings for NR”

[2] R2-2305120	Cell DTX-DRX Mechanism