3GPP TSG-RAN WG2 Meeting #123	                                                   R2-2307148

Toulouse, France, August 21-25, 2023

Agenda item:	7.11.2.2

Source:	TD Tech, Chengdu TD Tech

Title:	User plane for multicast reception in RRC_INCTIVE state

Document for:	Discussion and Decision

- Introduction

During the RAN2#122 meeting, the following agreements on multicast reception in RRC_INACTIVE state were made for the 3GPP R18 WI “Enhancements of NR Multicast and Broadcast services”.

The multicast MCCH configuration takes the broadcast MCCH configuration structure (i.e., mcch-Config-r17) as baseline.

To notify the multicast MCCH change, change notification mechanism for Rel-17 broadcast MCCH is the baseline.

Working assumption (to be confirmed by RAN1 via pending reply LS): One bit in the MCCH DCI is used to notify the change of the multicast MCCH. We reuse the bit used for MCCH change indication from Rel-17 MBS broadcast. This does not cover session deactivation which is FFS.

It is not supported to provide the PTM configuration of intra-gNB neighbour cells in the dedicated signalling.

For PTM configuration structure on the multicast MCCH, Rel-17 broadcast PTM configuration structure is taken as baseline.

As a baseline, The PTM configuration in the RRCRelease message with suspendconfig has the same structure as the PTM configuration in multicast MCCH.

FFS how existing MRBs are handled.

Introduce a new indication per tmgi in the group paging which informs Rel-18 UEs having a valid PTM configuration to receive the multicast in RRC_INACTIVE.

MCCH is used for notifying MC session deactivation for multicast reception in RRC_INACTIVE to enable Rel-18 UE to stay in RRC_INACTIVE and stop monitoring corresponding G-RNTI.

This is assumed to have no/minor impact on RAN1/PHY

In the contribution, the following content is discussed for multicast reception in RRC_INACTIVE state with the related proposals suggested.

- CFR configuration

- MAC operation

- Further discussion on PHY layer impacts

- CFR configuration

A UE in RRC_INACTIVE state needs to monitor multicast MCCH to obtain the PTM configuration of the multicast session it has joined. Then the UE receives the multicast session according to the PTM configuration of the multicast session. In order to avoid the CFR switching, it’s better that a same CFR is configured for multicast MCCH and multicast sessions in RRC_INACTIVE state. This same CFR can be named as the CFR for multicast sessions in RRC_INACTIVE state.

In order to ensure a UE in RRC_INACTIVE state can receive a multicast session in RRC_INACTVIE state, the CFR for multicast sessions in RRC_INACTIVE state can only be CORESET 0, initial DL BWP or contain initial DL BWP with the same SCS and CP type as the initial DL BWP.

In order to ensure that a UE in RRC_INACTIVE state can receive both multicast sessions in RRC_INACTIVE state and broadcast sessions, the CFR for multicast sessions in RRC_INACTIVE state and the CFR for broadcast sessions shall meet one of the following conditions:

- The two CFRs are same

- The CFR for multicast sessions in RRC_INACTIVE state contains the CFR for broadcast sessions.

- The CFR for multicast sessions in RRC_INACTIVE state is contained within the CFR for broadcast sessions.

For simplicity, there’s no need to support the case that the CFR for multicast sessions in RRC_INACTIVE state is contained within the CFR for broadcast sessions. Therefore the CFR for multicast sessions in RRC_INACTIVE state can be configured with an IE “CFR-Multicast-Inactive” as below:

For the cast that the CFR for multicast sessions in RRC_INACTIVE state contains the CFR for broadcast sessions, this IE is used to configure the starting position and bandwidth of the corresponding CFR. If this IE is absent, it indicates the case that the CFR for multicast sessions in RRC_INACTIVE state is same as the CFR for broadcast sessions.

A multicast session can be provided to both UEs in RRC_CONNECTED state and UEs in RRC_INACTIVE state. A UE in RRC_INACTIVE state can receive the multicast session on the CFR for multicast sessions in RRC_INACTIVE state. A UE in RRC_CONNECTED state can receive the multicast session on the CFR for multicast sessions in RRC_INACTIVE state if its active BWP contains the CFR for multicast sessions in RRC_INACTIVE state. Otherwise it can receive the multicast session on the configured CFR on its active BWP.

Based on the above discussion, the following proposals are suggested.

Proposal 1: The CFR for multicast sessions in RRC_INACTIVE state can ONLY be CORESET 0, initial DL BWP or contain initial DL BWP with the same SCS and CP type as the initial DL BWP. The CFR is used for both multicast MCCH and multicast sessions in RRC_INACTIVE state.

Proposal 2: The CFR for multicast sessions in RRC_INACTIVE state shall meet one of the following conditions:

- It is same as the CFR for broadcast sessions

- It contains the CFR for broadcast sessions

- It is contained within the CFR for broadcast sessions: for simplicity, there’s no need to support this case.

Proposal 3: Use an IE to configure the CFR for multicast sessions in RRC_INACTIVE state as below:

For the cast that the CFR for multicast sessions in RRC_INACTIVE state contains the CFR for broadcast sessions, this IE is used to configure the starting position and bandwidth of the corresponding CFR. If this IE is absent, it indicates the case that the CFR for multicast sessions in RRC_INACTIVE state is same as the CFR for broadcast sessions.

Proposal 4: For the UE joining a multicast session, if it’s in RRC_INACTIVE state, it can receive the multicast session on the CFR for multicast sessions in RRC_INACTIVE state. If it’s in RRC_CONNECTED state, it can receive the multicast session on the CFR for multicast sessions in RRC_INACTIVE state if its active BWP contains the CFR for multicast sessions in RRC_INACTIVE state. Otherwise it can receive the multicast session on the configured CFR on its active BWP.

- MAC operation

For a multicast session and the UEs joining the multicast session, if the CFR for multicast sessions in RRC_INACTIVE state is only used to send the multicast session with PTM mode to the UEs in RRC_INACTIVE state, neither HARQ feedback nor retransmission are supported for the multicast session on the said CFR. Under such case, DCI format 4_0 can be used to schedule the multicast session with PTM mode on the said CFR. Only one HARQ process is used for receiving the TB of the multicast session. Which HARQ process is used for receiving the TB of the multicast session is left to UE implementation. The DRX mode of the multicast session on the said CFR doesn't support retransmission with PTM mode. If SPS is used for the multicast session, gNB has no need to configure several HARQ processes for the TB sent with SPS because retransmission with PTM mode is not supported for the multicast session and thus no parallel HARQ processes are needed by SPS.

For a multicast session and the UEs joining the multicast session, if the active DL BWPs of some RRC_CONNECTED UEs contain the CFR for multicast sessions in RRC_INACTIVE state, the said CFR can be used to send the multicast session with PTM mode to both RRC_INACTIVE UEs and these RRC_CONNECTED UEs.

If the CFR for multicast sessions in RRC_INACTIVE state is used to send the multicast session with PTM mode to both RRC_INACTIVE UEs and some/all RRC_CONNECTED UEs, the following three cases need to be considered:

Case 1: neither HARQ feedback nor retransmission are used for the multicast session on the said CFR

Case 2: HARQ feedback from RRC_CONNECTED UEs and retransmission with PTP mode to RRC_CONNECTED UEs are used for the multicast session on the said CFR

Case 3: HARQ feedback from RRC_CONNECTED UEs and retransmission with PTM mode to both RRC_CONNECTED UEs and RRC_INACTIVE UEs are used for the multicast session on the said CFR

Under case 1, DCI format 4_0 can be used to schedule the multicast session with PTM mode on the said CFR. Only one HARQ process is used for receiving the TB of the multicast session. Which HARQ process is used for receiving the TB of the multicast session is left to UE implementation. The DRX mode of the multicast session on the said CFR doesn't support retransmission with PTM mode.

Under case 2, DCI format 4_1 can be used to schedule the multicast session with PTM mode on the said CFR. But the HARQ feedback related information on DCI format 4_1 is only applied for RRC_CONNECTED UEs. RRC_INACTIVE UEs ignore the HARQ feedback related information and just receive the related PDSCH according to DCI format 4_1.

Under case 2, gNB can make the retransmission of a TB with PTP mode on the said CFR based on the HARQ feedback from the corresponding RRC_CONNECTED UE. gNB needs to determine the number of the HARQ processes (P) and HARQ process offset (HARQprocessOffset) used for the multicast session because several parallel HARQ processes are needed by the multicast session for the PTP retransmission of a TB. Under case 2, the DRX mode of the multicast session on the CFR for multicast sessions in RRC_INACTIVE state doesn't support retransmission with PTM mode.

Under case 3, DCI format 4_1 can be used to schedule the multicast session. But the HARQ feedback related information on DCI format 4_1 is only applied for RRC_CONNECTED UEs. RRC_INACTIVE UEs ignore the HARQ feedback related information and just receive the related PDSCH according to DCI format 4_1.

Under case 3, gNB can make the retransmission of a TB with PTM mode on the said CFR based on the HARQ feedbacks from RRC_CONNECTED UEs. Therefore, RRC_INACTIVE UEs need to receive the retransmission of a TB with PTM mode. Correspondingly the DRX mode of the multicast session on the said CFR supports retransmission with PTM mode.

Under case 3, RRC_INACTIVE UEs has no need to send the HARQ feedback but it needs to execute the DRX operation according to the DRX mode just as if it sends the HARQ feedback. gNB needs to determine the number of the HARQ processes (P) and HARQ process offset (HARQprocessOffset) used for the multicast session because several parallel HARQ processes are needed by the multicast session for the PTM retransmission of a TB.

.When SPS is used for a multicast session in RRC_INACTIVE state and the SPS resources for the multicast session are configured to the UEs joining the multicast session, DCI format 4_1 can be used to activate the corresponding SPS resource if the multicast session supports HARQ feedback and retransmission where retransmission can be made with PTP mode or PTM mode. Otherwise, DCI format 4_0 with a new field named HARQ Process number field can be used to activate the corresponding SPS resource.

Based on the above discussion, the following proposals are suggested:

Proposal 5: For a multicast session and the UEs joining the multicast session, if the CFR for multicast sessions in RRC_INACTIVE state is only used to send the multicast session with PTM mode to RRC_INACTIVE UEs, the following conclusions are supported:

- DCI format 4_0 is used to schedule the multicast session with PTM mode on the CFR for multicast sessions in RRC_INACTIVE state.

- Only one HARQ process is used for receiving the TB of the multicast session. Which HARQ process is used for receiving the TB of the multicast session is left to UE implementation.

- The DRX mode of the multicast session on the CFR for multicast sessions in RRC_INACITVE state doesn't support retransmission.

Proposal 6: For a multicast session and the UEs joining the multicast session, if the CFR for multicast sessions in RRC_INACTIVE state is used to send the multicast session with PTM mode to both RRC_INACTIVE UEs and some/all RRC_CONNECTED UEs and if neither HARQ feedback nor retransmission are used for the multicast session, the following conclusions are supported:

- DCI format 4_0 is used to schedule the multicast session with PTM mode on the CFR for multicast sessions in RRC_INACTIVE state.

- Only one HARQ process is used for receiving the TB of the multicast session. Which HARQ process is used for receiving the TB of the multicast session is left to UE implementation.

- The DRX mode of the multicast session on the CFR for multicast sessions in RRC_INACITVE state doesn't support retransmission.

Proposal 7: For a multicast session and the UEs joining the multicast session, if the CFR for multicast sessions in RRC_INACTIVE state is used to send the multicast session with PTM mode to both RRC_INACTIVE UEs and some/all RRC_CONNECTED UEs and if HARQ feedbacks from RRC_CONNECTED UEs and retransmission with PTP mode to RRC_CONNECTED UEs are used for the multicast session, the following conclusions are supported:

- DCI format 4_1 can be used to schedule the multicast session with PTM mode on the CFR for multicast sessions in RRC_INACTIVE state.

- The HARQ feedback related information on DCI format 4_1 is only applied for RRC_CONNECTED UEs. RRC_INACTIVE UEs ignore the HARQ feedback related information and just receive the related PDSCH according to DCI format 4_1.

- gNB makes the retransmission of a TB with PTP mode based on the HARQ feedback from the corresponding RRC_CONNECTED UE.

- The DRX mode of the multicast session on the CFR for multicast sessions in RRC_INACTIVE state doesn't support retransmission with PTM mode.

- gNB needs to determine the number of the HARQ processes (P) and HARQ process offset (HARQprocessOffset) used for the multicast session.

Proposal 8: For a multicast session and the UEs joining the multicast session, if the CFR for multicast sessions in RRC_INACTIVE state is used to send the multicast session with PTM mode to both RRC_INACTIVE UEs and some/all RRC_CONNECTED UEs and if HARQ feedback from RRC_CONNECTED UEs and retransmission with PTM mode to both RRC_CONNECTED UE and RRC_INACTIVE UEs are used for the multicast session, the following conclusions can be made:

- DCI format 4_1 can be used to schedule the multicast session.

- The HARQ feedback related information is only applied for RRC_CONNECTED UEs. RRC_INACTIVE UEs can ignore the HARQ feedback related information and just receive the related PDSCH according to DCI format 4_1.

- gNB makes the retransmission of a TB with PTM mode on the CFR for multicast sessions in RRC_INACTIVE state based on the feedbacks from RRC_CONNECTED UEs.

- RRC_INACTIVE UEs need to receive the retransmission of a TB with PTM mode.

- The DRX mode of the multicast session on the CFR for multicast sessions in RRC_INACITVE state supports retransmission with PTM mode.

- A RRC_INACTIVE UE has no need to send the HARQ feedback but it needs to execute the DRX operation according to the DRX mode just as if it sends the HARQ feedback.

- gNB needs to determine the number of the HARQ processes (P) and HARQ process offset (HARQprocessOffset) used for the multicast session.

Proposal 9: When SPS is used for the multicast session in RRC_INACTIVE state and the SPS resources for the multicast session are configured to the UEs joining the multicast session, DCI format 4_1 can be used to activate the corresponding SPS resource if the multicast session supports HARQ feedback and retransmission. Otherwise DCI format 4_0 with a new field named HARQ Process number field can be used to activate the corresponding SPS resource if the multicast session doesn't support HARQ feedback and retransmission.

- Further discussion on PHY layer impact

If SPS is supported for multicast sessions in RRC_INACTIVE state and the SPS resources for a multicast session are configured to the UEs joining the multicast session, DCI format 4_0 with a new field named HARQ Process number field can be used to activate the SPS resource if the multicast session doesn’t support HARQ feedback and retransmission. Send an LS to RAN1 to check the following question:

Question 1: If the SPS resources for a multicast session are configured to UEs and the multicast session doesn't support HARQ feedback and retransmission, is it feasible that DCI format 4_0 with a new field named HARQ Process number field can be used to activate the SPS resource where HARQ process number field is of 4 bits and used for indicating the index of the activated SPS resource?

Based on the above discussion, the following proposal is suggested:

Proposal 10: Send an LS to RAN1 to check the following question:

Question 1: If the SPS resources for a multicast session are configured to UEs and the multicast session doesn't support HARQ feedback and retransmission, is it feasible that DCI format 4_0 with a new field named HARQ Process number field can be used to activate the SPS resource where HARQ process number field is of 4 bits and used for indicating the index of the activated SPS resource?

- Conclusion

Based on the discussion in the former sections, the following proposals are suggested.

Proposal 1: The CFR for multicast sessions in RRC_INACTIVE state can ONLY be CORESET 0, initial DL BWP or contain initial DL BWP with the same SCS and CP type as the initial DL BWP. The CFR is used for both multicast MCCH and multicast sessions in RRC_INACTIVE state.

Proposal 2: The CFR for multicast sessions in RRC_INACTIVE state shall meet one of the following conditions:

- It is same as the CFR for broadcast sessions

- It contains the CFR for broadcast sessions

- It is contained within the CFR for broadcast sessions: for simplicity, there’s no need to support this case.

Proposal 3: Use an IE to configure the CFR for multicast sessions in RRC_INACTIVE state as below:

For the cast that the CFR for multicast sessions in RRC_INACTIVE state contains the CFR for broadcast sessions, this IE is used to configure the starting position and bandwidth of the corresponding CFR. If this IE is absent, it indicates the case that the CFR for multicast sessions in RRC_INACTIVE state is same as the CFR for broadcast sessions.

Proposal 4: For the UE joining a multicast session, if it’s in RRC_INACTIVE state, it can receive the multicast session on the CFR for multicast sessions in RRC_INACTIVE state. If it’s in RRC_CONNECTED state, it can receive the multicast session on the CFR for multicast sessions in RRC_INACTIVE state if its active BWP contains the CFR for multicast sessions in RRC_INACTIVE state. Otherwise it can receive the multicast session on the configured CFR on its active BWP.

Proposal 5: For a multicast session and the UEs joining the multicast session, if the CFR for multicast sessions in RRC_INACTIVE state is only used to send the multicast session with PTM mode to RRC_INACTIVE UEs, the following conclusions are supported:

- DCI format 4_0 is used to schedule the multicast session with PTM mode on the CFR for multicast sessions in RRC_INACTIVE state.

- Only one HARQ process is used for receiving the TB of the multicast session. Which HARQ process is used for receiving the TB of the multicast session is left to UE implementation.

- The DRX mode of the multicast session on the CFR for multicast sessions in RRC_INACITVE state doesn't support retransmission with PTM mode.

Proposal 6: For a multicast session and the UEs joining the multicast session, if the CFR for multicast sessions in RRC_INACTIVE state is used to send the multicast session with PTM mode to both RRC_INACTIVE UEs and some/all RRC_CONNECTED UEs and if neither HARQ feedback nor retransmission are used for the multicast session, the following conclusions are supported:

- DCI format 4_0 is used to schedule the multicast session with PTM mode on the CFR for multicast sessions in RRC_INACTIVE state.

- Only one HARQ process is used for receiving the TB of the multicast session. Which HARQ process is used for receiving the TB of the multicast session is left to UE implementation.

- The DRX mode of the multicast session on the CFR for multicast sessions in RRC_INACITVE state doesn't support retransmission with PTM mode.

Proposal 7: For a multicast session and the UEs joining the multicast session, if the CFR for multicast sessions in RRC_INACTIVE state is used to send the multicast session with PTM mode to both RRC_INACTIVE UEs and some/all RRC_CONNECTED UEs and if HARQ feedbacks from RRC_CONNECTED UEs and retransmission with PTP mode to RRC_CONNECTED UEs are used for the multicast session, the following conclusions are supported:

- DCI format 4_1 can be used to schedule the multicast session with PTM mode on the CFR for multicast sessions in RRC_INACTIVE state.

- The HARQ feedback related information on DCI format 4_1 is only applied for RRC_CONNECTED UEs. RRC_INACTIVE UEs ignore the HARQ feedback related information and just receive the related PDSCH according to DCI format 4_1.

- gNB makes the retransmission of a TB with PTP mode based on the HARQ feedback from the corresponding RRC_CONNECTED UE.

- The DRX mode of the multicast session on the CFR for multicast sessions in RRC_INACTIVE state doesn't support retransmission with PTM mode.

- gNB needs to determine the number of the HARQ processes (P) and HARQ process offset (HARQprocessOffset) used for the multicast session.

Proposal 8: For a multicast session and the UEs joining the multicast session, if the CFR for multicast sessions in RRC_INACTIVE state is used to send the multicast session with PTM mode to both RRC_INACTIVE UEs and some/all RRC_CONNECTED UEs and if HARQ feedback from RRC_CONNECTED UEs and retransmission with PTM mode to both RRC_CONNECTED UEs and RRC_INACTIVE UEs are used for the multicast session, the following conclusions can be made:

- DCI format 4_1 can be used to schedule the multicast session.

- The HARQ feedback related information is only applied for RRC_CONNECTED UEs. RRC_INACTIVE UEs can ignore the HARQ feedback related information and just receive the related PDSCH according to DCI format 4_1.

- gNB makes the retransmission of a TB with PTM mode on the CFR for multicast sessions in RRC_INACTIVE state based on the feedbacks from RRC_CONNECTED UEs.

- RRC_INACTIVE UEs need to receive the retransmission of a TB with PTM mode.

- The DRX mode of the multicast session on the CFR for multicast sessions in RRC_INACITVE state supports retransmission with PTM mode.

- A RRC_INACTIVE UE has no need to send the HARQ feedback but it needs to execute the DRX operation according to the DRX mode just as if it sends the HARQ feedback.

- gNB needs to determine the number of the HARQ processes (P) and HARQ process offset (HARQprocessOffset) used for the multicast session.

Proposal 9: When SPS is used for the multicast session in RRC_INACTIVE state and the SPS resources for the multicast session are configured to the UEs joining the multicast session, DCI format 4_1 can be used to activate the corresponding SPS resource if the multicast session supports HARQ feedback and retransmission. Otherwise DCI format 4_0 with a new field named HARQ Process number field can be used to activate the corresponding SPS resource if the multicast session doesn't support HARQ feedback and retransmission.

Proposal 10: Send an LS to RAN1 to check the following question:

Question 1: If the SPS resources for a multicast session are configured to UEs and the multicast session doesn't support HARQ feedback and retransmission, is it feasible that DCI format 4_0 with a new field named HARQ Process number field can be used to activate the SPS resource where HARQ process number field is of 4 bits and used for indicating the index of the activated SPS resource?

- References

- RP-221458, Revised WID: Enhancements of NR Multicast and Broadcast Services, RAN Meeting#96-e

- R2-2211271, Analysis of options for sending PTM configuration, RAN2#120, TD Tech, Chengdu TD Tech,